# State of Ohio

Department of Transportation

~

Construction Inspection Forms Booklet

~

3/1/2013

Ohio Department of Transportation

Division of Construction Management

1980 W. Broad Street

Columbus, Ohio 43223

Phone: (614) 466-3598

Web Address: WWW.DOT.STATE.OH.US

## Construction Documentation Forms

### Introduction

The Ohio Department of Transportation is devoted to maintaining high levels of project inspection and documentation across the State. The forms and instructions contained herein are provided to project personnel to assist in the proper determination of compliance with the provisions of the contract, the measurement of quantities for payment, and the documentation of compliance and measurements.

In no instance does this document alter the provisions of the contract.

These procedures will be used to provide a uniform and fair basis of operations wherever the Ohio Department of Transportation provides supervision or engineering services for the acquisition, construction, or maintenance of materials or services. This is not a contract document, and is not to be construed in any way to obligate the Ohio Department of Transportation to perform any duties. These forms should be used in conjunction with the Construction Administration Manual of Procedures.

Reference Documents

Documents normally used for detailed interpretation of substantiation requirements, practices, and test methods are:

- Ohio Manual of Uniform Traffic Control Devices

- Sampling and Testing Manual, Testing Administration Manual

- Traffic Engineering Manual

- Construction Administration Manual of Procedures

Other documents may also be used as they are developed and published.

More testing forms (those beginning with TE) can be found online here:

http://www.dot.state.oh.us/Divisions/ConstructionMgt/Materials/Pages/TE_Forms.aspx

Districts are permitted to customize forms as long as all of the data listed on the standard forms published here is also included on the modified forms.

Many of these forms are available in Excel format with automated calculation fields.  They can be found online here:

http://www.dot.state.oh.us/Divisions/ConstructionMgt/Admin/Pages/InspectionForms.aspx

Forms called for in the Contract Administration area such as DBE Certification, Payroll, EDGE, and EEO may be found online here:

http://www.dot.state.oh.us/Divisions/ContractAdmin/Contracts/Pages/DBE.aspx

### Instruction Codes

The following instructions, grouped by measurement type, supplement the general instructions and have particular application to specific pay items. Match the Instruction Code listed in the Index of Pay Items tables (starting on page 4) to the following Instruction Codes to determine specific documentation requirements. Special and unlisted pay items will be documented using the guidelines outlined in the Construction Administration Manual of Procedures (MOP).

| Instruction Code | Description |
| --- | --- |
|  | Conversion Type Measurements |
| C-1 | Summarize the final weight to nearest ton (metric ton) from validated weight tickets and determine the final volume to the nearest cubic yard (cubic meter) by using conversion factor. |
| C-2 | Summarize and convert to nearest ton (metric ton) for the final quantity by conversion from validated tickets. Convert sack and brine shipments to weight as specified. |
| C-3 | Summarize the final quantity to nearest cubic yard (cubic meter) by conversion from validated weight tickets. Include a daily coordination of weight tickets with placement and location. Where there is a moisture limitation, at least one moisture test must be made (more if necessary) to provide for an appropriate deduction. |
| C-4 | Determine volume to nearest gallon (liter) by conversion from validated weight tickets. Where source validation is missing, or for partial loads, determine volume by conversion from weigh-back or by measurement of tank. Where basic measurement is by volume, record temperature. |
| C-5 | Determine the final quantity to nearest cubic yard (cubic meter) from plan quantity adjusted for errors and changes. Use for new construction on new subgrade. For variable thickness or resurfacing determine quantity to nearest cubic yard (cubic meter) by conversion from validated weight tickets. |
| C-6 | For direct volume measurements determine volume to nearest cubic yard (cubic meter) from cross-sections before and after removal. For converted volume measurements, summarize validated weight tickets and convert to nearest cubic yard (cubic meter) using density tests as basis for conversion. Adjust measured quantity by deducting embankment in excess of allowable tolerances, if any, in accordance with the Earthwork section of the MOP. |
| C-7 | Determine the area by field measurement to nearest square yard (square meter). Substantiate rates of application by means of at least one test area made at time of application. Determine rate of application on test area by calculation from verified quantities of material placed and measured in the area over which it is applied. For seeding and mulching, record number of square yards (square meters) having substantiated rates of application. |
|  | Volume Type Measurements |
| V-1 | Determine volume to nearest cubic yard (cubic meter) using recorded verified plan dimensions, which include a verification of depth and thickness. |
| V-2 | Record verified plan dimensions of the forms and calculate the volume of each unit. Summarize the final quantity to the nearest cubic yard (cubic meter) or decimal as indicated by the estimated bid quantity (Ref. No.). |
| V-3 | Determine the volume before removal or determine the volume from verified plan dimensions to the nearest cubic yard (cubic meter). |
| V-4 | Plan quantity must be paid as per specification 203.09. Any plan changes must be documented using cross-sections per C-6 or measurements per V-1. |
| V-5 | Determine the volume loose in a vehicle at the point of delivery and summarize to the nearest cubic yard (cubic meter). |
| V-6 | Determine the volume to the nearest cubic yard (cubic meter) using specified dimensions and vertical measurements between bottom plane of footing and top of rock or shale excavation. |
| V-7 | Determine the volume to the nearest cubic yard (cubic meter) from summarization of validated delivery tickets. Tickets must record cubic yard (cubic meter) from predetermined tank size or sizes, converted weight, or meter readings. Tickets for each load must be issued by the contractor and must be collected, identified, and validated by the inspector at the point of delivery. |
| V-8 | Determine the volume to the nearest cubic yard (cubic meter) by using horizontal dimensions established by 503.10 of the specifications. |
|  | Weight Type Measurements |
| W-1 | Summarize the weight to the nearest 100 pounds (50 kilograms) by using validated scale weight tickets or weights calculated from the verified dimensions and steel handbook. |
| W-2 | Obtain a copy of a memorandum from the Office of Structural Engineering for approved pay weights. |
| W-3 | Summarize the validated delivery tickets to the nearest ton (metric ton). Determine the volume of the embankment, if any, in excess of the allowable tolerances, convert to tons (metric tons), and deduct from the summarized weight. |
| W-4 | Summarize to the nearest ton (metric ton) from the validated weight tickets. |
| W-5 | Summarize to the nearest pound (kilogram) from the validated weight tickets. |
|  | Linear Type Measurements |
| L-1 | Measure the length of the item and summarize to the nearest foot (meter). |
| L-2 | Measure the length of an item to the nearest foot (meter), include data as to its re-use and storage. |
| L-3 | Measure the length of the item to the nearest 1/100th mile (1/100 kilometer, e.g. 19.98 kilometer), etc. |
| L-4 | Measure the length to the nearest 0.1 ft (0.1 meter, e.g. 14.8 meter), etc. |
| L-5 | Guardrail quantities are normally determined in standard 12 feet 6 inch (3.81 meter) panel lengths with adjustments for end assemblies, in multiples of 6.25 feet (1.905 meter). |
| L-6 | Measure the length of an item to the nearest 1/100th mile (1/100 kilometer, e.g. 19.98 kilometer), etc. and include deficiency data as per the specifications. |
| L-7 | Measure the length of piles between extreme point and cut-off to the nearest 0.1 ft (25 millimeters). Measure the inside of cast-in-place piles and make a conformance statement relative to the measurement and length marks. Summarize the measurements to the nearest foot (meter). Locate the piles by use of the numbering system on the piling layout. |
| L-8 | For items 603 and 605 measure the length of the item and round up to the next foot (0.5 meter). |
|  | Area Type Measurements |
| A-1 | Measure the surface dimensions of the item, calculate the area, and summarize to the nearest square yard (square meter). |
| A-2 | Adjust the plan area for changes, errors, and deviations in excess of the allowable tolerances, and summarize to the nearest square yard (square meter). |
| A-3 | Determine the specified plan areas of each size and type from verified shop drawings and summarize to the nearest 1/10 square yard (1/10 square meter). |
| A-4 | Plan quantity as per specification 659.24. Check application rates and deduct area if the application rates are below specifications for seed, mulch, and emulsion. |
|  | Unit (each) Type Measurements |
| U-1 | Determine the number of units. |
| U-2 | Determine the number of units visible on completion and make a statement relative to the conformance with the contract requirements. |
| U-3 | Determine the number of units and include the data as to reuse or storage or disposal by Contractor. |
|  | Lump Sum Type Measurements |
| LS-1 | Verify that the work is in accordance with the plans. |
| LS-2 | Record driving log of first test pile or the results of the test loads and locate by use of the numbering system on the piling layout. |
|  | Miscellaneous Type Measurements |
| M-1 | Summarize the final rolling time to the nearest hour. Include the data on weight and tire pressure. |

### Index of Pay Items for the Construction Inspection Forms

Earthwork (200)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 201 | Clearing and Grubbing |  |  |  |  |
| 201 | trees or stumps removed, _____ size | each | U-1 | CA-D-1A/1B or CA-D-3SM |  |
| 201 | clearing and grubbing | lump | LS-1 | CA-D-3SM |  |
| 202 | Removal of Structures and Obstructions |  |  |  |  |
| 202 | structures removed | lump | LS-1 | CA-D-3SM |  |
| 202 | portions of structures removed portions of structures removed portions of structures removed | lump 1 yd3 100 pound | LS-1 V-3 W-1 | CA-D-3SM CA-D-1A/1B CA-D-1A/1B weight | X |
| 202 | pipe removed for reuse or storage | 1 foot | L-2 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | pipe removed | 1 foot | L-1 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | pavement removed | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 202 | wearing course removed | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 202 | base removed | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 202 | walk removed | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 202 | steps removed | lump | LS-1 | CA-D-3SM |  |
| 202 | curb removed | 1 foot | L-1 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | curb and gutter removed | 1 foot | L-1 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | gutter removed gutter removed | 1 foot 0.1 yd2 | L-1 A-1 | CA-D-1A/1B or CA-D-3SM CA-D-1A/1B |  |
| 202 | curb removed for storage | 1 foot | L-2 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | precast traffic dividers removed for re-use and storage | each | U-3 | CA-D-1A/1B or CA-D-3SM |  |
| 202 | buildings demolished | lump | LS-1 | CA-D-3SM |  |
| 202 | underground storage tank removed | each | U-1 | CA-D-3SM |  |
| 202 | regulated storage tank removed | each | U-1 | CA-D-3SM |  |
| 202 | septic tank removed | each | U-1 | CA-D-3SM |  |
| 202 | privy vault removed | each | U-1 | CA-D-3SM |  |
| 202 | guardrail removed | 1 foot | L-1 | CA-D-2 or CA-D-3SM |  |
| 202 | guardrail removed for re-use or storage | 1 foot | L-2 | CA-D-2 or CA-D-3SM |  |
| 202 | fence removed for re-use or storage | 1 foot | L-2 | CA-D-2 or CA-D-3SM |  |
| 202 | rpm’s removed for disposal by contractor | each | U-3 | CA-D-2 or CA-D-3SM |  |
| 202 | manhole removed | each | U-1 | CA-D-3SM |  |
| 202 | manhole abandoned | each | U-1 | CA-D-3SM |  |
| 202 | catch basin of inlet removed | each | U-1 | CA-D-3SM |  |
| 202 | catch basin or inlet abandoned | each | U-1 | CA-D-3SM |  |
| 203 | Roadway Excavation and Embankment |  |  |  |  |
| 203 | excavation | 1 yd3 | V-4 | CA-EW-1, CA-D-3SM |  |
| 203 | embankment | 1 yd3 | V-4 | CA-EW-1, CA-D-3SM |  |
| 203 | granular embankment | 1 yd3 | V-4 | CA-EW-1 | X |
|  |  |  |  | CA-D-3SM |  |
| 203 | granular material, type-__ | 1 yd3 | V-4 | CA-EW-1 | X |
|  |  |  |  | CA-D-3SM |  |
| 203 | borrow | 1 yd3 | C-6 | Cross-Sections |  |
|  | borrow | 1 ton | W-3 | WEIGHT |  |
| 204 | Subgrade Compaction and Proof Rolling |  |  |  |  |
| 204 | subgrade compaction | 1 yd2 | A-2 | CA-D-1A/1B |  |
| 204 | proof rolling | 0.1 hour | M-1 | CA-EW-2 |  |
| 204 | excavation of subgrade | 1 yd3 | V-4 | CA-EW-8 |  |
|  |  |  |  | CA-D-3SM |  |
| 204 | embankment | 1 yd3 | V-4 | CA-EW-8 | X |
|  |  |  |  | CA-D-3SM |  |
| 204 | granular embankment | 1 yd3 | V-4 | CA-EW-8 | X |
|  |  |  |  | CA-D-3SM |  |
| 204 | granular material, type-__ | 1 yd3 | V-4 | CA-EW-8 | X |
|  |  |  |  | CA-D-3SM |  |
| 204 | geotextile fabric | 1 yd2 | A-2 | CA-D-3SM |  |
| 205 | Lime Modified Soil |  |  | CA-EW-8 |  |
| 205 | lime modified soil-dry method | 1 yd3 | V-1 | CA-D-1A/1B, CA-EW-1 |  |
| 205 | lime modified soil-wet method | 1 yd3 | V-1 | CA-D-1A/1B, CA-EW-1 |  |
| 205 | Contractor designed lime soil | Lump | LS-1 | CA-D-3SMA |  |
| 205 | lime | 0.1 ton | W-4 | WEIGHT | X |
| 205 | water | 0.1 m gal | V-7 | CA-D-1 and CA-D-2 |  |
| 206 | Lime Stabilized Subgrade |  |  |  |  |
| 206 | lime soil stabilized subgrade | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 206 | Contractor designed lime soil | Lump | LS-1 | CA-D-3SM |  |
| 206 | lime | 0.1 ton | W-4 | WEIGHT | X |
| 206 | water | 0.1 m gal | V-7 | CA-D-1 and CA-D-2 |  |
| 206 | Test rolling | 0.1 hour | M-1 | CA-EW-2 |  |
| 804 | Cement Stabilized Subgrade |  |  |  |  |
| 804 | Cement stabilized subgrade | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 804 | Cement | 0.1 ton | W-4 | Weight | X |
| 804 | Test rolling | 0.1 hour | M-1 | CA-EW-2 |  |
| 804 | Contractor designed cement soil | Lump | LS-1 | CA-D-3SM |  |
| 832 | Temporary Erosion Control |  |  |  |  |
| 832 | construction seeding and mulching | 1 yd2 | C-7 A-1 | CA-EC-2 |  |
| 832 | slope drains | 1 foot | L-1 | CA-D-3SM |  |
| 832 | benches, dams, sediment basin | 1 yd3 | V-1 | CA-D-1A/1B |  |
| 832 | straw or hay bales | each | U-1 | CA-D-3SM |  |
| 832 | filter fabric fence | 1 foot | L-1 | CA-D-3SM |  |
| 832 | temporary dikes | 1 yd3 | V-1 | CA-D-1A/1B |  |
| 832 | Construction ditch protection | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 832 | rock channel protection | 1 yd3 | V-1 | CA-D-1A/1B |  |
| 832 | sediment removal | 1 yd3 | V-1 | CA-D-1A/1B |  |
| 832 | filter fabric ditch check | 1 foot | L-1 | CA-D-3SM |  |
| 832 | sediment basins and dams | 1 yd3 | V-1 | CA-D-1A/1B |  |
| 832 | bale filter dike | 1 foot | L-1 | CA-D-3SM |  |
| 832 | inlet protection | 1 foot | L-1 | CA-D-3SM |  |
| 832 | construction fence | 1 foot | L-1 | CA-D-3SM |  |
| 208 | Rock Blasting |  |  |  |  |
| 208 | pre-splitting | 1 yd2 | A-1 | CA-D-1 and CA-D-2 |  |
| 208 | pre-blast condition survey | lump sum | LS-1 | CA-D-3SM |  |
| 208 | Scaling | 0.5 hr. | M-1 | CA-D-3SM |  |
| 208 | blasting consultant | lump sum | LS-1 | CA-D-3SM |  |
| 208 | airblast and noise control | lump sum | LS-1 | CA-D-3SM |  |
| 208 | vibration control and monitoring | lump sum | LS-1 | CA-D-3SM |  |
| 208 | Hydrologist | lump sum | LS-1 | CA-D-3SM |  |

Pavement Repairs (250)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 251 | Partial Depth Pavement Repair |  |  |  |  |
| 251 | partial depth pavement repair | 0.1 yd2 | A-1 | CA-D-6 |  |
| 252 | Full Depth Rigid Pavement Removal and Flexible Replacement |  |  |  |  |
| 252 | full depth rigid pavement removal and flexible replacement | 0.1 yd2 | A-1 | CA-D-6 |  |
| 252 | full depth pavement sawing | 0.1 foot | L-1 | CA-D-6 |  |
| 253 | Pavement Repair |  |  |  |  |
| 253 | pavement repair pavement repair | 0.1 yd2 0.1 yd3 | A-1 V-1 | CA-D-6 CA-D-6 |  |
| 254 | Pavement Planing |  |  |  |  |
| 254 | pavement planing, bituminous | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 254 | pavement planing, concrete | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 254 | patching planed surfaces | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 255 | Full Depth Pavement Removal and Rigid Replacement |  |  |  |  |
| 255 | full depth pavement removal and rigid replacement, class ______ | 0.1 yd2 | A-1 | CA-D-6 |  |
| 255 | full depth pavement sawing | 0.1 foot | L-1 | CA-D-6 |  |
| 256 | Bonded Patching of Portland Cement Concrete Pavement |  |  |  |  |
| 256 | bonded patching of Portland cement conc., type __ | 0.1 ft2 | A-1 | CA-D-6 |  |
| 257 | Diamond Grinding Portland Cement Concrete Pavement |  |  |  |  |
| 257 |  |  |  |  |  |
| 258 | Dowel Bar Retrofit |  |  |  |  |
| 258 |  |  |  |  |  |

Bases (300)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 301 | Asphalt Concrete Base |  |  |  |  |
| 301 | bituminous aggregate base | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3, 4, and 5 | X |
| 302 | Asphalt Concrete Base |  |  |  |  |
| 302 | asphalt concrete base | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3, 4, and 5 | X |
| 304 | Aggregate Base |  |  |  |  |
| 304 | aggregate base | 0.1 yd3 | V-1 C-5 | CA-D-1 and 2 | X |
| 305 | Portland Cement Concrete Base |  |  |  |  |
| 305 | concrete base | 0.1 yd2 | A-2 | CA-D-1A/1B |  |
| 320 | Rubblize and Roll |  |  |  |  |
| 320 | rubblize and roll | 0.1 yd2 | A-1 | CA-D-3SM |  |
| 320 | filler aggregate | 0.1 yd3 | V-1 | CA-D-3SM |  |
| 321 | Cracking and Seating Existing Plain Concrete Pavement |  |  |  |  |
| 321 | cracking and seating existing plain concrete pavement | 0.1 yd2 | A-1 | CA-D-1A/1B |  |

Flexible Pavement (400)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 407 | Tack Coat |  |  |  |  |
| 407 | tack coat | 1 gallon | C-4 | CA-FP-6 | X |
| 407 | tack coat for intermediate course | 1 gallon | C-4 | CA-FP-6 | X |
| 408 | Prime Coat |  |  |  |  |
| 408 | bituminous prime coat | 1 gallon | C-4 | CA-FP-6 | X |
| 409 | Sawing and Sealing Asphalt Concrete Pavement Joints |  |  |  |  |
| 409 | Saw and seal asphalt joints | 0.1 ft | L-4 | CA-D-1A/1B | X |
| 410 | Traffic Compacted Surface |  |  |  |  |
| 410 | traffic compacted surface type a or b | 0.1 yd3 0.1 ton | C-3 W-4 | CA-D-1A/1B WEIGHT | X X |
| 410 | traffic compacted surface type c | 0.1 yd3 0.1 ton | C-3 W-4 | CA-D-1A/1B WEIGHT | X X |
| 411 | Stabilized Crushed Aggregate |  |  |  |  |
| 411 | stabilized crushed aggregate | 0.1 yd3 | C-5 | CA-D-1A/1B | X |
| 421 | Microsurfacing |  |  |  |  |
| 421 | microsurfacing, surface course | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 421 | microsurfacing, leveling course | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 421 | microsurfacing, rut fill course | 0.1 ton | W-4 | CA-D-1A/1B | X |
| 422 | Chip Seal with Polymer Binder |  |  |  |  |
| 422 | single chip seal with polymer binder | 0.1 yd2 | A-1 | CA-D-1A/1B, CA-D-2 |  |
| 422 | double chip seal with polymer binder | 0.1 yd2 | A-1 | CA-D-1A/1B, CA-D-2 |  |
| 423 | Crack Sealing, Hot Applied |  |  |  |  |
| 423 | crack sealing | 1 pound | W-5 | CA-D-1A/1B | X |
| 424 | Fine Graded Polymer Asphalt Concrete |  |  |  |  |
| 424 | Fine Graded Polymer Asphalt Concrete, A or B | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4 | X |
| 442 | SuperPave Asphalt Concrete |  |  |  |  |
| 442 | asphalt concrete, intermediate course, __ mm, type __ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-2 | X |
| 442 | asphalt concrete, surface course, __ mm, type __ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-2 | X |
| 443 | Stone Matrix Asphalt Concrete |  |  |  |  |
| 443 | Stone Matrix Asphalt Concrete | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4, | X |
| 446 | Asphalt Concrete |  |  |  |  |
| 446 | asphalt concrete, intermediate course type_ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4, TE 217 | X |
| 446 | asphalt concrete, surface course type_ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4, TE 217 | X |
| 448 | Asphalt Concrete |  |  |  |  |
| 448 | asphalt concrete, intermediate. course type_ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4 | X |
| 448 | asphalt concrete, surface course type_ | 0.1 yd3 | C-5 | CA-D-1A/1B CA-FP-3 and 4 | X |

Rigid Pavement (450)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 451 | Reinforced Portland Cement Concrete Pavement |  |  |  |  |
| 451 | reinforced concrete pavement | 0.1 yd2 | A-2 | CA-D-3SM, CA-D-9 |  |
| 452 | Non-Reinforced Portland Cement Concrete Pavement |  |  |  |  |
| 452 | plain concrete pavement | 0.1 yd2 | A-2 | CA-D-3SM, CA-D-9 |  |
| 455 | Concrete Quality Control |  |  |  |  |
| 499 | Concrete General |  |  |  |  |

Structures (500)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 502 | Structures for Maintaining Traffic |  |  |  |  |
| 502 | temporary structures | lump sum | LS-1 | CA-D-3SM |  |
| 503 | Excavation for Structures |  |  |  |  |
| 503 | cofferdams, cribs, and sheeting | lump sum | LS-1 | CA-D-3SM |  |
| 503 | unclassified excavation | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 503 | unclassified excavation | lump sum | LS-1 | CA-D-3SM |  |
| 503 | unclassified excavation including rock | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 503 | unclassified excavation including shale | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 503 | unclassified excavation including shale or rock | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 503 | rock excavation | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 503 | shale excavation | 1 yd3 | V-8 | CA-D-1A/1B |  |
| 504 | Sheet Piling Left in Place |  |  |  |  |
| 504 | steel sheet piling left in place _____ | 0.1 ft2 | A-1 | CA-D-1A/1B |  |
| 505 | Pile Driving Equipment Mobilization |  |  |  |  |
| 505 | pile driving equipment mobilization | lump sum | LS-2 | CA-D-3SM |  |
| 506 | Static Load Test |  |  |  |  |
| 506 | static load test | lump sum | LS-2 | CA-D-3SM |  |
| 506 | subsequent static load test | each | LS-2 | CA-D-3SM |  |
| 507 | Bearing Piles |  |  |  |  |
| 507 | steel piles hp______x______, furnished | 0.1 foot | L-7 | CA-D-3SM |  |
| 507 | steel piles hp_____x_____, driven | 0.1 foot | L-7 | CA-S-3, BR-2-75A |  |
| 507 | __in cast-in-place, reinforced concrete piles, furnished | 0.1 foot | L-7 | CA-D-3SM |  |
| 507 | __in cast-in-place, reinforced concrete piles, driven | 0.1 foot | L-7 | CA-S-3, BR-2-75A, TE-45 |  |
| 507 | timber piles, creosoted | 0.1 foot | L-7 | CA-S-3, BR-2-75A |  |
| 507 | timber piles, untreated | 0.1 foot | L-7 | CA-S-3, BR-2-75A |  |
| 507 | pre-bored holes | 0.1 foot | L-1 | CA-D-3SM |  |
| 507 | steel points or shoes | each | U-1 | CA-D-3SM |  |
| 508 | Falsework and Forms |  |  |  |  |
| 509 | Reinforcing Steel |  |  |  |  |
| 509 | epoxy coated reinforcing steel | 1 pound | W-5 | CA-D-1A/1B, CA-S-9 |  |
| 510 | dowel holes with cement grout | each | U-1 | CA-D-3SM |  |
| 510 | dowel holes with nonmetallic grout | each | U-1 | CA-D-3SM |  |
| 511 | Concrete for Structures |  |  |  |  |
| 511 | class ____ concrete,____________________ | 0.1 yd3 | V-2 | CA-D-1A/1B, TE-45 |  |
| 511 | class_____ concrete, bridge deck | 0.1 yd3 0.1 yd2 | V-2 A-2 | CA-D-1A/1B, TE-45, CA-S-4, CA-S-6, CA-S-22, D10-S-31 |  |
| 512 | Waterproofing |  |  |  |  |
| 512 | type______ waterproofing | 0.1 yd2 lump sum | A-1 LS-1 | CA-D-1A/1B CA-D-3SM |  |
| 512 | Sealing of Concrete Surfaces | 0.1 yd2 | A-1 | CA-D-1A/1B CA-D-3SM, CA-S-21 |  |
| 513 | Structural Steel Members |  |  |  |  |
| 513 | structural steel (___________________) | 100 pound lump sum | W-2 LS-1 | SEE W-2 CA-D-3SM, CA-S-20 |  |
| 513 | welded stud shear connectors | each | U-1 | CA-D-3SM |  |
| 514 | Painting of Structural Steel |  |  |  |  |
| 514 | surface preparation, existing steel, system __ | lump sum Sq. Ft. | LS-1 | CA-D-3SM, CA-S-7, CA-S-11, CA-S-12,CA-S-13,CA-S-14, CA-S-32 |  |
| 514 | field painting of existing steel, prime coat, system_______ | lump sum Sq. Ft. | LS-1 | CA-D-3SM, CA-S-2, CA-S-7, CA-S-15,CA-S-32 |  |
| 514 | Field painting of structural steel, intermediate coat, system_______ | lump sum Sq. Ft. | LS-1 | CA-D-3SM, CA-S-2, CA-S-7, CA-S-16, CA-S-17, CA-S-37 |  |
| 514 | field painting of structural steel, finish coat, system _______ | lump sum Sq. Ft. | LS-1 | CA-D-3SM, CA-S-2, CA-S-7, CA-S-17, CA-S-32 |  |
| 514 | Grinding fins, tears, slivers on existing structural steel | Lump sum | LS-1 | CA-D-3SM, CA-S-16 |  |
| 514 | Final Inspection Repair | Each | U-1 | CA-D-3SM, CA-S-18, CA-S-19 |  |
| 515 | Prestressed Concrete Bridge Members |  |  |  |  |
| 515 | prestressed concrete bridge members | each 0.1 foot | U-1 L-4 | CA-D-3SM, CA-S-20 CA-D-3SM |  |
| 515 | intermediate diaphragms | each | U-1 | CA-D-3SM |  |
| 516 | Expansion and Contraction Joints, Joint Sealers, and Bearing Devices |  |  |  |  |
| 516 | structural steel expansion joints | 0.1 foot 100 pound | L-4 W-2 | CA-D-3SM SEE W-2 |  |
| 516 | structural expansion joints including elastomeric ________ seals | 0.1 foot | L-4 | CA-D-3SM |  |
| 516 | elastomeric compression seals for structural steel joints, _____ width | 0.1 foot | L-4 | CA-D-3SM |  |
| 516 | folder copper strip __________ | 0.1 foot | L-4 | CA-D-3SM |  |
| 516 | vertical extension of structural expansion joints | 0.1 foot | L-4 | CA-D-3SM |  |
| 516 | _____ preformed expansion joint filler | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 516 | joint sealer | 0.1 foot | L-4 | CA-D-3SM |  |
| 516 | bearing devices | each 0.1 foot 0.1 yd2 100 pound | U-1 L-4 A-1 W-2 | CA-D-3SM CA-D-3SM CA-D-1A/1B SEE W-2 |  |
| 516 | _____in elastomeric bearing pad | each 0.1 yd2 | U-1 A-1 | CA-D-3SM CA-D-1A/1B |  |
| 516 | elastomeric bearing with internal laminates only ___x___x___ | each | U-1 | CA-D-3SM |  |
| 516 | elastomeric bearing with internal laminates with load plate___x___x___ | each | U-1 | CA-D-3SM |  |
| 516 | 1/8 inch preformed bearing pads | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 517 | Railings |  |  |  |  |
| 517 | railing (____________________________) | 0.1 foot | L-1 | CA-D-3SM |  |
| 518 | Drainage of Structures |  |  |  |  |
| 518 | porous backfill | 0.1 yd3 lump sum | V-1 LS-1 | CA-D-1A/1B CA-D-3SM |  |
| 518 | porous backfill with filter fabric | 0.1 yd3 lump sum | V-1 LS-1 | CA-D-1A/1B CA-D-3SM |  |
| 518 | ____in _____ pipe include specials | 0.1 foot | L-4 | CA-D-3SM |  |
| 518 | scuppers, including supports | each | U-1 | CA-D-3SM |  |
| 518 | trough horizontal conductors | 100 pound 0.1 foot | W-2 L-1 | SEE W-2 CA-D-3SM |  |
| 518 | pipe horizontal conductors | 100 pound 0.1 foot | W-2 L-1 | SEE W-2 CA-D-3SM |  |
| 518 | ___ in ___ pipe downspout | 1 foot | L-1 | CA-D-3SM |  |
| 519 | Patching Concrete Structures |  |  |  |  |
| 519 | patching concrete structures | 0.1 ft2 | A-1 | CA-D-1A/1B |  |
| 520 | Pneumatically Placed Mortar |  |  |  |  |
| 520 | pneumatically placed mortar | 0.1 ft2 | A-1 | CA-D-1A/1B |  |
| 522 | Structural Plate Corrugated Metal Structures on Footings |  |  |  |  |
| 522 | ___in ___ structural plate corrugated metal structure, _____in | 0.1 foot | L-1 | CA-D-3SM |  |
| 523 | Dynamic Load Test |  |  |  |  |
| 523 | dynamic load testing | each | U-1 | CA-D-3SM |  |
| 523 | re-strike | each | U-1 | CA-D-3SM |  |
| 524 | Drilled Shafts |  |  |  |  |
| 524 | drilled shafts, ___in diameter, above bedrock | 0.1 foot | L-1 | CA-S-1, TE-45, CA-S-30 |  |
| 524 | drilled shafts, ___in diameter, into bedrock | 0.1 foot | L-1 | CA-S-1, TE-45, CA-S-30 |  |
| 524 | drilled shafts, ___in diameter | 0.1 foot | L-1 | CA-S-1, TE-45, CA-S-30 |  |
| 526 | Approach Slabs |  |  |  |  |
| 526 | reinforced concrete approach slabs | 0.1 yd2 | A-1 | CA-D-1A/1B |  |

Incidentals (600)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 601 | Slope and Channel Protections |  |  |  |  |
| 601 | riprap | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 601 | crushed aggregate slope protection | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 601 | concrete slope protection | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 601 | dump rock fill, type ______ | 0.1 yd3 | V-1 | CA-D-1A/1B |  |
| 601 | rock channel protection, type ____ with filter | 0.1 yd3 | V-1 | CA-D-1A/1B |  |
| 601 | rock channel protection, type ____ without filter | 0.1 yd3 | V-1 | CA-D-1A/1B |  |
| 601 | paved gutter | 0.1 foot | L-1 | CA-D-3SM |  |
| 602 | Masonry |  |  |  |  |
| 602 | brick masonry | 0.1 yd3 | V-2 | CA-D-1A/1B |  |
| 602 | block masonry | 0.1 yd3 | V-2 | CA-D-1A/1B |  |
| 602 | concrete masonry | 0.1 yd3 | V-2 | CA-D-1A/1B |  |
| 605 | Underdrains |  |  |  |  |
| 605 | ___in unclassified pipe underdrains | 0.1 foot | L-8 | CA-P-2 |  |
| 605 | ___in shallow pipe underdrains | 0.1 foot | L-8 | CA-P-2 |  |
| 605 | ___in deep pipe underdrains | 0.1 foot | L-8 | CA-P-2 |  |
| 605 | aggregate drains | 0.1 foot | L-8 | CA-D-3SM |  |
| 605 | ___in rock cut underdrain | 0.1 foot | L-8 | CA-P-2 |  |
| 605 | ___in prefabricated edge underdrain | 0.1 foot | L-8 | CA-P-2 |  |
| 606 | Guardrail |  |  |  |  |
| 606 | guardrail, type_____ | 0.1 foot | L-5 | CA-D-2 OR CA-D-3SM |  |
| 606 | guardrail, barrier design, type_____ | 0.1 foot | L-5 | CA-D-2 OR CA-D-3SM |  |
| 606 | guardrail, rebuilt, type______ | 0.1 foot | L-5 | CA-D-2 OR CA-D-3SM |  |
| 606 | anchor assembly, type_____ | each | U-1 | CA-D-3SM |  |
| 606 | anchor assembly, type ___, barrier design | each | U-1 | CA-D-3SM |  |
| 606 | bridge terminal assembly, type _____ | each | U-1 | CA-D-3SM |  |
| 606 | guardrail post | each | U-1 | CA-D-3SM |  |
| 607 | Fence |  |  |  |  |
| 607 | fence, type _____ | 0.1 foot | L-1 | CA-D-2 OR CA-D-3SM |  |
| 607 | gate, type _____ | each | U-1 | CA-D-3SM |  |
| 608 | Walks, Curb Ramps, and Steps |  |  |  |  |
| 608 | concrete walk | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 608 | bituminous walk | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 608 | aggregate walk | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 608 | curb ramps | 0.1 yd2 each | A-1 U-1 | CA-D-1A/1B CA-D-3SM |  |
| 608 | concrete steps | 0.1 foot | L-1 | CA-D-3SM |  |
| 609 | Curbing, Concrete Medians, and Traffic Islands |  |  |  |  |
| 609 | sandstone curb | 0.1 foot | L-1 | CA-D-3SM |  |
| 609 | curb, type______ | 0.1 foot | L-1 | CA-D-3SM |  |
| 609 | combination curb and gutter | 0.1 foot | L-1 | CA-D-3SM |  |
| 609 | asphalt concrete curb | 0.1 foot | L-1 | CA-D-3SM |  |
| 609 | concrete traffic island | 0.1 yd2 0.1 yd3 | A-1 V-1 | CA-D-1A/1B CA-D-1A/1B |  |
| 609 | concrete median | 0.1 yd2 0.1 yd3 | A-1 V-1 | CA-D-1A/1B CA-D-1A/1B |  |
| 610 | Cellular Retaining Walls |  |  |  |  |
| 610 | cellular retaining wall | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 611 | Pipe Culverts, Sewers, and Drains |  |  |  |  |
| 611 | ____ in conduit, type ________ | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ________x________ conduit, type _____ | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ____ in conduit reconstructed, type ___ | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | type a precast reinforced concrete flat topped three-sided culvert ___in span x ___in rise | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | type a precast reinforced concrete arch section ___in span x ___in rise | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ___in rise x ___in span conduit, type a corrugated steel box culvert, ___in min. cover, ___in max. cover | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ___in rise x ___in span conduit, type a corrugated aluminum box culvert, ___in min. cover, ___in max. cover | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ___in conduit, type ___, field paving | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | ___in conduit, type ___, field paving of existing pipe | 0.1 foot | L-8 | CA-P-1(611) by Contractor |  |
| 611 | manholes | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | inlets | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | catch basins | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | manhole, catch basin, or inlet reconstructed to grade | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | manhole, catch basin, inlet, or monument bas adjusted to grade | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | precast reinforced concrete outlet | each | U-1 | CA-P-3(611) by Contractor |  |
| 611 | inspection wells | each | U-1 | CA-D-3SM |  |
| 611 | junction chambers | each | U-1 | CA-D-3SM |  |
| 613 | Low Strength Mortar Backfill |  |  |  |  |
| 613 | low strength mortar backfill | 0.1 yd3 | V-1 | CA-D-1A/1B |  |
| 613 | low strength mortar backfill, type___ | 0.1 yd3 | V-1 | CA-D-1A/1B |  |
| 614 | Maintaining Traffic |  |  |  |  |
| 614 | maintaining traffic | lump sum | LS-1 | CA-D-3SM |  |
| 614 | temporary pavement markings | each 1 foot 0.01 mile | U-1 L-1 L-3 | CA-D-3SM CA-D-3SM CA-D-3SM |  |
| 614 | temporary raised pavement marker | each | U-1 | CA-D-3SM |  |
| 614 | portable changeable message board | Each 1 month | U-1 U-1 | CA-D-3SM CA-D-3SM |  |
| 614 | Work zone speed limit sign | Each | U-1 | CA-D-3SM |  |
| 614 | Work zone marking sign | Each | U-1 | CA-D-3SM |  |
| 614 | law enforcement officer w/patrol car | 1 hour | U-1 | CA-D-3SM |  |
| 614 | barrier reflector | Each | U-1 | CA-D-3SM |  |
| 614 | temporary crossover lighting system | Each | U-1 | CA-D-3SM |  |
| 614 | temporary impact attenuator | Each | U-1 | CA-D-3SM |  |
| 614 | temporary lane line class_____ | 0.01 mile | L-3 | CA-D-3SM |  |
| 614 | temporary center line class _____ | 0.01 mile | L-3 | CA-D-3SM |  |
| 614 | temporary channelizing line, class I ___ | 1 foot | L-1 | CA-D-3SM |  |
| 614 | temporary edge line class I, ______ | 0.01 mile | L-3 | CA-D-3SM |  |
| 614 | temporary gore marking, class II , ____ | 1 foot | L-1 | CA-D-3SM |  |
| 614 | temporary stop line, class I , _____ | 1 foot | L-1 | CA-D-3SM |  |
| 614 | temporary crosswalk line, class I , _____ | 1 foot | L-1 | CA-D-3SM |  |
| 614 | temporary dotted line, class I ,_____ | 1 foot | L-1 | CA-D-3SM |  |
| 614 | bituminous concrete for maintaining traffic | 0.1 yd3 | C-5 | CA-D-1A/1B | X |
| 615 | Roads and Pavements for Maintaining Traffic |  |  |  |  |
| 615 | temporary pavement, class a | 0.1 yd2 | A-1 | CA-D-1A/1B | X |
| 615 | temporary pavement, class b | 0.1 yd2 | A-1 | CA-D-1A/1B | X |
| 615 | temporary roads | lump sum | LS-1 | CA-D-3SM |  |
| 616 | Dust Control |  |  |  |  |
| 616 | water | 0.1 m gal | V-7 | CA-D-3SM | X |
| 616 | calcium chloride | 0.1 ton | C-2 | CA-D-3SM | X |
| 617 | Reconditioning Shoulders |  |  |  |  |
| 617 | shoulder preparation | 1 yd2 | A-1 | CA-D-1 and 2 |  |
| 617 | compacted aggregate, type___ | 0.1 yd3 | C-1 | WEIGHT | X |
| 617 | water | 0.1 m gal | V-7 | CA-D-1 and 2 | X |
| 618 | Rumble Strips on Shoulders |  |  |  |  |
| 618 | rumble strips, type 1 | 1 foot 0.1 mile | L-1 L-3 | CA-D-3SM CA-D-3SM |  |
| 618 | rumble strips, type 2 (asphalt) | 1 foot 0.1 mile | L-1 L-3 | CA-D-3SM CA-D-3SM |  |
| 618 | rumble strips, type 2 (concrete) | 1 foot 0.1 mile | L-1 L-3 | CA-D-3SM CA-D-3SM |  |
| 618 | rumble strips, type 3 | 1 foot 0.1 mile | L-1 L-3 | CA-D-3SM CA-D-3SM |  |
| 619 | Field Office |  |  |  |  |
| 619 | field office, type _____ | lump sum | LS-1 | CA-D-3SM |  |
| 619 | field office, type _____ | ½ month | U-1 | CA-D-3SM |  |
| 620 | Delineators |  |  |  |  |
| 620 | delineator, type____, post mounted | Each | U-1 | CA-D-3SM |  |
| 620 | delineator, type ____, bracket mounted | Each | U-1 | CA-D-3SM |  |
| 620 | temporary delineator, type_____ | Each | U-1 | CA-D-3SM |  |
| 620 | delineator removed for storage or removal | Each | U-3 | CA-D-3SM |  |
| 620 | reflector, type _____ | Each | U-1 | CA-D-3SM |  |
| 621 | Raised Pavement Markers (RPM) |  |  |  |  |
| 621 | raised pavement marker | Each | U-1 | CA-D-3SM |  |
| 621 | raised pavement marker, reflector | Each | U-1 | CA-D-3SM |  |
| 621 | two-way radio equipment | lump sum | LS-1 | CA-D-3SM |  |
| 622 | Concrete Barrier |  |  |  |  |
| 622 | concrete barrier, type_____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 622 | portable concrete barrier, _____in | 0.1 foot | L-1 | CA-D-3SM |  |
| 622 | portable concrete barrier, __in bridge mount | 0.1 foot | L-1 | CA-D-3SM |  |
| 623 | Construction Layout Stakes |  |  |  |  |
| 623 | construction layout stakes | lump sum | LS-1 | CA-D-3SM |  |
| 623 | monuments assemblies | each | U-1 | CA-D-3SM |  |
| 623 | reference monuments | each | U-1 | CA-D-3SM |  |
| 623 | Right of Way monuments | each | U-1 | CA-D-3SM |  |
| 624 | Mobilization |  |  |  |  |
| 624 | mobilization | lump sum | LS-1 | CA-D-3SM |  |
| 625 | Highway Lighting |  |  |  |  |
| 625 | light pole | Each | U-2 | CA-D-3SM |  |
| 625 | light pole foundation | Each | U-2 | CA-D-3SM |  |
| 625 | light tower | Each | U-2 | CA-D-3SM |  |
| 625 | light tower foundation | Each | U-2 | CA-D-3SM |  |
| 625 | light tower maintenance platform | Each | U-2 | CA-D-3SM |  |
| 625 | luminaire | Each | U-2 | CA-D-3SM |  |
| 625 | bracket arm | Each | U-2 | CA-D-3SM |  |
| 625 | glare shield | Each | U-2 | CA-D-3SM |  |
| 625 | ground rod | Each | U-2 | CA-D-3SM |  |
| 625 | pull box, (type), (size) | Each | U-2 | CA-D-3SM |  |
| 625 | trench | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | trench in paved areas, type____ | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | conduit, (type), (size) | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | conduit jacked or drilled under pavement (size ___) | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | no.___awg,__ volt distribution cable | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | no.___awg,__, pole and bracket cable | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | __in duct-cable w/ __no.__awg,__-volt cable | 0.1 foot | L-1 | CA-D-2 |  |
| 625 | connector kit, type____ | Each | U-2 | CA-D-2 |  |
| 625 | cable splicing kit | Each | U-2 | CA-D-2 |  |
| 625 | power service | Each | U-2 | CA-D-3SM |  |
| 625 | structural grounding system | Each | U-2 | CA-D-3SM |  |
| 625 | light pole anchor bolts on structure | Each | U-2 | CA-D-3SM |  |
| 625 | junction box | Each | U-2 | CA-D-3SM |  |
| 625 | portable power unit | Each | U-2 | CA-D-3SM |  |
| 625 | high voltage test | lump sum | LS-1 | CA-D-3SM |  |
| 626 | Barrier Reflectors |  |  |  |  |
| 626 | barrier reflector, type a | Each | U-1 | CA-D-3SM |  |
| 626 | barrier reflector, type b | Each | U-1 | CA-D-3SM |  |
| 626 | barrier reflector, type a2 | Each | U-1 | CA-D-3SM |  |
| 626 | barrier reflector, type b2 | Each | U-1 | CA-D-3SM |  |
| 630 | Traffic Signs and Sign Supports |  |  |  |  |
| 630 | ground mounted beam support foundation. | Each | U-1 | CA-D-3SM |  |
| 630 | rigid overhead sign support foundation | Each | U-1 | CA-D-3SM |  |
| 630 | span wire sign support foundation | Each | U-1 | CA-D-3SM |  |
| 630 | ground mounted support, ______post | 0.1 foot | L-4 | CA-D-2 |  |
| 630 | ground mounted support, _____beam | 0.1 foot | L-4 | CA-D-2 |  |
| 630 | one way support, _____post | 0.1 foot | L-4 | CA-D-2 |  |
| 630 | temporary sign support | 0.1 foot Each | L-4 U-1 | CA-D-2 CA-D-3SM |  |
| 630 | street name sign support | 0.1 foot | L-4 | CA-D-2 |  |
| 630 | breakaway beam connection | Each | U-1 | CA-D-3SM |  |
| 630 | overhead sign support, type tc-_______, design _____ | Each | U-1 | CA-D-3SM |  |
| 630 | combination overhead sign support, type___, design____ | Each | U-1 | CA-D-3SM |  |
| 630 | sign attachment assembly | Each | U-1 | CA-D-3SM |  |
| 630 | luminaire support assembly, type tc-____ | Each | U-1 | CA-D-3SM |  |
| 630 | span wire sign support, type tc-17.10, design___ | Each | U-1 | CA-D-3SM |  |
| 630 | overpass structure mounted sign support, type tc - _____, design___ | Each | U-1 | CA-D-3SM |  |
| 630 | sign hanger assembly, (span, wire, mast arm) | Each | U-1 | CA-D-3SM |  |
| 630 | sign support assembly, (pole or bridge mount) | Each | U-1 | CA-D-3SM |  |
| 630 | sign, (flat sheet, extrusheet, temp. overlay) | 0.1 ft2 | A-3 | CA-D-2 |  |
| 630 | sign, double faced, (street name, mile marker) | Each | U-1 | CA-D-3SM |  |
| 630 | sign erected, (flat sheet, extrusheet, temp. overlay) | 0.1 ft2 | A-3 | CA-D-2 |  |
| 630 | sign backing assembly | Each | U-1 | CA-D-3SM |  |
| 630 | covering sign | 0.1 ft2 | A-3 | CA-D-1A/1B |  |
| 630 | barrier wall assembly, type tc-design | each | U-1 | CA-D-3SM |  |
| 630 | removal of ground mounted (major) sign and (storage, re-erection, or disposal) | each | U-1 | CA-D-2 |  |
| 630 | removal of ground mounted (beam, post) support and (storage or disposal) | each | U-1 | CA-D-2 |  |
| 630 | removal of overhead mounted sign and (storage, re-erection, or disposal) | each | U-1 | CA-D-2 |  |
| 630 | removal of overhead sign support and (storage, re-erection, of disposal), type tc- _____ | each | U-1 | CA-D-2 |  |
| 630 | removal of overlay sign | each | U-1 | CA-D-2 |  |
| 631 | Sign Lighting and Electrical Signs |  |  |  |  |
| 631 | sign service | each | U-1 | CA-D-2 |  |
| 631 | sign wired | each | U-1 | CA-D-2 |  |
| 631 | sign wired, overpass structure mounted | each | U-1 | CA-D-2 |  |
| 631 | disconnect switch with enclosure, type_ | each | U-1 | CA-D-2 |  |
| 631 | switch enclosure mounting bracket assembly | each | U-1 | CA-D-2 |  |
| 631 | ballast (integral or remote), type ___ | each | U-1 | CA-D-2 |  |
| 631 | ballast wiring enclosure, type ___ | each | U-1 | CA-D-2 |  |
| 631 | ballast wiring enclosure mounting bracket | each | U-1 | CA-D-2 |  |
| 631 | photoelectric control | each | U-1 | CA-D-3SM |  |
| 631 | mercury vapor luminaire, type ___, with _____-watt lamp | each | U-1 | CA-D-2 |  |
| 631 | changeable message sign, electrical type(limited, unlimited) message | each | U-1 | CA-D-3SM |  |
| 631 | changeable message sign, drum type | each | U-1 | CA-D-3SM |  |
| 631 | internally illuminated fixed message sign, type ______ | each | U-1 | CA-D-3SM |  |
| 631 | sign flasher assembly | each | U-1 | CA-D-3SM |  |
| 631 | school speed limit sign assembly, __in | each | U-1 | CA-D-3SM |  |
| 631 | timer with enclosure | each | U-1 | CA-D-3SM |  |
| 631 | removal of (luminaire, disconnect switch, ballast, etc.) and (storage or re-erection) | each | U-1 | CA-D-2 |  |
| 632 | Traffic Signal Equipment |  |  |  |  |
| 632 | vehicular signal head,__-section___ in lens _____-way | each | U-1 | CA-D-3SM |  |
| 632 | vehicular signal head, optically programmed, __ -section, ___in lens, __way | each | U-1 | CA-D-3SM |  |
| 632 | pedestrian signal head, type ____ | each | U-1 | CA-D-3SM |  |
| 632 | pedestrian pushbutton | each | U-1 | CA-D-3SM |  |
| 632 | loop detector unit | each | U-1 | CA-D-3SM |  |
| 632 | detector loop | each | U-1 | CA-D-3SM |  |
| 632 | magnetometer detector unit | each | U-1 | CA-D-3SM |  |
| 632 | magnetometer sensor probe | each | U-1 | CA-D-3SM |  |
| 632 | strain pole foundation | each | U-1 | CA-D-3SM |  |
| 632 | signal support foundation | each | U-1 | CA-D-3SM |  |
| 632 | pedestal foundation | each | U-1 | CA-D-3SM |  |
| 632 | signal support, type tc-____,design___ | each | U-1 | CA-D-3SM |  |
| 632 | combination signal support, type tc-___, design___ | each | U-1 | CA-D-3SM |  |
| 632 | strain pole, type tc-___, design___ | each | U-1 | CA-D-3SM |  |
| 632 | combination strain pole, type tc-___, design ____ | each | U-1 | CA-D-3SM |  |
| 632 | strain pole embedded, type tc-__,design_ | each | U-1 | CA-D-3SM |  |
| 632 | comb. strain pole embedded, type tc-___ design -____ | each | U-1 | CA-D-3SM |  |
| 632 | wood pole, class ____, (length) ft | each | U-1 | CA-D-3SM |  |
| 632 | down guy | each | U-1 | CA-D-3SM |  |
| 632 | pedestal, (length) ft | each | U-1 | CA-D-3SM |  |
| 632 | pedestal, (length) ft, transformer base | each | U-1 | CA-D-3SM |  |
| 632 | conduit riser, __________in dia. | each | U-1 | CA-D-3SM |  |
| 632 | messenger wire, (no.) strand _____in | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | signal cable, _____-conductor no.__awg | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | interconnect cable,____-conductor no.__ | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | interconnect cable, integral messenger wire type, ____-conductor no.___awg | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | loop detector lead-in cable | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | magnetometer lead-in cable | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | power cable, ___-conductor no.___awg | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | service cable, ___-conductor no.____awg | 0.1 foot | L-1 | CA-D-3SM |  |
| 632 | power service | each | U-1 | CA-D-3SM |  |
| 632 | covering of vehicular signal head | each | U-1 | CA-D-3SM |  |
| 632 | removal of traffic signal installation | each | U-1 | CA-D-3SM |  |
| 632 | removal of (item) and (storage or re-erection) | each | U-3 | CA-D-3SM |  |
| 632 | reuse of (item) | each | U-3 | CA-D-3SM |  |
| 633 | Traffic Signal Controllers |  |  |  |  |
| 633 | controller unit, type ___, with cabinet, type ___ | each | U-1 | CA-D-3SM |  |
| 633 | controller unit, type ___ | each | U-1 | CA-D-3SM |  |
| 633 | controller unit, type ___, furnish only | each | U-1 | CA-D-3SM |  |
| 633 | cabinet, type ___ | each | U-1 | CA-D-3SM |  |
| 633 | cabinet, type___, furnish only | each | U-1 | CA-D-3SM |  |
| 633 | cabinet riser | each | U-1 | CA-D-3SM |  |
| 633 | controller, master, traffic responsive | each | U-1 | CA-D-3SM |  |
| 633 | controller, master, traffic responsive, furnish only | each | U-1 | CA-D-3SM |  |
| 633 | remote monitoring station | each | U-1 | CA-D-3SM |  |
| 633 | telephone service | each | U-1 | CA-D-3SM |  |
| 633 | training | each | U-1 | CA-D-3SM |  |
| 633 | system analysis | each | U-1 | CA-D-3SM |  |
| 633 | cabinet foundation | each | U-1 | CA-D-3SM |  |
| 633 | controller work pad | each | U-1 | CA-D-3SM |  |
| 633 | flasher controller | each | U-1 | CA-D-3SM |  |
| 638 | Water Mains and Service Branches |  |  |  |  |
| 638 | ___in water main, ductile iron, ANSI class___, joints and fittings | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in water main, polyvinyl chloride pipe and fittings | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in copper service branch | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in polyethylene service branch | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in polybutylene service branch | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | polyethylene encasement | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in steel pipe encasement (open cut, bored) | 0.1 foot | L-1 | CA-D-3SM |  |
| 638 | ___in gate valve and valve box | each | U-1 | CA-D-3SM |  |
| 638 | ___in inserting valve and valve box | each | U-1 | CA-D-3SM |  |
| 638 | ___in cutting-in sleeve, valve, and box | each | U-1 | CA-D-3SM |  |
| 638 | __in x __in tapping sleeve, valve & box | each | U-1 | CA-D-3SM |  |
| 638 | meter, setting, stop and chamber | each | U-1 | CA-D-3SM |  |
| 638 | fire hydrant extended and adjusted | each | U-1 | CA-D-3SM |  |
| 638 | fire hydrant adjusted to grade | each | U-1 | CA-D-3SM |  |
| 638 | fire hydrant removed and reset | each | U-1 | CA-D-3SM |  |
| 638 | fire hydrant and gate valve removed and reset | each | U-1 | CA-D-3SM |  |
| 638 | fire hydrant removed and disposed of | each | U-1 | CA-D-3SM |  |
| 638 | valve box adjusted to grade | each | U-1 | CA-D-3SM |  |
| 638 | service box adjusted to grade | each | U-1 | CA-D-3SM |  |
| 638 | meter and chamber removed and reset | each | U-1 | CA-D-3SM |  |
| 638 | sheeting and bracing left in place | 0.1 yd3 | V-2 | CA-D-1A/1B |  |

Pavement Marking (640)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 642 | Traffic Paint |  |  |  |  |
| 642 | edge line, type____ | 0.01 mile | L-6 | CA-D-3SM |  |
| 642 | lane line, type _____ | 0.01 mile | L-6 | CA-D-3SM |  |
| 642 | center line, type ___ | 0.01 mile | L-6 | CA-D-3SM |  |
| 642 | channelizing line, type _____ | 1 foot | L-1 | CA-D-3SM |  |
| 642 | stop line, type _____ | 1 foot | L-1 | CA-D-3SM |  |
| 642 | crosswalk line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 642 | transverse line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 642 | curb marking, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 642 | island marking, type _____ | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 642 | handicap symbol marking, type _____ | each | U-1 | CA-D-3SM |  |
| 642 | railroad symbol marking, type _____ | each | U-1 | CA-D-3SM |  |
| 642 | school symbol marking, __in, type ____ | each | U-1 | CA-D-3SM |  |
| 642 | parking lot stall marking, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 642 | lane arrow, type _____ | each | U-1 | CA-D-3SM |  |
| 642 | word on pavement, ____in, type _____ | each | U-1 | CA-D-3SM |  |
| 642 | dotted line, ___in, type _____ | 1 foot | L-1 | CA-D-3SM |  |
| 642 | removal of pavement marking | 1 foot 0.1 yd2 | L-1 A-1 | CA-D-3SM CA-D-1A/1B |  |
| 642 | two-way radio equipment | lump sum | LS-1 | CA-D-3SM |  |
| 643 | Polyester Pavement Marking |  |  |  |  |
| 643 | edge line | 0.01 mile | L-6 | CA-D-3SM |  |
| 643 | lane line | 0.01 mile | L-6 | CA-D-3SM |  |
| 643 | center line | 0.01 mile | L-6 | CA-D-3SM |  |
| 643 | channelizing line | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | stop line | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | crosswalk line | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | transverse line | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | curb marking | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | island marking | 0.1 yd2 | A-1 | CA-D-3SM |  |
| 643 | handicap symbol marking | each | U-1 | CA-D-3SM |  |
| 643 | railroad symbol marking | each | U-1 | CA-D-3SM |  |
| 643 | school symbol marking, ____in | each | U-1 | CA-D-3SM |  |
| 643 | parking lot stall marking | 0.1 foot | L-1 | CA-D-3SM |  |
| 643 | lane arrow | each | U-1 | CA-D-3SM |  |
| 643 | word on pavement ____in | each | U-1 | CA-D-3SM |  |
| 643 | dotted line, ___in | 1 foot | L-1 | CA-D-3SM |  |
| 643 | removal of pavement marking | 1 foot 0.1 yd2 | L-1 A-1 | CA-D-3SM CA-D-1A/1B |  |
| 643 | two-way radio equipment | lump sum | LS-1 | CA-D-3SM |  |
| 644 | Thermoplastic Pavement Marking |  |  |  |  |
| 644 | edge line | 0.01 mile | L-6 | CA-D-3SM |  |
| 644 | lane line | 0.01 mile | L-6 | CA-D-3SM |  |
| 644 | center line | 0.01 mile | L-6 | CA-D-3SM |  |
| 644 | channelizing line | 0.1 foot | L-1 | CA-D-3SM |  |
| 644 | stop line | 0.1 foot | L-1 | CA-D-3SM |  |
| 644 | crosswalk line | 0.1 foot | L-1 | CA-D-3SM |  |
| 644 | transverse line | 1 foot | L-1 | CA-D-3SM |  |
| 644 | handicap symbol marking | each | U-1 | CA-D-3SM |  |
| 644 | railroad symbol marking, ____in | each | U-1 | CA-D-3SM |  |
| 644 | school symbol marking, ____in | each | U-1 | CA-D-3SM |  |
| 644 | parking lot stall marking | 0.1 foot | L-1 | CA-D-3SM |  |
| 644 | lane arrow | each | U-1 | CA-D-3SM |  |
| 644 | word on pavement, ____in | each | U-1 | CA-D-3SM |  |
| 644 | dotted line, _____in | 1 foot | L-1 | CA-D-3SM |  |
| 644 | removal of pavement marking | 1 foot 0.1 yd2 | L-1 A-1 | CA-D-3SM CA-D-1A/1B |  |
| 644 | two-way radio equipment | lump sum | LS-1 | CA-D-3SM |  |
| 645 | Preformed Pavement Marking |  |  |  |  |
| 645 | edge line, type_____ | 0.01 mile | L-6 | CA-D-3SM |  |
| 645 | lane line, type_____ | 0.01 mile | L-6 | CA-D-3SM |  |
| 645 | center line, type _____ | 0.01 mile | L-6 | CA-D-3SM |  |
| 645 | channelizing line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 645 | stop line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 645 | crosswalk line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 645 | transverse line, type _____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 645 | handicap symbol marking, type _____ | each | U-1 | CA-D-3SM |  |
| 645 | railroad symbol marking, type _____ | each | U-1 | CA-D-3SM |  |
| 645 | school symbol marking, ___in, type ___ | each | U-1 | CA-D-3SM |  |
| 645 | parking lot stall marking, type_____ | 0.1 foot | L-1 | CA-D-3SM |  |
| 645 | lane arrow, type _____ | each | U-1 | CA-D-3SM |  |
| 645 | word on pavement, ___in , type ___ | each | U-1 | CA-D-3SM |  |
| 645 | dotted line, _____in, type _____ | 1 foot | L-1 | CA-D-3SM |  |
| 645 | removal of pavement markings | 1 foot 0.1 yd2 | L-1 A-1 | CA-D-3SM CA-D-1A/1B |  |
| 645 | two-way radio equipment | lump sum | LS-1 | CA-D-3SM |  |
| 646 | Epoxy Pavement Marking |  |  |  |  |
| 646 | epoxy pavement markings | see 642 above |  |  |  |
| 647 | Heat-Fused Preformed Plastic Pavement Marking |  |  |  |  |
| 647 | heat-fused preformed plastic pavement markings | see 642 above |  |  |  |

Roadsides (650)

| I T E M | DESCRIPTION | UNITS Required Accuracy Daily Entries | C O D E | F O R M | TKTS |
| --- | --- | --- | --- | --- | --- |
| 651 | Topsoil Stockpiled |  |  |  |  |
| 651 | topsoil stockpiled | 1 yd3 | V-3 | CA-D-1A/1B |  |
| 652 | Placing Stockpiled Topsoil |  |  |  |  |
| 652 | placing stockpiled topsoil | 1 yd3 | V-5 | CA-D-1A/1B |  |
| 653 | Topsoil Furnished and Placed |  |  |  |  |
| 653 | topsoil furnished and placed | 1 yd3 | V-5 | CA-D-1A/1B |  |
| 654 | Renovating Existing Soil |  |  |  |  |
| 654 | renovation existing sod | 1 ft2 | A-1 | CA-D-1A/1B |  |
| 654 | commercial fertilizer | 0.01 ton | W-4 | CA-D-3SM |  |
| 656 | Roadside Cleanup |  |  |  |  |
| 656 | roadside cleanup | 1 ft2 | A-1 | CA-D-1A/1B |  |
| 657 | Riprap for Tree Protection |  |  |  |  |
| 657 | riprap for tree protection | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 658 | Tree Root Aeration |  |  |  |  |
| 658 | tree root aeration | 1 yd3 | V-5 | CA-D-1A/1B |  |
| 659 | Seeding and Mulching |  |  |  |  |
| 659 | Commercial fertilizer | 0.01 ton | W-4 | CA-D-3SM |  |
| 659 | Agricultural liming | 0.01 ton | W-4 | CA-D-3SM |  |
| 659 | seeding and mulching | 1 yd2 | A-4 | CA-D-1A/1B |  |
| 659 | seeding and mulching for wildlife | 1 yd2 | A-4 | CA-D-1A/1B |  |
| 659 | repair seeding and mulching | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 659 | Water | 1 m gal | V-7 | CA-D-3SM | X |
| 659 | Mowing | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 660 | Sodding |  |  |  |  |
| 660 | sodding unstaked | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 660 | sodding staked | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 660 | sodding reinforced | 1 yd2 | A-1 | CA-D-1A/1B |  |
| 661 | Planting Trees, Shrubs, and Vines |  |  |  |  |
| 661 | tree seedling, (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | perennials, (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | ground cover , (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | deciduous shrub , (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | evergreen shrub, (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | deciduous tree, (size), (species) | each | U-2 | CA-D-3SM |  |
| 661 | evergreen tree, (size), (species) | each | U-2 | CA-D-3SM |  |
| 662 | Landscape Watering |  |  |  |  |
| 662 | landscape watering | gallon | V-7 | CA-D-3SM | X |
| 666 | Pruning Existing Trees |  |  |  |  |
| 666 | pruning existing trees __in to __in dia. | each | U-2 | CA-D-3SM |  |
| 670 | Erosion Protection |  |  |  |  |
| 670 | slope erosion control | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 670 | ditch erosion control | 0.1 yd2 | A-1 | CA-D-1A/1B |  |
| 671 | Temporary Erosion Control Mats |  |  |  |  |
| 671 | seeding and paper blanket | 0.1 yd2 | A-1 | CA-D-1A/1B |  |

List of Forms

CA-S-1  Inspection Record for Drilled Shafts	29

CA-S-1  Inspection Record for Drilled Shafts (back)	30

CA-S-2  Paint Thickness (QCP #5, #8, #10)	31

CA-S-2  Paint Thickness  (QCP #5, #8, #10) (back)	32

CA-S-3  (BR-2-75) Pile Driving Log	33

CA-S-3  (BR-2-75) Pile Driving Log (back)	34

CA-S-3A  (BR-2-75A) Pile Driving Log Summary	35

CA-S-4  High Performance Concrete Pre-Pour Meeting   -  1/3	37

CA-S-4  High Performance Concrete Pre-Pour Meeting   -  2/3	38

CA-S-4  High Performance Concrete Pre-Pour Meeting  -  3/3	39

CA-S-4  High Performance Concrete Pre-Pour Meeting  -  3/3 (back)	40

CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  1/3	41

CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  2/3	42

CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  3/3	43

CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  3/3 (back)	44

CA-S-6  Superstructure Concrete Pre-Pour Meeting  -  1/2	45

CA-S-6  Superstructure Concrete Pre-Pour Meeting  -  2/2	46

CA-S-7  QCS Inspection Documentation	47

CA-S-7  QCS Inspection Documentation (back)	48

CA-S-8  (BR-5) Piling Record	49

CA-S-8  (BR-5) Piling Record (back)	50

CA-S-11  QCS & Visual Standards Information	51

CA-S-11  QCS & Visual Standards Information (back)	52

CA-S-12  Bridge Painting Quality Control Points (QCP #1 & #2)	53

CA-S-12  Bridge Painting Quality Control Points (QCP #1 & #2) (back)	54

CA-S-13  Abrasive Blasting (QCP#3)	55

CA-S-13  Abrasive Blasting (QCP#3) (back)	56

CA-S-14  Disposal of Hazardous / Non- Hazardous Waste for Bridge Painting (QCP#4)	57

CA-S-14  Disposal of Hazardous / Non-Hazardous Waste for Bridge Painting (back)	58

CA-S-15  Prime Coat Application (QCP#5)	59

CA-S-15  Prime Coat Application (QCP#5) (back)	60

CA-S-16  Bridge Painting: Grinding Fins, Tears, and Slivers; and Caulking (QCP #6, #9)	61

CA-S-16  Bridge Painting: Grinding Fins, Tears, and Slivers and Caulking (QCP #6, #9) (back)	62

CA-S-17  Intermediate & Finish Coat Application (QCP #8 & #10)	63

CA-S-17  Intermediate & Finish Coat Application (QCP #8 & #10) (back)	64

CA-S-18   Bridge Painting Destructive Test Log (QCP #11)	65

CA-S-18   Bridge Painting Destructive Test Log (QCP #11) (back)	66

CA-S-19   Bridge Painting Final Review (QCP #11)	67

CA-S-19   Bridge Painting Final Review (QCP #11) (back)	68

CA-S-20   Erection (Demolition) Procedure Checklist	69

CA-S-20   Erection (Demolition) Procedure Checklist (back)	69

CA-S-21   Sealing of Concrete Surfaces Checklist	71

CA-S-21   Sealing of Concrete Surfaces Checklist (back)	71

CA-S-22   Bridge Deck Concrete Placement Dry Run Form (Outline)	73

CA-S-22   Bridge Deck Concrete Placement Dry Run Form (Back)	73

CA-S-30   Reinforcing Steel Verification	75

CA-S-30   Reinforcing Steel Verification (back)	76

CA-S-31   Air Tests for Superstructures	77

CA-S-31   Air Tests for Superstructures (back)	78

CA-S-32   Structure Temperature Records	79

CA-S-32   Structure Temperature Records (back)	80

CA-C-1   Concrete Control Test Form	81

CA-C-1   Concrete Control Test Form (back)	82

CA-C-2   QC/QA Concrete QCP Checklist – 1/4	83

CA-C-2   QC/QA Concrete QCP Checklist - 2/4	84

CA-C-2   QC/QA Concrete QCP Checklist - 3/4	85

CA-C-2   QC/QA Concrete QCP Checklist - 4/4	86

CA-C-2   QC/QA Concrete QCP Plan Activity Log - 1/2	87

CA-C-2   QC/QA Concrete QCP Plan Activity Log- 2/2	88

CA-EW-1   Earthwork Quantity Calculations	89

CA-EW-1   Earthwork Quantity Calculations (back)	89

CA-EW-2   Proof Rolling Documentation	91

CA-EW-2   Proof Rolling Documentation (back)	92

CA-EW-3   Log of Test Pit Investigation	93

CA-EW-3   Log of Test Pit Investigation (back)	93

CA-EW-4   Moisture Density Curve Calculation	95

CA-EW-4   Moisture Density Curve Calculation (back)	96

CA-EW-6   Nuclear Gauge Compaction with Aggregate Correction	99

CA-EW-6   Nuclear Gauge Compaction with Aggregate Correction	(back)	100

Zero Air Voids Curve	102

CA-EW-7   Non-Stabilized Drainage Base Compaction	103

CA-EW-7   Non-Stabilized Drainage Base Compaction (back)	104

CA-EW-8   Authorization of Undercuts	105

CA-EW-8   Authorization of Undercuts (back)	106

CA-EW-9   Rock Blasting Inspection Form	107

CA-EW-9   Rock Blasting Inspection Form (back)	108

CA-EW-10   Rock Blasting Drilling Log	109

CA-EW-10   Rock Blasting Drilling Log (back)	110

CA-EW-11   Blast Site Security Plan	111

CA-EW-11   Blast Site Security Plan (back)	112

CA-EW-12   Daily Earthwork Inspection Form (back)	114

CA-EC-1   Weekly and Rain Event Checklist	115

CA-EC-1   Weekly and Rain Event Checklist (back)	116

CA-EC-2   Seeding Calculations	117

CA-EC-2   Seeding Calculations (back)	117

CA-FP-1   Warranty Asphalt Checklist	119

CA-FP-1   Warranty Asphalt Checklist (back)	120

CA-FP-2   Random Selection of Asphalt Field Samples (448, 403)	121

CA-FP-2   Random Selection of Asphalt Field Samples (448, 403) (back)	122

CA-FP-3   Summary of Asphalt Concrete Quantities	123

CA-FP-3   Summary of Asphalt Concrete Quantities (back)	124

CA-FP-4   Asphalt Concrete inspection	125

CA-FP-5   Roller Capacity and Placement Rate	127

CA-FP-6   Calculation of Liquid Asphalt Materials	129

CA-FP-6   Calculation of Liquid Asphalt Materials (back)	130

TE-217   Bituminous Concrete Density Determination - 446 Cores	131

TE-217   Bituminous Concrete Density Determination - 446 Cores	132

TE-217   Bituminous Concrete Density Determination - 446 Form Instructions	133

TE-217   Bituminous Concrete Density Determination - 446 Cores Instructions	134

TE-217   Bituminous Concrete Density Determination - 446 Random Number Table	135

TE-217   Bituminous Concrete Density Determination	136

CA-P-1(603)   Pipe Construction Inspection Form  -  1/2	137

CA-P-1(603)   Pipe Construction Inspection Form  -  2/2	137

CA-P-2   Underdrain Construction Inspection Form  -  1/2	139

CA-P-2   Underdrain Construction Inspection Form  -  2/2	139

CA-P-3(604)   Drainage Structure Inspection Form  -  1/2	141

CA-P-3(604)   Drainage Structure Inspection Form  -  2/2	141

CA-P-3(611)    Drainage Structure Inspection Form  -  1/2	143

CA-P-3(611)   Drainage Structure Inspection Form  -  2/2	144

CA-P-1(611)   Pipe Construction Inspection Form  -  1/2	145

CA-P-1(611)    Pipe Construction Inspection Form  -  2/2	145

CA-D-1A   Field Calculation and Measurement	147

CA-D-1A   Field Calculation and Measurement (back)	148

CA-D-1B   Field Calculation and Measurement	149

CA-D-1B    Field Calculation and Measurement (back)	150

CA-D-2   Field Calculation and Measurement	151

CA-D-2   Field Calculation and Measurement (back)	152

CA-D-3SM-A   ODOT Inspectors Daily Report	153

CA-D-3SM-A   ODOT Inspectors Daily Report (back)	154

CA-D-4SM   ODOT P.E. / P.S. Daily Work Report	155

CA-D-5   Daily Account of Force Account Work	157

CA-D-5   Daily Account of Force Account Work (back)	158

CA-D-6  Pavement Repair and Sawing Measurement	159

CA-D-6  Pavement Repair and Sawing Measurement (back)	160

CA-D-7  Short Term Work Zone Review	161

CA-D-7  Short Term Work Zone Review (back)	162

CA-D-8  Long Term Work Zone Review  -  1/2	163

CA-D-8   Long Term Work Zone Review  -  2/2	164

CA-D-9   Daily Concrete Pavement Documentation Form   - 1/5	165

CA- D-9  Daily Concrete Pavement Documentation Form   -  2/5	166

CA- D-9  Daily Concrete Pavement Documentation Form   -  3/5	167

CA- D-9  Daily Concrete Pavement Documentation Form   -  4/5	168

CA- D-9  Daily Concrete Pavement Documentation Form   -  5/5	169

CA- D-9  Daily Concrete Pavement Documentation Form  -  (Back)	170

CA-L-1   Report of Electrical Tests	171

CA-L-1   Report of Electrical Tests  (back)	172

CA-L-2   Report of Electrical Tests	173

CA-L-2  Report of Electrical Tests (back)	174

CA-L-3   Report of Electrical Tests	175

CA-L-3   Report of Electrical Tests   (back)	176

CA-L-4   Report on Sign Lighting	177

CA-L-4   Report on Sign Lighting  (back)	178

CA-L-5   Report on High Voltage Direct Current Tests	179

CA-L-5   Report on High Voltage Direct Current Tests (back)	179

CA-T-1   DLS Report Format  -  Weight-Based System	181

CA-T-2  DLS Short Report Format  -  Weight-Based System	182

CA-T-3  DLS Report Format   -  Stroke Counter System	183

CA-T-4  DLS Short Report Format  -  Stroke Counter System	184

CA-T-5  DLS Report Format  -  Flow Meter Based System	185

CA-T-6  DLS Short Report Format  -  Flow Meter Based System	186

CA-T-7  DLS Report Format  -  ThermoPlastic System	187

CA-T-8  DLS Short Report Format – ThermoPlastic System	188

TE-31   Sample Data	189

TE-31   Sample Data   (back)	190

Noise Barrier Wall Shop Drawing Review Checklist	191

Noise Barrier Wall Shop Drawing Review Checklist  -  (Back)	192

CA-G-1   Preconstruction Meeting Agenda / Checklist   -  1/3	193

CA-G-1  Preconstruction Meeting Agenda / Checklist  -  2/3	194

CA-G-1  Preconstruction Meeting Agenda / Checklist  -  3/3	195

CA-G-1  Preconstruction Meeting Agenda / Checklist  -  3/3  back	196

#### CA-S-1  Inspection Record for Drilled Shafts

| Project No: | Bridge No: | SFN: |
| --- | --- | --- |
| Drilling Contractor: | Project Engineer: | Date: |
| Type of Equipment: | Inspector: | Item No: |
| Max Continuous Torque (FtLb)(Nm): |  | Cost Above Rock($/LF): |
| CROWD (Max cont. Downward Force )-(Lbs)(N): |  | Cost in Rock($/LF): |
| Cost of Concrete pumping($/LF): |  | Type of Rock: |

| Drilled Shaft Number: |  |  |  | Units |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date & Time of Drilling |  | Started |  | Date |  |  |  |  |  |
|  |  |  |  | Time |  |  |  |  |  |
|  |  | Finished |  | Date |  |  |  |  |  |
|  |  |  |  | Time |  |  |  |  |  |
| Elevation of Top of Overburden |  |  |  | Ft (m) |  |  |  |  |  |
| Length of Drilled Shafts above Bedrock Socket |  | Through air |  | Ft (m) |  |  |  |  |  |
|  |  | Through Overburden |  | Ft (m) |  |  |  |  |  |
|  |  | Pay Length |  | Ft (m) |  |  |  |  |  |
| Obstructions Encountered |  | Number |  | - |  |  |  |  |  |
|  |  | Size |  | Ft (m) |  |  |  |  |  |
|  |  | Time of Removal |  | Hours |  |  |  |  |  |
| Length of Drilled Shaft in the Rock Socket |  | Elev. Top of Socket |  | Ft (m) |  |  |  |  |  |
|  |  | Elev. Bottom of Socket |  | Ft (m) |  |  |  |  |  |
|  |  | Length of Socket |  | Ft (m) |  |  |  |  |  |
| Steel Casing |  | Casing Thickness |  | In(mm) |  |  |  |  |  |
|  |  | Casing Left in Place |  | Ft (m) |  |  |  |  |  |
| Reinforcing Steel | Vertical | Bar Size Number |  | --- |  |  |  |  |  |
|  |  | No. of Bars |  | --- |  |  |  |  |  |
|  | Spiral | Bar Size Number |  | --- |  |  |  |  |  |
|  |  | Pitch |  | In(mm) |  |  |  |  |  |
| Concrete |  | Slump |  | In(mm) |  |  |  |  |  |
|  |  | Cylinder Strength |  | PSI(MPa) |  |  |  |  |  |
|  |  | Air Temp |  | F(C) |  |  |  |  |  |
|  |  | Time to Place Concrete |  | Hr |  |  |  |  |  |
| Tolerances | Deviations from Plumb |  | N-S | In(mm) |  |  |  |  |  |
|  |  |  | E-W | In(mm) |  |  |  |  |  |
|  | Deviations of column top center from plan |  |  | In(mm) |  |  |  |  |  |
| Plan Shaft Diameter (Bedrock / Overburden) |  |  |  | In(mm) |  |  |  |  |  |
| Constructed Diameter (Bedrock / Overburden) |  |  |  | In(mm) |  |  |  |  |  |

#### CA-S-1  Inspection Record for Drilled Shafts (back)

#### CA-S-2  Paint Thickness (QCP #5, #8, #10)

#### CA-S-2  Paint Thickness  (QCP #5, #8, #10) (back)

#### CA-S-3  (BR-2-75) Pile Driving Log

| Project No: | County/Route/Section: |
| --- | --- |
| Bridge No: | Object Bridge Over: |
| SFN: | Date: |

| Substructure Unit: | Date Driven: | Pile Number: |
| --- | --- | --- |
| Pile Type: | Wall Thickness: | Hammer: |
| Required Ultimate Bearing: | Batter: | Drop Hammer Ram Weight: |
| Cutoff Elevation: | Ground Elevation: | No. of Splices: |

| Penetration | Blows /Ft | Stroke or Pressure | Penetration | Blows/Ft | Stroke or Pressure | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Inspectors Name: |  |  |  | Signature: |  |  |

#### CA-S-3  (BR-2-75) Pile Driving Log (back)

#### CA-S-3A  (BR-2-75A) Pile Driving Log Summary

CA-S-3A  (BR-2-75A) Pile Driving Log Summary (back)

#### CA-S-4  High Performance Concrete Pre-Pour Meeting   -  1/3

PROJECT:  ________________________________ 	DATE:  ___________________

LOCATION:  _______________________________    	TIME:   ___________________

PRESENT: ________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

DRY RUN:

‒ Check height of rail over enddams at all four corners - *Equal heights for left rear and fwd and for right rear and fwd

‒ Check any change in cross-slope and discuss method of performance

–     Confirm appropriate depth of roller fins

–     Check reinforcing steel clearances

TIME AND LOCATION OF POUR:

‒ Date: __________________    Time:__________________

‒ Pour to begin on __________________     side and commence on the __________________     side

‒ Location of pumps, if used

‒ Lighting plan if at night

PLAN AND SPECIFICATION REVIEW:

‒ Need a written statement, from the admixture supplier verifying the compatibility of materials, a mix design, and a batching sequence

‒ Need successful test pour prior to concrete placement

‒ A technical representative is required during concrete placement

‒ Calendar restrictions for flyash are waived

‒ Need verification of vibration frequencies on finishing machine

‒ No rain to be forecasted during placement; if rain occurs, operations cease

‒ Surface evaporation rate must be equal to or less than 0.1 pound per square foot per hour DURING the pour as determined and documented by the contractor

‒ Vibrators shall be used

‒ Mix characteristics shall be adjusted off the deck before placement

‒ Maximum mix temperature is 90 degrees

‒ 7 day water cure: 1 layer of burlap with continuous water covered with plastic sheeting, apply membrane cure as per 511.19 method (b) within 12 hours of burlap removal

‒ Prior to opening to traffic, check top and bottom for cracks and, if necessary, reseal from the top

‒ Can open to traffic after membrane cure is applied, unless between October 15 and March 30, then must wait 30 days

–    Seal joints with HMWM

–    Re-apply membrane cure after grooving, unless concrete is older than 30 days

#### CA-S-4  High Performance Concrete Pre-Pour Meeting   -  2/3

PROJECT:  __________________________________________  	DATE:  ___________________

CONCRETE PLANT:

‒ Presoak limestone 24 hours and allow to drain overnight   -   *Don’t charge bins with dry material

‒ Batching to begin at   _____________

‒ Expel all water from the mixers before loading / reloading (especially important for micro-silica mixes)

‒ Dissolvable micro-silica bags are not allowed

‒ A slow ribbon feed of cement is required

‒ Mix 5 minutes at plant

‒ Pour consists of approximately  _____________   C.Y.

‒ Time between trucks  _____________  minutes

‒ Pour _____________  cy./hr.

‒ Radio communication with plant for mix design changes

‒ Haul distance:  _____________ minutes

QUALITY CONTROL

‒ High Performance Concrete

‒ Mix no.  _____________

‒ Proportioning and batching changes are contractor’s responsibility

‒ W/C max = 0.40, (mix 3 and 4), based on total cementitious material

‒ Maximum slump = 8 inches

‒ Air 7% ± 2

‒ Flyash, unless use slag, shall be type C

‒ Type F or G admixture is required for workability; suggest using type A for finishing advantages

‒ Any admixtures added at the jobsite shall be mixed a minimum of 5 minutes

‒ Air and slump tests are to be taken at point of placement

‒ Discharge time = 90 minutes

–    No “balling” of mix, if balling occurs load will be rejected and mixing process revised

CONTRACTOR’S OPERATION

‒ Tools

*  broom finish

*  vibrating pan or rollers

*  straightedge

*  presoaked burlap

*  plastic for bad weather protection

*  extra vibrator

#### CA-S-4  High Performance Concrete Pre-Pour Meeting  -  3/3

PROJECT:  _____________________________________________  	DATE:  ___________________

CONTRACTOR’S OPERATION (Continued)

‒ Concrete Placement

*  wet deck continuously in front of placement

*  place  plastic sheeting to avoid deck contamination

*  straightedge checking at bulkheads and enddams

*  need 2 walk-bridges (1 for finishers and 1 for burlap)

*  smooth finish along curbs and/or parapets only

*  burlap shall follow as close as possible to deck overlay

PROJECT INSPECTION	Personnel   (If available)

‒ Moisture tests to be run at _____________

‒ 1 Plant Inspector

*  Checking aggregate moistures

*  checking revolutions

*  checking no wash water, no wash down water after batching

‒ 2 Concrete Quality Control Testers  __________________________

*  chase air on every truck	 _____________

*  Air, slump, yield

*  Cylinders made every 200 CY.

*  various pump, if used, configuration correlation tests

*  make beams

*  check concrete temperature

‒ 1 Concrete Quality Control Inspector  __________________________

*  documenting any added materials

*  check batch tickets

*  time from start to discharge

*  insuring 5 minute mixing time if materials added

*  documenting test results

*  fill out TE-45

‒ 1 Placement Inspector	__________________________

*  make depth checks

*  make reinforcing steel depth checks

*  insure curing is placed ASAP

MISCELLANEOUS

‒ Traffic Control

#### CA-S-4  High Performance Concrete Pre-Pour Meeting  -  3/3 (back)

#### CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  1/3

PROJECT:  ____________________________________________________  	DATE:  ___________________

LOCATION:  __________________________________________________      	TIME:   ___________________

PRESENT: ___________________________________________________________________________________

______________________________________________________________________________________________

______________________________________________________________________________________________

DRY RUN:

‒ Check height of rail over enddams at all four corners - *Equal heights for left rear and fwd and for right rear and fwd

‒ Check any change in cross-slope and discuss method of performance

–     Confirm appropriate depth of roller fins

TIME AND LOCATION OF POUR:

‒ Date: __________________    Time:__________________

‒ Pour to begin on __________________     side and commence on the __________________     side

‒ Lighting plan if at night

PLAN AND SPECIFICATION REVIEW:

‒  The Contractor shall obtain a written statement from the manufacturer of the micro-silica admixture stating that he is satisfied with the compatibility of the combination of materials and the sequence in which they are combined.

‒ At the option of the laboratory, a trial batch may be made (4) four days prior to overlay

‒ Manufacturer representative shall be present during overlay

‒ No micro-silica overlays after October 15

‒ No rain to be forecasted during placement; if rain occurs, operations cease

‒ Air temperature must be below 85 degrees and not predicted to go over during concrete placement

* Must be a minimum of 45 degrees during placement and curing period

‒ Surface evaporation rate must be equal to or less than 0.1 pound per square foot per hour as determined and documented by the contractor

‒ Wet deck prior to concrete placement but no ponding

‒ Mix characteristics shall be adjusted off the deck before placement of the overlay

‒ Maximum mix temperature is 90 degrees

‒ Vibrators shall be used in variable depth areas, edges and along enddams or bulkheads

‒ During short delays, overlay and grout shall be covered with wet burlap

*Excess delays require bulkheads

‒ 3 day water cure:  1 layer of burlap with continuous water covered with plastic sheeting

‒ Contractor to stencil date of construction and the letters MS into the overlay

‒ Deck to be sounded after overlay to insure no delaminated areas

#### CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  2/3

PROJECT:  ____________________________________________  	DATE:  ___________________

CONCRETE PLANT:

‒ Presoak limestone 24 hours and allow to drain overnight 	*Don’t charge bins with dry material

‒ Batching to begin at  ________________

‒ Expel all water from the mixers before loading / reloading

‒ Add super on jobsite if long haul distance

‒ Minimum of 70 revolutions on mixers before leaving plant unless central mixed

‒ Pour consists of approximately  ________________ CY

‒ Time between trucks  ________________ minutes

‒ Pour   ________________ cy./hr.

‒ Radio communication with plant for mix design changes

‒ Haul distance:   ________________  minutes

QUALITY CONTROL

‒ Modified micro-silica concrete

‒ W/C max = 0.36

‒ Slump 6 inches ± 2

‒ Air 8% ± 2

‒ Type A or D and F admixture shall be used

‒ Transit mixers are limited to carrying ¾ of its rated capacity or 6 cy whichever is less

‒ Any admixtures added at the jobsite shall be mixed a minimum of 5 minutes

‒ Discharge time = 90 minutes

CONTRACTOR’S OPERATION

‒ Tools

*  broom finish,  * vibrating pan or roller, *  straightedge,  *  presoaked burlap

*  plastic for bad weather protection,  *  extra vibrator

‒ Concrete Placement

*  abrasive or water blast deck within 24 hours of overlay

*  air blast deck immediately before overlay

*  place  plastic sheeting to avoid deck contamination

*  straightedge checking at bulkheads and enddams

*  need 2 walk-bridges (1 for finishers and 1 for burlap)

*  texture deck with tining tool (0.15" deep)

*  smooth finish along curbs and/or parapets only

*  burlap shall follow as close as possible to deck overlay

#### CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  3/3

PROJECT:  ________________________________________  	DATE:  ___________________

PROJECT INSPECTION	Personnel (If available)

‒ Moisture tests to be run at  _______________  followed by design of the mix.

‒ 1 Plant Inspector   ____________________________

*  check moistures

*  checking revolutions

*  checking no wash water

‒ 2 Concrete Control Testers    _____________________________

*  full test on every truck	 ______________________________

* Cylinders as required every 50 CY

*  check concrete temperature

‒ 1 Concrete Quality Control Inspector  _____________________________

*  documenting any added materials

*  check batch tickets

*  time from start to discharge

*  insuring 5 minute mixing time if materials added

*  documenting test results

*  fill out TE-45

‒ 1 Placement Inspector  _____________________________

*  make depth checks

*  Ensure curing is placed ASAP

MISCELLANEOUS

‒ Traffic Control

#### CA-S-5  Micro-Silica Overlay Pre-Pour Meeting  -  3/3 (back)

#### CA-S-6  Superstructure Concrete Pre-Pour Meeting  -  1/2

PROJECT:  ____________________________________________________  	DATE:  ___________________

LOCATION:  ________________________________________________      	TIME:   ___________________

PRESENT: _______________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

DRY RUN: _________________

‒ Check height of rail over enddams at all four corners - *Equal heights for left rear and fwd and for right rear and fwd

‒ Check any change in cross-slope and discuss method of performance

–     Confirm appropriate depth of roller fins

–     Check reinforcing steel clearances

TIME AND LOCATION OF POUR:

‒ Date: __________________    Time:__________________

‒ Pour to begin on __________________     side and commence on the __________________     side

‒ Location of pumps, if used

‒ Lighting plan if at night

PLAN AND SPECIFICATION REVIEW:

‒ Air temperature must be below 85o and not predicted to go over during concrete placement

‒ Surface evaporation rate must be equal to or less than 0.2 lbs/sq. ft. per hour

‒ Must water cure:  2 layers of burlap with continuous water, or plastic coated/covered burlap with positive moisture seal.

–     Cure (5) five days with beam test

CONCRETE PLANT:

‒ Presoak limestone 24 hours and allow to drain overnight

*Don’t charge bins with dry material

‒ Batching to begin at  __________________

‒ Expel all water from the mixers before loading / reloading

‒ Add super on jobsite if long haul distance

‒ Minimum of 70 revolutions on mixers before leaving plant unless central mixed

‒ Pour consists of approximately __________________  C.Y.

‒ Time between trucks __________________  minutes

‒ Pour __________________   cy./hr.

‒ Radio communication with plant for mix design changes

‒ Haul distance:__________________   minutes

#### CA-S-6  Superstructure Concrete Pre-Pour Meeting  -  2/2

ROJECT:  ____________________________________________  	DATE:  ___________________

QUALITY CONTROL  (Air and Slump tests to be checked at the point of Placement)

‒ Class QC2 or QC 3 Concrete	‒ Slump 2-4 inches (6" nom. And 7" max. with addition of type F or G admixture)  ‒ W/C max = 0.44	‒ Type B or D admixture (retarder) to be used if over 60o F.

‒ Air 6% ± 2

‒ Discharge time = 60 minutes (90 minutes with retarder)

CONTRACTOR’S OPERATION

‒ Tools

*  broom finish,   *  straightedge,     *  wet burlap,   *  plastic for bad weather protection,   *  extra vibrator,

*  weather gauges

‒ Concrete Placement

*  air blast deck before pour

*  wet forms before placement

*  straightedge checking at bulkheads and enddams

*  need 2 walk-bridges (1 for finishers and 1 for burlap)

*  Groove as per 511.20

*  smooth finish along curbs and/or parapets only

*  wet burlap shall follow immediately after finishing

PROJECT INSPECTION	Personnel (if available)

‒ Moisture tests to be run at  ______________________________________

followed by design of the mix.

‒ 1 Plant Inspector:	 ______________________________________

*  writing tickets,   *  checking revolutions,    *  checking no wash water

‒ 2 Concrete Control Testers  ______________________________________

*  chase air on every truck,  *  various pump, if used, configuration correlation tests

*  make beams,  *  check concrete temperature

‒ 1 Concrete Quality Control Inspector  ______________________________________

*  documenting any added materials,    *  checking rev’s

*  documenting test results,  *  fill out TE-45

‒ 1 Placement Inspector  ______________________________________

*  make depth checks,  *  make reinforcing steel depth checks

*  insure curing is placed ASAP without marring surface

MISCELLANEOUS

‒	Traffic Control

#### CA-S-7  QCS Inspection Documentation

#### CA-S-7  QCS Inspection Documentation (back)

#### CA-S-8  (BR-5) Piling Record

| Project No: | County/Route/Section: |
| --- | --- |
| Bridge No: | Object Bridge Over: |
| SFN: | Date: |

| Substructure Unit: | Date Driven: | Hammer: |
| --- | --- | --- |
| Pile Type: | Required Resistance: |  |
| Required Ultimate Bearing: | Batter: | Drop Hammer Ram Weight: |
| Cutoff Elevation: | Ground Elevation: |  |

| Pile Number | Pile Length | Blows/Ft at end of Driving | Stroke or Pressure at end of Driving | Batter | No. of Splices | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Inspectors Name: |  |  |  | Signature: |  |  |

#### CA-S-8  (BR-5) Piling Record (back)

#### CA-S-11  QCS & Visual Standards Information

Diary Date _____________

Page _______ of ________

Quality Control Specialist

Names/Names  _____________________________________________________________________________

Formal Training by  _________________________________________________________________________

Dates of Training  ________________________________________________________________

Note: Obtain copy of training certificate for project records.

QCS listed on Office of Construction Administration Website?  ______________________________________

REMARKS:  ______________________________________________________________________________

__________________________________________________________________________________________

__________________________________________________________________________________________

Test Section of Abrasive Blasting  (Job Site Visual Standards)

Bridge Number _________________   Proj. Number  ____________________

Location  _________________________________________________________________________________

Test Section compared to SSPC Visual Standards?  ______________

Substrate Deduction _______________________________________

Surface Profile ___________________________________________

Test Section Approved? ___________________________________

Photos taken? ___________________________________________

Signature ___________________________________________     Date ____________________________

REMARKS: _______________________________________________________________________________

__________________________________________________________________________________________

__________________________________________________________________________________________

#### CA-S-11  QCS & Visual Standards Information (back)

#### CA-S-12  Bridge Painting Quality Control Points (QCP #1 & #2)

#### CA-S-12  Bridge Painting Quality Control Points (QCP #1 & #2) (back)

#### CA-S-13  Abrasive Blasting (QCP#3)

#### CA-S-13  Abrasive Blasting (QCP#3) (back)

#### CA-S-14  Disposal of Hazardous / Non- Hazardous Waste for Bridge Painting (QCP#4)

#### CA-S-14  Disposal of Hazardous / Non-Hazardous Waste for Bridge Painting (back)

#### CA-S-15  Prime Coat Application (QCP#5)

#### CA-S-15  Prime Coat Application (QCP#5) (back)

#### CA-S-16  Bridge Painting: Grinding Fins, Tears, and Slivers; and Caulking (QCP #6, #9)

#### CA-S-16  Bridge Painting: Grinding Fins, Tears, and Slivers and Caulking (QCP #6, #9) (back)

#### CA-S-17  Intermediate & Finish Coat Application (QCP #8 & #10)

#### CA-S-17  Intermediate & Finish Coat Application (QCP #8 & #10) (back)

#### CA-S-18   Bridge Painting Destructive Test Log (QCP #11)

#### CA-S-18   Bridge Painting Destructive Test Log (QCP #11) (back)

#### CA-S-19   Bridge Painting Final Review (QCP #11)

#### CA-S-19   Bridge Painting Final Review (QCP #11) (back)

#### CA-S-20   Erection (Demolition) Procedure Checklist

Name (print clearly):   ____________________________________________________________

Project:  _________ Bridge (Co, Rt, Section): _________________________________________

Description:  _____________________________________________________________________

Items Inspected:

Plan:

□ Contractor’s Erection (Demolition) Plan stamped by two PE’s.

□ Contractor’s Erection (Demolition) Plan accepted by regulatory agencies, Railroads, Coast Guard,

U.S. Army Corps of Engineers, etc., if applicable.

Details:

□ Erection (Demolition) sequence for all members.

□ Maintenance of Traffic during erection (demolition) operations.

□ Location of permanent support structures, roads, railroads, waterways, utilities.

□ Member delivery location and orientation for erection.

□ Member removal location and orientation for demolition.

□ Location and radius of each crane during each pick.

□ Location of crane support (barges, mats, etc.) , during each pick.

□ Crane capacities shown for each crane configuration and boom length used.

□ Lifting weights of primary member picks, including all rigging.

□ Rigging weights, capacities, and arrangements for primary member picks.

□ Locations of  the centers of gravity and lifting points for primary members.

□ Temporary supports or bracing.

□  Blocking for bridge bearings.

Calculations:

□ Load capacity and stability of crane(s), temporary supports and rigging for each pick and release.

□ Structural adequacy and stability of members for each erection (demolition) step.

Signature: _______________________________________ Date: ___________

CA-S-20   Erection (Demolition) Procedure Checklist (back)

#### CA-S-21   Sealing of Concrete Surfaces Checklist

Name (print clearly):   _________________________________________________________

Project:  _________ Bridge (Co, Rt, Section): ______________________________________

Description:  _________________________________________________________________

Items Inspected:

Storage:

□ Thermometer to check for manufacturer’s acceptable range

□ Sealer stored not frozen nor in sun

□ Shelf life

Submittals made:

□ Technical and application data

□ Material Safety Data Sheet

□ 100 Grit Sandpaper supplied by Contractor for comparison

Equipment for proper preparation:

□ Correct water blast equipment (7000 psi minimum)

□ Correct abrasive blast equipment (followed by cleaning)

Equipment mixing and application:

□ Equipment meets manufacturer’s requirements:

Surface Preparation, clean concrete with 100 grit sandpaper texture:

□ Test site set up

□ Test all surfaces, (not just vertical)

□ Surface checked for texture

□ Acid Test performed if curing compound used

Mixing:

□ Material used within manufacturer’s shelf life  □ Mix time and pot life marked

□ Mixed per manufacturer’s recommended procedures

Application:

□ Conditions acceptable

□ Material applied within pot life

□ Minimum coating rates met in test sections

- Epoxy – 120 Ft2/gal, (150 Ft2/gal if authorized)

- Urethane  - 200 Ft2/gal,(even coverage, no skips or sags)

- Non-Epoxy – 100 Ft2/gal, Decks, 125 Ft2/gal, Vert. Surfaces

Signature: ____________________________________________ Date: ____________

CA-S-21   Sealing of Concrete Surfaces Checklist (back)

#### CA-S-22   Bridge Deck Concrete Placement Dry Run Form (Outline)

Name (print clearly):   __________________________________________________

Project:  _________ Bridge (Co, Rt, Section): _______________________________

Description Side/Phase):________________________________________________

Record of measurements of Deck thicknesses, Reinforcing Steel clearances, under rollers of deck finishing machine during dry run.

Plan Deck Thickness________

- Check Frequency of Vibrating Pan or Roller

- Check Depth of Roller Fins

Use other forms of documentation as necessary

Signature:_____________________________  Date:_______________________

CA-S-22   Bridge Deck Concrete Placement Dry Run Form (Back)

#### CA-S-30   Reinforcing Steel Verification

#### CA-S-30   Reinforcing Steel Verification (back)

#### CA-S-31   Air Tests for Superstructures

#### CA-S-31   Air Tests for Superstructures (back)

#### CA-S-32   Structure Temperature Records

#### CA-S-32   Structure Temperature Records (back)

#### CA-C-1   Concrete Control Test Form

Project No.: ____________________________ Co./Rt./Sec.: ______________________________

Name: ________________________________   Date: ___________   Truck No.: ______________

Arrival Time: _______________________   Discharge Time: ______________________________

Placement Description:  ____________________________________________________________

Placement Location: _______________________________________________________________

Cylinder Sample Numbers: ________________________________________ Class  ____ Concrete

Full Pot Weight     =  (+) ___________________ lbs.

Empty Pot Weight =  (-) ___________________ lbs.

Concrete Weight   =       ___________________ lbs.

Air Pot Factor       =     ____________________

Weight of One Cubic Foot (Unit Weight) = ____________________  lbs / ft3

Batch Weight

Fine Aggregate Weight:  	________  lbs / _______  yd3  =  _________________  lbs

Coarse Aggregate Weight  	________  lbs / _______  yd3  =  _________________  lbs

Cement Weight		________  lbs / _______  yd3  =  _________________  lbs

Water Batch Weight		________  lbs / _______  yd3  =  _________________  lbs

Additional Weight		________  lbs / _______  yd3  =  _________________  lbs

One Cubic Yard Batch Weight    =   _________________  lbs

Yield Test Results

____________________ Cubic Feet

Air = ____ % ,  Slump = _____ inch ,  Air Temp. = ____ , Concrete Temp.= _____

#### CA-C-1   Concrete Control Test Form (back)

#### CA-C-2   QC/QA Concrete QCP Checklist – 1/4

Project No.: ____________________________ Co./Rt./Sec.: ______________________________

Contractor: ___________________________________________________________________

Submission Date: ______________   Original Approval Date: ______________

A.  Basic Information

|  | - The name and location of the Department inspected and approved concrete producer. |
| --- | --- |
|  | - The Department accepted Job Mix Formula (JMF) to be used for each item. |
|  | - The name and accreditation of the AMRL Accredited Laboratory to be used for testing the strength samples for structures or pavements. |
|  | - Name of the person(s) who is responsible for compliance with the QCP; acting as liaison to the Department; reporting any non-specification test results and assuring that adjustments are made to remedy problems. |
|  | - Names of all technicians who will perform plant or field site inspection, sampling and testing. Use ACI Grade I certified technicians to perform concrete sampling and testing. Provide copies of their ACI certificates. |
|  | - Calibration records of test equipment to be used on the project. |
|  | - Develop and provide example forms for reporting QC test results to the Engineer conforming to 455.03.C or 455.04.G. |

#### CA-C-2   QC/QA Concrete QCP Checklist - 2/4

B.  Minimum Quality Control at the Concrete Source

|  | - Verify that the material sources are certified for the type of work in which they are to be used. |
| --- | --- |
|  | - Verify that aggregates, cementitious materials and admixtures sources and design weights match the proposed JMF. |
|  | - Describe how aggregates will be hauled, stockpiled and handled to minimize segregation, avoid contamination, and assure a uniform gradation. |
|  | - Describe procedures and frequency used to control and measure aggregate moistures. |
|  | - Provide the batching sequence and mixing procedures to be used to assure that material balling does not occur. |
|  | - Describe how adjustments to the SSD aggregate design weights in the JMF are made to compensate for moisture contained in the aggregates during batching. |
|  | - Describe how adjustments to mix water will be made to compensate for aggregate moistures to assure the JMF’s water-cementitious ratio (W/Cm) is not exceeded. |
|  | - Define how the batching tolerances of 499.06 are assured. |
|  | - Provide the information reported on the batch ticket and how it will be validated that it meets the requirements of 499.07. |
|  | - Describe the method and frequency of assuring that the combined aggregate gradation remains within Zone II of the Coarseness Factor Chart. If adjustments are made to the JMF proportions, provide the individual aggregate gradations, combined aggregate gradation, and verification that the proportions remain within Zone II of the Coarseness Factor Chart to the Engineer prior to placement. |
|  | - If using a non-potable water source, describe the procedures and frequency to verify that the water meets the requirements of ASTM C1602. - If a reclaiming system is used, describe the method and frequency of testing to ensure that the water contains no more than 0.06% chlorides. |
|  | - Describe how and when the water is removed from the mixer prior to batching a new load of concrete. |
|  | - Describe methods to verify that the storage and dispensing methods for the admixtures comply with the manufacturer’s recommendations. |
|  | - Describe methods to keep the concrete temperature within specifications and how to mitigate effects of changes to the set time. |
|  | - Define the desired slump and tolerance for concrete in each construction item. |
|  | - Define the acceptable yield range, testing frequency to verify that the concrete is maintained within the range. |

#### CA-C-2   QC/QA Concrete QCP Checklist - 3/4

C.  Additional Quality Control Plan Requirements for Concrete Pavement

|  | - In the QCP define the lots for the project. The Engineer will approve the lot division. |
| --- | --- |
|  | - Provide the placement sequence and placement widths for the pavement work and determine the sublot division conforming to Supplement 1064. Provide the Engineer the proposed sublots. The Engineer will approve the sublot division. |
|  | Material Control Requirements |
|  | - Check the aggregate stockpile conditions, gradation and moisture condition daily. - Provide the name and OAIMA Level II certification of the person(s) performing the aggregate gradation. - Define the methods of reporting results to the Engineer including whether the concrete aggregate proportions still conform to the mix’s well graded requirements. |
|  | - For portable plants, assure that the plant is inspected by the Department prior to placing concrete. Describe the procedures and frequencies to verify the mixer blades condition and the scale, gage, meter and admixture dispenser operation. |
|  | - During paving, perform the following: - Yield. Define the tolerance and determine the frequency for testing the yield. At a minimum, test the first day of paving, then every fifth day of placement thereafter. - Air, Slump and Temperature. Sample and test the concrete at least each ½ day of operation at the placement location. |
|  | - Testing for opening the pavement to traffic early according to 451.18. Describe the methods to be used to assure the modulus of rupture obtains 600 psi (4.2 MPa). Define the methods to report results to the Engineer. |
|  | Pavement Cores for Compression and Thickness. Define at what age the sublot cores for strength and thickness will be taken. Define the age the cores will be tested for compressive strength. Take strength cores at the same location as the cores for thickness determination. Determine sublot core locations according to Supplement 1064. |
|  | - Pavement Thickness Measurement. Define who will be measuring the pavement thickness according to Supplement 1064. Define the frequency of reporting pavement thickness results to the Engineer. When the sublot core’s thickness is deficient, follow the requirements of 451.18 for additional core locations to determine the deficiency’s limits. |
|  | - Define how the Engineer will be provided access to witness the measurements. |
|  | - Define the method and frequency of reporting pavement thickness results to the Engineer |
|  | Concrete Strength. For each sublot, test the core strengths using an AMRL accredited laboratory. |
|  | - Propose the method and frequency of reporting the results to the Engineer for acceptance. |

#### CA-C-2   QC/QA Concrete QCP Checklist - 4/4

C.  Additional Quality Control Plan Requirements for Concrete Pavement

|  | Construction Process Requirements |
| --- | --- |
|  | - Define the minimum required rate of concrete delivery for continuous placement and assure that the equipment and transportation is adequate to provide the concrete at that rate. |
|  | - Describe the methods of protecting the concrete in the case of inclement weather. |
|  | - Describe the methods and frequency of controlling and checking the plastic thickness during paving and reporting issues to the Engineer. |
|  | - Define fine grading methods and equipment |
|  | - Define the procedure and frequency of conditioning the subbase or subgrade before pavement placement. |
|  | - Define who will perform the HIPERPAV analysis required in 451.09 and the proposed timeframe the Engineer will have to review the report. |
|  | - Describe equipment and methods for consistently delivering and evenly spreading the concrete in front of the paver. |
|  | - Describe the procedure for dowel bar or load transfer device installation and methods for determining proper placement location after concrete placement. Assure methods conform with 451.09.B. |
|  | - Describe methods of monitoring the vibrator operation and frequency, time of day, station location and track speed according to 451.05.B |
|  | - Define materials, methods and controls for curing and joint sealing and assuring application requirements. |
|  | - Describe joint sawing methods and proposed timing to the sawing operation. |
|  | - Describe finishing and pavement grooving methods. |
|  | - Pavement Smoothness. - Define methods to check pavement smoothness conforming to 451.13 and reporting to the Engineer - If other smoothness tolerances are required in the contract define the methods to measure, evaluate, and report the results to the Engineer |

#### CA-C-2   QC/QA Concrete QCP Plan Activity Log - 1/2

Project No.: ___________________________ Co./Rt./Sec.: ______________________________

Contractor: ___________________________________________________________________

Submission Date: ______________   Original Approval Date: ______________

| Date: | Activity Description |
| --- | --- |

#### CA-C-2   QC/QA Concrete QCP Plan Activity Log- 2/2

Project No.: ___________________________ Co./Rt./Sec.: ______________________________

Contractor: ___________________________________________________________________

Submission Date: ______________   Original Approval Date: ______________

| Date: | Activity Description |
| --- | --- |

Completed by: _________________________________   Date: __________________

#### CA-EW-1   Earthwork Quantity Calculations

CA-EW-1   Earthwork Quantity Calculations (back)

#### CA-EW-2   Proof Rolling Documentation

#### CA-EW-2   Proof Rolling Documentation (back)

#### CA-EW-3   Log of Test Pit Investigation

CA-EW-3   Log of Test Pit Investigation (back)

#### CA-EW-4   Moisture Density Curve Calculation

#### CA-EW-4   Moisture Density Curve Calculation (back)

CA-EW-5   Nuclear Gauge Compaction Form

CA-EW-5   Nuclear Gauge Compaction Form (back)

#### CA-EW-6   Nuclear Gauge Compaction with Aggregate Correction

#### CA-EW-6   Nuclear Gauge Compaction with Aggregate Correction	(back)

Typical Moisture Density Curves - Set C - May, 1949

#### Zero Air Voids Curve

#### CA-EW-7   Non-Stabilized Drainage Base Compaction

#### CA-EW-7   Non-Stabilized Drainage Base Compaction (back)

#### CA-EW-8   Authorization of Undercuts

#### CA-EW-8   Authorization of Undercuts (back)

#### CA-EW-9   Rock Blasting Inspection Form

#### CA-EW-9   Rock Blasting Inspection Form (back)

#### CA-EW-10   Rock Blasting Drilling Log

#### CA-EW-10   Rock Blasting Drilling Log (back)

#### CA-EW-11   Blast Site Security Plan

#### CA-EW-11   Blast Site Security Plan (back)

CA-EW-12   Daily Earthwork Inspection Form

#### CA-EW-12   Daily Earthwork Inspection Form (back)

#### CA-EC-1   Weekly and Rain Event Checklist

#### CA-EC-1   Weekly and Rain Event Checklist (back)

#### CA-EC-2   Seeding Calculations

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

| Left |  |  |  |  |  | Right |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | Width | Sum | Dist | Sq. Yd. |  | Station | Width | Sum | Dist | Sq. Yd. |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
|  | ---- |  |  |  |  |  | ---- |  |  |  |
|  |  | ---- | ---- | ---- |  |  |  | ---- | ---- | ---- |
| Total |  |  |  |  |  | Total |  |  |  |  |

Remarks: _______________________________________________________________________

________________________________________________________________________________

| Test conducted by: |  |  |  |
| --- | --- | --- | --- |
| Inspected By: |  | Date: |  |
| Checked By: |  | Date: |  |

CA-EC-2   Seeding Calculations (back)

#### CA-FP-1   Warranty Asphalt Checklist

#### CA-FP-1   Warranty Asphalt Checklist (back)

#### CA-FP-2   Random Selection of Asphalt Field Samples (448, 403)

#### CA-FP-2   Random Selection of Asphalt Field Samples (448, 403) (back)

#### CA-FP-3   Summary of Asphalt Concrete Quantities

#### CA-FP-3   Summary of Asphalt Concrete Quantities (back)

#### CA-FP-4   Asphalt Concrete inspection

CA-FP-4   Asphalt Concrete inspection (back)

#### CA-FP-5   Roller Capacity and Placement Rate

CA-FP-5   Roller Capacity and Placement Rate (back)

#### CA-FP-6   Calculation of Liquid Asphalt Materials

#### CA-FP-6   Calculation of Liquid Asphalt Materials (back)

Notes:

This formula calculates volume at the specified pay temperature (109.01) for the asphalt material.  If the actual temperature, as measured in the tank truck or distributor, differs from the specified pay temperature, the pay volume must be adjusted as per Supplement 1060.

Values for Temperature Factor, ""K"", and Coefficient of Expansion are included in ODOT Supplement 1060, Table A.  Table B provides Pay Temperature and an index of formulas to be used for calculating volume based on actual measured temperature."

#### TE-217   Bituminous Concrete Density Determination - 446 Cores

#### TE-217   Bituminous Concrete Density Determination - 446 Cores

#### TE-217   Bituminous Concrete Density Determination - 446 Form Instructions

#### TE-217   Bituminous Concrete Density Determination - 446 Cores Instructions

#### TE-217   Bituminous Concrete Density Determination - 446 Random Number Table

#### TE-217   Bituminous Concrete Density Determination

#### CA-P-1(603)   Pipe Construction Inspection Form  -  1/2

Project # _____________________	County, Route & Section _________________________________________

Sample ID  _____________________   Personnel ID _____________________ Date Sampled   _____________________

Item # _____________________  Ref. # 	 Producer Code		  Contractor  _____________________

Description ____________________________________________________________________________________

Location _________________ Balloon Ref. # __________Plan Page __________ Pipe Material (RCP, Metal, HDPE, Other)

Type of Pipe (circle one)    A      B      C      D      E      F          Pipe Markings   ____________

Provide a 2 to 1 slope from top of pipe in a cut to the top of excavation

Provide a 2 to 1 slope from spring-line of pipe in a fill to the top of excavation

W   = Width of trench at Top of Pipe. ______ Refer to Spec. 603.05

B = Thickness of bedding under pipe. _______					________________

T = Outside diameter of pipe to trench wall. _____					________________

S = Depth of bedding.  ____________

OD = Outside diameter of pipe. ____________					________________

*  Type 2 Bedding is Shown See Specification and SCD For Type 1,3,4

For Type 2 Bedding Plastic: B= 6"(150 mm) S= B + 30% OD

For Type 2 Bedding Non Plastic: B= 3"(75 mm) S= B + 30% OD

TRENCH & BEDDING CHECKS  (ACTUAL FIELD MEASUREMENTS)

| Check Stations & Location * | T | W | B | S | O.D. | Initial | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |

* Recommended checks every 50 feet, with a minimum of two checks per run.  Check locations may be related to starting point such as outlet end, catch basin, C/L station, etc.

| Field Measurements Lf. (meter) | Plan Pay Length Lf. (meter) | Grade Check Method | Backfill |  |  | Compaction Form# | Initials & Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Compaction Method Tamp, Flood, etc. | Type: Soil or Structural Backfill 1,2 or 3 | Lift Depth |  |  |

Type of Joints (603.08) _____________________ Joints Installed Satisfactorily?   Yes  No

Materials:  All tested and approved prior to incorporation into the project.

CA-P-1(603)   Pipe Construction Inspection Form  -  2/2

Balloon Ref. # (s):______________________ Plan Page (s):___________________________________

Station to Station: _____________________________________________________________________	_______

Plan Quantity:  __________________________ LF (meters)

Remarks:  __________________________________________________________________________  	_________

Daily Total Field Measured: ________________ LF (meters) *

Remarks: ___________________________________________________________________________  	_________

Daily Total Authorized Pay: ________________ LF (meter)**:_________________________________

*	Explain field measurements and authorized pay.  Pay in accordance with 603.14 Method of Measurement?

a) Structure or pipe end location moved or least cost to State when accommodating full sections.

b) Measured to C/L of structure when inside structure dimension is under 6 linear feet (2 meters) in direction of flow or only one structure on the run of pipe.

c) You must account to "Lab" for pipe length due to creep & measurements to C/L of structures.

**	Round all Items Reference to nearest 0.1 foot (0.03 meter).

Remarks: Record problems, soft foundations, under cuts, rock, instructions, utilities encountered, etc.  Describe and note      location of existing conduits encountered, whether connected, plugged and abandoned, reconnected, etc.

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

Sketch area:

____________________________________________   ___________________________________

Inspector’s Signature			Date	   	    P.E./P.S. Signature

#### CA-P-2   Underdrain Construction Inspection Form  -  1/2

Project # __________________ County, Route & Section __________________________________________________

Sample ID_________ Personnel ID__________________ Date Sampled______________________________________

Item # _________ Ref. #_________ Producer Code_________  Contractor_____________________________________

Description  ______________________________________________________________________________________

Location _________Balloon Ref. # _________Plan Page_________ Pipe Material (RCP, Metal, HDPE, Other)

Underdrain Markings_______ Filter Fabric Required _______Underdrain Joint Type_________ (coupler, bell & Spigot )

W  = Width of trench.__________

D= Depth of Trench ____________

D						T =Outside pipe to trench wall. _____________

ID = Inside diameter of pipe.	___________

TRENCH & BACKFILL CHECKS  (ACTUAL FIELD MEASUREMENTS)

| Station to Station Right, Left Side Every 50 feet (20m) | T | W | I.D. | Backfill Material 8,9,89’s | Outlet Station | How Outlet Catch Basin Slope | Initial | Date | Date Pav’t Placed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

CA-P-2   Underdrain Construction Inspection Form  -  2/2

Balloon Ref. # (s):_____________________ Plan Page (s):____________________________________

Station to Station: _____________________________________________________________________	_______

Plan Quantity:  __________________________ LF (meters)

Remarks:  	________________________________________________________________________________________

Daily Total Field Measured: ________________ LF (meters) *

Remarks:  	_______________________________________________________________________________________

Daily Total Authorized Pay: ________________ LF (meter)**:	____________________________________________

*	Explain field measurements and authorized pay.  Pay in accordance with 603.12 Method of Measurement?

a) Structure or pipe end location moved or least cost to State when accommodating full sections.

b) Measured to C/L of structure when inside structure dimension is over 6 linear feet (2 meters) in direction of flow or only one structure on the run of pipe.

c) You must account to "Lab" for pipe length due to creep & measurements to C/L of structures.

**	Round all Items Reference to nearest 0.1 foot (0.03 meter).

Remarks: Record problems, soft foundations, under cuts, rock, instructions, utilities encountered, etc.  Describe and note      location of existing conduits encountered, whether connected, plugged and abandoned, reconnected, etc.

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

Sketch area:

____________________________________________	______________________________________________

Inspector’s Signature			Date	   	    P.E./P.S. Signature

#### CA-P-3(604)   Drainage Structure Inspection Form  -  1/2

Project # _____________________	County, Route & Section

Sample ID			Personnel ID			Date Sampled

Item # 			Ref. # 	 Producer Code		     Contractor

Description

Location _________________ Balloon Ref. # ____Plan Page ____  Material (Precast, Brick-Block, Other)

Temperature  _________ Structure Temperature  ______ Mortar Temperature ______________

Type of Pipe being placed in structure (circle all that apply) A B C  D E F ,  Structure Markings   ______

Lifting Devices are to remain in place.            Manhole top type furnished _________________________

W  = Width of trench at Top of the Structure.___________							B = Thickness of bedding under Structure. ___________							T = Structure to trench wall thickness ___________							        	Type of Bedding Material Furnished  _______________							Type of Backfill material furnished  _______________						 	Type of Structure (Catch Basin, Inlet, Manhole, 604.02) ________                                                                              					Type of Joint Material being used (604.06)  ________

| Pipe OD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Structure Opening Dia. |  |  |  |  |  |

Backfill

| Compaction Method Tamp, Flood, etc. | Type: Soil or Structural Backfill 1, 2, 3 | Lift Depth | Compaction Form | Initials And Date |
| --- | --- | --- | --- | --- |

CA-P-3(604)   Drainage Structure Inspection Form  -  2/2

Circle the Application: 1 Proposed Structure, 2 Reconstruction Structure, 3 Adjustment to Grade

Balloon Ref. # ___________ Plan Page ________ Plan Station __________ Offset ___________  Elevation  ____________

Structure Top Field Station _______________ Field Offset___________________  Field Elevation____________________

Manhole Plan Base Station _____________________ Offset _____________________ Elevation  ____________________

Manhole Field Base Station _________________  Field Offset__________________  Field Elevation__________________

Reconstruction Structure removal depth________  Outlet Pipe Elevation  __________ Field Outlet Pipe Elevation________

Is the Sump Finished Per the Standard Drawing: YES __  NO__   Explain:  _______________________________________

Is the Consults Finished Per the Standard Drawing: YES __  NO__   Explain:  _____________________________________

Authorized Pay: YES _______NO______ Reference Number____________________________

Explain field measurements and authorized pay.  Pay in accordance with 604.08 Method of Measurement

Remarks: Record problems, soft foundations, under cuts, rock, instructions, utilities encountered, etc.  Describe and note location of existing conduits encountered, whether connected, plugged and abandoned, reconnected, etc. Provide locations and size of any laterals that are not part of a 603 pay Item in a sketch.

____________________________________________________________________________________

Sketch area:

____________________________________________	______________________________________________

Inspector’s Signature			Date	   	    P.E./P.S. Signature

#### CA-P-3(611)    Drainage Structure Inspection Form  -  1/2

Project # _____________________	County, Route & Section

Sample ID			Personnel ID			Date Sampled

Item # 			Ref. # 	 Producer Code		     Contractor

Description

Location _________________ Balloon Ref. # ____Plan Page ____  Material (Precast, Brick-Block, Other)

Temperature  _________ Structure Temperature  ______ Mortar Temperature ______________

Type of Pipe being placed in structure (circle all that apply) A B C  D E F ,  Structure Markings   ______

Lifting Devices are to remain in place.            Manhole top type furnished _________________________

W  = Width of trench at Top of the Structure.___________							B = Thickness of bedding under Structure. ___________							T = Structure to trench wall thickness ___________							        	Type of Bedding Material Furnished  _______________							Type of Backfill material furnished  _______________						 	Type of Structure (Catch Basin, Inlet, Manhole, 611.10) ________                                                                              					Type of Joint Material being used (611.10)  ________

| Pipe OD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Structure Opening Dia. |  |  |  |  |  |

Backfill

| Compaction Method Tamp, Flood, etc. | Backfill Type Description 611.04 B | Lift Depth | Compaction Form | Initials And Date |
| --- | --- | --- | --- | --- |

#### CA-P-3(611)   Drainage Structure Inspection Form  -  2/2

Circle the Application: 1 Proposed Structure, 2 Reconstruction Structure, 3 Adjustment to Grade

Balloon Ref. # ___________ Plan Page ________ Plan Station __________ Offset ___________  Elevation  ____________

Structure Top Field Station _______________ Field Offset___________________  Field Elevation____________________

Manhole Plan Base Station _____________________ Offset _____________________ Elevation  ____________________

Manhole Field Base Station _________________  Field Offset__________________  Field Elevation__________________

Reconstruction Structure removal depth________  Outlet Pipe Elevation  __________ Field Outlet Pipe Elevation________

Is the Sump Finished Per the Standard Drawing: YES __  NO__   Explain:  _______________________________________

Is the Concrete bottom shaped per the Standard Drawing: YES __  NO__   Explain:  ________________________________

Payment will be made in accordance with 611.16 Method of Measurement

Remarks: Record problems, soft foundations, under cuts, rock, instructions, utilities encountered, etc.  Describe and note location of existing conduits encountered, whether connected, plugged and abandoned, reconnected, etc. Provide locations and size of any laterals that are not part of a 611 pay Item in a sketch.

____________________________________________________________________________________

Sketch area:

611.04 C:

“I certify that the information on this form is accurate and all deviations from the current installation plan have been noted.”

____________________________________________   _______________________________________

Contractor’s Representative		Date	   	Print Contractor’s Representative Name

#### CA-P-1(611)   Pipe Construction Inspection Form  -  1/2

Project # _____________________	County, Route & Section _________________________________________

Sample ID  _____________________   Personnel ID _____________________ Date Sampled   _____________________

Item # _____________________  Ref. # 	 Producer Code		  Contractor  _____________________

Description ____________________________________________________________________________________

Location _________________ Balloon Ref. # __________Plan Page __________ Pipe Material (RCP, Metal, HDPE, Other)

Type of Pipe (circle one)    A      B      C      D      E      F          Pipe Markings   ____________

W   = Width of trench at Top of Pipe. ______ Refer to Spec. 611.05

B = Thickness of bedding under pipe. _______					________________

T = Outside diameter of pipe to trench wall. _____					________________

S = Depth of bedding.  ____________

OD = Outside diameter of pipe. ____________					________________

TRENCH & BEDDING CHECKS  (ACTUAL FIELD MEASUREMENTS)

| Check Stations & Location * | T | W | B | S | O.D. | Initial | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |

* Check every 50 feet, with a minimum of two checks per run.  Check locations may be related to starting point such as outlet end, catch basin, C/L station, etc.

| Field Measurements Lf. (meter) | Plan Pay Length Lf. (meter) | Grade Check Method | Backfill |  |  | Compaction Form# | Initials & Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Compaction Method Tamp, Flood, etc. | Material type as described in the installation plan | Lift Depth |  |  |

Type of Joints (611.08) _____________________ Joints Installed Satisfactorily?   Yes  No

Materials:  All tested and approved prior to incorporation into the project.

611.06

611.06

CA-P-1(611)    Pipe Construction Inspection Form  -  2/2

Balloon Ref. # (s):______________________ Plan Page (s):___________________________________

Station to Station: _____________________________________________________________________	_______

Plan Quantity:  __________________________ LF (meters)

Remarks:  __________________________________________________________________________  	_________

Daily Total Field Measured: ________________ LF (meters) *

Remarks: ___________________________________________________________________________  	_________

611.16

*	The Department will measure conduit by the number of feet, measured center-to-center of small drainage structures or

between open ends including the length of pipe bends and branches.  The Department will not deduct conduit length for catch

basins, inlets, or manholes where the distance measured in the direction of flow, including bends, is 6 feet or less.  Where the

location of a drainage structure or an open end is changed with the approval of the Engineer to accommodate full conduit

sections, the Department will measure the length placed.  Conduits placed on slopes steeper than 3H:1V or with beveled or

skewed ends will be measured along the invert.

Remarks: Record problems, soft foundations, under cuts, rock, instructions, utilities encountered, etc.  Describe and note      location of existing conduits encountered, whether connected, plugged and abandoned, reconnected, etc.

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

Sketch area:

611.04:

“I certify that the information on this form is accurate and all deviations from the current installation plan have been noted.”

____________________________________________   _______________________________________

Contractor’s Representative		Date	   	Print Contractor’s Representative Name

#### CA-D-1A   Field Calculation and Measurement

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

| Station Location |  |  | Plan Sheet | Plan Ref. | Plan Quantity | Field Quantity | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Side |  |  |  |  |  |
| Describe Location |  |  |  |  |  |  |  |

| Calculation/Sketch/Additional Remarks: |  |
| --- | --- |
| Special Notes: |  |
| Inspected By: | Date: |
| Checked By: | Date: |

#### CA-D-1A   Field Calculation and Measurement (back)

#### CA-D-1B   Field Calculation and Measurement

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

| Station Location |  |  | Plan Sheet | Plan Ref. | Plan Quantity | Field Quantity | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Side |  |  |  |  |  |
| Describe Location |  |  |  |  |  |  |  |

| Calculation/Sketch/Additional Remarks: |
| --- |
| Special Notes: |  |
| Prepared By: | Date: |
| Checked By: | Date: |

#### CA-D-1B    Field Calculation and Measurement (back)

#### CA-D-2   Field Calculation and Measurement

| Project No: | County/Route/Section: |
| --- | --- |

| Location |  |  | Ref. #: |  | Ref. #: |  | Ref. #: |  | Ref. #: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Item: |  | Item: |  | Item: |  | Item: |  |
|  |  |  | Unit: |  | Unit: |  | Unit: |  | Unit: |  |
| Date | Station or Plan Page | L/R | Plan Quantity | Measured Quantity | Plan Quantity | Measured Quantity | Plan Quantity | Measured Quantity | Plan Quantity | Measured Quantity |
| TOTALS |  |  |  |  |  |  |  |  |  |  |

| Special Notes: |  |
| --- | --- |
| Inspected By: | Date: |
| Checked By: | Date: |

#### CA-D-2   Field Calculation and Measurement (back)

#### CA-D-3SM-A   ODOT Inspectors Daily Report

#### CA-D-3SM-A   ODOT Inspectors Daily Report (back)

#### CA-D-4SM   ODOT P.E. / P.S. Daily Work Report

CA-D-4SM   ODOT P.E. / P.S. Daily Report (back)

#### CA-D-5   Daily Account of Force Account Work

| Project No: | County/Route/Section: | Date: |
| --- | --- | --- |
| Contractor: |  |  |
| Sub-Contractor: |  |  |
| Description of Work: |  |  |

Labor

| Name | Class | From | To | Regular Hrs | OT Hrs |
| --- | --- | --- | --- | --- | --- |

Equipment				A-Owned/ Bid Work,   B-Rented/ Bid Work,   C-Owned/ Non-Bid work,   D-Rented/ Non-Bid work

| A,B,C,D | Hrs Used | Hrs Idle | Year | Type | Model | HP, GVW, Capacity | Gas/Diesel/Elect | Equip No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Material

| Quantity | Unit | Description |  | Signatures/Date |
| --- | --- | --- | --- | --- |
|  |  |  |  | Contractor: |
|  |  |  |  | ODOT: |

Original to ODOT File, Copy to Contractor

#### CA-D-5   Daily Account of Force Account Work (back)

#### CA-D-6  Pavement Repair and Sawing Measurement

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

| Station | Station | Lane | Length (ft) | Width (ft) | Depth (in) | Quantity of Repair (sy) | Length of Sawing (ft) | Plan Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TOTALS |  |  |  |  |  |  |  |  |

| Special Notes: |  |
| --- | --- |
| Inspected By: | Date: |
| Checked By: | Date: |

#### CA-D-6  Pavement Repair and Sawing Measurement (back)

#### CA-D-7  Short Term Work Zone Review

| Project No: | County/Route/Section: |  | Date: |
| --- | --- | --- | --- |
| Weather: |  | Temperature: | Time: |
| Type Of Traffic Control: Road Closed / Lane Closed / Stationary / Moving / Shoulder / Other |  |  |  |
| Work performed by: ODOT / Permit / Utility / Contractor Name: |  |  |  |

A. Traffic Control / Safety Devices  Signs, Flaggers, Cones, Drums, Arrow Boards, Signals, PCMS, etc.

| None | Deficiency And Corrective Action To Be Taken | Date Corrected |
| --- | --- | --- |

B. Traffic Flow	Evidence of crashes, incidents, congestion points, delays, etc.

| None | Deficiency And Corrective Action To Be Taken | Date Corrected |
| --- | --- | --- |

C. Conformance with Standards   Tapers, buffer areas, etc

| None | Deficiency And Corrective Action To Be Taken | Date Corrected |
| --- | --- | --- |

D. Interaction of Work Vehicles and Traffic   Entering/exiting work zone, mud on road, etc

| None | Deficiency And Corrective Action To Be Taken | Date Corrected |
| --- | --- | --- |

E. Storage of Equipment and Materials    Protected or outside of clear zone

| None | Deficiency And Corrective Action To Be Taken | Date Corrected |
| --- | --- | --- |

Action taken

| Notification: Verbal / Written |  |  | Correct By Date: |
| --- | --- | --- | --- |
| Section Requiring Action: | A / B / C / D / E | Corrected promptly? Yes / No |  |
| Work Stoppage Orders? | Yes / No | Date corrected: |  |
| Estimate Held? | Yes / No | Field Review by: |  |
| Copy to: DWZTM, County Manager, Contractor , Construction or Other (Identify) |  |  |  |

#### CA-D-7  Short Term Work Zone Review (back)

#### CA-D-8  Long Term Work Zone Review  -  1/2

#### CA-D-8   Long Term Work Zone Review  -  2/2

#### CA-D-9   Daily Concrete Pavement Documentation Form   - 1/5

Project: ___________________   County/Route/Section: ___________________________

Reference #: _______________	Item #: ________________

Contractor: __________________________________________

Station: ______________ to _______________ 	Lane: _______

Placement Width: __________   Placement Depth: __________

Date: ____________

Temperature and Weather: __________________________________________

PCJMF: __________________________

SUBBASE

Subbase material and condition: ______________________________________

Was the subbase thoroughly moistened before placing concrete? __________ (451.06)

Method used to water subbase: __________________________________________________

JOINTS (451.08)

Longitudinal Joints (451.08A) – Tie Bars

Length: __________ Size: __________ Depth placed: __________

Are tie bars epoxy coated? __________

Method of placing tie bars: ______________________________________________________

Are tie bars being kept horizontal in the slab? ___________

Tie bar spacing? ___________        Number of tie bars placed per typical panel? ___________

Longitudinal joint located at _________ feet from left edge of slab

_________ feet from right edge of slab

#### CA- D-9  Daily Concrete Pavement Documentation Form   -  2/5

Load Transfer Devices (451.08 B) - Transverse Joints

Dowel Length: __________ Diameter: __________   Depth placed: __________

Are dowel bars epoxy coated? __________

Were the dowel bars oiled? __________ Entire bar or alternating ends? _______________

Are dowel bars being kept in the proper alignment? ____________________________

Method of placing dowel bars? ________________________________________________

Dowel bar spacing? _______________    beginning ______________ from edge of slab

Number of dowels placed per panel? 	_________ for _________ panel width

_________ for _________ panel width

Transverse joint spacing: ______________

Expansion Joints (451.08C)

Dowel Length: __________ Diameter: __________   Depth placed: __________

Are dowel bars epoxy coated? __________

Were the dowel bars oiled? __________

Were expansion sleeves installed? __________

On opposite ends of adjacent dowels? _______

Transverse joint spacing: ______________

Construction Joints (451.08E)

Construction joint type: ______________________________________________________

Method of doweling construction joint: __________________________________________

Location of construction joint with respect to contraction joint: _______________________

#### CA- D-9  Daily Concrete Pavement Documentation Form   -  3/5

PLACING CONCRETE (451.06)

Method of Construction – Fixed Form or Slip Formed? ________________________________

Formed Construction

Forms made of steel? ___________

Form depth equal to pavement thickness? _______

Forms cleaned and oiled? _________

Slip Form Construction

Slip form paver make/ model: _________________________________________

Number of internal vibrators: __________________________________________

Vibrator monitors working? ___________

Were all vibrators working properly? _______

Vibration frequency range _______ to ______ ipm

Was vibrator information recorded? ________

Method of aligning paver: _________________________________________________

Paver advancement rate: ____________ feet/minute

Is the longitudinal face of the slab being kept vertical? ____________

Any projections or ridges exceeding tolerances in 451.03B? ____________

Method of depositing concrete:

Is a spreader required/used? _____________________________________________

Was its use waived? / Reason: _______________________________________________

#### CA- D-9  Daily Concrete Pavement Documentation Form   -  4/5

FINISHING (451.09)

Hand finishing methods used: ____________________________________________________

Was 10’ straightedge used? _____________

Were pavement edges rounded? ___________

Micro texture type: _____________________ applied using: ____________________________

Macro texture type: _____________________ applied using: ____________________________

Tine groove spacing: ____________________ Tine depth: ______________________________

Were station numbers stamped into the shoulder pavement? ______________ Spacing? _______

Station number position: ________________________________________________________

CURING (451.10)

Method of applying curing compound: _____________________________________________

Curing compound required: Width = _____ (slab width) + 2 x _____ (slab thickness) = _____

Gallons Required = ________ (length) x ______ (width) / 150 = _________

Gallons Used = __________ (begin tank) - __________ (end tank) = ___________

Curing compound applied approximately ___________ minutes after final finishing

JOINT SAWING (451.08)

Was a HIPERPAV file submitted for this pour? ___________

HIPERPAV predicts early age cracking beginning at _____________ without joint sawing.

Joint sawing began at ____________

Method of sawing:  Early-entry saw? ______ Wet Sawing? ________

Width of Saw Cut		Depth of Saw Cut

Longitudinal Joints: _________________		________________

Contraction Joints:	_________________		________________

#### CA- D-9  Daily Concrete Pavement Documentation Form   -  5/5

INSTRUCTIONS GIVEN TO THE CONTRACTOR

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

COMMENTS

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Signature: ________________________________ Date: ___________________

#### CA- D-9  Daily Concrete Pavement Documentation Form  -  (Back)

#### CA-L-1   Report of Electrical Tests

For Traffic Signals

| Project No: |  |  |  |  | County/Route/Section: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. No: |  |  |  |  | Item & Description: |  |  |  |  |  |  |
| Short Circuit Test (CMS 632.28C) (Infinite or very high OHMS required) |  |  |  |  |  |  | Circuit Continuity Test (CMS 632.28D) (Zero or negligible OHMS required) |  |  |  |  |  |
| Pairs Measured |  | OHMS |  | Pairs Measured | OHMS |  | Pairs Measured | Zero-OHMS |  | Pairs Measured |  | Zero-OHMS |
| Signal Cable |  |  |  | Interconnect Cable (7 cond.) |  |  | LOOP UNSPL. |  |  | Interconnect Cable (7 cond.) |  |  |
| W/R |  |  |  | W/R |  |  | loop unspl. |  |  | W/R |  |  |
| W/ORG |  |  |  | W/ORG |  |  | loop unspl. |  |  | W/ORG |  |  |
| W/G |  |  |  | W/G |  |  | loop unspl. |  |  | W/G |  |  |
| W/BK |  |  |  | W/BL |  |  | loop + L.I. |  |  | W/BL |  |  |
| R/ORG |  |  |  | W/WBK |  |  | loop + L.I. |  |  | W/WBK |  |  |
| R/G |  |  |  | W/BK |  |  | loop + L.I. |  |  | W/BK |  |  |
| R/BK |  |  |  | R/ORG |  |  | loop + L.I. |  |  | R/ORG |  |  |
| ORG/G |  |  |  | R/G |  |  | Signal Cable |  |  | R/G |  |  |
| ORG/BK |  |  |  | R/BL |  |  | W/R |  |  | R/BL |  |  |
| G/BK |  |  |  | R/WBK |  |  | W/ORG |  |  | R/WBK |  |  |
| ETC. |  |  |  | R/BK |  |  | W/G |  |  | R/BK |  |  |
|  |  |  |  | ORG/G |  |  | W/BK |  |  | ORG/G |  |  |
|  |  |  |  | ORG/BL |  |  | R/ORG |  |  | ORG/BL |  |  |
|  |  |  |  | ORG/WBK |  |  | R/G |  |  | ORG/WBK |  |  |
|  |  |  |  | ORG/WB |  |  | R/BK |  |  | ORG/WB |  |  |
|  |  |  |  | G/BL |  |  | ORG/G |  |  | G/BL |  |  |
| ALL GRD |  |  |  | G/WBK |  |  | ORG/BK |  |  | G/WBK |  |  |
|  |  |  |  | G/BK |  |  | G/BK |  |  | G/BK |  |  |
| Power Cable |  |  |  | BL/WBK |  |  | Power Cable |  |  | BL/WBK |  |  |
| W/BK |  |  |  | BL/BK |  |  | W/BK |  |  | BL/BK |  |  |
| W/R (3 cond.) |  |  |  | WBK/BK |  |  | W/R (3 cond.) |  |  | WBK/BK |  |  |
| BK/R (3 cond.) Jumpered together |  |  |  | (9 cond. ETC.) Add sheet |  |  | BK/R (3 cond.) |  |  | (9 cond. ETC.) Add sheet |  |  |
| Test conducted by: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspected By: |  |  |  |  |  |  |  |  |  |  |  | Date: |  |
| Checked By: |  |  |  |  |  |  |  |  |  |  |  | Date: |  |

#### CA-L-1   Report of Electrical Tests  (back)

#### CA-L-2   Report of Electrical Tests

Signal-Ten Day Test (CMS 632.28.G)

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

Test Conducted By: ______________  Representing _________________________

Test Witnessed By:________________ Representing _______________________

Certification

I hereby certify that the above test was conducted in conformance with the plans and specifications for Project No.____________, that the test results as indicated below and that the requirements of the plans and specifications have been met.

Contractor’s Representative: _________________________________________

Title: _________________________________________________________________________

Signal Control System Ten Day Performance Test

Time and Date Started: __________________________________________________________

Time and Date Completed: _______________________________________________________

Details of Outages, Trouble, Repairs, Etc., Including Locations:

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

NOTE: If this report does not include all of the (electrical) (lighting) systems proposed on the project, indicate here the separate circuits or components covered by this test:

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

Test Conducted By: ________________________  Representing: _________________________

#### CA-L-2  Report of Electrical Tests (back)

#### CA-L-3   Report of Electrical Tests

Sign Lighting Test (CMS 625.19)

| Project No: | County/Route/Section: |
| --- | --- |
| Ref. No: | Item & Description: |

Note: This report shall be accompanied by a written certification that test equipment used was last calibrated by an acceptable testing agency not more than 60 days prior to the date of the test.

|  |  | Ground Test [] Circuit Test [] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Location |  |  |  |
| Light/Sign Structure No. | Circuit No. | Station | Route, Street, Etc | LT./RT. | Resistance in: Ohms-  Megohms- |

Test Conducted By: _______________________________     Date: ________________________

Representing: ____________________________________________________________________

Test Witnessed By: _____________________________________    Date: ___________________

Representing: ____________________________________________________________________

Certification

I hereby certify that the above test was conducted in conformance with the plans and specifications for Project No.____________, that the test results were as indicated above and that the requirements of the plans and specifications have been met.

Contractor’s Representative: ___________________________________    Date: ____________

Title: ___________________________________________________________________________

#### CA-L-3   Report of Electrical Tests   (back)

#### CA-L-4   Report on Sign Lighting

(CMS 625.09 and CMS 631)

| Project No: |  |  | County/Route/Section: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. No: |  |  | Item & Description: |  |  |  |  |  |
| Ref |  | Ground Rod Reading: |  |  |  |  |  |  |
| Ref |  | Disconnect Switch Amps: |  |  | Type Enclosure: |  |  |  |
| SIGN “A” - (Length) __________ X (Width) __________ = __________ SQ. FT. |  |  |  |  |  |  |  |  |
| Ref |  | Ballast Type |  |  |  |  |  |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| HEIGHT: BOTTOM OF SIGN TO TOP OF PAVEMENT: __________ |  |  |  |  |  |  |  |  |
| Ref |  | Sign Wired Complete |  |  |  |  |  |  |
| SIGN “B” - (Length) __________ X (Width) __________ = __________ SQ. FT. |  |  |  |  |  |  |  |  |
| Ref |  | Ballast Type |  |  |  |  |  |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| HEIGHT: BOTTOM OF SIGN TO TOP OF PAVEMENT: __________ |  |  |  |  |  |  |  |  |
| Ref |  | Sign Wired Complete |  |  |  |  |  |  |
| SIGN “C” - (Length) __________ X (Width) __________ = __________ SQ. FT. |  |  |  |  |  |  |  |  |
| Ref |  | Ballast Type |  |  |  |  |  |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| HEIGHT: BOTTOM OF SIGN TO TOP OF PAVEMENT: __________ |  |  |  |  |  |  |  |  |
| Ref |  | Sign Wired Complete |  |  |  |  |  |  |
| SIGN “D” - (Length) __________ X (Width) __________ = __________ SQ. FT. |  |  |  |  |  |  |  |  |
| Ref |  | Ballast Type |  |  |  |  |  |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| Ref |  | Lamp Size |  |  | Type |  | Quantity |  |
| HEIGHT: BOTTOM OF SIGN TO TOP OF PAVEMENT: __________ |  |  |  |  |  |  |  |  |
| Ref |  | Sign Wired Complete |  |  |  |  |  |  |

| Test Conducted By: |  |
| --- | --- |
| Inspected by: | Date: |
| Checked by: | Date: |

#### CA-L-4   Report on Sign Lighting  (back)

#### CA-L-5   Report on High Voltage Direct Current Tests

(CMS 625.19E and SS 1003)

| Project No: |  |  | County/Route/Section: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ref. No: |  |  | Item & Description: |  |  |  |
| Circuit Number | Power 1 | Power 2 |  | Neutral | Air Temperature | Relative Humidity |

Notes: __________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

| Test Conducted By: |  |
| --- | --- |
| Inspected by: | Date: |
| Checked by: | Date: |

CA-L-5   Report on High Voltage Direct Current Tests (back)

#### CA-T-1   DLS Report Format  -  Weight-Based System

#### CA-T-2  DLS Short Report Format  -  Weight-Based System

#### CA-T-3  DLS Report Format   -  Stroke Counter System

#### CA-T-4  DLS Short Report Format  -  Stroke Counter System

#### CA-T-5  DLS Report Format  -  Flow Meter Based System

#### CA-T-6  DLS Short Report Format  -  Flow Meter Based System

#### CA-T-7  DLS Report Format  -  ThermoPlastic System

#### CA-T-8  DLS Short Report Format – ThermoPlastic System

#### TE-31   Sample Data

Envelope  No: __________

Sample ID: __________     Sample Origin: _____________________  Personnel ID: _________

Type of Inspection: ______________________________________  Date Sampled: __________

P/S Code (1):  _______________________________    at ________________________________

Mix Plant Code (1): __________________________    at ________________________________

Material Code: __________________________________________________________________

Brand Name: ___________________________________________________________________

Description 1: ___________________ 2: __________________ 3: _________________________

Represents Quantity (2): __________________________   Unit of Measure:________________

Number of Items: __________________________  Consigned to: _________________________

Sampled from: ______________________________ Mfg. Control Number: ________________

Responsible Location: ________________________________  Test Lab: ___________________

| Assign To | Project PO | PO Ind | Item Code | Ref No | Quantity |
| --- | --- | --- | --- | --- | --- |

Lot/Day (3): ____________________________   JMF (3): _______________________________

Concrete Cylinder Specimen Numbers (4): _____________   Bill of Lading No: ____________

Remarks: ______________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

- Same materials are used at a Bituminous or PC Concrete mix plant and are sampled and tested by plant and not by project. For these samples the producer/supplier is identified in the P/S field and the mix plant where the material is sampled is identified in the mix plant field.

- Job control samples require a quantity.

- Rice and Extraction Samples require a JMF,  411 materials require Lot/Day No.

- Identifies concrete cylinders being shipped to the Test lab (TE-31 not to be entered into CIVIS - data already entered from corresponding TE-45)

#### TE-31   Sample Data   (back)

#### Noise Barrier Wall Shop Drawing Review Checklist

The purpose of this checklist is to assist the Engineer with the Department’s review of shop drawings submitted by the Contractor in accordance with Standard Bridge Drawing, NBS-1-09.

- The shop drawings are signed, sealed and dated by an Ohio Registered Professional Engineer.  See footnote ( ).

- The Contractor has accepted the shop drawings in writing. See footnote (1).

- The acceptance letter documents all issues raised by the contractor, fabricator and Department including disposition of issues. See footnote (1).

- For absorptive walls, the absorptive wall material supplier is a pre-approved supplier in accordance with NBS-1-09 or all certified test data and performance history documentation has been provided and is in accordance with NBS-1-09.  See footnote (1).

- The shop drawing top of wall elevation at each panel meets or is above the plan requirements.  See footnote ( ).

- The shop drawing bottom of wall elevation at each panel meets or is lower than the plan requirements.  See footnote (2).

- The total panel height at each location meets the plan requirements and is consistent with the top and bottom of wall elevations as provided in the shop drawings. See footnote (2).

- The shop drawing post height at each location meets the plan requirements. See footnote (2).

- The shop drawing individual panel designs are in accordance with NBS-1-09. See footnote (3).

- The shop drawing individual post designs are in accordance with NBS-1-09. See footnote (3).

- The shop drawing wall alignment is in accordance with the contract requirements.  See footnote (3).

- The shop drawing material requirements are in accordance with NBS-1-09.  See footnote (3).

- The shop drawing aesthetic treatments are in accordance with the contract requirements, including alignment of horizontal joint lines for at least 96-ft. See footnote ( ).

- The shop drawing base plate and anchor bolt requirements are in accordance with NBS-1-09. See footnote (3).

If all checklist items are met, mark documents as “ACCEPTED”.

Footnotes:

1.   If missing, mark documents as “NOT ACCEPTED”.

2.    Document all discrepancies and mark documents “ACCEPTED AS NOTED”. Resubmittal is not required.

3. Document all discrepancies and mark documents “ACCEPTED AS NOTED – RESUBMIT”.

#### Noise Barrier Wall Shop Drawing Review Checklist  -  (Back)

#### CA-G-1   Preconstruction Meeting Agenda / Checklist   -  1/3

Preconstruction Meeting Agenda/Minutes

Attach signed attendance sheet with Name/Representing/Phone

Date of Meeting: _________ Location: __________________________________

Name of Project Engineer: ____________________________________________

Name of Area Engineer: ______________________________________________

Designer of Record: __________________________________________________

State Project No: __________, C-R-S: _____________, PID: ________________

Project Type / Description: ___________________________________________

Bid date: _________ Awarded date: _________ Completion date: ____________

Estimate dates: __________ and _____________ of each month,

accepted payroll and certification of estimates is required (109.09)

Field Offices: _______________________________________________________

Public Relations: ____________________________________________________

__________________________________________________________________

Distribute and discuss all applicable contract documents: ____________________

__________________________________________________________________

__________________________________________________________________

Explanation of Scope and Future Intents (if any): ___________________________

__________________________________________________________________

Safety Requirements and Protocols: _____________________________________

__________________________________________________________________

__________________________________________________________________

Emergency Information: ______________________________________________

__________________________________________________________________

Status of Utilities, Right of Way, RR and Local requirements: ________________

__________________________________________________________________

__________________________________________________________________

Discussion of all DBE Goals/EEO/Prevailing Wage Requirements: ____________

__________________________________________________________________

__________________________________________________________________

Protection and Restoration Provisions: ___________________________________

__________________________________________________________________

#### CA-G-1  Preconstruction Meeting Agenda / Checklist  -  2/3

Prime Contractor: ____________________________Phone No: ______________

Superintendent: ______________________________Phone No: ______________

Normal Work Hours: _________  to  _____________ Weekdays

Normal Work Hours: _________  to  _____________ Weekends

Expected Construction Start Date: ______________

Environmental permit and SWPPP requirements and commitments: ____________

__________________________________________________________________

__________________________________________________________________

Erosion Control and Best Management Practices discussion: _________________

__________________________________________________________________

__________________________________________________________________

Initial Schedule received? _____ (Bar Chart or CPM)

Review of the Schedule: ______________________________________________

__________________________________________________________________

Partnering: _________________________________________________________

__________________________________________________________________
Review of the Disputes and Claims Process: ______________________________

__________________________________________________________________

Review of the Change Order Process: ___________________________________

__________________________________________________________________

Time Extensions: ____________________________________________________

__________________________________________________________________

Force Account: _____________________________________________________

__________________________________________________________________

Price Adjustments: ___________________________________________________

__________________________________________________________________

Locations, Dates, and Descriptions of major work phases: ___________________

__________________________________________________________________

__________________________________________________________________

Haul roads designated: _______________________________________________

__________________________________________________________________

__________________________________________________________________

Borrow/ Waste/ Agreements: __________________________________________

__________________________________________________________________

__________________________________________________________________

List Subcontractors to be used: _________________________________________

__________________________________________________________________

#### CA-G-1  Preconstruction Meeting Agenda / Checklist  -  3/3

AC / PCC plants to be used: ___________________________________________

__________________________________________________________________

__________________________________________________________________

List of major material suppliers: ________________________________________

__________________________________________________________________

__________________________________________________________________

Material approval, inspection, and payment processes: ______________________

__________________________________________________________________

__________________________________________________________________

Bridge Related Items/Issues: ___________________________________________

__________________________________________________________________

__________________________________________________________________

Maintenance of Traffic: _______________________________________________

__________________________________________________________________

__________________________________________________________________

Other Traffic Related Items: ___________________________________________

__________________________________________________________________

Final Inspection/Punch List Requirements: ________________________________

__________________________________________________________________

Progress Meetings frequency: 	_________________________________________

Contractor Evaluation and C-95: ________________________________________

Other items for Discussion: ________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Distribute the Preconstruction Minutes with all attachments to:  Project Engineer, Prime Contractor Superintendent, Designer, Construction Office, LPA, and others expressing interest.

#### CA-G-1  Preconstruction Meeting Agenda / Checklist  -  3/3  back