Item 255 Full Depth Rigid Pavement Removal and Rigid Replacement

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Are diamond saws used to accomplish the full depth pavement sawing? |  | 255.03 |  |  |  |  |
|  | Is the lift out method being used for pavement removal? |  | 255.03 |  |  |  |  |
|  | If the pavement edge is damaged does Contractor’s re-saw with deduct (255.03)? Is the repair area compacted after removal of pavement? |  | 255.04 |  |  |  |  |
|  | Are hydraulic or electric drills used for drilling dowels/tiebar holes? |  | 255.05 |  |  |  |  |
|  | Are dowel/tiebar holes drilled to the specified diameter of 1 5/8 inch? |  | BP-2.5 |  |  |  |  |
|  | Are clear or white opaque plastic grout retention disc used? |  | BP-2.5 |  |  |  |  |
|  | Is sufficient dowel grout pneumatically injected into the rear of hole? |  | 255.05 |  |  |  |  |
|  | Are smooth dowels coated with a thin coating of oil prior to placing? |  | BP-2.5 |  |  |  |  |
|  | Does the grout firmly anchor the dowel or tiebar within 30 minutes? |  | 255.02 |  |  |  |  |
|  | Are dowels/tiebars held in proper alignment until grout has hardened? |  | 255.05 |  |  |  |  |
|  | Is the dowel grout allowed to harden prior to placing the concrete? |  | 255.06 |  |  |  |  |
|  | Do patches > 10 feet long have tiebars in the joint between lanes? |  | BP-2.5 |  |  |  |  |
|  | Is the pavement edge formed? |  |  |  |  |  |  |
|  | Is the area and full depth repair consolidated with an internal vibrator? |  |  |  |  |  |  |
|  | Are patch areas tested with a 10 foot straight edge a smooth ride over? |  |  |  |  |  |  |
|  | Surface texture on the new patch the same as the adjacent pavement? |  |  |  |  |  |  |
|  | Is curing compound applied to the surface at the proper rate? |  |  |  |  |  |  |
|  | Steel mesh installed in all patches > 10 feet in length? |  | BP-2.5 |  |  |  |  |
|  | Steel mesh installed in repairs opened to traffic within 24 hours? |  | BP-2.5 |  |  |  |  |
|  | Are insulating boards used to cover fast setting (Class FS) repairs? |  | 499.05 A |  |  |  |  |
|  | Are the vertical faces of the joints cleaned by abrasive blasting? |  | 255.06 |  |  |  |  |
|  | Joint seal material maintained ¼ inch below the top of pavement? |  | 255.06 |  |  |  |  |
|  | Bond breaker tape installed under the hot applied joint sealer? |  | BP-2.5 |  |  |  |  |
|  | Are unfilled repairs < 10 feet long covered with steel plates overnight? |  | 255.08 |  |  |  |  |
|  | Repairs filled when work is suspended for a Holiday or a weekend? |  | 255.08 |  |  |  |  |