Item 401 Asphalt Pavement

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Is the surface properly cleaned prior to asphalt placement? |  | 401.14, 407.05 |  |  |  |  |
|  | Is the proper tack material used ? |  | 407.02 |  |  |  |  |
|  | Is the tack coat uniformly applied? |  | 407.06 |  |  |  |  |
|  | How uniform is the tack application (5= uniform, 0= thin ribbons)? |  |  |  |  |  |  |
|  | If cover aggregate was used, was it uniformly applied? |  | 407.07 |  |  |  |  |
|  | If a surface course is being laid, is the vertical face of the longitudinal cold joint sealed? |  | 401.07 |  |  |  |  |
|  | Is the intermediate course tack coated prior to placing the surface course? |  | 401.14, 407.06 |  |  |  |  |
|  | If the existing pavement is concrete, was rubberized asphalt emulsion, 702.13, used? |  | 407.06 |  |  |  |  |
|  | Is the existing pavement dry? |  | 401.06 |  |  |  |  |
|  | Does the existing pavement surface temperature exceed the minimum allowed? |  | 401.06 |  |  |  |  |
|  | If a surface course is being placed, does the air temperature exceed 40 F? |  | 401.06 |  |  |  |  |
|  | Is the haul distance from the asphalt plant less than 50 miles? |  | 401.11 |  |  |  |  |
|  | Is the truck tarp covering the mix when the truck arrives at the paver? |  | 401.11 |  |  |  |  |
|  | If air temps below 50 F and the haul distance is greater than 20 miles, are the truck beds insulated? |  | 401.11 |  |  |  |  |
|  | Is a screed or screed extension used to match a previously placed pavement course? |  | 401.15 |  |  |  |  |
|  | Does screed extension meet the auger and heating requirements? |  | 401.15 |  |  |  |  |
|  | Is the asphalt concrete uniform in composition and surface texture (no segregation)? |  | 401.15 |  |  |  |  |
|  | What is the severity or segregation (5= None; 0 = High Severity)? |  | 401.15 |  |  |  |  |
|  | Is the mix delivered at a uniform, continuous rate? |  | 401.15 |  |  |  |  |
|  | Are delivery trucks bumping the paving machine? |  | 401.15 |  |  |  |  |
|  | Is the paving machine leaving any unusual marks or streaks in the pavement? |  | 401.15 |  |  |  |  |
|  | Is mix temperature < 325 F (163 C) for normal mixes or < 350 F (170 C) for polymer modified mixes? |  | 401.15 702.00 |  |  |  |  |
|  | Is “blue smoke” observed during the delivery of the asphalt mix? |  | 401.15 |  |  |  |  |
|  | Is “drain down” of the asphalt cement observed during the delivery of the mix? |  | 401.15 |  |  |  |  |
|  | Is all aggregate uniformly coated when delivered to the paving machine? |  | 401.15 |  |  |  |  |
|  | Was the mix slumped down in the truck when it was delivered? |  | 401.15 |  |  |  |  |
|  | Is the delivered asphalt mix workable? |  | 401.15 |  |  |  |  |
|  | Is the delivered mix tender? |  | 401.15 |  |  |  |  |
|  | Has any asphalt been rejected on the project? |  | 401.15 |  |  |  |  |
|  | Has the required placement rate been calculated? |  | 401.19 |  |  |  |  |
|  | Has the actual placement rate been calculated? |  | 401.19 |  |  |  |  |
|  | How often is the actual placement rate calculated? |  | 401.19 |  |  |  |  |
|  | Is the actual placement rate within the proper tolerance from the required placement rate? |  | 401.19 |  |  |  |  |
|  | If traffic is permitted to cross the edge of the new asphalt mat, is the adjacent lane placed within 24 hours? |  | 401.17 |  |  |  |  |
|  | If a hot joint is made by the use of 2 pavers, are delivery trucks alternating between them? |  | 401.17 |  |  |  |  |
|  | On 446 projects, is a hot longitudinal joint made between the mainline lane and the adjoining berm? |  | 401.17 446.06 |  |  |  |  |
|  | Are the random sample locations determined by the project within 24 hours after the pavement is placed? |  | 446.05 |  |  |  |  |
|  | Does the Contractor obtain core samples within 48 hours after the asphalt mat is placed? |  | 446.05 |  |  |  |  |
|  | Are core sample holes properly filled by the Contractor? |  | 446.05 |  |  |  |  |
|  | Are core samples properly stored? |  | 446.05 |  |  |  |  |
|  | Are core samples transported to the District in a timely manner? |  | 446.05 |  |  |  |  |
|  | Is a daily plate sample being taken? |  | 446.05 |  |  |  |  |
|  | Have sample locations been properly picked? |  | 448.04 |  |  |  |  |
|  | If mat samples are being obtained, are proper procedures being used? |  | 448.04 |  |  |  |  |
|  | If paver hopper samples are taken, are the proper procedures being used? |  | 448.04 |  |  |  |  |
|  | Are samples properly identified, wrapped and shipped to the District Lab? |  | 448.04 |  |  |  |  |
|  | Has the rate of placement been calculated in square yards (square meters) per hour (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Has the maximum capacity of the roller train been calculated (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Is the maximum capacity greater than the rate of placement (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Does the roller train meet the specifications for the mix being placed (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Base course: Type 1 pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | Intermediate and Surface Courses: three wheel roller and pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | Variable depth courses: Steel and pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | If a vibratory roller is used, is the course thickness less than 1.5 inches (38 mm) (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Is the longitudinal joint being compacted first? |  | 401.13, 401.16 |  |  |  |  |
|  | On 446 projects, is the berm compacted with the same roller coverage as the mainline pavement? |  | 401.13, 401.16 446.05 |  |  |  |  |