Item 451 Rigid Pavement Construction

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Proper Diameter Dowels |  | Table 451.08-1 |  |  |  |  |
|  | Dowels and Baskets wires epoxy coated |  | 709.13 |  |  |  |  |
|  | Dowels anchored to base using approved method |  | 451.08B |  |  |  |  |
|  | Are dowels are parallel to the subbase |  | BP 2.2 |  |  |  |  |
|  | Are free end of dowels coated with light oil within 2 hours of paving |  | 451.08 B |  |  |  |  |
|  | At expansion joints, board fits tight, caps installed & dowels oiled |  | 451.08 C |  |  |  |  |
|  | Shipping wires removed immediately prior to paving |  | 451.08 B |  |  |  |  |
|  | Dowel basket exposure to weather and sunlight limited to 180 days |  | 709.13 |  |  |  |  |
|  | Are tiebars or hook bolt spaced properly @ 26 or 20 inch centers |  | BP-2.1, TABLE A |  |  |  |  |
|  | Are tiebars epoxy coated |  | 451.02, 709.00 |  |  |  |  |
|  | Are tiebars or hook bolts the proper diameter and length |  | BP- 2.1 |  |  |  |  |
|  | Tiebars installed by mechanical equipment or secured by chairs |  | 451.08 A |  |  |  |  |
|  | Tiebars being installed at mid-depth at longitudinal joints |  | BP-2.1 |  |  |  |  |
|  | Tiebars kept proper distance from dowel baskets |  | BP-2.1)? (12 to 18 inches |  |  |  |  |
|  | Paving machine is self propelled and has internal vibrators |  | 451.03 |  |  |  |  |
|  | Paving machine has a working vibration monitoring system |  | 451.03 |  |  |  |  |
|  | The Contractor has a separate spreader in the paving train |  | 451.03)? (>10,000 SY) |  |  |  |  |
|  | The vibrators on the paving machine shut off when the paver stops |  | 451.06 |  |  |  |  |
|  | Vibrators are within frequency range of 7000 to 11,000 impulses per minute |  | 451.03 |  |  |  |  |
|  | Concrete forms are the proper depth for the thickness specified |  | 451.03 A |  |  |  |  |
|  | Concrete forms cleaned and oiled prior to use |  | 451.03 A |  |  |  |  |
|  | Concrete forms base width is equal to depth of the form |  | 451.03 A |  |  |  |  |
|  | Slip form paver is controlled from a preset grade line |  | 451.03 B |  |  |  |  |
|  | Self propelled curing machine with wind protection and shielding used |  | 451.10 |  |  |  |  |
|  | Has the subbase been thoroughly moistened with water prior to paving |  | 451.06 |  |  |  |  |
|  | Concrete tests performed on concrete taken from the point of use |  | MOP |  |  |  |  |
|  | Concrete is spread uniformly full width of the pavement without voids |  | MOP |  |  |  |  |
|  | Paving mesh is being installed at the proper depth and location |  | BP-1.1 |  |  |  |  |
|  | Thickness checks made during paving to verify pavement thickness |  | MOP |  |  |  |  |
|  | Temperature of the plastic concrete maintained at 90 F or less |  | 451.06 |  |  |  |  |
|  | Two concrete test beams made every 7500 square yards |  | 700 |  |  |  |  |
|  | Coarse aggregates used has been tested for d-crack susceptibility |  | 499.02, 703.13 |  |  |  |  |
|  | Dowel basket assemblies vibrated by separate internal vibrator |  | 451.06 |  |  |  |  |
|  | Is a 10-ft checking straight edge used after finishing? Pavement edge rounded with edging tool and tool marks removed? |  |  |  |  |  |  |
|  | Broom or turf drag used to provide a uniform gritty surface before grooving? |  |  |  |  |  |  |
|  | Transverse grooves applied at the correct depth, width and spacing? |  |  |  |  |  |  |
|  | Station numbers stenciled at proper location? |  |  |  |  |  |  |
|  | Curing compound applied after water sheen has disappeared |  | 451.10 |  |  |  |  |
|  | Curing compound properly agitated prior to use |  | 451.10 |  |  |  |  |
|  | Daily checks made on usage to assure application rate is maintained |  | MOP |  |  |  |  |
|  | Pavement edges sprayed with curing membrane after form removal |  | 451.10 |  |  |  |  |
|  | Traffic kept off concrete pavement for 7 days |  | 451.16 |  |  |  |  |
|  | Pavement not opened for 7 days? ...5 days |  | 451.14 |  |  |  |  |
|  | Does the contractor have a standby saw |  | 451.08 D |  |  |  |  |
|  | Sawing depth checked to assure compliance with the specifications |  | 451.08 D |  |  |  |  |
|  | Timely sawing so that minimal raveling occurs |  | 451.08 D |  |  |  |  |
|  | Joints sealed prior to opening the pavement to construction traffic |  | 451.15 |  |  |  |  |
|  | Saw cuts straight and perpendicular to the pavement edge |  | 451.08 |  |  |  |  |
|  | Are saw cuts cleaned properly prior to sealing the joint opening |  | 451.08 D |  |  |  |  |
|  | Elongation of 705.11, preformed elastomeric sealer limited to 5 % |  | 451.15 A |  |  |  |  |