Item 524 Drilled Shafts

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Did contractor submit installation plan at least 14 days prior constructing drilled shafts? |  | 524.03 |  |  |  |  |
|  | Did contractor’s installation plan contain include all the required information? |  | 524.03 |  |  |  |  |
|  | If the dry method was used, were the sides stable and any flow of water into the excavation less then 300 mm (12 inches) per hour? |  | 524.04A |  |  |  |  |
|  | If wet construction method is used for a shaft which is not socketed into bedrock, was the water or slurry fluid level during the drilling operation inside the drilled shaft higher then the static water table? |  | 524.04B |  |  |  |  |
|  | If temporary casing is used, was at least a 5 foot (1.5 meter) head of concrete maintained above the bottom of the casing? |  | 524.04C |  |  |  |  |
|  | If permanent casing is required, did the contractor excavate material beyond the limits of the casing? |  | 524.04D |  |  |  |  |
|  | Were spacer attached to the reinforcing cage at quarter points? |  | 524.09 |  |  |  |  |
|  | Were round spacers of the proper size used to maintain the required clearance, ie.3 inches (75 mm) for shaft diameters up to 4 feet (1.2meters) and 6 inches (150 mm) for shaft diameters larger than 4 feet (1.2 meters)? |  | 524.09 |  |  |  |  |
|  | If concrete is placed underwater, is it placed in one continuous operation? |  | 524.10 |  |  |  |  |
|  | If a tremie is used to place concrete underwater, is the end of the tremie maintained at least 10 feet (3 meters) below the surface of the concrete? |  | 524.12 |  |  |  |  |
|  | If a pump is used to place concrete underwater, is the end of the pump maintained at least 10 feet (3 meters) below the surface of the fresh concrete? |  | 524.13 |  |  |  |  |
|  | If a pump or tremie is used to place concrete under water, was the end of the pump or tremie properly plugged when it was placed into the water? |  | 524.12 / 524.13 |  |  |  |  |
|  | Where bottoms and sides of excavation checked for cleanliness? |  | 524.08 |  |  |  |  |
|  | Where tolerances checked? |  | 524.14 |  |  |  |  |
|  | If wet method is used, after the shaft is full is concrete continually placed into the shaft until acceptable quality of concrete is evident at the top of the shaft? |  | 524.10 |  |  |  |  |