Item 621 Raised Pavement Markers

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Materials conform to following specs: 721.01, 721.02, 721.03, 721.04 |  | 621.02 |  |  |  |  |
|  | 621 Raised Pavement Markers (RPM) |  |  |  |  |  |  |
|  | Check depth and width of saw cut per Standard Const, Drawing TC 6510. |  | SCD 65.10 |  |  |  |  |
|  | Check before placement of the epoxy, the saw cut is clean of all loose material and dry. Saw cut should have 1/8 inch all sides clearance of the casting. |  | 621.04 |  |  |  |  |
|  | Verify ambient air temperature are at least 40degrees Fahrenheit (5 degrees Centigrade) and the pavement is dry. |  | 621.04 |  |  |  |  |
|  | Check epoxy is an A+B mixture, throughly mixed (grey color) and in accordance with manufacture’s recommendations. |  | 621.04 |  |  |  |  |
|  | Check that sufficient epoxy is in and between the slots to ensure that all voids beneath and around the casting are filled. |  | 621.04 |  |  |  |  |
|  | Placement of RPM castings shall be 6" from any construction joint (Lateral or Longitudinal). |  |  |  |  |  |  |
|  | Location and / stations are per Standard Const. Drawing. |  | SCD TC 65.11, TC 65.12 |  |  |  |  |
|  | Verify that all dirt, dust, oil, grease, rust, moisture, parts of damaged reflectors, or any foreign matter is removed that impairs adhesion of the reflector to the casting. |  | 621.04 |  |  |  |  |
|  | Verify reflector area of the castings shall be sandblasted to 80% bare metal. |  | 621.04 |  |  |  |  |
|  | Verify the application of adhesive is in a single bead, sufficient to squeeze out on all sides of the reflector when pressure is applied, to seat the reflector and seal out moisture. |  | 621.04 |  |  |  |  |