Item 203 Roadway Excavation and Embankment

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was specification material used? |  | 203.02/ 95 |  |  |  |  |
|  | Were the restrictions on the material types as per 203.03 implemented? |  | 203.03/ 127 |  |  |  |  |
|  | Was the work properly drained and maintained? |  | 203.04.A/ 128 |  |  |  |  |
|  | Was the benching correctly performed? |  | 203.05/ 136 |  |  |  |  |
|  | Was the foundation properly evaluated and compacted? |  | 203.05/ 130 |  |  |  |  |
|  | Was soil or granular material placed in 8” lifts? |  | 203.06/ 142 |  |  |  |  |
|  | Was shale placed in 8” lifts? |  | 203.06.B/ 142 |  |  |  |  |
|  | Was the bucket test correctly performed? |  | 703.16.D/ 144 |  |  |  |  |
|  | Was shale properly watered and broken down? |  | 203.06.B/ 145 |  |  |  |  |
|  | Was shale and rock properly separated? |  | 203.06.B/ 146 |  |  |  |  |
|  | Was the correct rock lift thickness used? |  | 203.06.C/ 145 |  |  |  |  |
|  | Was the rock fill correctly constructed? |  | 203.06.C/ 145 |  |  |  |  |
|  | Were the RPCC pieces less than 3’ X 3’? |  | 203.06.D/ 146 |  |  |  |  |
|  | Was soil properly mixed with RPCC or RACP? |  | 203.06.D/ 146 |  |  |  |  |
|  | Was the correct lift thickness used for the RPCC/RACP and soil mixtures in the fill? |  | 203.06.D/ 146 |  |  |  |  |
|  | Are proper moisture control testing and practices being followed? |  | 203.07.A/ 146 |  |  |  |  |
|  | Are the results of the test section being applied throughout the production areas of the embankment construction? |  | 203.07.B/ 147 |  |  |  |  |
|  | Were the plan cross sections verified? |  | 203.08/ 158 |  |  |  |  |
|  | Were the tolerances in 203.08 correctly checked? |  | 203.08/ 158 |  |  |  |  |
|  | Were the interim and final quantities correctly calculated? |  | 203.09/ 157 |  |  |  |  |

Item 204 Subgrade Compaction and Proof Rolling

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was specification material used? |  | 204.02 |  |  |  |  |
|  | If a test section was used, was the correct roller weights and number of passes used? |  | 204.03 |  |  |  |  |
|  | Did the subgrade pass compaction testing? |  | 204.03 |  |  |  |  |
|  | Was rock, shale, or coal removed 2’ below the bottom of the pavement? |  | 204.05/ 167 |  |  |  |  |
|  | Was the undercut underdrains at least 6” into the existing material? |  | 204.05/ 168 |  |  |  |  |
|  | Were the existing underdrains cleaned out or were additional construction underdrains properly used? |  | 204.03/ 166 |  |  |  |  |
|  | Was the moisture content at the time of proof rolling correct? |  | 204.06/ 168 |  |  |  |  |
|  | Was the correct proof roller weight and tire pressure used? |  | 204.06/ 170 |  |  |  |  |
|  | Was form CA-EW-2 properly filed out? |  | 204.06/ 169 |  |  |  |  |
|  | Was the responsibility for failed areas administrated correctly? |  | 204.06/ 174 |  |  |  |  |
|  | Was the correct failure criteria used to evaluate the subgrade? |  | 204.06/ 171 |  |  |  |  |
|  | Was the failed areas investigated properly? |  | 204.06/ 174 |  |  |  |  |
|  | Were the test pits properly constructed? |  | 204.06/ 175 |  |  |  |  |
|  | Was form CA-EW-3 correctly filled out? |  | 204.06/ 176 |  |  |  |  |
|  | Was the information entered into Figure 204.H correctly obtained? |  | 204.06/ 178 |  |  |  |  |
|  | Was the correct undercut depth determined using Figure 204.H? |  | 204.06/ 180 |  |  |  |  |
|  | Was the undercut material and geotextile properly placed? |  | 204.07/ 182 |  |  |  |  |
|  | Was the undercut properly drained? |  | 204.07/ 183 |  |  |  |  |
|  | Were the finished undercut locations proof rolled? |  | 204.06/ 184 |  |  |  |  |
|  | Were the total quantities calculated correctly? |  | 204.08 |  |  |  |  |

Item 206 Lime Stabilized Subgrade

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was the correct soil type stabilized? |  | 206.02/ 189 |  |  |  |  |
|  | Was specification lime used? |  | 206.02/ 189 |  |  |  |  |
|  | Did the soil weight at least 100 lbs/ft3? |  | 206.02/ 190 |  |  |  |  |
|  | Were the limits in 206.03 followed? |  | 206.03/ 190 |  |  |  |  |
|  | Was the subgrade line and grade properly checked? |  | 206.03/ 237 |  |  |  |  |
|  | Was the subgrade properly test rolled? |  | 206.03/ 190 |  |  |  |  |
|  | Was the Contractor lime percentage report accepted by the Project? |  | 206.06/ 192 |  |  |  |  |
|  | Was the correct percentage of lime used? |  | 206.03/ 190 |  |  |  |  |
|  | Was the initial mixing performed correctly? |  | 206.04/ 191 |  |  |  |  |
|  | Was the lime mixture lightly compacted after the initial mixing? |  | 206.04/ 191 |  |  |  |  |
|  | Was the final mixing performed correctly? |  | 206.04/ 191 |  |  |  |  |
|  | Was the stabilized subgrade compacted properly? |  | 206.05/ 191 |  |  |  |  |
|  | Did the stabilized subgrade meet the compaction requirements? |  | 206.05/ 191 |  |  |  |  |
|  | Was the final rolling performed correctly? |  | 206.05/ 191 |  |  |  |  |
|  | Were test pits used to verify the depth of the stabilization? |  | /192 |  |  |  |  |
|  | Was the stabilized subgrade kept moist until the prime coat was applied? |  | /192 |  |  |  |  |
|  | Was the correct prime coat applied in a timely manner? |  | 206.02 & 206.05 /192 |  |  |  |  |
|  | Was traffic kept off the stabilized subgrade until the end of the cure period? |  | 206.05/ 192 |  |  |  |  |
|  | Was the stabilized subgrade proof rolled after stabilization? |  | /184 & 190 |  |  |  |  |
|  | Was the total pay quantities calculated correctly? |  | 206.08 |  |  |  |  |

Item 804 Cement Stabilized Subgrade

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was the correct soil type stabilized? |  | 804.02/ 193 |  |  |  |  |
|  | Was specification cement used? |  | 804.02/ 193 |  |  |  |  |
|  | Were the limits in 804.03.A followed? |  | 804.03.A/193 |  |  |  |  |
|  | Was the subgrade line and grade properly checked? |  | 804.03.A/237 |  |  |  |  |
|  | Was the subgrade properly test rolled? |  | 804.03.A/194 |  |  |  |  |
|  | Was the Contractor cement percentage report accepted by the Project? |  | 804.06/ 196 |  |  |  |  |
|  | Was the correct percentage of cement used? |  | 804.02/ 193 |  |  |  |  |
|  | Was a canvas used to determine the amount of cement used? |  | 804.03.B/194 |  |  |  |  |
|  | Was the initial mixing performed correctly? |  | 804.03.C/194 |  |  |  |  |
|  | Was water added during the final mixing? |  | 804.03.C/195 |  |  |  |  |
|  | Was the final mixing performed correctly? |  | 804.03.C/195 |  |  |  |  |
|  | Was the stabilized subgrade compacted properly? |  | 804.03.D/195 |  |  |  |  |
|  | Did the stabilized subgrade meet the compaction requirements? |  | 804.03.D/195 |  |  |  |  |
|  | Was the final rolling performed correctly? |  | 804.03.D/195 |  |  |  |  |
|  | Were test pits used to verify the depth of the stabilization? |  | 804.03.C/195 |  |  |  |  |
|  | Was the stabilized subgrade kept moist until the prime coat was applied? |  | 804.04/ 195 |  |  |  |  |
|  | Was the correct prime coat applied in a timely manner? |  | 804.02& 804.04 /193 & 195 |  |  |  |  |
|  | Was traffic kept off the stabilized subgrade until the end of the cure period? |  | 804.04 |  |  |  |  |
|  | Was the stabilized subgrade proof rolled after stabilization? |  | 804.05/184 |  |  |  |  |
|  | Were the total pay quantities calculated correctly? |  | 804.07 |  |  |  |  |

Item 208 Rock Blasting

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was the blasting plan accepted by the Department? |  | 208.05/ 238 |  |  |  |  |
|  | Was the pre-blast survey correctly performed? |  | 208.14/ 238 |  |  |  |  |
|  | Was the hydrologist work performed correctly? |  | 208.17 |  |  |  |  |
|  | Was the test section plan accepted? |  | 208.07/ 238 |  |  |  |  |
|  | Was the burden distance checked? |  | 208.06.C/228 |  |  |  |  |
|  | Was the blasting hole spacing checked? |  | 208.06/ 229 |  |  |  |  |
|  | Was the blasting hole depth checked? |  | 208.06.B/228 |  |  |  |  |
|  | Was the blasting hole diameter checked? |  | 208.06.D/229 |  |  |  |  |
|  | Was the stemming depth checked? |  | 208.06.E/228 |  |  |  |  |
|  | Was the correct type of stemming used? |  | 208.06.E/230 |  |  |  |  |
|  | Were the blasting vibration levels under the specification requirement? |  | 208.15.F/233 |  |  |  |  |
|  | Did the project control the air blast to under 134 dB? |  | 208.16.A/232 |  |  |  |  |
|  | Was the blast controlled enough not to minimize back slope damage? |  | 208.06.F/231 |  |  |  |  |
|  | Was the blast recorded? |  | 208.20.D |  |  |  |  |
|  | Were the rifling of the holes minimized? |  | 208.16/ 231 |  |  |  |  |
|  | Was fly rock dealt with according to 208.18? |  | 208.18/ 231 |  |  |  |  |
|  | Was the pre-split holes properly drilled and detonated? |  | 208.09/ 233 |  |  |  |  |
|  | Do the pre-split cut faces have uniform slope and shear face? |  | 208.07 & 208.09/ 233 |  |  |  |  |
|  | For all slopes that do not require pre-splitting, do the slopes have a neat and smooth appearance? |  | 208.01 |  |  |  |  |
|  | Was the blasting consultant utilized properly? |  | 208.13 |  |  |  |  |

Item 304 Aggregate Base

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was specification material used? |  | 304.02/ 260 |  |  |  |  |
|  | Was a moisture-density curve made from the stock piles material? |  | 304.03/ 261 |  |  |  |  |
|  | Was the stock piled material at the correct moisture content? |  | 304.03/ 261 |  |  |  |  |
|  | Was the correct lift thickness placed? |  | 304.04/ 261 |  |  |  |  |
|  | Was the correct roller weight used as per the lift thickness? |  | 304.04/ 261 |  |  |  |  |
|  | If multiple lifts are needed, were the lifts equivalent? |  | 304.04/ 262 |  |  |  |  |
|  | Was the proper self propelled spreading machine used? |  | 304.04/ 262 |  |  |  |  |
|  | If a grader was used without a spreader box, were in place gradation tests taken? |  | 304.04/ 262 |  |  |  |  |
|  | If the material is segregated, were in placed gradation tests taken? |  | 304.04/ 262 |  |  |  |  |
|  | Was the material at the correct moisture content prior to compaction? |  | 304.05/ 262 |  |  |  |  |
|  | Was the test section correctly constructed? |  | 304.05/ 262 |  |  |  |  |
|  | Was the minimum roller weights and number of passes used for the test section? |  | 304.05/ 262 |  |  |  |  |
|  | Was the same or more compactive effort used in the production area as found in the test section? |  | 304.05/ 263 |  |  |  |  |
|  | Did the compaction tests in the production areas pass? |  | 304.05/ 263 |  |  |  |  |
|  | Was the 304 properly maintained? |  | 304.05/ 263 |  |  |  |  |
|  | Did the finished surface meet the requirements of 304.06? |  | 304.06/ 264 |  |  |  |  |
|  | Were the depth checks made correctly? |  | 304.06/ 264 |  |  |  |  |
|  | Were the width checks made correctly? |  | 304.06/ 265 |  |  |  |  |
|  | Were the final quantities correctly calculated? |  | 304.07 |  |  |  |  |
|  | If a variable depth thickness was specified, was the conversion chart used correctly? |  | 304.07 |  |  |  |  |

SS-1015 Compaction Testing of Unbound Materials

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was a standard count taken every week? |  | 1015.01/ 797 |  |  |  |  |
|  | Was the testing location properly prepared? |  | 1015.02/ 799 |  |  |  |  |
|  | Were the gauge readings taken at the proper depths? |  | 1015.02/ 801 |  |  |  |  |
|  | Was the proctor soil taken from under the gauge? |  | 1015.02.A/804 |  |  |  |  |
|  | Was the soil properly screened through a ¾” sieve? |  | 1015.02.A/804 |  |  |  |  |
|  | Was the proctor correctly made? |  | 1015.02.A/776 |  |  |  |  |
|  | Was a large concrete block used for the proctor foundation? |  | 1015.02.A/777 |  |  |  |  |
|  | Was the scale properly balanced and leveled? |  | 1015.02.A/804 |  |  |  |  |
|  | Was the total weight calculation correctly made? |  | 1015.02.A/804 |  |  |  |  |
|  | Was the correct curve chosen? |  | 1015.02.A/805 |  |  |  |  |
|  | Was the correct optimum moisture and maximum density chosen? |  | 1015.02.A/806 |  |  |  |  |
|  | Was the correct compaction calculated? |  | /806 |  |  |  |  |
|  | Was the zero air voids checked? |  | /807 |  |  |  |  |
|  | If an aggregate correction was needed, was the percentage of rock correctly calculated? |  | 1015.03/ 817 |  |  |  |  |
|  | Was the corrected maximum density and optimum moisture chosen? |  | 1015.03/ 819 |  |  |  |  |
|  | Were the test section tests properly taken? |  | 1015.05/ 831 |  |  |  |  |
|  | Was the correct test used for shale having < 25 % retained on the ¾” sieve? |  | 1015.08.A/835 |  |  |  |  |
|  | Was the correct test used for shale having 25 to 75 % retained on the ¾” sieve? |  | 1015.08.A/835 |  |  |  |  |
|  | Was the correct test used for shale having > 75 % retained on the ¾” sieve? |  | 1015.08.A/835 |  |  |  |  |
|  | Were the correct number of compaction tests taken? |  | 1015.10/ 836 |  |  |  |  |

Mechanically Stabilized Earth Walls

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Did the panels arrive with a TE-24? |  | 2.1 |  |  |  |  |
|  | Were the panels rejected or repaired as per the specifications? |  | 2.1.7 |  |  |  |  |
|  | Was the Select Granular Material approved? |  | 2.5 |  |  |  |  |
|  | If in a cut was required to construct the wall, were the side walls properly protected? |  | 4.1 |  |  |  |  |
|  | Was the foundation properly prepared? |  | 4.2 |  |  |  |  |
|  | Was the drainage properly constructed? |  | 2.7 |  |  |  |  |
|  | Was the filter fabric properly placed? |  | 2.7 |  |  |  |  |
|  | Was the foundation undercut properly constructed? |  | 3.2 |  |  |  |  |
|  | Was the leveling pad placed as specified? |  | 4.2 |  |  |  |  |
|  | Were the wall panels placed according to the plan and markings on the back of the panels? |  | 4.3/2.1.8 |  |  |  |  |
|  | Was external bracing used for the first lift of panels? |  | 4.3 |  |  |  |  |
|  | Were the horizontal and vertical tolerances met? |  | 4.3 |  |  |  |  |
|  | Were the geogrids placed normal to the wall face? |  | 4.3 |  |  |  |  |
|  | Was the backfill placed in 8” lifts? |  | 4.4 |  |  |  |  |
|  | Was the backfill compacted to the specification requirements? |  | 4.4 |  |  |  |  |
|  | Was the backfill within 3 feet of the wall compacted with a light tamper? |  | 4.4 |  |  |  |  |
|  | Did a manufacture’s representative inspect the site during the wall construction? |  | 4.0 |  |  |  |  |
|  | Did the soils consultant properly take the compaction tests? |  | 4.0 |  |  |  |  |
|  | Was the coping and traffic barrier constructed properly? |  | 2.8 |  |  |  |  |
|  | Were the pile sleeves constructed properly? |  | 6.0 |  |  |  |  |

SS 832 Temporary Sediment and Erosion Control

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Have you read the 401, 404 permits? |  | 832.01 |  |  |  |  |
|  | Has the contractor furnished a permit to fill in water areas (ponds, lakes) out side the project work limits? See 107.19 |  | 832.01 |  |  |  |  |
|  | Are there SWPPP and a Co-Permittee form required |  | 832.01 |  |  |  |  |
|  | Has a NOI been filed? |  | 832.02 |  |  |  |  |
|  | Does the filter fabric material or hay or straw bale material for bale filter dikes meet the standard drawing DM-4.3/4.4 30 inch (0.8m). |  | 832.03 |  |  |  |  |
|  | Does the Construction Fence meet standard drawing DM-4.3 |  | 832.03 |  |  |  |  |
|  | Are the dikes per standard drawing DM-4.3 |  | 832.03 |  |  |  |  |
|  | Does the filter fabric material for the ditch checks meet the standard drawing DM-4.4 30 inch (0.8m). |  | 832.03 |  |  |  |  |
|  | Does the rock material for ditch checks meet the standard drawing DM-4.4 size No.1 through No. 4 rock? |  | 832.03 |  |  |  |  |
|  | Does the filter fabric material for inlet protection meet the standard drawing DM4.4 18 in. (0.5m) |  | 832.03 |  |  |  |  |
|  | Does the filter fabric material for perimeter controls meet the standard drawing DM-4.4 30 inch (0.8m) |  | 832.03 |  |  |  |  |
|  | Is the rock channel protection type C or D per standard drawing DM-4.3/4.4? |  | 832.03 |  |  |  |  |
|  | Does the excavation and embankment material for sediment basins and dams the meet the standard drawing DM-4.3. |  | 832.03 |  |  |  |  |
|  | Does the rock material for sediment basins and dams the meet the standard drawing DM-4.3 601 rock channel protection type C or D. |  | 832.03 |  |  |  |  |
|  | Does the pipe material for slope drains meet the standard drawing DM-4.3 corrugated steel pipe or corrugated or smooth plastic pipe with pipe caps? |  | 832.03 |  |  |  |  |
|  | Does the rock channel protection material meet the slope drain standard drawing DM-4.3 601 Type C or D.? |  | 832.03 |  |  |  |  |
|  | Has the contractor furnished TSEC BMP? |  | 832.04 |  |  |  |  |
|  | Have any provisions been enacted. |  | 832.05 |  |  |  |  |
|  | Which requirement is this project under A- G.? |  | 832.06 |  |  |  |  |
|  | Have all TSEC BMP met the material specifications? |  | 832.07 |  |  |  |  |
|  | Which TSEC BMP has been furnished? |  | 832.08 |  |  |  |  |
|  | Perimeter Controls in place |  | 832.08 |  |  |  |  |
|  | Inlet Protection in place |  | 832.08 |  |  |  |  |
|  | Construction Seeding and Mulching in place |  | 832.08 |  |  |  |  |
|  | Slope Protection in place |  | 832.08 |  |  |  |  |
|  | Ditch Checks and Ditch Protection in place |  | 832.08 |  |  |  |  |
|  | Filter fabric Fence in place |  | 832.08 |  |  |  |  |
|  | Rock in place |  | 832.08 |  |  |  |  |
|  | Bale Filter Dike in place |  | 832.08 |  |  |  |  |
|  | Straw or Hay Bales in place |  | 832.08 |  |  |  |  |
|  | Filter Fabric Fence in place |  | 832.08 |  |  |  |  |
|  | Sediment Basins in place |  | 832.08 |  |  |  |  |
|  | River, Stream, and Water Body Protection in place |  | 832.08 |  |  |  |  |
|  | Stream Relocation in place |  | 832.08 |  |  |  |  |
|  | Concrete Washout Areas in place |  | 832.08 |  |  |  |  |
|  | TSEC BMP in place |  | 832.08 |  |  |  |  |
|  | Project Access TSEC in place in place |  | 832.08 |  |  |  |  |
|  | BMP Locations in place |  | 832.08 |  |  |  |  |
|  | Project Fueling and Refueling TSEC BMP in place |  | 832.08 |  |  |  |  |
|  | All other TSEC BMP in place |  | 832.08 |  |  |  |  |
|  | Has a permit been issued for a Causeways or Access Fills to the Department, or the Contractor? |  | 832.09 |  |  |  |  |
|  | No payment has been made for Causeways and Access Fills |  | 832.10 |  |  |  |  |
|  | Has all sediment been removed per CMS 105.16? |  | 832.11 |  |  |  |  |
|  | Is a SWPPP required for this project? |  | 832.12 |  |  |  |  |
|  | Has the SWPPP been accepted with a P.E. stamp? |  | 832.13 |  |  |  |  |
|  | Is the project receiving a copy of the required inspection reports? |  | 832.14 |  |  |  |  |
|  | Is the amount due being ducted from the 832 erosion control amount? |  | 832.15 |  |  |  |  |
|  | Has one each been made for a SWPPP? |  | 832.16 |  |  |  |  |
|  | Has measurements been made for all the TSEC BMP? |  | 832.16 |  |  |  |  |
|  | Has payment been made for a SWPPP once only? |  | 832.17 |  |  |  |  |
|  | Has Payment been made for all the TSEC BMP? |  | 832.17 |  |  |  |  |

Item 603 Pipe Culverts, Sewers, and Drains

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Do you have the current copy of the Inspection Documentation Form? |  | 603.01 |  |  |  |  |
|  | Do you know what Type of Conduit is on the job? Culverts, Storm Sewer under Pavement, Storm Sewer not under Pavement, Drive Pipes, Small Drain Connections, Underdrain Outlets |  | 603.02 |  |  |  |  |
|  | Is the Inspection Documentation Form being filled out? |  | 603.02 |  |  |  |  |
|  | Has the Trench Excavation Method been recorded? |  | 603.03 |  |  |  |  |
|  | Has the Bedding Construction been recorded? |  | 603.04 |  |  |  |  |
|  | Has the Laying Conduit Method been recorded? |  | 603.05 |  |  |  |  |
|  | Has the Jointing Conduit Method been recorded? |  | 603.06 |  |  |  |  |
|  | Has the Waterproofing Method been recorded? |  | 603.07 |  |  |  |  |
|  | Has the type of the Backfill Method been recorded? |  | 603.08 |  |  |  |  |
|  | Has the compaction checks of the bedding been recorded? |  | 603.09 |  |  |  |  |
|  | Has the compaction checks of the backfill been recorded? |  | 603.09 |  |  |  |  |
|  | How the site was restored (type of pavement replacement) been recorded? |  | 603.10 |  |  |  |  |
|  | Has the existing or proposed pipe being field paved method been recorded? |  | 603.11 |  |  |  |  |
|  | Has pipe measured method (invert or center to center) been recorded? |  | 603.12 |  |  |  |  |
|  | Has the length paid for per day of work been recorded? |  | 603.13 |  |  |  |  |

Item 401 Asphalt Pavement

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Is the surface properly cleaned prior to asphalt placement? |  | 401.14, 407.05 |  |  |  |  |
|  | Is the proper tack material used ? |  | 407.02 |  |  |  |  |
|  | Is the tack coat uniformly applied? |  | 407.06 |  |  |  |  |
|  | How uniform is the tack application (5= uniform, 0= thin ribbons)? |  |  |  |  |  |  |
|  | If cover aggregate was used, was it uniformly applied? |  | 407.07 |  |  |  |  |
|  | If a surface course is being laid, is the vertical face of the longitudinal cold joint sealed? |  | 401.07 |  |  |  |  |
|  | Is the intermediate course tack coated prior to placing the surface course? |  | 401.14, 407.06 |  |  |  |  |
|  | If the existing pavement is concrete, was rubberized asphalt emulsion, 702.13, used? |  | 407.06 |  |  |  |  |
|  | Is the existing pavement dry? |  | 401.06 |  |  |  |  |
|  | Does the existing pavement surface temperature exceed the minimum allowed? |  | 401.06 |  |  |  |  |
|  | If a surface course is being placed, does the air temperature exceed 40 F? |  | 401.06 |  |  |  |  |
|  | Is the haul distance from the asphalt plant less than 50 miles? |  | 401.11 |  |  |  |  |
|  | Is the truck tarp covering the mix when the truck arrives at the paver? |  | 401.11 |  |  |  |  |
|  | If air temps below 50 F and the haul distance is greater than 20 miles, are the truck beds insulated? |  | 401.11 |  |  |  |  |
|  | Is a screed or screed extension used to match a previously placed pavement course? |  | 401.15 |  |  |  |  |
|  | Does screed extension meet the auger and heating requirements? |  | 401.15 |  |  |  |  |
|  | Is the asphalt concrete uniform in composition and surface texture (no segregation)? |  | 401.15 |  |  |  |  |
|  | What is the severity or segregation (5= None; 0 = High Severity)? |  | 401.15 |  |  |  |  |
|  | Is the mix delivered at a uniform, continuous rate? |  | 401.15 |  |  |  |  |
|  | Are delivery trucks bumping the paving machine? |  | 401.15 |  |  |  |  |
|  | Is the paving machine leaving any unusual marks or streaks in the pavement? |  | 401.15 |  |  |  |  |
|  | Is mix temperature < 325 F (163 C) for normal mixes or < 350 F (170 C) for polymer modified mixes? |  | 401.15 702.00 |  |  |  |  |
|  | Is “blue smoke” observed during the delivery of the asphalt mix? |  | 401.15 |  |  |  |  |
|  | Is “drain down” of the asphalt cement observed during the delivery of the mix? |  | 401.15 |  |  |  |  |
|  | Is all aggregate uniformly coated when delivered to the paving machine? |  | 401.15 |  |  |  |  |
|  | Was the mix slumped down in the truck when it was delivered? |  | 401.15 |  |  |  |  |
|  | Is the delivered asphalt mix workable? |  | 401.15 |  |  |  |  |
|  | Is the delivered mix tender? |  | 401.15 |  |  |  |  |
|  | Has any asphalt been rejected on the project? |  | 401.15 |  |  |  |  |
|  | Has the required placement rate been calculated? |  | 401.19 |  |  |  |  |
|  | Has the actual placement rate been calculated? |  | 401.19 |  |  |  |  |
|  | How often is the actual placement rate calculated? |  | 401.19 |  |  |  |  |
|  | Is the actual placement rate within the proper tolerance from the required placement rate? |  | 401.19 |  |  |  |  |
|  | If traffic is permitted to cross the edge of the new asphalt mat, is the adjacent lane placed within 24 hours? |  | 401.17 |  |  |  |  |
|  | If a hot joint is made by the use of 2 pavers, are delivery trucks alternating between them? |  | 401.17 |  |  |  |  |
|  | On 446 projects, is a hot longitudinal joint made between the mainline lane and the adjoining berm? |  | 401.17 446.06 |  |  |  |  |
|  | Are the random sample locations determined by the project within 24 hours after the pavement is placed? |  | 446.05 |  |  |  |  |
|  | Does the Contractor obtain core samples within 48 hours after the asphalt mat is placed? |  | 446.05 |  |  |  |  |
|  | Are core sample holes properly filled by the Contractor? |  | 446.05 |  |  |  |  |
|  | Are core samples properly stored? |  | 446.05 |  |  |  |  |
|  | Are core samples transported to the District in a timely manner? |  | 446.05 |  |  |  |  |
|  | Is a daily plate sample being taken? |  | 446.05 |  |  |  |  |
|  | Have sample locations been properly picked? |  | 448.04 |  |  |  |  |
|  | If mat samples are being obtained, are proper procedures being used? |  | 448.04 |  |  |  |  |
|  | If paver hopper samples are taken, are the proper procedures being used? |  | 448.04 |  |  |  |  |
|  | Are samples properly identified, wrapped and shipped to the District Lab? |  | 448.04 |  |  |  |  |
|  | Has the rate of placement been calculated in square yards (square meters) per hour (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Has the maximum capacity of the roller train been calculated (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Is the maximum capacity greater than the rate of placement (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Does the roller train meet the specifications for the mix being placed (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Base course: Type 1 pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | Intermediate and Surface Courses: three wheel roller and pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | Variable depth courses: Steel and pneumatic tire roller? |  | 401.13, 401.16 |  |  |  |  |
|  | If a vibratory roller is used, is the course thickness less than 1.5 inches (38 mm) (NA for 446)? |  | 401.13, 401.16 |  |  |  |  |
|  | Is the longitudinal joint being compacted first? |  | 401.13, 401.16 |  |  |  |  |
|  | On 446 projects, is the berm compacted with the same roller coverage as the mainline pavement? |  | 401.13, 401.16 446.05 |  |  |  |  |

Item 255 Full Depth Rigid Pavement Removal and Rigid Replacement

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Are diamond saws used to accomplish the full depth pavement sawing? |  | 255.03 |  |  |  |  |
|  | Is the lift out method being used for pavement removal? |  | 255.03 |  |  |  |  |
|  | If the pavement edge is damaged does Contractor’s re-saw with deduct (255.03)? Is the repair area compacted after removal of pavement? |  | 255.04 |  |  |  |  |
|  | Are hydraulic or electric drills used for drilling dowels/tiebar holes? |  | 255.05 |  |  |  |  |
|  | Are dowel/tiebar holes drilled to the specified diameter of 1 5/8 inch? |  | BP-2.5 |  |  |  |  |
|  | Are clear or white opaque plastic grout retention disc used? |  | BP-2.5 |  |  |  |  |
|  | Is sufficient dowel grout pneumatically injected into the rear of hole? |  | 255.05 |  |  |  |  |
|  | Are smooth dowels coated with a thin coating of oil prior to placing? |  | BP-2.5 |  |  |  |  |
|  | Does the grout firmly anchor the dowel or tiebar within 30 minutes? |  | 255.02 |  |  |  |  |
|  | Are dowels/tiebars held in proper alignment until grout has hardened? |  | 255.05 |  |  |  |  |
|  | Is the dowel grout allowed to harden prior to placing the concrete? |  | 255.06 |  |  |  |  |
|  | Do patches > 10 feet long have tiebars in the joint between lanes? |  | BP-2.5 |  |  |  |  |
|  | Is the pavement edge formed? |  |  |  |  |  |  |
|  | Is the area and full depth repair consolidated with an internal vibrator? |  |  |  |  |  |  |
|  | Are patch areas tested with a 10 foot straight edge a smooth ride over? |  |  |  |  |  |  |
|  | Surface texture on the new patch the same as the adjacent pavement? |  |  |  |  |  |  |
|  | Is curing compound applied to the surface at the proper rate? |  |  |  |  |  |  |
|  | Steel mesh installed in all patches > 10 feet in length? |  | BP-2.5 |  |  |  |  |
|  | Steel mesh installed in repairs opened to traffic within 24 hours? |  | BP-2.5 |  |  |  |  |
|  | Are insulating boards used to cover fast setting (Class FS) repairs? |  | 499.05 A |  |  |  |  |
|  | Are the vertical faces of the joints cleaned by abrasive blasting? |  | 255.06 |  |  |  |  |
|  | Joint seal material maintained ¼ inch below the top of pavement? |  | 255.06 |  |  |  |  |
|  | Bond breaker tape installed under the hot applied joint sealer? |  | BP-2.5 |  |  |  |  |
|  | Are unfilled repairs < 10 feet long covered with steel plates overnight? |  | 255.08 |  |  |  |  |
|  | Repairs filled when work is suspended for a Holiday or a weekend? |  | 255.08 |  |  |  |  |

Item 451 Rigid Pavement Construction

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Proper Diameter Dowels |  | Table 451.08-1 |  |  |  |  |
|  | Dowels and Baskets wires epoxy coated |  | 709.13 |  |  |  |  |
|  | Dowels anchored to base using approved method |  | 451.08B |  |  |  |  |
|  | Are dowels are parallel to the subbase |  | BP 2.2 |  |  |  |  |
|  | Are free end of dowels coated with light oil within 2 hours of paving |  | 451.08 B |  |  |  |  |
|  | At expansion joints, board fits tight, caps installed & dowels oiled |  | 451.08 C |  |  |  |  |
|  | Shipping wires removed immediately prior to paving |  | 451.08 B |  |  |  |  |
|  | Dowel basket exposure to weather and sunlight limited to 180 days |  | 709.13 |  |  |  |  |
|  | Are tiebars or hook bolt spaced properly @ 26 or 20 inch centers |  | BP-2.1, TABLE A |  |  |  |  |
|  | Are tiebars epoxy coated |  | 451.02, 709.00 |  |  |  |  |
|  | Are tiebars or hook bolts the proper diameter and length |  | BP- 2.1 |  |  |  |  |
|  | Tiebars installed by mechanical equipment or secured by chairs |  | 451.08 A |  |  |  |  |
|  | Tiebars being installed at mid-depth at longitudinal joints |  | BP-2.1 |  |  |  |  |
|  | Tiebars kept proper distance from dowel baskets |  | BP-2.1)? (12 to 18 inches |  |  |  |  |
|  | Paving machine is self propelled and has internal vibrators |  | 451.03 |  |  |  |  |
|  | Paving machine has a working vibration monitoring system |  | 451.03 |  |  |  |  |
|  | The Contractor has a separate spreader in the paving train |  | 451.03)? (>10,000 SY) |  |  |  |  |
|  | The vibrators on the paving machine shut off when the paver stops |  | 451.06 |  |  |  |  |
|  | Vibrators are within frequency range of 7000 to 11,000 impulses per minute |  | 451.03 |  |  |  |  |
|  | Concrete forms are the proper depth for the thickness specified |  | 451.03 A |  |  |  |  |
|  | Concrete forms cleaned and oiled prior to use |  | 451.03 A |  |  |  |  |
|  | Concrete forms base width is equal to depth of the form |  | 451.03 A |  |  |  |  |
|  | Slip form paver is controlled from a preset grade line |  | 451.03 B |  |  |  |  |
|  | Self propelled curing machine with wind protection and shielding used |  | 451.10 |  |  |  |  |
|  | Has the subbase been thoroughly moistened with water prior to paving |  | 451.06 |  |  |  |  |
|  | Concrete tests performed on concrete taken from the point of use |  | MOP |  |  |  |  |
|  | Concrete is spread uniformly full width of the pavement without voids |  | MOP |  |  |  |  |
|  | Paving mesh is being installed at the proper depth and location |  | BP-1.1 |  |  |  |  |
|  | Thickness checks made during paving to verify pavement thickness |  | MOP |  |  |  |  |
|  | Temperature of the plastic concrete maintained at 90 F or less |  | 451.06 |  |  |  |  |
|  | Two concrete test beams made every 7500 square yards |  | 700 |  |  |  |  |
|  | Coarse aggregates used has been tested for d-crack susceptibility |  | 499.02, 703.13 |  |  |  |  |
|  | Dowel basket assemblies vibrated by separate internal vibrator |  | 451.06 |  |  |  |  |
|  | Is a 10-ft checking straight edge used after finishing? Pavement edge rounded with edging tool and tool marks removed? |  |  |  |  |  |  |
|  | Broom or turf drag used to provide a uniform gritty surface before grooving? |  |  |  |  |  |  |
|  | Transverse grooves applied at the correct depth, width and spacing? |  |  |  |  |  |  |
|  | Station numbers stenciled at proper location? |  |  |  |  |  |  |
|  | Curing compound applied after water sheen has disappeared |  | 451.10 |  |  |  |  |
|  | Curing compound properly agitated prior to use |  | 451.10 |  |  |  |  |
|  | Daily checks made on usage to assure application rate is maintained |  | MOP |  |  |  |  |
|  | Pavement edges sprayed with curing membrane after form removal |  | 451.10 |  |  |  |  |
|  | Traffic kept off concrete pavement for 7 days |  | 451.16 |  |  |  |  |
|  | Pavement not opened for 7 days? ...5 days |  | 451.14 |  |  |  |  |
|  | Does the contractor have a standby saw |  | 451.08 D |  |  |  |  |
|  | Sawing depth checked to assure compliance with the specifications |  | 451.08 D |  |  |  |  |
|  | Timely sawing so that minimal raveling occurs |  | 451.08 D |  |  |  |  |
|  | Joints sealed prior to opening the pavement to construction traffic |  | 451.15 |  |  |  |  |
|  | Saw cuts straight and perpendicular to the pavement edge |  | 451.08 |  |  |  |  |
|  | Are saw cuts cleaned properly prior to sealing the joint opening |  | 451.08 D |  |  |  |  |
|  | Elongation of 705.11, preformed elastomeric sealer limited to 5 % |  | 451.15 A |  |  |  |  |

Item 509 Reinforcing Steel

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was damaged epoxy coating repaired when required? |  | 509.09 |  |  |  |  |
|  | Was patched epoxy coating allowed to cure prior to placing concrete? |  | 509.09 |  |  |  |  |
|  | Was reinforcing steel properly stored and kept free of dirt? |  | 509 |  |  |  |  |
|  | Were the proper clearances maintained? |  | 509.04 |  |  |  |  |
|  | Were the supports properly spaced? |  | 509.08 |  |  |  |  |
|  | Were epoxy coated rebars tied at every intersection except when spacing of intersections is less then 1 foot (0.3 m) in each direction in which case they should be tied at every other intersection? |  | 509.04 |  |  |  |  |
|  | Was reinforcing steel adequately tied so that it has firmly held in position during the placing and setting of concrete? |  | 509.04 |  |  |  |  |

Item 511 Concrete

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was concrete finished without water or evaporation retardant being improperly used? |  | 511.10 |  |  |  |  |
|  | Was proper curing material properly applied as soon as possible 511.17, |  | 511.17 |  |  |  |  |
|  | If water curing was used, was burlap kept wet during the required curing period? |  | 511.17 |  |  |  |  |
|  | Was superstructure concrete properly textured? |  | 511.19, 511.20 |  |  |  |  |
|  | Was a dry run made prior to placing superstructure concrete? |  | 511 |  |  |  |  |
|  | Was a vibrating roller or vibrating pan used to place high performance concrete? |  | 511.19 |  |  |  |  |
|  | If a vibrating roller was used, did the fins protrude more then 1/4 from the roller? |  | 511.19 |  |  |  |  |
|  | Did the contractor provide the necessary verification pertaining to the vibrating frequencies of the vibrating pan or roller |  | 511.19 |  |  |  |  |

Item 513 Structural Steel

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Were welders qualified? |  | 513, S 1011 |  |  |  |  |
|  | Prior to welding, was required preheating of the structural steel accomplished? |  | 513 |  |  |  |  |
|  | Are fillet welds the proper size? |  | 513 |  |  |  |  |
|  | After the electrodes have been exposed to the atmosphere, were they properly dried and stored? |  | 513 |  |  |  |  |
|  | Were 50% of the bolt holes filled with a combination of drift pins and snug tight bolts? |  | 513.20 |  |  |  |  |
|  | Were all bolts replaced which were installed prior to inserting drift pins? |  | 513.20 |  |  |  |  |
|  | Were bolts properly match marked? |  | 513.22 |  |  |  |  |
|  | Were bend tests on shear studs performed at the beginning of each work day? |  | 513.22 |  |  |  |  |
|  | Were bend tests on shear studs performed when welding has been interrupted for an hour or more? |  | Bridge Welding Code 7.8.1 |  |  |  |  |
|  | Were all studs inspected for a 360 degree flash? |  | Bridge Welding Code 7.8.1 |  |  |  |  |
|  | Were all stud welding operators qualified? |  | Bridge Welding Code 7.7.4 |  |  |  |  |
|  | Were all studs given a light blow with a hammer? |  | S 1011.7.8.1 |  |  |  |  |

Item 514 Structural Steel Painting

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was the Engineer provided with all the required testing equipment? |  | 514.04 |  |  |  |  |
|  | Was the quality control specialist properly trained? |  | 514.03 |  |  |  |  |
|  | If production work was taking place, was the quality control specialist involved in any other activities not related to quality control? |  | 514.03 |  |  |  |  |
|  | Was the quality control specialist properly equipped? |  | 514.03 |  |  |  |  |
|  | Was the quality control specialist able to climb? |  | 514.03 |  |  |  |  |
|  | Was there a MSDS provided for all abrasives, paints, and thinners? |  | 514.08 |  |  |  |  |
|  | Was the steel temperature at least 5 degrees above dew point before blasting or painting the steel? |  | 514.05 |  |  |  |  |
|  | Were air compressors checked every four hours? |  | 514.03 |  |  |  |  |
|  | Was the thinner from the same manufacture as the paint? |  | 514.14 |  |  |  |  |
|  | Were printed instructions from the manufacture available pertaining to thinning of the paint? |  | 514.14 |  |  |  |  |
|  | Were shear mixers used to mix the paint? |  | 514.14 |  |  |  |  |
|  | Was primer continuously mixed during application? |  | 514.14 |  |  |  |  |
|  | Was relative humidity below 85% during application of paint? |  | 514.06 B |  |  |  |  |
|  | Was proper scaffolding used for inspection? |  | 514.09 |  |  |  |  |
|  | Were the tarps installed correctly? |  | 514.11D |  |  |  |  |
|  | Was all equipment parked on ground covers? |  | 514.11D |  |  |  |  |
|  | Was the debris collected and stored at the end of each day? |  | 514.11D |  |  |  |  |
|  | Was the debris properly stored? |  | 514.11D |  |  |  |  |
|  | Were debris sampled within the first week of production blasting? |  | 514.11D |  |  |  |  |
|  | Was hazardous waste disposed of within 60 days after it was generated? |  | 514.11D |  |  |  |  |
|  | Were all damaged areas properly repaired? |  | 514.19 |  |  |  |  |

Item 524 Drilled Shafts

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Did contractor submit installation plan at least 14 days prior constructing drilled shafts? |  | 524.03 |  |  |  |  |
|  | Did contractor’s installation plan contain include all the required information? |  | 524.03 |  |  |  |  |
|  | If the dry method was used, were the sides stable and any flow of water into the excavation less then 300 mm (12 inches) per hour? |  | 524.04A |  |  |  |  |
|  | If wet construction method is used for a shaft which is not socketed into bedrock, was the water or slurry fluid level during the drilling operation inside the drilled shaft higher then the static water table? |  | 524.04B |  |  |  |  |
|  | If temporary casing is used, was at least a 5 foot (1.5 meter) head of concrete maintained above the bottom of the casing? |  | 524.04C |  |  |  |  |
|  | If permanent casing is required, did the contractor excavate material beyond the limits of the casing? |  | 524.04D |  |  |  |  |
|  | Were spacer attached to the reinforcing cage at quarter points? |  | 524.09 |  |  |  |  |
|  | Were round spacers of the proper size used to maintain the required clearance, ie.3 inches (75 mm) for shaft diameters up to 4 feet (1.2meters) and 6 inches (150 mm) for shaft diameters larger than 4 feet (1.2 meters)? |  | 524.09 |  |  |  |  |
|  | If concrete is placed underwater, is it placed in one continuous operation? |  | 524.10 |  |  |  |  |
|  | If a tremie is used to place concrete underwater, is the end of the tremie maintained at least 10 feet (3 meters) below the surface of the concrete? |  | 524.12 |  |  |  |  |
|  | If a pump is used to place concrete underwater, is the end of the pump maintained at least 10 feet (3 meters) below the surface of the fresh concrete? |  | 524.13 |  |  |  |  |
|  | If a pump or tremie is used to place concrete under water, was the end of the pump or tremie properly plugged when it was placed into the water? |  | 524.12 / 524.13 |  |  |  |  |
|  | Where bottoms and sides of excavation checked for cleanliness? |  | 524.08 |  |  |  |  |
|  | Where tolerances checked? |  | 524.14 |  |  |  |  |
|  | If wet method is used, after the shaft is full is concrete continually placed into the shaft until acceptable quality of concrete is evident at the top of the shaft? |  | 524.10 |  |  |  |  |

Item 614 Maintaining Traffic

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | DRIVE THROUGH |  |  |  |  |  |  |
|  | ARE MANEUVERS DIFFICULT OR UNEXPECTED? |  |  |  |  |  |  |
|  | ADEQUATE WARNING OF CONDITIONS? |  |  |  |  |  |  |
|  | IS SIGNING CLEAR / UNCLUTTERED AND PROPERLY SPACED? |  |  |  |  |  |  |
|  | ARE TRAFFIC CONTROL DEVICES SUFFICIENTLY VISIBLE? |  |  |  |  |  |  |
|  | ANY INCIDENTS, CONGESTION POINTS OR DELAYS? |  |  |  |  |  |  |
|  | SIGNS |  |  |  |  |  |  |
|  | NEED TO BE: REMOVED / REPOSITIONED /COVERED |  |  |  |  |  |  |
|  | NEED: CLEANING / REPLACEMENT |  |  |  |  |  |  |
|  | ADDITIONAL SIGNS NEEDED |  |  |  |  |  |  |
|  | CONFLICTING SIGNS: PERMANENT / TEMPORARY |  |  |  |  |  |  |
|  | NON-APPROVED SIGN SUPPORT |  |  |  |  |  |  |
|  | VIEW BLOCKED BY VEGETATION OR OTHER SIGNS |  |  |  |  |  |  |
|  | WHAT KIND OF SHEET DOES THE SIGN HAVE TYPE G OR H |  |  |  |  |  |  |
|  | PORTABLE CHANGEABLE MESSAGE SIGNS |  |  |  |  |  |  |
|  | OPERATION DOES NOT MEET GUIDELINES? |  |  |  |  |  |  |
|  | INAPPROPRIATE MESSAGE? |  |  |  |  |  |  |
|  | TOO MUCH INFORMATION? |  |  |  |  |  |  |
|  | NOT DIMMED AT NIGHT |  |  |  |  |  |  |
|  | NOT DELINEATED (NO CONES / BARRELS) |  |  |  |  |  |  |
|  | ARROW PANEL |  |  |  |  |  |  |
|  | INCORRECT PLACEMENT? |  |  |  |  |  |  |
|  | MALFUNCTION (BULB OUT, ETC.) |  |  |  |  |  |  |
|  | NOT DIMMED AT NIGHT |  |  |  |  |  |  |
|  | DRUMS = D / CONES = C / TUBULAR MARKERS = TM |  |  |  |  |  |  |
|  | TAPER LENGTH TOO SHORT |  |  |  |  |  |  |
|  | DEVICE SPACING TOO LONG |  |  |  |  |  |  |
|  | ADDITIONAL DEVICES NEEDED |  |  |  |  |  |  |
|  | MISALIGNED |  |  |  |  |  |  |
|  | CONDITION (REPAIR / CLEAN / REPLACE) |  |  |  |  |  |  |
|  | REFLECTIVE BANDS (MISSING / DAMAGED) |  |  |  |  |  |  |
|  | TRAFFIC BARRIER |  |  |  |  |  |  |
|  | IMPROPER BARRIER WALL FLARE |  |  |  |  |  |  |
|  | TERMINAL TREATMENT IN CLEAR ZONE |  |  |  |  |  |  |
|  | BARRIER NEEDS TO BE REALIGNED /REMOVED / CONNECTED |  |  |  |  |  |  |
|  | ATTENUATOR (REPAIR / REPLACE) |  |  |  |  |  |  |
|  | DELINEATORS (CLEAN / ADDITIONAL NEEDED) |  |  |  |  |  |  |
|  | OBJECT MARKERS (CLEAN / ADDITIONAL NEEDED /REPLACE) |  |  |  |  |  |  |
|  | GLARE SCREEN (CLEAN / REPAIR) |  |  |  |  |  |  |
|  | FLAGGING OPERATION |  |  |  |  |  |  |
|  | ADDITIONAL SIGNING NEEDED? |  |  |  |  |  |  |
|  | ARE FLAGGERS POSITIONED CORRECTLY? |  |  |  |  |  |  |
|  | ARE FLAGGERS HIGHLY VISIBLE? |  |  |  |  |  |  |
|  | ARE FLAGGERS PROPERLY ATTIRED? |  |  |  |  |  |  |
|  | ARE FLAGGERS FLAGGING CORRECTLY? |  |  |  |  |  |  |
|  | PAVEMENT MARKINGS |  |  |  |  |  |  |
|  | PERMANENT = PERM. / TEMPORARY = TEMP. MATERIAL TYPE: PAINT = P / TAPE = T |  |  |  |  |  |  |
|  | REMOVE |  |  |  |  |  |  |
|  | REPAIR |  |  |  |  |  |  |
|  | NEED ADDITIONAL |  |  |  |  |  |  |
|  | CONFUSING |  |  |  |  |  |  |
|  | RAISED PAVEMENT MARKERS PERMANENT = P / CONSTRUCTION = C |  |  |  |  |  |  |
|  | MISSING |  |  |  |  |  |  |
|  | NEED ADDITIONAL |  |  |  |  |  |  |
|  | REMOVE |  |  |  |  |  |  |
|  | DO NOT CORRESPOND TO PAVEMENT MARKINGS |  |  |  |  |  |  |
|  | CRASH INFORMATION |  |  |  |  |  |  |
|  | OH-1 CRASH REPORTS RETREIVED WEEKLY FROM ALL JURISDICTIONS? |  |  |  |  |  |  |
|  | CRASH REPORTS EVALUATED WEEKLY? |  |  |  |  |  |  |
|  | POTENTIAL SOLUTIONS DISCOVERED? |  |  |  |  |  |  |
|  | CRASH REPORTS & SOLUTIONS DOCUMENTED AND PRESENTED THE ENGINEER |  |  |  |  |  |  |
|  | SOLUTION(S) TO BE IMPLEMENTED WITHIN ONE WEEK? |  |  |  |  |  |  |
|  | MISCELLANEOUS |  |  |  |  |  |  |
|  | ADEQUATE BUFFER SPACE? |  |  |  |  |  |  |
|  | IS THE WORK AREA PROTECTED? |  |  |  |  |  |  |
|  | EQUIPMENT PROPERLY STORED / PROTECTED? |  |  |  |  |  |  |
|  | ARE LANE CLOSURES IN ACCORDANCE WITH ALLOWED HOURS (PLCM, PLANS, CONTRACT)? |  |  |  |  |  |  |
|  | OTHER (COMMENT ON ANY OTHER DEVICES / SITUATIONS (DESCRIBE IN COMMENTS) |  |  |  |  |  |  |

Item 621 Raised Pavement Markers

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Materials conform to following specs: 721.01, 721.02, 721.03, 721.04 |  | 621.02 |  |  |  |  |
|  | 621 Raised Pavement Markers (RPM) |  |  |  |  |  |  |
|  | Check depth and width of saw cut per Standard Const, Drawing TC 6510. |  | SCD 65.10 |  |  |  |  |
|  | Check before placement of the epoxy, the saw cut is clean of all loose material and dry. Saw cut should have 1/8 inch all sides clearance of the casting. |  | 621.04 |  |  |  |  |
|  | Verify ambient air temperature are at least 40degrees Fahrenheit (5 degrees Centigrade) and the pavement is dry. |  | 621.04 |  |  |  |  |
|  | Check epoxy is an A+B mixture, throughly mixed (grey color) and in accordance with manufacture’s recommendations. |  | 621.04 |  |  |  |  |
|  | Check that sufficient epoxy is in and between the slots to ensure that all voids beneath and around the casting are filled. |  | 621.04 |  |  |  |  |
|  | Placement of RPM castings shall be 6" from any construction joint (Lateral or Longitudinal). |  |  |  |  |  |  |
|  | Location and / stations are per Standard Const. Drawing. |  | SCD TC 65.11, TC 65.12 |  |  |  |  |
|  | Verify that all dirt, dust, oil, grease, rust, moisture, parts of damaged reflectors, or any foreign matter is removed that impairs adhesion of the reflector to the casting. |  | 621.04 |  |  |  |  |
|  | Verify reflector area of the castings shall be sandblasted to 80% bare metal. |  | 621.04 |  |  |  |  |
|  | Verify the application of adhesive is in a single bead, sufficient to squeeze out on all sides of the reflector when pressure is applied, to seat the reflector and seal out moisture. |  | 621.04 |  |  |  |  |

Item 640 Pavement Marking (641-647)

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Pavement Marking - General |  | 641 |  |  |  |  |
|  | Ambient air temperature, pavement temperature, type, and condition of existing pavement (1) |  | 641 |  |  |  |  |
|  | Location and/or stationing where work performed (2) |  | 641 |  |  |  |  |
|  | Width of line, lines straight, edges sharp, uniform retroreflective (3) |  | 641.03 |  |  |  |  |
|  | Type of Line - solid, dashed, dotted (4) |  | 641.03 |  |  |  |  |
|  | State if brooming was required (5) |  | 641.05 |  |  |  |  |
|  | Show amount of paint and beads required and amount used for each item (6) |  | 641.08 |  |  |  |  |
|  | Traffic Paint |  | 642 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5,6) |  | 642.01 |  |  |  |  |
|  | Material as per 740.02 and 740.09 |  | 642.02 |  |  |  |  |
|  | Paint quantity increased 25% for new pavement |  | 642.04 |  |  |  |  |
|  | Temperature above 40 F (4 C) |  | 642.04 |  |  |  |  |
|  | Polyester Pavement Marking |  | 643 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5,6) |  | 643.01 |  |  |  |  |
|  | Material as per 740.03 and 740.09 |  | 643.02 |  |  |  |  |
|  | Temperature above 50 F (10 C) |  | 643.04 |  |  |  |  |
|  | Thermoplastic Pavement Marking |  | 644 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5) |  | 644.01 |  |  |  |  |
|  | Material as per 740.04 and 740.09, Type C |  | 644.02 |  |  |  |  |
|  | Amount of thermoplastic and beads required and used. |  | 644.04 |  |  |  |  |
|  | Temperature requirements are outlined in 644.04 |  | 644.04 |  |  |  |  |
|  | Preformed Pavement Marking |  | 645 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5) |  | 645 |  |  |  |  |
|  | Materials as per 740.05 and 740.06 |  | 645.02 |  |  |  |  |
|  | Pavement markings applied per manufacturer’s recommendation. |  | 645.03 |  |  |  |  |
|  | Temperature requirements are outlined in 645.03 or per manufacturer’s recommendations |  | 645.03 |  |  |  |  |
|  | Epoxy Pavement Marking |  | 646 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5) |  | 646.01 |  |  |  |  |
|  | Materials as per 740.07 and 740.09 |  | 646.02 |  |  |  |  |
|  | Pavement and air temperature above 50 F (10 C) |  | 646.05 |  |  |  |  |
|  | Heat Fused Preformed Plastic Pavement Marking |  | 647 |  |  |  |  |
|  | General requirements per 641 (1,2,3,4,5) |  | 647.01 |  |  |  |  |
|  | Materials as per 740.08 and 740.09. |  | 647.02 |  |  |  |  |
|  | State type of material used, A, B, or C |  | 647.04 |  |  |  |  |