2007 Technical Process Review Checklist

Item 251 Partial Depth Pavement Repair

| District |  | Date |  | Project No. |  |  | Co/Rt/Sec. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |  |
|  | Review “√” |  | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |
| Removal of Existing Pavement |  |  |  |  |  |  |  |  |
| 1 |  |  | Rectangular areas that incorporate surface deterioration are marked. |  | 251.02 |  |  |  |
| 2 |  |  | Repairs at joints are the full width of the lane. |  | 251.02 |  |  |  |
| 3 |  |  | Pavement removed to the plan depth. |  | 251.02 |  |  |  |
| 4 |  |  | Removal done without damage to the existing pavement. |  | 251.02 |  |  |  |
| 5 |  |  | Transverse joints are trimmed to vertical 1.5 inches minimum |  | 251.03 |  |  |  |
| Placement of Asphalt concrete |  |  |  |  |  |  |  |  |
| 6 |  |  | Surfaces on which the asphalt concrete is to be placed are clean and free of contaminants. |  | CIM |  |  |  |
| 7 |  |  | Proper tack coat is used and thoroughly applied to exposed surfaces and fills cracks and joints. |  | 251.03 |  |  |  |
| 8 |  |  | 448 asphalt concrete temperature at time of placement. |  | JMF |  |  |  |
| 9 |  |  | 448 asphalt is used to an elevation that results in pavement flush to the top of existing pavement after compaction. |  | 251.03 |  |  |  |
| 10 |  |  | 10 foot straight edge is being used. Surface flush + ¼”. |  | CIM 251.03 |  |  |  |
| 11 |  |  | Proper compaction was performed. Type I pneumatic tire roller. |  | CIM 251.03 |  |  |  |
| 12 |  |  | Joints perimeters were sealed properly using approved 702.04 RS-1, RS-2, CRS-1 or CRS-2 |  | 251.03 |  |  |  |
| Documentation(Measurement & Payment) |  |  |  |  |  |  |  |  |
| 13 |  |  | Areas of repairs measured and converted to square yards by project personnel. |  | 251.04 251.05 |  |  |  |
| 14 |  |  | The location of the repair, dimensions and calculations are documented on a CA-D -6. |  | CIM |  |  |  |