2007 Technical Process Review Checklist

Item 253 Pavement Repair

| District |  | Date |  | Project No. |  |  | Co/Rt/Sec. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |  |
|  | Review “√” |  | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |
| Removal of Existing Pavement |  |  |  |  |  |  |  |  |
| 1 |  |  | Repair areas are marked. (Location and Limit). |  | 253.02 |  |  |  |
| 2 |  |  | Repairs are full depth. (Unless plan details otherwise.) |  | 253.02 |  |  |  |
| 3 |  |  | Adjacent Pavement or shoulder is damaged during the pavement sawing or pavement removal. |  | 253.02 |  |  |  |
| 4 |  |  | The subgrade/subbase is Shaped and compacted properly. |  | 253.03 |  |  |  |
| 5 |  |  | Transverse joints are trimmed to vertical 1.5 inches minimum |  | CIM |  |  |  |
| Placement of Asphalt concrete |  |  |  |  |  |  |  |  |
| 6 |  |  | Surfaces on which the asphalt concrete is to be placed are clean and free of contaminants. |  | 253.03401.14 |  |  |  |
| 7 |  |  | Proper tack coat is used and applied to exposed surfaces. |  | 253.03401.14 |  |  |  |
| 8 |  |  | Asphalt concrete used and thicknesses conform to plans. |  | 253.01 |  |  |  |
| 9 |  |  | Asphalt concrete temperature at time of placement. |  | CIM |  |  |  |
| 10 |  |  | Asphalt material is used to an elevation that results in pavement flush to the top of existing pavement after compaction. |  | 253.03 |  |  |  |
| 11 |  |  | 10 foot straight edge is being used. Surface flush + ¼”. |  | CIM |  |  |  |
| 12 |  |  | Proper compaction was performed. Type I pneumatic tire roller. |  | 253.03 401.13 |  |  |  |
| 13 |  |  | Joints perimeters were sealed properly using approved 702.04 Rs-1, RS-2, CRS-1 or CRS-2 |  | 253.03 702.04 |  |  |  |
| Documentation(Measurement & Payment) |  |  |  |  |  |  |  |  |
| 14 |  |  | Areas of repairs measured and converted to square yards by project personnel. |  | 253.04 253.05 |  |  |  |
| 15 |  |  | The location of the repair, dimensions and calculations are documented on a CA-D -6. |  | CIM |  |  |  |