2007 Technical Process Review Checklist

Item 401 Asphalt Concrete Pavements General

| District |  | Date |  |  | Project No. |  |  | Co/Rt/Sec. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item #: Ref #: |  |  |  | Contractor: |  |  | Project/District Contacts: |  |  |
|  | Review “√” |  | Specification Conformance Statement |  |  | CMS/ CIM | Measurements |  | Conformance Comments |
| Tack Coat |  |  |  |  |  |  |  |  |  |
| 1 |  |  | A notification received 24 hours and a meeting was held. |  |  | 401.07 MOP |  |  |  |
| 2 |  |  | The surface variations are within tolerances, dry and properly cleaned prior to asphalt placement. |  |  | 401.06 401.14 |  |  |  |
| 3 |  |  | Proper tack material is being used. Rubberized asphalt emulsion was used on existing concrete pavement. |  |  | 407.02 407.06 702.13 |  |  |  |
| 4 |  |  | The intermediate course is tacked prior to placing the surface course. Course that is open to traffic for more than 30 days. |  |  | 401.14 407.06 401.17 |  |  |  |
| 5 |  |  | Joint is tacked (0.25 gal/yd2) Cold joints on surface courses. |  |  | 401.17 |  |  |  |
| 6 |  |  | Contact surfaces (curbing, manholes, and vertical face of existing pavement…) are painted with a thin, uniform coating of asphalt material. |  |  | 401.14 |  |  |  |
| 7 |  |  | Tack is uniformly applied. (5= uniform, 0= thin ribbons) |  |  | 407.06 |  |  |  |
| 8 |  |  | Cover aggregate conforms to 703.06 is uniformly applied. 4 to 8 lbs/yd2 |  |  | 407.02 407.07 |  |  |  |
| Weather Limitations |  |  |  |  |  |  |  |  |  |
| 9 |  |  | The existing pavement surface temperature meets the minimum allowed. |  |  | 401.06 |  |  |  |
| 10 |  |  | Surface course is being placed, the air temperature does exceed 40 F. |  |  | 401.06 |  |  |  |
| 11 |  |  | Type 1H or polymer binder surface course is being placed, the surface and air temp. does exceed 50 F. |  |  | 401.06 |  |  |  |
| 12 |  |  | Type 1H or polymer binder surface course is being placed before November 1st. |  |  | 401.06 |  |  |  |
| Hauling |  |  |  |  |  |  |  |  |  |
| 13 |  |  | The haul distance from the asphalt plant to the project is less than 50 miles. |  |  | 401.11 |  |  |  |
| 14 |  |  | A truck tarp is covering the mix when the truck arrives at the paving machine. |  |  | 401.11 |  |  |  |
| 15 |  |  | Truck beds are insulated. (Temp. < 50 F) (Haul distance is > 20 miles) |  |  | 401.11 |  |  |  |
| 16 |  |  | Materials are excessively sticking to the truck beds. |  |  | 401.11 |  |  |  |
| Spreading and finishing |  |  |  |  |  |  |  |  |  |
| 17 |  |  | Spreading equipment is being approved & modified to prevent segregation. |  |  | 401.12 |  |  |  |
| 18 |  |  | A screed or screed extension is used to match a previously placed pavement course. |  |  | 401.15 |  |  |  |
| 19 |  |  | The screed extension meets the auger and heating requirements. |  |  | 401.15 |  |  |  |
| 20 |  |  | Material Transfer Vehicle is used (Nighttime Paving) |  |  | MOP |  |  |  |
| 21 |  |  | Lighting system provided (Nighttime Paving) |  |  | MOP |  |  |  |
| 22 |  |  | The asphalt concrete is uniform in composition and surface texture. (No segregation) |  |  | 401.15 |  |  |  |
| 23 |  |  | The rating of segregation (5 = None; 0 = High Severity) |  |  | 401.15 |  |  |  |
| 24 |  |  | The mix is delivered at a uniform and continuous rate. |  |  | 401.15 |  |  |  |
| 25 |  |  | The delivery trucks do not bump the paving machine. |  |  | 401.15 |  |  |  |
| 26 |  |  | The paving machine is not leaving any unusual marks or streaks in the pavement. |  |  | 401.15 |  |  |  |
| 27 |  |  | Max temperature does not exceed. 325 F for normal mixes 350 F for polymer modified mixes |  |  | 702.00 |  |  |  |
| 28 |  |  | Min temperature exceeded. 250 F for Item 301& 302 290 F for 1H & polymer Check BCJMF other mixes |  |  | 301.04 302.04 401.16 |  |  |  |
| 29 |  |  | “Blue smoke” is observed during the delivery of the asphalt mix. |  |  | 401.15 |  |  |  |
| 30 |  |  | “Drain down” of the asphalt cement is observed during the delivery. |  |  | 401.15 |  |  |  |
| 31 |  |  | Aggregate uniformly coated when delivered to the paving machine. |  |  | 401.15 |  |  |  |
| 32 |  |  | The mix is slumped down in the truck when delivered. |  |  | 401.15 |  |  |  |
| 33 |  |  | The delivered mix is workable. |  |  | 401.15 |  |  |  |
| 34 |  |  | The delivered mix is tender. |  |  | 401.15 |  |  |  |
| 35 |  |  | Asphalt mix was rejected on the project site. |  |  | 401.15 |  |  |  |
| 36 |  |  | Pavement is reviewed & inspected during daylight. (Nighttime Paving) |  |  | MOP |  |  |  |
| Rolling Train Capacity/ Placement Rate Calculation |  |  |  |  |  |  |  |  |  |
| 37 |  |  | The roller train conforms to the specifications. |  |  | 401.13 401.16 |  |  |  |
| 38 |  |  | Mat compacted thickness. Type II 3 inch Max (301) 6 inch Max (302) 4 inch Min & 7.75 inch Max |  |  | 401.14 301.04 302.04 |  |  |  |
| 39 |  |  | Three wheel roller and pneumatic tire roller are being used. Variable depth course requirement. |  |  | 401.13 401.16 |  |  |  |
| 40 |  |  | Vibratory roller is being used. Course thickness >1.5 inches |  |  | 401.13 401.16 |  |  |  |
| 41 |  |  | Type 1 pneumatic tire roller is being used. Base course requirement. |  |  | 401.13 401.16 |  |  |  |
| 42 |  |  | The maximum capacity (yd2/hr) of the roller train is calculated and converted to (Tons/hr). |  |  | 401.13 401.16 |  |  |  |
| 43 |  |  | The spreading rate of placement is calculated (Tons/hr). |  |  | 401.13 401.16 |  |  |  |
| 44 |  |  | Roller train capacity is greater than the rate of placement. |  |  | 401.13 401.16 |  |  |  |
| 45 |  |  | The shoulder is being compacted with the same roller coverage as the mainline pavement. (446) requirement. |  |  | 401.13 401.16 446.05 |  |  |  |
| 448 Acceptance (Density) |  |  |  |  |  |  |  |  |  |
| 46 |  |  | Nuclear or PQI Model 300 is being used. |  |  | 448.03 |  |  |  |
| 47 |  |  | QC tests every 1000 Ft are being recorded. |  |  | 448.03 |  |  |  |
| 48 |  |  | Two QA tests are being performed. |  |  | 448.03 |  |  |  |
| 49 |  |  | Any deductions as per SS 1055 have been made. |  |  | 448.03 |  |  |  |
| 50 |  |  | Does the contractor completed forms TE-Min Density Target and TE-Mat Density QCQA? |  |  | S 1055 |  |  |  |
| 446 Acceptance (Density) |  |  |  |  |  |  |  |  |  |
| 51 |  |  | Random sample locations are determined by the project within 48 hours after the pavement is placed. |  |  | 446.05 |  |  |  |
| 52 |  |  | The Contractor obtains core samples within 48 hours after the asphalt mat is placed. |  |  | 446.05 |  |  |  |
| 53 |  |  | The core sample holes are properly filled by the Contractor. |  |  | 446.05 |  |  |  |
| 54 |  |  | The core samples are properly stored. |  |  | 446.05 |  |  |  |
| 55 |  |  | The core samples are transported to the District in a timely manner. |  |  | 446.05 |  |  |  |
| Joints |  |  |  |  |  |  |  |  |  |
| 56 |  |  | The longitudinal joint is being compacted first. |  |  | 401.13 401.16 |  |  |  |
| 57 |  |  | Adjacent lane is placed within 24 hours whenever traffic is permitted to cross the edge of the mat. |  |  | 401.17 |  |  |  |
| 58 |  |  | Hot longitudinal joint is made between the mainline/ramp lane and the adjoining shoulder. (446) requirement |  |  | 401.17 446.06 |  |  |  |
| 59 |  |  | Delivery trucks are alternating between two pavers. Hot joint requirement. |  |  | 401.17 |  |  |  |
| 60 |  |  | Joints are set up at the proper height above the adjacent construction to receive maximum compaction. |  |  | 401.17 |  |  |  |
| 61 |  |  | The longitudinal joint is being compacted first w/3 wheel roller. Surface & intermediate only. |  |  | 401.17 |  |  |  |
| Surface Tolerances |  |  |  |  |  |  |  |  |  |
| 62 |  |  | Does surface smoothness apply to this project? |  |  | PN 420 |  |  |  |
| 63 |  |  | Did the contractor furnish 10 ft straight edge equipped with level? |  |  | 401.19 |  |  |  |
| 64 |  |  | Are surface tolerances as per spec. Long ¼” Trans 3/8” in 10 Ft |  |  | 401.19 |  |  |  |
| 65 |  |  | Did the contractor check the intermediate course at 50-Ft intervals before surface course? |  |  | 401.19 |  |  |  |
| 66 |  |  | Does longitudinal profile deviate more than 7/8 inch in 50 Ft? |  |  | 401.19 |  |  |  |
| Documentation(Measurement & Payment) |  |  |  |  |  |  |  |  |  |
| 67 |  |  | The required placement rate (yd3/Station) is calculated and converted to (Tons/Station) |  |  | 401.19 |  |  |  |
| 68 |  |  | The actual placement rate (Tons/Station) was calculated. |  |  | 401.19 |  |  |  |
| 69 |  |  | The actual placement rate is calculated _____ times a day. |  |  | 401.19 |  |  |  |
| 70 |  |  | The actual placement rate is within the proper tolerance (+ 5%). |  |  | 401.19 |  |  |  |
| 71 |  |  | Quantities paid do not exceed the quantities calculated from plan lines and dimensions. |  |  | 401.21 |  |  |  |
| 72 |  |  | Quantities (tons) are converted to yd3 & paid in accordance with contract items for each type. |  |  | 401.21 401.22 |  |  |  |
| 73 |  |  | The location & placement are documented on a CA-D-3A/3B and CA-FP-4. |  |  | CIM |  |  |  |