Critical Inspection Item List

| Item | Work Type | Critical Activities | Low Risk Activities |
| --- | --- | --- | --- |
| 202 | Removals | Structures over traffic | Increased when in roadway |
| 203, 208 | Excavation | Near Utilities, or Blasting |  |
| 204, 205, 206 | Embankment/ Subgrade compaction/ Lime & Cement Stabilized Subgrade/ Proof Rolling | Fills over 3 feet, proof rolling | Linear grading |
| 207, 670, 671 | Temporary Erosion & Sediment Controls | All initial installations and plan review | Maintenance and repair |
| 251, 252, 253, 255, 256 | Pavement Repair | Layout and reinforcement placement, material quality | Pavement removal, sawing and sealing. |
| 300 through 452 | Base Materials, Asphalt & Concrete Pavements | Placement, grade and thickness checks, material quality |  |
| 254 | Pavement Planing |  | Layout and profile checks |
| 500 through 526 | Structure | All dimensions, and plan checks, material quality | Setting bridge rail, sealing of concrete |
| 514 | Bridge Painting | Coating removal process, coating thickness, material quality | Debris collection, |
| 519 | Structure Patching | Material quality | Methods and materials and limit areas. |
| 602, 603, 604, 605, 638 | Drainage, Drainage Structures, and Waterlines | Bedding and backfilling, material quality |  |
| 606, 607 | Guardrail and Fence | Layout and material quality. | Post installation |

| 608, 609, 622 | Incidental Concrete | Layout and material quality |  |
| --- | --- | --- | --- |
| 610 | Retaining Walls | Reinforcing, backfill placement | Re-steel and forming as long it can be checked prior to concrete placement. |
| 526 | Approach Slabs | Reinforcing, thickness, and material quality |  |
| 615 | Temporary Roads & Pavements | In-depth inspection required |  |
|  | Incidental Items |  | Occasional basis |
| 614 | Maintenance of Traffic |  |  |
| 621 | Raised Pavement Markers | Casting depth and epoxy placement |  |
| 630, 631 | Traffic Signs & Supports | Stop signs and foundation location and construction. | Sign location can be done normal work day. Does not require continuous inspection. |
| 625 | Highway Lighting | Attention paid to proper foundation and underground work. | Does not require continuous inspection. |
| 632, 633 | Traffic Signals | Portions of work do not require continuous inspection. As per PE/PS |  |
| 641 through 647 | Pavement Marking | Lay out of new markings most critical. | Can be inspected after placement. |
| 651 through 660 | Seeding and Mulching |  | If work can be completed in less than one day additional time maybe required to determine quantities. |
| 661 through 666 | Landscape |  | Proper layout and assure items are being installed properly. Overtime not required. |