2007 Technical Process Review Checklist

Item 507 Bearing Piles

| District: Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: |  | Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Prime Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| # | Review “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements | Conformance Comments |  |
| Piles |  |  |  |  |  |  |  |
| 1 | √ | Piles of specified type, length and size were used. Tapered piles diameter ≥ 8”. |  | 507.02 507.06 |  |  |  |
| 2 | √ | Pile wall thickness (T) T ≥ R/900,00 ≥ ¼” Fluted wall thickness = T/1.4 |  | 507.06 |  |  |  |
| 3 | √ | Shoes or points projection ≤ ¼”. |  | 507.06 |  |  |  |
| 4 | √ | Mill certifications show domestic origin. |  | 507.03 |  |  |  |
| 5 | √ | Piles were driven to refusal or to ultimate bearing value. |  | 507.04 |  |  |  |
| 6 | √ | Piles were driven minimum 80% of estimated depth. |  | 507.04 |  |  |  |
| 7 | √ | Adjacent piles were rising while driving. |  | 507.04 |  |  |  |
| 8 | √ | Piles cut off at the plan angle and elevation. |  | 507.04 |  |  |  |
| 9 | √ | Piles are free of accumulated water or debris & Covered. |  | 507.06 |  |  |  |
| 10 | √ | Radius between placing concrete in piles and driving piles ≥ 15 FT. |  | 507.06 |  |  |  |
| 11 | √ | Piles spliced ≥ 3 FT above ground & driven ≥ 150 Blows. |  | 507.09 |  |  |  |
| 12 | √ | Splices for H-piles performed by a pre-qualified welder. |  | 507.09 513.21 |  |  |  |
| 13 | √ | Reinforcing steel erected per 524.09 and concrete placed per 524.11. |  | 507.06 |  |  |  |
| 14 | √ | Concrete used is Class C w/superplasticizer & Slump 7 + 1”. |  | 507.03 |  |  |  |
| Hammer |  |  |  |  |  |  |  |
| 15 | √ | Air-Operated Ram = 2700 Lb. |  | 507.04 |  |  |  |
| 16 | √ | Diesel W/ Ram = 2700 Lb. Stroke measure within 6” Gauge & graph convert pressure to energy |  | 507.04 |  |  |  |
| 17 | √ | Drop Hammer Ultimate load ≤ 70 Tons. Ram = 3000 Lb. Stroke ≤ 7 Ft. |  | 507.04 |  |  |  |
| 18 | √ | Hydraulic w/Impact Energy monitor Stroke measure within 2”. |  | 507.04 |  |  |  |
| 19 | √ | Hammer included follower. Director approval is needed. |  | 507.04 |  |  |  |
| 20 | √ | Hammer included cushion. |  | 507.04 |  |  |  |
| 21 | √ | Hammer was providing minimum 30 blows/FT. |  | 507.04 |  |  |  |
| 22 | √ | Hammer travel is aligned with piles’ axis. |  | 507.04 |  |  |  |
| 23 | √ | Hammer was damaging piles while driving. |  | 507.04 |  |  |  |
| Tests |  |  |  |  |  |  |  |
| 24 | √ | Friction Piles (Blows/FT) Ultimate bearing was determined using dynamic load test Item 523. |  | 507.05 |  |  |  |
| 25 | √ | Consultant performing the dynamic load is certified as per 523.03. |  | 507.04 |  |  |  |
| 26 | √ | Bearing Piles (20 Blows/Inch) Refusal as shown on plan. |  | 507.05 |  |  |  |
| 27 | √ | Static Load test is required. Drove anchor & test piles only. |  | 507.04 |  |  |  |
| Defective Piles |  |  |  |  |  |  |  |
| 28 | √ | Piles are within plan location limits. Underground ≤ 1 FT. Above Ground ≤ 3 Inch. Cross sectional area ≥ 80 %. Footer edge clearance ≥ 75 % (plan) Water tight. |  |  |  |  |  |
| Documentation |  |  |  |  |  |  |  |
| 29 | √ | Forms used: CA-S-3 Piling Driving Log. CA-S-8 Pile Record. |  | CIM |  |  |  |