Item 604 Manholes, Catch Basins, Inlets, Inspection Wells, Junction Chambers, Precast Reinforced Concrete Outlets, or Monuments

| District: |  | Date: |  |  | Project No.: |  | Co/Rt/Sec: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Do you have the current copy of the Inspection Documentation Form? (CA-P-3) |  | 604.01 |  |  |  |  |
|  | Is the Inspection Documentation Form being filled out? |  | 604.01 |  |  |  |  |
|  | Is the H & V control in place? |  | 604.03 |  |  |  |  |
|  | Catch Basins, Inlets: Are the pad dimensions correct (LxWxDepth)? |  | 604.04 |  |  |  |  |
|  | Catch Basins, Inlets: Were the blocks wetted prior to mortar and joints full? |  | 604.05 |  |  |  |  |
|  | Catch Basins, Inlets: Was the mortar properly mixed (1 Portland: 3 sand)? |  | 604.02 |  |  |  |  |
|  | Catch Basins: Was the correct curing method used and recorded? |  | 604.05 |  |  |  |  |
|  | Catch Basins, Inlets: Were the conditions appropriate for the work (frost, temp, curing)? |  | 604.05 511 |  |  |  |  |
|  | Catch Basins: Was the backfill placed in accordance w/ 603.10 and / or 603.11 |  | 604.04 603.10 603.11 |  |  |  |  |
|  | Catch Basins, Inlets: Were the inside dimensions checked and recorded? |  | 604.01 |  |  |  |  |
|  | Was the Precast basin or manhole placed on the correct type of bedding material? |  | 604.06 604.02 |  |  |  |  |
|  | Does the Precast have a TE-24? |  | 604.06 |  |  |  |  |
|  | Precast manhole or catch basin: Following pipe placement, were the openings properly grouted? |  | 604.06 |  |  |  |  |
|  | Precast manhole or catch basin, Inlets: Was the backfill correctly installed (tamped 8” lifts)? |  | 604.04 |  |  |  |  |
|  | Are the riser sections / joints properly sealed (rubber gasket or mastic)? |  | 604.02 604.06 |  |  |  |  |
|  | Cast in place: Were steps installed correctly? |  | 604.07 |  |  |  |  |