Item 507 Bearing Piles

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Were piles of specified type and size? |  | 507.02 |  |  |  |  |
|  | Were mill certifications on file to prove domestic origin of piling? |  | 507.03 |  |  |  |  |
|  | Is there a means in place to measure stroke height or bounce chamber energy of the hammer? |  | 507.04 |  |  |  |  |
|  | If friction piles, has a dynamic load test been run per CMS 523 to determine the proper driving criteria? |  | 507.05 |  |  |  |  |
|  | Does the person performing the dynamic load test have the necessary certification per CMS 523.03? |  | 507.05 |  |  |  |  |
|  | Is the wall thickness of the CIP shells the greater of ½” and the ultimate bearing value (in lbs.) divided by 900,000? |  | 507.06 |  |  |  |  |
|  | Were splices for H-piles performed by a pre-qualified welder? |  | 507.09 513.21 |  |  |  |  |
|  | Were splices made 3 feet above ground? |  | 507.09 |  |  |  |  |
|  | Were forms CA-S-3 Piling Driving Log and CA-S-8 Pile Record being used? |  | CIM |  |  |  |  |
|  | Were piles within location tolerances (1’ for below grade piles and 3” for projecting piles)? |  | 507.10 |  |  |  |  |

Item 509 Reinforcing Steel

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was damaged epoxy coating repaired when required? |  | 509.09 |  |  |  |  |
|  | Was patched epoxy coating allowed to cure prior to placing concrete? |  | 509.09 |  |  |  |  |
|  | Was reinforcing steel properly stored and kept free of dirt? |  | 509.03 |  |  |  |  |
|  | Were the proper clearances maintained? |  | 509.04 |  |  |  |  |
|  | Were the supports properly spaced? |  | 509.08 |  |  |  |  |
|  | Were epoxy coated rebars tied at every intersection except when spacing of intersections is less then 1 foot (0.3 m) in each direction in which case they should be tied at every other intersection? |  | 509.04 |  |  |  |  |
|  | Was reinforcing steel adequately tied so that it has firmly held in position during the placing and setting of concrete? |  | 509.04 |  |  |  |  |

Item 511 Concrete

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was a dry run made prior to placing superstructure concrete? |  | CIM 511 |  |  |  |  |
|  | Were the proper number of concrete test specimens being produced? |  | 511.06 |  |  |  |  |
|  | Was evaporation rate determined and documented using Figure 1 in ACI 308? |  | 511.10 |  |  |  |  |
|  | Was concrete finished without water or evaporation retardant being improperly used? |  | 511.10 |  |  |  |  |
|  | Was superstructure concrete properly textured? |  | 511.19, 511.20 |  |  |  |  |
|  | Was proper curing material properly applied as soon as possible? |  | 511.17 |  |  |  |  |
|  | Was a vibrating roller or vibrating pan used to place high performance concrete? |  | 511.19 |  |  |  |  |
|  | If a vibrating roller was used, did the fins protrude more then 1/4 from the roller? |  | 511.19 |  |  |  |  |
|  | Did the contractor provide the necessary verification pertaining to the vibrating frequencies of the vibrating pan or roller |  | 511.19 |  |  |  |  |

Item 513 Structural Steel

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Were welders qualified? |  | 513, S 1011 |  |  |  |  |
|  | Prior to welding, was required preheating of the structural steel accomplished? |  | 513 |  |  |  |  |
|  | Are fillet welds the proper size? |  | 513 |  |  |  |  |
|  | After the electrodes have been exposed to the atmosphere, were they properly dried and stored? |  | 513 |  |  |  |  |
|  | Were 50% of the bolt holes filled with a combination of drift pins and snug tight bolts? |  | 513.20 |  |  |  |  |
|  | Were all bolts replaced which were installed prior to inserting drift pins? |  | 513.20 |  |  |  |  |
|  | Were bolts properly match marked? |  | 513.22 |  |  |  |  |
|  | Were bend tests on shear studs performed at the beginning of each work day? |  | 513.22 |  |  |  |  |
|  | Were bend tests on shear studs performed when welding has been interrupted for an hour or more? |  | Bridge Welding Code 7.8.1 |  |  |  |  |
|  | Were all studs inspected for a 360 degree flash? |  | Bridge Welding Code 7.8.1 |  |  |  |  |
|  | Were all stud welding operators qualified? |  | Bridge Welding Code 7.7.4 |  |  |  |  |
|  | Were all studs given a light blow with a hammer? |  | S 1011.7.8.1 |  |  |  |  |

Item 514 Structural Steel Painting

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Was the Engineer provided with all the required testing equipment? |  | 514.05 |  |  |  |  |
|  | Was the quality control specialist properly trained? |  | 514.04 |  |  |  |  |
|  | If production work was taking place, was the quality control specialist involved in any other activities not related to quality control? |  | 514.04 |  |  |  |  |
|  | Was the quality control specialist properly equipped? |  | 514.04 |  |  |  |  |
|  | Was the quality control specialist able to climb? |  | 514.04 |  |  |  |  |
|  | Was there a MSDS provided for all abrasives, paints, and thinners? |  | 514.09 |  |  |  |  |
|  | Was the steel temperature at least 5 degrees above dew point before blasting or painting the steel? |  | 514.06 |  |  |  |  |
|  | Were air compressors checked every four hours? |  | 514.04 |  |  |  |  |
|  | Was the thinner from the same manufacture as the paint? |  | 514.16 |  |  |  |  |
|  | Were printed instructions from the manufacture available pertaining to thinning of the paint? |  | 514.16 |  |  |  |  |
|  | Were shear mixers used to mix the paint? |  | 514.16 |  |  |  |  |
|  | Was primer continuously mixed during application? |  | 514.16 |  |  |  |  |
|  | Was relative humidity below 85% during application of paint? |  | 514.06 B |  |  |  |  |
|  | Was proper scaffolding used for inspection? |  | 514.10 |  |  |  |  |
|  | Was PE stamped inspection access plan submitted? |  | 514.10 |  |  |  |  |
|  | Were the tarps installed correctly? |  | 514.13D |  |  |  |  |
|  | Was all equipment parked on ground covers? |  | 514.13D |  |  |  |  |
|  | Was the debris collected and stored at the end of each day? |  | 514.13D |  |  |  |  |
|  | Was the debris properly stored? |  | 514.13D |  |  |  |  |
|  | Were debris sampled within the first week of production blasting? |  | 514.13D |  |  |  |  |
|  | Was hazardous waste disposed of within 60 days after it was generated? |  | 514.13D |  |  |  |  |
|  | Were all damaged areas properly repaired? |  | 514.22 |  |  |  |  |

Item 524 Drilled Shafts

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Did contractor submit installation plan at least 14 days prior constructing drilled shafts? |  | 524.03 |  |  |  |  |
|  | Did contractor’s installation plan contain include all the required information? |  | 524.03 |  |  |  |  |
|  | Was reinforcing steel properly stored and kept free of dirt? |  | 524..09 509.03 |  |  |  |  |
|  | If the dry method was used, were the sides stable and any flow of water into the excavation less then 300 mm (12 inches) per hour? |  | 524.04A |  |  |  |  |
|  | If wet construction method is used for a shaft which is not socketed into bedrock, was the water or slurry fluid level during the drilling operation inside the drilled shaft higher then the static water table? |  | 524.04B |  |  |  |  |
|  | If temporary casing is used, was at least a 5 foot (1.5 meter) head of concrete maintained above the bottom of the casing? |  | 524.04C |  |  |  |  |
|  | If permanent casing is required, did the contractor excavate material beyond the limits of the casing? |  | 524.04D |  |  |  |  |
|  | Were spacer attached to the reinforcing cage at quarter points? |  | 524.09 |  |  |  |  |
|  | Were round spacers of the proper size used to maintain the required clearance, ie.3 inches (75 mm) for shaft diameters up to 4 feet (1.2meters) and 6 inches (150 mm) for shaft diameters larger than 4 feet (1.2 meters)? |  | 524.09 |  |  |  |  |
|  | If concrete is placed underwater, is it placed in one continuous operation? |  | 524.10 |  |  |  |  |
|  | If a tremie is used to place concrete underwater, is the end of the tremie maintained at least 10 feet (3 meters) below the surface of the concrete? |  | 524.12 |  |  |  |  |
|  | If a pump is used to place concrete underwater, is the end of the pump maintained at least 10 feet (3 meters) below the surface of the fresh concrete? |  | 524.13 |  |  |  |  |
|  | If a pump or tremie is used to place concrete under water, was the end of the pump or tremie properly plugged when it was placed into the water? |  | 524.12 / 524.13 |  |  |  |  |
|  | Where bottoms and sides of excavation checked for cleanliness? |  | 524.08 |  |  |  |  |
|  | Where tolerances checked? |  | 524.14 |  |  |  |  |
|  | If wet method is used, after the shaft is full is concrete continually placed into the shaft until acceptable quality of concrete is evident at the top of the shaft? |  | 524.10 |  |  |  |  |

Item 526 Approach Slabs

| District: |  | Date: |  |  | Project No.: PID: |  | Co/Rt/Sec: Bridge: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contractor: Sub: |  |  |  |  | Project/District Contacts: |  |  |
| Reviewed “√” | Specification Conformance Statement |  | CMS/ CIM | Measurements |  | Conformance Comments |  |
|  | Has subgrade, or subbase, been accepted? |  | 526.01 |  |  |  |  |
|  | Were epoxy coated rebars tied at every intersection except when spacing of intersections is less then 1 foot (0.3 m) in each direction in which case they should be tied at every other intersection? |  | 526.03 |  |  |  |  |
|  | Was subgrade wetted immediately prior to concrete placement? |  | 526.04 |  |  |  |  |
|  | Was evaporation rate determined and documented using Figure 1 in ACI 308? |  | 526.04 511.10 |  |  |  |  |
|  | Was concrete finished without water or evaporation retardant being improperly used? |  | 526.04 511.10 |  |  |  |  |
|  | Was approach slab concrete properly textured? |  | 526.05 |  |  |  |  |
|  | Was proper curing material properly applied as soon as possible 511.17.A, |  | 526.05 511.17.A |  |  |  |  |
|  | Were curbs placed monolithically with approach slab? |  | 526.06 |  |  |  |  |