Chapter 3745-19 
Open Burning Standards

    - Definitions

As used in Chapter 3745-19 of the Administrative Code

- "Agricultural waste" means any matter generated by crop, horticultural, or livestock production practices, and includes such items as woody debris and plant matter from stream flooding, bags, cartons, structural materials, and landscape wastes that are generated in agricultural activities, but does not include land clearing waste; buildings; garbage; dead animals; motor vehicles and parts thereof; nor economic poisons and containers thereof, unless the manufacturer has identified open burning as a safe disposal procedure.

- "Economic poisons" include but are not restricted to pesticides such as insecticides, fungicides, rodenticides, miticides, nematocides and fumigants; herbicides; seed disinfectants; and defoliants.

- "Garbage" means any matter resulting from the handling, processing preparation, cooking and consumption of food or food products.

- "Landscape waste" means any plant matter, except garbage, including trees, tree trimmings, branches, stumps, brush, weeds, leaves, grass, shrubbery, yard trimmings, and crop residues.

- "Land clearing waste" means plant matter which is removed from land, including plant matter removed from stream banks during projects involving more than one property owner, for the purpose of rendering the land useful for residential, commercial, or industrial development.

- "Ohio EPA" means the Ohio environmental protection agency director or agencies delegated authority by the director of the Ohio environmental protection agency pursuant to section 3704.03 of the Revised Code or the chief of any Ohio environmental protection agency district office.

- "Open burning" means the burning of any materials wherein air contaminants resulting from combustion are emitted directly into the ambient air without passing through a stack or chimney. Open burning includes the burning of any refuse or salvageable material in any device not subject to or designed specifically to comply with the requirements of rule 3745-17-09 or 3745-17-10 of the Administrative Code.

- "Residential waste" means any matter, including landscape wastes, generated on a one-, two- or three-family residence as a result of residential activities, but not including garbage.

- "Restricted area" means the following

  - Except as provided in paragraph (I) (2) of this rule, the area within the boundary of any municipal corporation established in accordance with the provisions of title 7 of the revised code, plus a zone extending one thousand feet beyond the boundaries of any such municipal corporation having a population of one thousand to ten thousand persons and a zone extending one mile beyond any such municipal corporation having a population of ten thousand persons or more according to the latest federal census.

  - "Restricted area" shall not include any municipal corporation the territory of which is located on an island in Lake Erie except that, during the yearly period between Memorial Day and Labor Day, any such municipal corporation shall be required to comply with the requirements of rule 3745-19-03 of the Administrative Code.

- "Unrestricted area" means all areas outside the boundaries of a restricted area as defined in paragraph (I) of this rule.

HISTORY:  Eff 6-21-76; 10-30-87; 12-20-88; 11-27-00

Rule promulgated under:  RC Chapter 119

Rule authorized by:  RC 3704.03

Rule amplifies:  RC Chapter 3704.

119.032 review dates: 7-11-2000, 7-11-2005

    - Relations to other prohibitions

- Notwithstanding any provision in Chapter 3745-19 of the Administrative Code, no open burning shall be conducted in an area where an air alert, warning, or emergency under Chapter 3745-25 of the Administrative Code is in effect.

- No provisions of Chapter 3745-19 of the Administrative Code, permitting open burning, and no permission to open burn granted by the Ohio EPA, shall exempt any person from compliance with any section of the Revised Code, or any regulation of any state department, or any local ordinance or regulation dealing with open burning.

HISTORY:  Eff 9-21-73; 10-20-87

Rule promulgated under:  RC Chapter 119

Rule authorized by:  RC 3704.03

Rule amplifies:  RC Chapter 3704

    - Open burning in restricted areas

- No person or property owner shall cause or allow open burning in a restricted area except as provided in paragraphs (B) to (D) of this rule or in section 3704.11 of the Revised Code.

- Open burning shall be allowed for the following purposes without notification to or permission from the Ohio EPA:

    - Cooking for human consumption

    - Heating tar, welding, acetylene torches, highway safety flares, heating for warmth of outdoor workers and strikers, smudge pots and similar occupational needs.

Fires allowed by paragraphs (B)(1) and (B)(2) of this rule shall not be used for waste disposal purposes and shall be of minimum size sufficient for their intended purpose; the fuel shall be chosen to minimize the generation and emission of air contaminants.

- Open burning shall be allowed for the following purposes with prior notification to the Ohio EPA in accordance with paragraph (B) of rule 3745-19-05 of the Administrative Code:

      - Prevention or control of disease or pests, with written or verbal verification to the Ohio EPA from the local health department, cooperative extension service, Ohio department of agriculture, or U.S. department of agriculture, that open burning is the only appropriate disposal method.

      - Ceremonial fires provided the following conditions are met

        - The ceremonial fires shall be less than five feet by five feet in dimension and shall burn no longer than three hours;

        - The ceremonial fires shall not be used for waste disposal purposes; and

        - The fuel shall be chosen so as to minimize the generation and emission of air contaminants.

      - Disposal of agricultural waste generated on the premises if the following conditions are observed:

          - The fire is set only when atmospheric conditions will readily dissipate contaminants;

          - The fire does not create a visibility hazard on the roadways, railroad tracks, or air fields;

          - The fire is located at a point on the premises no less than one thousand feet from any inhabited building not located on said premises;

          - The wastes are stacked and dried to provide the best practicable condition for efficient burning; and

          - No materials are burned which contain rubber, grease, asphalt or liquid petroleum products.

- Open burning shall be allowed for the following purposes upon receipt of written permission from the Ohio EPA, in accordance with paragraph (A) of rule 3745-19-05 of the Administrative Code, provided that any conditions specified in the permission are followed:

      - Disposal of ignitable or explosive materials where the Ohio EPA determines that there is no practical alternate method of disposal;

      - Instruction in methods of fire fighting or for research in the control of fires;

      - In emergency or other extraordinary circumstances for any purpose determined to be necessary by the Ohio EPA; and

      - Recognized horticultural, silvicultural, range, or wildlife management practices; and

      - Fires and/or pyrotechnic effects, for purposes other than waste disposal, set as part of commercial film-making or video production activities for motion pictures and television.

HISTORY:  Eff 6-21-76; 10-30-87; 12-20-88; 11-27-00

Rule promulgated under:  RC Chapter 119.

Rule authorized by:  RC 3704.03

Rule amplifies:  RC Chapter 3704.

119.032 review date: 7-11-2000, 7-11-2005

Research Aids

Open dumping and burning

O-Jur3d: Envirn Pro §§ 109, 113

3745-19-04  Open burning in unrestricted areas

(A) No person or property owner shall cause or allow open burning in an unrestricted area except as provided in paragraphs (B) to (C) of this rule or in section 3704.11 of the Revised Code.

(B) Open burning shall be allowed for the following purposes without notification to or permission from the Ohio EPA:

(1) Cooking for human consumption

(2) Heating tar, welding, acetylene torches, highway safety flares, heating for warmth of outdoor workers and strikers, smudge pots and similar occupational needs.

Fires allowed by paragraphs (B)(1) and (B)(2) of this rule shall not be used for waste disposal purposes, and shall be of the minimum size sufficient for their intended purpose; the fuel shall be chosen to minimize the generation and emission of air contaminants.

(3) Disposal of residential waste or agricultural waste generated on the premises if the following conditions are observed:

(a) The fire is set only when atmospheric conditions will readily dissipate contaminants;

(b) The fire does not create a visibility hazard on the roadways, railroad tracks, or air fields;

(c) The fire is located at a point on the premises no less than one thousand feet from any inhabited building not located on said premises;

(d) The wastes are stacked and dried to provide the best practicable condition for efficient burning; and

(e) No materials are burned which contain rubber, grease, asphalt or liquid petroleum products.

(4) Ceremonial purposes, if the following conditions are met

(a) The ceremonial fires shall be less than five feet by five feet in dimension and shall burn no longer than three hours;

(b) The ceremonial fires shall not be used for waste disposal purposes; and

(c) The fuel shall be chosen so as to minimize the generation and emission of air contaminants.

(C) Open burning shall be allowed for the following purposes upon receipt of written permission from the Ohio EPA, in accordance with paragraph (A) of rule 3745-19-05 of the Administrative Code, provided that any conditions specified in the permission are followed:

- Disposal of ignitable or explosive materials where the Ohio EPA determines that there is no practical alternate method of disposal;

- Instruction in methods of fire fighting or for research in the control of fire;

- In emergency or other extraordinary circumstances for any purpose determined to be necessary by the Ohio EPA;

- Disposal of land clearing waste if the following conditions are observed

    - The fire is set only when atmospheric conditions will readily dissipate contaminants;

    - The fire does not create a visibility hazard on roadways, railroad tracks, or air fields;

    - The fire is located at a point on the premises no less than one thousand feet from any inhabited building not located on said premises; and

    - An air curtain destructor or other device or method determined by the director to be at least as effective is used to curtail release of air contaminants; and

- Recognized horticultural, silvicultural, range, or wildlife management practices

- Fires and/or pyrotechnic effects, for purposes other than waste disposal, set as part of commercial film-making or video production activities for motion pictures and television.

(D) Open burning shall be allowed for the prevention or control of disease or pests with written or verbal verification to the Ohio EPA from the local health department, cooperative extension service, Ohio department of agriculture, or U.S. department of agriculture, that open burning is the only appropriate disposal method.

HISTORY:  Eff 6-21-76; 10-30-87; 12-20-88; 11-27-00

Rule promulgated under:  RC Chapter 119

Rule authorized by:  RC 3704.03

Rule amplifies:  RC Chapter 3704.

    - Permission to individuals and notification to the Ohio EPA

- Permission

  - An application for permission to open burn shall be submitted in writing at least ten days before the fire is to be set. It shall be in such form and contain such information as required by the Ohio EPA.

  - Except as provided in paragraph (A)(6) of this rule. Such applications shall contain, as a minimum, information regarding:

    - The purpose of the proposed burning

    - The nature of quantities of material to be burned

    - The date or dates when such burning will take place

    - The location of the burning site, including a map showing distances to residences, populated areas, roadways, air fields, and other pertinent landmarks; and

    - The methods or actions which will be taken to reduce the emissions of air contaminants.

  - Permission to open burn shall not be granted unless the applicant demonstrates to the satisfaction of the Ohio EPA that open burning is necessary to the public interest; will be conducted in a time, place, and manner as to minimize the emission of air contaminants; and will have no serious detrimental effect upon adjacent properties or the occupants thereof. The Ohio EPA may impose such conditions as may be necessary to accomplish the purpose of Chapter 3745-19 of the Administrative Code.

  - Except as provided in paragraph (A)(6) of this rule, permission to open burn must be obtained for each specific project. In emergencies where public health or environmental quality will be seriously threatened by delay while written permission is sought, the fire may be set with oral permission of the Ohio EPA.

  - Violations of any of the conditions set forth by the Ohio EPA in granting permission to open burn shall be grounds for revocation of such permission and refusal to grant future permission, as well as for the imposition of other sanctions provided by law.

  - The Ohio department of commerce, division of state fire marshal, may request permission to open burn on an annual basis for the purpose of training firefighters on pre-flashover conditions using the Ohio fire academy's mobile training laboratory at either the academy or at other training sites in Ohio. The annual application required pursuant to paragraph (A)(1) of this rule shall contain information as required in paragraph (A)(2) of this rule, except the information required in paragraphs (A)(2)(c) and (A)(2)(d) of this rule need not be provided unless it is available at the time of submittal of the application. The academy shall contact the appropriate Ohio EPA district office or local air agency at least five days before each training session of the date or dates when the training session will take place and its location.

- Notification

  - Notification shall be submitted in writing at least ten days before the fire is to be set. It shall be in such form and contain such information as shall be required by the Ohio EPA.

  - Such notification shall inform the Ohio EPA regarding

(a) The purpose of the proposed burning

(b) The nature and quantities of materials to be burned

(c) The date or dates when such burning will take place; and

(d) The location of the burning site

  - The Ohio EPA, after receiving notification, may determine that the open burning is not allowed under Chapter 3745-19 of the Administrative Code and the Ohio EPA shall notify the applicant to this effect.

HISTORY:  Eff 9-21-73; 10-30-87; 11-27-00

Rule promulgated under:  RC Chapter 119

Rule authorized by:  RC 3704.03

Rule amplifies:  RC Chapter 3704.

119.032 review date: 7-11-2000, 7-11-2005

Top of Form

Bottom of Form