OFFICE OF STRUCTURAL ENGINEERING

BRIDGE INVENTORY CODING GUIDE

Broad Street				US Grant

Cherry Street				Veterans’ Glass City Skyway

Published December 1989

Revised August 2012

ACKNOWLEDGMENTS

Excerpts and/or items from the following publications were used as guidelines in preparing this guide:

- "Bridge Inventory Coding Guide", October 1988, 1989, 1992, 2010, 2011 Ohio Department of Transportation.

- "Bridge Inspection Manual", 1987, 1989, 1992, 1995, 1998, 2006, 2010 Ohio Department of Transportation.

- "Recording and Coding Guide for the Structure Inventory and Appraisal of the Nations Bridges",   1995 U.S. Department of Transportation, Federal Highway Administration.

- "Census of Population and Housing 2000", U.S. Department of Commerce, Bureau of Census.

TABLE OF CONTENTS

INVENTORY RESPONSIBILITY	7

DEFINITION OF A BRIDGE	8

BRIDGE INVENTORY CODING GUIDE AND FORMS	8

UPDATING THE ODOT BRIDGE MANAGEMENT SYSTEM	9

BRIDGE INVENTORY CODING INSTRUCTIONS	10

TRANSACTION CODE FUNCTION	10

ADDITION (Transaction Code "A" on BR87 & 87A & in BMS)	10

CHANGE (Transaction Code "C" on BR87 & 87A & “U” in BMS)	11

RETIRE (Transaction Code "R" on BR87 & “U” in BMS)	11

RESTORE OR "UNRETIRE" (Transaction Code “U” in BMS)	12

DELETE (Transaction Code “D” in BMS)	12

GUIDELINES ON STRUCTURE FILE NUMBER (SFN)	13

PROCEDURE FOR SUBMITTING AND PROCESSING BR87/87A FORMS	14

ITEM #1 - OHIO'S DESIGNATED MPOs [2 digits]	15

ITEM #2 - CITY OR TOWN (FIPS CODE) [5 digits]	16

ITEM #3 - INVENTORY ROUTE [5 digits]	17

ITEM #4 - FEATURE(S) INTERSECTED	20

ITEM #5 - INVENTORY ROUTE BRIDGE NUMBER	21

ITEM #6 - AVERAGE DAILY TRAFFIC [6 digits]	24

ITEM #7 - AVERAGE DAILY TRAFFIC YEAR [2 digits]	25

ITEM #8 - AVERAGE DAILY TRUCK TRAFFIC [5 digits]	26

ITEM #9 - DIRECTION OF TRAFFIC [1 digit]	27

ITEM #10 - TEMPORARY STRUCTURE DESIGNATION [1 digit]	28

ITEM #11 - DESIGNATED NATIONAL NETWORK [1 digit]	29

ITEM #12 - PARALLEL STRUCTURES [1 digit]	30

ITEM #14 - NATIONAL HIGHWAY SYSTEM (NHS) [1 digit]	32

ITEM #15 - MACRO LEVEL CORRIDOR [1 digit]	33

ITEM #16 - FUNCTIONAL CLASSIFICATION [2 digits]	34

ITEM #19 - STRAHNET HIGHWAY DESIGNATION [1 digit]	36

ITEM #22 - INTERSECTED ROUTE (5 digits)	38

ITEM #23 - FEATURE(S) INTERSECTED [24 digits]	39

ITEM #24 - INTERSECTED ROUTE BRIDGE NUMBER [5 digits]	40

ITEM #25 – INTERSECTED ADT [6 digits]	41

ITEM #26 - INTERSECTED ADT YEAR [2 digits]	42

ITEM #27 - INTERSECTED TRUCK ADT [5 digits]	43

ITEM #28 - INTERSECTED NATIONAL HIGHWAY SYSTEM (NHS) [1 digit]	44

ITEM #29 - INTERSECTED MACRO LEVEL CORRIDOR (1digit)	45

ITEM #30 - INTERSECTED FUNCTIONAL CLASSIFICATION [2 digits]	46

ITEM #32 - INTERSECTED DIRECTION OF TRAFFIC (1 digit)	48

ITEM #33 - INTERSECTED DESIGNATED NATIONAL NETWORK [1 digit]	49

ITEM #34 - INTERSECTED PARALLEL STRUCTURES [1 digit]	50

ITEM #36 - INTERSECTED STRAHNET HIGHWAY DESIGNATION [1 digit]	52

ITEM #38 - BYPASS LENGTH [2 digits]	54

ITEM #39 - COORDINATES [16 digits]	56

ITEM #40 - TOLL [1 digit]	57

ITEM #41 - DATE BUILT [8 digits]	58

ITEM #42 - DATE OF MAJOR REHABILITATION [8 digits]	59

ITEM #43 - NUMBER OF LANES [4 digits]	60

ITEM #44 – DEGREE OF HORIZONTAL CURVE [4 digits]	61

ITEM #45 - SKEW [2 digits]	62

ITEM #46 – UTILITIES (1 digit)	64

ITEM #48 - DESIGN LOADING [1 digit]	66

ITEM #49 - APPROACH ROADWAY WIDTH [3 digits]	67

ITEM #50 - BRIDGE ROADWAY WIDTH (CURB TO CURB)	68

ITEM #51 - DECK WIDTH (OUT/OUT) [4 digits]	69

ITEM #52 - MEDIAN TYPE (ON BRIDGE) [3 digits]	72

ITEM #53 - BRIDGE MEDIAN [1 digit]	73

ITEM #54 - CURBS OR SIDEWALKS [2*3 digits]	75

ITEM #55 - TYPE OF CURB OR SIDEWALKS [2*2 digits]	78

ITEM #56 - FLARED [1 digit]	79

ITEM #57 - COMPOSITE STRUCTURE [1 digit]	80

ITEM #58 - BRIDGE RAILING TYPE [1 digit]	81

ITEM #59 - DECK DRAINAGE [1 digit]	82

ITEM #60 - DECK TYPE [1 digit]	83

ITEM #61 - DECK PROTECTION [2 digits]	84

ITEM #62 - WEARING SURFACE	85

ITEM #63 - STRUCTURE TYPE	86

ITEM #65 - MAXIMUM SPAN LENGTH [4 digits]	89

ITEM #66 - OVERALL STRUCTURE LENGTH [6 digits]	90

ITEM #67 - MINIMUM VERTICAL CLEARANCE ON BRIDGE [2*4 digits]	92

ITEM #69 - NBIS BRIDGE LENGTH [1 digit]	95

ITEM #70 - SUBSTRUCTURE	98

ITEM #71 - FOUNDATIONS [5 digits]	100

ITEM #72 - AVERAGE LENGTH ABUTMENT FOUNDATIONS  [2 digits]	101

ITEM #73 - AVERAGE LENGTH PIER FOUNDATIONS[2 digits]	102

ITEM #74 - SCOUR CRITICAL SUSCEPTIBILITY [1 digit]	103

ITEM #75 - TYPE OF CHANNEL PROTECTION [1 digit]	106

ITEM #77 - MINIMUM VERTICAL CLEARANCE UNDER BRIDGE [2*4 digits]	108

ITEM #78 - MINIMUM LATERAL UNDERCLEARANCE (TO EDGE OF LANE) [2*6 digits]	111

ITEM #80 - MINIMUM LATERAL CLEARANCE ON BRIDGE (TO EDGE OF LANE) [2 x 6 digits]	114

ITEM #81 - VERTICAL CLEARANCE AT VERTICAL LIFT BRIDGE [3 digits]	115

ITEM #82 - BLANK	116

ITEM #83 - LOAD RATING [5 digits]	117

ITEM #84 - METHOD OF RATING [1 digit]	119

ITEM #85 - SOFTWARE OF RATING ANALYSIS [1 digit]	120

ITEM #86 - STREAM VELOCITY [3 digits]	121

ITEM #87 – PLAN INFORMATION AVAILABLE [1 digit]	122

ITEM #88 - WATERWAY ADEQUACY [1 digit]	123

ITEM #89 - APPROACH ALIGNMENT [1 digit]	125

ITEM #90 - PROPOSED IMPROVEMENTS	126

Type of Work [2 digits]	126

Method [1 digit]	126

Length of Improvements [6 digits]	127

Roadway Improvement Costs [6 digits]	129

Total Project Costs [6 digits]	129

Year Of Improvement Cost Estimate [2 digits]	130

ITEM #91 - DESIGNATED INSPECTION FREQUENCY [2 digits]	131

ITEM #92 – OPERATING LOAD RATING FACTOR [5 digits]	132

ITEM #93 –INVENTORY LOAD RATING FACTOR [5 digits]	134

ITEM #94 - INSPECTION PROGRAM RESPONSIBILITY [3digits]	136

ITEM #95 - MAJOR MAINTENANCE PROGRAM RESPONSIBILITY [3 digits]	138

ITEM #96 – ROUTINE MAINTENANCE RESPONSIBILITY [3digits]	140

ITEM #97 - [BLANK]	142

ITEM #98 - TYPE SERVICE [2 digits]	143

ITEM # 99 – [BLANK]	144

ITEM # 100 – [BLANK]	145

ITEM #101 - LOCATION OF STRUCTURE [25 digits]	147

ITEM #102 - FACILITY CARRIED BY STRUCTURE [18 digits]	148

ITEM #103 - ROUTE ON THE BRIDGE [2 digits]	149

ITEM #104 - ROUTE UNDER THE BRIDGE [2 digits]	150

ITEM #105 - BORDER BRIDGE [5 digits]	151

ITEM #106 - BORDER BRIDGE STRUCTURE NUMBER [15 digits]	152

ITEM #107 – [BLANK]	153

Item #108 – [BLANK]	154

ITEM #109 - APPROACH GUARDRAIL [1 digit]	155

ITEM #110 - APPROACH PAVEMENT WEARING SURFACE [1 digit]	156

ITEM #111 - APPROACH PAVEMENT GRADE [1 digit]	157

ITEM #112 - APPROACH SLAB [1 digit]	158

ITEM #113 - LENGTH OF APPROACH SLAB [3 digits]	159

ITEM #114 - #118 BLANK	160

ITEM #119 - DATE OF WEARING SURFACE [8 digits]	161

ITEM #120 – BLANK	162

ITEM #121 - MAIN MEMBER TYPE [1 digit]	163

ITEM #122 - MOMENT PLATES [1 digit]	164

ITEM #123 – [BLANK]	165

ITEM #124 - BEARING DEVICE [2 digits]	166

ITEM #126 - NAVIGATION	168

ITEM #127 - LENGTH OUT/OUT OF CULVERT [4 digits]	170

ITEM #129 - DEPTH OF FILL OVER CULVERT [2 digits]	172

ITEM #130 - HEADWALLS OR ENDWALLS [1 digit]	173

ITEM #131 - CULVERT/BRIDGE TYPE (1 digit)	174

ITEM #135 - HINGES [1 digit]	176

ITEM #138 - TYPE OF MAIN LONGITUDINAL BRIDGE MEMBERS [1 digit]	178

ITEM #139 - FRAMING TYPE [1 digit]	179

ITEM #141 - STRUCTURAL STEEL	181

ITEM #142 - NAME OF FABRICATOR [14 digits]	183

ITEM #143 - NAME OF CONTRACTOR [14 digits]	184

ITEM #144 - OHIO ORIGINAL CONSTRUCTION PROJECT NUMBER [4 & 2 digits]	185

ITEM #145 - RAILROAD IDENTIFICATION [2 digits]	186

ITEM #148 - DECK CONCRETE TYPE [1 digit]	188

ITEM #151 - STANDARD DRAWING NUMBER [11 digits]	190

ITEM #152 - DRAINAGE AREA [3 digits]	191

ITEM #153 – WORK HISTORY	192

ITEM #154 - TOTAL MINIMUM HORIZONTAL CLEARANCE ON BRIDGE [6 digits]	196

ITEM #155 - PRACTICAL MAXIMUM VERTICAL CLEARANCE ON BRIDGE [4 digits]	198

ITEM #156 - TOTAL MINIMUM HORIZONTAL CLEARANCE UNDER BRIDGE [6 digits]	199

ITEM #157 - PRACTICAL MAXIMUM VERTICAL CLEARANCE UNDER BRIDGE [4 digits]	201

ITEM #159 - TEMPORARY BRIDGE CONDITION [4 * 1 digit]	203

ITEM #161 - SPECIAL FEATURES [6*1 digit]	205

ITEM #162 - HEIGHT OF FENCING ON BRIDGE [2 digits]	206

ITEM #163 - NOISE BARRIER WALLS [1 digit)]	207

ITEM #165 - POST TENSIONED [1 digit]	209

ITEM #167 - SCENIC RIVER [1 digit]	211

ITEM #169 - EXPANSION JOINT [3 digits]	213

ITEM #171 - JOINT RETROFIT [6 digits]	215

ITEM #173 - HAUNCHED GIRDER [1 digit]	217

ITEM #174 - HAUNCH GIRDER DEPTH [4 digits]	218

ITEM #175 - MAIN MEMBER DEPTH [4 digits]	219

ITEM #178 - PILE LOG [1 digit]	221

ITEM #179 - ABUTMENT/PILE LOAD TESTING [1 digit]	222

ITEM #181 - NON-CARDINAL RAMP	224

ITEM #182 - CARDINAL RAMP	225

ITEM #184 - SIGN ON/UNDER BRIDGE [2 digits]	227

ITEM #185 - WATER FLOW DIRECTION  [1 digit]	228

ITEM #186 - SLOPE PROTECTION [1 digit]	229

ITEM #188 - FRACTURE CRITICAL INSPECTION	231

ITEM #189 - UNDERWATER INSPECTION	233

ITEM #190 - INSPECTION FROM SNOOPER [1 digit]	235

ITEM #191 - INSPECTION FROM BOAT [1 digit]	236

ITEM #193 - SPECIAL INSPECTION [1 digit]	238

ITEM #194 – SEISMIC SUSCEPTIBLE[1 digit]	239

APPENDIX A	240

APPENDIX B	243

APPENDIX C	245

APPENDIX D	258

APPENDIX E	302

APPENDIX F	307

APPENDIX G	308

APPENDIX H	310

APPENDIX I	311

APPENDIX J	312

APPENDIX K	313

APPENDIX L	316

INDEX	321

BR87	324

BR87A	325

BR86	325

BR86S	327

INTRODUCTION TO THE OHIO BRIDGE INVENTORY SYSTEM

The Bridge Inventory System has been developed by the Ohio Department of Transportation Office of Structural Engineering (OSE) in cooperation with the Division of Information Technology (DoIT). The system provides for a collection of bridge data to be used for producing various analytical and statistical reports which aid in the design, planning, programming, and financing of bridge maintenance and construction in Ohio, as well as a funding selection base by FHWA.

Federal legislation, and publication by the U.S. Department of Transportation, Federal Highway Administration (FHWA) of the "Recording and Coding Guide for The Structure Inventory and Appraisal of the Nation's Bridges", July 1972, (rev. JAN. 1979, DEC. 1988, DEC. 1995, DEC. 2000) and the Surface Transportation Assistance Act of 1978 render it mandatory that particular data herein specified, be collected and maintained for all bridges on the Public Highway and Street Systems in Ohio.  It will be possible to accomplish this task only through the complete cooperation of all governmental and local agencies involved.  Most of the identification and structural data required will be available from existing bridge inventories of the various agencies, as established under Section(s) 723.54, 5501.47, 5543.20 of the Ohio Revised Code. Some items required may compel an agency to make field measurements to obtain accurate and up-to-date data.

Provisions have been made to collect and maintain information on all bridges on the State & Non-State Highway Systems. All inventory and inspection data of bridges is stored in ODOT Bridge Management System (BMS).

Each agency (ODOT District, County, Municipality, Turnpike Commission, etc.) shall be responsible for the validity of data for its own bridges.

All bridges and culverts carrying or going over public streets and highways must be inventoried.  It is necessary that all bridges be inventoried and coded with respect to a particular route known as the "Inventory Route" which is defined in more detail in this BIC Guide.

In all cases structure description items shall be coded from the viewpoint of the structure as a bridging unit, regardless of its orientation over, under or on the inventory route.

## INVENTORY RESPONSIBILITY

In order to eliminate duplication of records the following rule shall apply. For the purpose of this inventory each agency, State, County and Municipality shall inventory only the structures for which it has inspection responsibility under Ohio law, or any special written agreement legally transferring inspection responsibility.  Agencies charged by law with maintenance of a structure must not place that bridge record on file.

Exceptions to this rule are private and industrial structures over or under an agency route and for which the inspection responsibility has been assigned by law or mutual agreement to the private agency.  In this case, the agency whose route goes over or under shall inventory the structure.  Structures separating grades of two intersecting routes under the same agency jurisdiction shall be inventoried on the route carried by the bridge, i.e. code using the route on the structure as the "Inventory route."

For bridges with joint Inspection Responsibility, a public agency would be responsible to do the bridge inventory as per hierarchy (State, Turnpike, County, and Municipality) unless a special agreement is in place.

In special cases a public agency may be required to inventory a bridge inspected or maintained by a border state (Indiana, Kentucky, Michigan, Pennsylvania, or West Virginia) for cross-reference or holding bridge clearance information.

## DEFINITION OF A BRIDGE

A structure including supports over a depression or an obstruction such as water, highway, or railway; and having an opening measured along the centerline of the roadway of 10 ft. [3.048 m] or more between under-copings of abutments or spring lines of arches, or extreme ends of openings for multiple boxes. It may also include multiple pipes, where the clear distance between openings is less than half of the smaller contiguous opening.

Multiple cell culverts, under a fill, with a length of 10 feet (3.05 meters) or more between extreme ends of openings, measured along the centerline of the highway, including multiple pipes where the clear distance between openings is less than half of the diameter of the smaller opening, shall be regarded as a bridge.

## BRIDGE INVENTORY CODING GUIDE AND FORMS

The Bridge Inventory Coding Guide (also referred to as “the BIC Guide”) has been prepared through the joint efforts of the Office of Structural Engineering and Division of Information Technology to establish policies and procedures for the creation and maintenance of a Bridge Management System.

The BIC Guide is intended for use by State, County and Municipal highway officials in coding bridge inventory in the BMS or "Bridge Inventory Code Sheets" (BR87 and BR87A).

It shall be the joint responsibility of the Bridge Inventory Section of the Office of Structural Engineering and the computer systems analyst assigned to the Bridge Management System to insure that the Guide is current.  Inventory Coding Guide will be published on the Inventory website.

Printed copies of the Inventory Coding Guide can be purchased for $15.00 from the Inventory Section in the Office of Structural Engineering on special request.

Inquiries concerning the material contained in this guide and the code sheets (BR87 and BR87A) should be directed by phone or in writing to:

Ohio Department of Transportation

Office of Structural Engineering

1980 West Broad Street

Columbus, OH 43223

Attn:  Bridge Inventory Section

Kathy Keller - 614-752-9973 - kkeller@dot.state.oh.us

Amjad Waheed, PE – 614-752-9972 – awaheed@dot.state.oh.us

## UPDATING THE ODOT BRIDGE MANAGEMENT SYSTEM

Updating the ODOT BMS consists of adding bridges, retiring bridges from the database, or changing/updating data of an existing bridge record. This includes inventory as well as inspection data of a bridge.

The agency (ODOT District, County, Municipality, Turnpike Commission, etc.) responsible for the inventory of a bridge shall also be responsible for maintaining the integrity of inventory data of the bridge.  Each agency shall maintain its portion of the database on a day-to-day basis.

There are three common methods to update BMS: 1) Direct data entry into the BMS (only done by authorized users of BMS); 2) Using County Engineers Association of Ohio (CEAO) program (by submitting an electronic copy of the data export file); and 3) submitting paper Bridge Inventory forms (BR87/87A) and Bridge Inspection forms (86/86S).

Direct data entry in the BMS and submission of electronic file of bridge data are the preferred methods to change BMS data.

A complete review and update of the bridge records shall be accomplished at least once a year.

F.H.W.A. requires prompt updating of data due to replacement, rehabilitation, load posting, etc.  The limit is 90 days for ODOT, and 180 days Non-ODOT agencies.

The Bridge Inventory Section of the Office of Structural Engineering will attempt to correct batch update edit errors caused by coding or data recording mistakes.  Any original data, changed accordingly, will be reported to the originating agency, as soon as practical.

## BRIDGE INVENTORY CODING INSTRUCTIONS

PROCESSING KEY - This item is the key to process all bridge data.  It includes the identification number for the structure's file, known as the Structure File Number, and also the Transaction Code.

Following is a complete listing of all items occurring on the "Bridge Inventory forms” (BR87 & BR87A) in the order they appear.  Items are described in sufficient detail to allow easy coding of structures by all the agencies involved.  Since all items do not apply to all agencies, it is necessary that each description be read carefully.  Each item should be coded in its entirety exactly as described.

For instructions on coding Bridge Inspection forms (BR86/BR86S), refer to ODOT Bridge Inspection Manual.

## TRANSACTION CODE FUNCTION

Updating the inventory items on the Bridge Management System consists of four types of transactions; Adding, Retiring, Restoring or Changing.  For all updates to the file, the Structure File Number and transaction code must be entered.  If either code is omitted, the update record will be ignored.

## ADDITION (Transaction Code "A" on BR87 & 87A & in BMS)

The addition of a bridge record to the Bridge Management System establishes the existence of the bridge for the system.  An addition of a bridge (i.e. Structure File Number) is a one-time entry and must be done before any updating (changes, inspection entries, etc.) can be performed.

An addition record must be coded completely and as correctly as possible since an extensive edit is performed on each bridge record added to the file.  All edit criteria is based on the coding guide instructions.  If errors are found, the computer will display the bridge record on an error listing and indicate the items in error.  Items found to be in error will not be entered onto the file, but will be voided and the entire bridge record will be flagged in error to insure corrective action.

In BMS you will use the Transaction Code of “A” and “ADD” in the Fast Path.

Duplicate bridge records, i.e., Structure File Numbers, will not be allowed.

## CHANGE (Transaction Code "C" on BR87 & 87A & “U” in BMS)

A bridge record can be changed by coding the Structure File Number, a "C" in Transaction code, and the complete data item(s) to be changed, or by accessing the appropriate on-line screen and directly entering the changes into BMS using Transaction Code “U”.

Changes must be made by item and the entire item must be coded to include the revised characters.

Any or all data items may be corrected on a change record either on-line or by batch forms.  Items to be changed will be edited for errors before altering the bridge record on the file.  Error items will be indicated on an error listing and the record will be in error status until the record is corrected.

A bridge record cannot be added and changed (transactions “A” and “C”) during the same computer process.

Some items on the bridge record allow blanks or no data entry in the item field

## RETIRE (Transaction Code "R" on BR87 & “U” in BMS)

A bridge record, i.e. Structure File Number, may be retired from the file by coding the Structure File Number and placing an “R” in the transaction code on the BR87 or using Transaction Code “U” on the “RET” screen in BMS.  In addition you must show the reason for retiring the structure by placing one of the following codes in the first space (character space 27) in the Feature(s) Intersected Item (Item #4) on card A of the BR87 or on the “RET” screen in BMS.

| Code | Description |
| --- | --- |
| 1 | Replaced (> 50% Sufficiency Rating) |
| 2 | Replaced (< 50% Sufficiency Rating) |
| 3 | Transferred to another agency (County, municipality, etc.) |
| 4 | Collapsed, not to be replaced. |
| 5 | Closed with no intent to replace. |
| 7 | Collapsed and replaced |
| 0 | Other |

When a SFN is retired from the BMS, its records do not get deleted but get frozen. No further additions/deletions/changes can be made to retired bridge records. The retired SFN cannot be re-assigned to any other bridge.

If a bridge gets accidentally retired, please contact the BIS in OSE to restore the retired bridge.

## RESTORE OR "UNRETIRE" (Transaction Code “U” in BMS)

A bridge record that has been erroneously retired may be restored to the active file.  Restore or "Un-retire" will only be done by the Bridge Inventory Section in ODOT, Central Office (CO).

## DELETE (Transaction Code “D” in BMS)

A bridge record may be deleted from the "Active and Inactive" files.  Deletes will only be done in the ODOT, Central Office (CO).  Please contact Bridge Inventory Section in the ODOT, CO.

A.  Structure File Number (SFN)

This is the identification number for the data file on a particular structure.  It is a seven (7) digit numeric which is assigned from a master list by the appropriate SFN control authority.  The first two (2) digits are the numeric code for the Ohio County (See Appendix "A") in which the structure is located.  The next four (4) digits make up the specific number for that structure.  The last or seventh digit is a computer calculated check digit which has no particular sequence, but is an integral and very necessary part of the file number.  Using this digit, the program can check the validity of the entire number before entering the data in the record.  Only numbers listed in the BM-116 report can be used.

The specific structure number (digits three thru six) will be assigned by the SFN Control Authority from predetermined blocks of numbers within any one county as follows:

| State Structures | 0001 thru 2999 |
| --- | --- |
| County Structures | 3000 thru 5999 |
| Municipal Structures | 6000 thru 8999 |
| Special Systems Structures | 9000 thru 9999 |

This block or set system of numbering will be duplicated in each County, thus permitting any record to be identified within a county as state, county or municipal.  The Structure File Number is a permanent number assigned to the structure when the bridge is first conceived and remains so until the entire structure is demolished or removed from the system.  If a structure is transferred from one jurisdiction to another, such as County to State or vice versa, it must be "Retired" from the first agency's record and "Added" to the receiving agency’s record with a totally new structure file number.  Structure file numbers for bridges transferred or demolished shall not be reassigned to new or other structures.  If it becomes necessary,   because of growth of an agency's system and through lack of availability of new unassigned numbers, a previously used number may be used for a new structure record, but only after approval is received for its reuse from the proper SFN control authority.

The State Structure Inventory section will serve as the SFN control authority for all structures on the State System and special systems state-wide.  County Bridge Engineers will control assignment of numbers to structures within their jurisdiction and the State District Bridge Engineers will control and assign small blocks of numbers for each Municipality within each County included in his/her district.  Each SFN control agency will be provided with initial lists of numbers available for assignment to structures under their jurisdiction.  Update listings showing assigned numbers to date as well as those still available for assignment will be provided to each SFN control agency by the "State" on a regularly scheduled basis.  Special listings can be obtained on a limited basis by contacting the State District Bridge Engineer.

It is imperative that the SFN is entered legibly and accurately on the forms BR87 and BR87A so that data will be entered in the proper structure’s file.

## GUIDELINES ON STRUCTURE FILE NUMBER (SFN)

When inventorying a bridge any structure(s) with a closed median is to be recorded as one bridge.  A closed median is where:

- The area between the two roadways on the structure is bridged over and is capable of supporting traffic, or

- The longitudinal deck opening at back-to-back median barriers is a maximum of 3 inches (75 mm) wide.

Separate superstructures with an open median (not meeting the closed median criteria above) sharing a common substructure unit or units are to be recorded as two bridges.  A structure carrying a ramp that merges into another structure, has at least one distinct abutment, and is greater than 10 feet (3.1 m) in length, is to be recorded as a separate bridge.  The separating point between bridges should be the closest deck joint or other logical and reasonable location as determined by the bridge owner.

A structure that divides into two (or more) separate structures (or vice versa) is to be recorded as two (or more) bridges.  The separating point between bridges should be the closest deck joint or other logical and reasonable location as determined by the bridge owner.

The SFN of a bridge should usually not change when work is performed, or the portions of a structure are reconstructed except in unusual circumstances.  A SFN is retained to maintain the history of the structure.  Existing SFN of a bridge should be retired and a new SFN shall be assigned when a completely new Structure is built to replace the old Structure. (Example: totally new super and sub structures or when the existing deck, superstructure and substructure are removed with the exception of the piles).

In case it is deemed necessary to retire the existing SFN and assign a new SFN to the bridge, a reference to the old SFN shall be made.

For other complex or unique situations not covered here, the bridge owner should contact the Office of Structural Engineering, Inventory Section.  See examples below:

| Example | Scope | Need New SFN? |
| --- | --- | --- |
| 1 | Deck replacement | No |
| 2 | Superstructure replaced with same type | No |
| 3 | Widening | No |
| 4 | Replacing superstructure and modifying piers and abutments | No |
| 5 | Replacing complete bridge including foundations | Yes |
| 6 | New Superstructure of different type | Yes |
| 7 | Replacing Wearing Surface | No |
| 8 | Replacing Super and Substructure except piles | Yes |
| 9 | Repairing, Replacing or removing joints, bearings, railings, parapets, sidewalks, etc. | No |
| 10 | Other than above | Contact OSE-BIS |

B.  Transaction Code

This is the second part of the record processing key and tells the computer program what action to take concerning the data shown on the Code Sheet.  The following codes shall be used:

| A | Add Record (for totally new records) |
| --- | --- |
| C | Change Record – Changes any item or all of an established record. Changes must be by item. The entire item must be recoded to include the revised characters. Record those items only which are being changed in current process. Do not fill unchanged items. |
| R | Retire Record – Removes entire record from active inventory. Also designates reason for retire. (See instructions for Retire in Transaction Code Functions) |

## PROCEDURE FOR SUBMITTING AND PROCESSING BR87/87A FORMS

“The Bridge Inventory Code Sheets" (BR87/87A) must precede or accompany the "Bridge Inspection Report" (BR86/86S) code form if a bridge record is being added to the file.

Code the "Bridge Inventory Code Sheets" (BR87 and BR87A) according to instructions.  County, municipal agencies and Ohio Turnpike should forward the coded sheets to the ODOT Central Office Structure Inventory Section.

FORM #BR87 - CARD A

## ITEM #1 - OHIO'S DESIGNATED MPOs [2 digits]

Enter a two (2) digit numeric code for the MPO in which the bridge is located.  If this item does not apply, code NN for None.  If the MPO is not known, code UU for Unknown.

See Appendix M or the link below for a list of MPOs by County.

http://www.dot.state.oh.us/Divisions/TransSysDev/ProgramMgt/CapitalPrograms/Pages/MPOMapContacts.aspx

| Code | Description |
| --- | --- |
| 01 | AMATS (Akron) |
| 02 | BHJTS (Steubenville-Weirton) |
| 03 | BOMTS (Wheeling-Bridgeport) |
| 04 | EDATA (Youngstown) |
| 05 | HAIATS (Huntington) |
| 06 | LCATS (Newark-Heath) |
| 07 | MORPC (Columbus) |
| 08 | NOACA (Cleveland) |
| 09 | OKI (Cincinnati) |
| 10 | SCATS (Canton) |
| 11 | MVRPC (Dayton) |
| 12 | TCC (Lima) |
| 13 | TCC (Mansfield) |
| 14 | TCC (Springfield) |
| 15 | TMACOG (Toledo) |
| 16 | WWW (Parkersburg-Belpre) |
| 17 | ERPC (Erie County-City of Vermilion) |
| NN | None |
| UU | Unknown |

## ITEM #2 - CITY OR TOWN (FIPS CODE) [5 digits]

Cities, towns, townships, villages, and other census designated places shall be identified using the Federal Information Processing Standards (FIPS) codes given in the current version of the Census of Population and Housing -- Geographic Identification Code Scheme.  If there is no FIPS place code, then code all zeros.

See Appendix D for list of F.I.P.S. codes.

## ITEM #3 - INVENTORY ROUTE [5 digits]

The Inventory route is any route along which structures are being inventoried.  Where possible, structures should be inventoried on the route carried on the structure.  The inventory will progress from the most important routes to the least important, by established hierarchy.  The inventory must start with the structure nearest the beginning of the route.  Remember a structure is to be inventoried only once and by only one agency.

When two (2) or more routes are concurrent (overlapped), the highest of the hierarchy of Systems as shown in the second position (System) below will be used.  If the concurrent routes are of the same hierarchy level, the lowest numbered route will be used. Routes chosen under these rules are commonly known as the “Preferred route".  Structures occurring in overlap areas shall be inventoried only on the preferred route.

A.	On /under [1 digit]

Enter the proper code to indicate if the route is carried by the structure or goes under it.

If there are two (2) or more physically separate routes under a structure you must inventory each route, using the same "Structure File Number" and coding appropriate A thru C "under" code for each route under the bridge.  The Inventory route can never be coded “B” or “C”.  It may be coded as “A” only under special circumstances when one of the multiple routes under the bridge is the inventory route.

| Code | Description |
| --- | --- |
| 1 | Route carried “on” the structure |
| 2 | Single route goes “under” the structure |
| A-C | Multiple routes go “under” the structure |
| “A” | Signifies the first of multiple routes under the structure |
| “B” | Signifies the second of multiple routes under the structure |
| “C” | Signifies the third of multiple routes under the structure |

"On" signifies that the inventory route is carried "on" the structure.  Each bridge structure carrying highway traffic must have a record identified with a type code = 1 (numeric).

"Under" signifies that the inventory route goes "under" the structure.  If an inventory route beneath the structure is on the National Highway System, is a defense route or is otherwise important, a record must be coded to identify it.  The type code must be 2 or alphabetic letter A through C.  When there is a single route under the bridge, code 2.  If 2 or more routes go under a structure on separate roadways, the code of 2 shall not be used.  Code A, B or C consecutively for multiple routes on separate roadways under the same structure.  For a non-highway system, structure (pedestrian, railroad, etc.), the most significant roadway should be coded “A” in the Inventory route and “B” and/or “C” as the Intersecting route.

The Inventory route is any route along which structures are being inventoried.  Where possible, structures should be inventoried on the route carried on the structure.  The inventory will progress from the most important routes to the least important, by established hierarchy.  The inventory must start with the structure nearest the beginning of the route.  Remember a structure is to be inventoried only once and by only one agency.

B.	System [1 digit]

| Code | Description |
| --- | --- |
| 1 | Interstate Highway |
| 2 | U.S. Numbered Highway |
| 3 | State Highway |
| 4 | County Highway (Township Highway) |
| 5 | Municipal Street (i.e. village, town, city) |
| 6 | Federal Lands Road |
| 7 | State Lands Road – ODNR |
| 8 | Other (including toll roads not otherwise identified) |
| 0 | Non-Highway Related (Example – A Park Pedestrian Bridge or a Bike Path over a waterway) |

C.	Route Number [5 digits]

The official Route Number as shown on the State's Straight Line Mileage diagrams or posted in the records of the Agency involved, shall be coded and right justified in the five (5) available positions using leading zeros where necessary.  The route number will normally be a one (1) or more digit numeric.  A single letter code must be added to the route numeric in the last digit position only for routes, designated as Alternate, Directional Alternate, By Pass, etc. (See Appendix I for official letter codes to be used for such routes), these codes have no directional connotation and must be coded in the fifth (5) position only of the five (5) character field.  Meaningful abbreviations for street and road names should be used by local agencies, for those streets or local roads without an established system route number.

For special off-highway system bridges, the fourth (4) and fifth (5) digit locations of the route number must be coded with a standardized alphabetic two (2) character suffix which identifies the public complex in which the bridge is located.

Example:  State Park - SP

D.	Directional Suffix (1 digit)

Code the directional suffix to the route number when one is part of the route number.  Leave blank if not applicable.

| Code | Description |
| --- | --- |
| 1 | North |
| 2 | East |
| 3 | South |
| 4 | West |

Note:	Parts "C" and "D" must be coded carefully since they will be used as part of the bridge number (Item #5) and will be the key for properly differentiating one route from another for listing purposes.

E.	Designation [1 digit]

Code a single numeric to designate the roadway carried by or going under the structure.

| Code | Description |
| --- | --- |
| 0 | Other |
| 1 | Mainline |
| 2 | Alternate |
| 3 | By-Pass |
| 4 | Spur |
| 5 | Toll Road |
| 6 | Business |
| 7 | Ramp or Wye |
| 8 | Service and/or unclassified frontage road |
| 9 | Truck Route |

Generally county and township roads and municipal streets are coded as mainline.

F.	Preferred Route [1 digit]

Code letter "P" if the inventory route is the preferred route in an overlap area.  Leave blank if not in overlap area.

## ITEM #4 - FEATURE(S) INTERSECTED

A.	Features Description (25 digits)

The information to be coded for the first twenty-four (24) characters of this item shall be the name or names of features intersected by the Inventory route at the structure. When a highway is one of the features intersected, the number of such highway (e.g., I.R. 71, C.R. 16, and S.R. 5) should be coded if known, then the local name or names of the route (Mill Rd., Stone Street, etc.).  If other features such as streams, canals and railroads are crossed, they should be coded next.  Abbreviations may be used, but should be meaningful.  If a single description is all that is required and it is shorter than twenty-five (25) characters it is preferable that it be left justified (i.e., started in the left most position), without trailing zeros.  If a bridge is closed to all traffic, code the word "closed" in parenthesis after the features description.

B.	Special (Reserved) [1 digit]

The 25th Character of this item must be left blank.

## ITEM #5 - INVENTORY ROUTE BRIDGE NUMBER

This is the Structure Location Number and will be incorporated with parts "C" and "D" of "Inventory route" to form the Bridge Number.  The Bridge Number should allow ready identification and easy location of any structure.

Note:	The following descriptions for coding bridge numbers apply only to State Highway Structures, but a similar scheme may be used by other agencies.  Counties, Cities and Townships must establish and follow some standard numbering system for their inventory. These agencies may ignore the sub-headings under this item and code their Bridge Number left justified, and using no more than the ten (10) characters allowed under this item.

A.	County [3 digits]

Code the three (3) characters alphabetic shown in Appendix B for the County in which the structure is located.  Counties may want to adopt a scheme similar to this one.  If so, the county could code a three (3) character (locally established standard) alphabetic code for the township in which the structure occurs.  If a municipality has a desire to adopt the same scheme, they should use a meaningful three (3) letter abbreviation for the name of the municipality.

B.	Unit Number

The Unit Number is the specific number of a structure established by its location along the inventory route and for its special location with respect to the inventory route.  It is made up of some or all of several parts as follows:

1.	Straight Line Mileage (SLM) [5 digits]

The first four (4) characters are coded as a numeric representing the distance from the South or West County Line or other beginning of the Route as established in the State's Straight Line Diagrams.  Code leading zeros where necessary.  The fifth position is reserved exclusively for coding a "B" for back station as shown in the Straight Line Diagrams.  If not coded, the fifth position will be blank.

2.	Special Designation [2 digits]

The first position shall be used for coding "L" or "R" for Left and Right parallel structures.  On divided highways with clearly separate and parallel structures for each direction of travel, each structure will carry the same straight line mileage designation, with the addition of the letter (L) for the left (or non-cardinal direction) structure and (R) for the right (or cardinal direction) structure.

On divided highways where the deck is continuous across both directions of travel and there is a longitudinal joint near the center of the bridge (closed median by F.H.W.A. definition), code as one structure.  (3" or less open joint at the centerline does not constitute parallel bridges.)  Both the positions may be coded to define ramp structures by using a one (1) or two (2) character alphabetic code.  Note:  For ramp definitions, the letters "L" and "R" may not be used in the first position.  Details on basic ramp designation schemes can be obtained from the Structure Inventory Section.  Final decision on ramp designations will be made by the Structure Inventory Section.

## ITEM #6 - AVERAGE DAILY TRAFFIC [6 digits]

Code a six (6) digit numeric that shows the average daily traffic volume for the Inventory route. Use your best estimate of the traffic volume where an actual count is not available. In coding traffic counts for parallel Left and Right bridges on Multi-lane divided highways, (physically separate structures filed under different Structure File Numbers) code one-half the official route traffic count for each bridge.  Right justify filling with leading zeros where necessary.  If the bridge is closed code the actual ADT from before the closure occurred.  If System = 0 for non-highway related traffic code A.D.T. all zero's.

If coding traffic in BMS, you must use Transaction Code “A” to add a new A.D.T.  BMS keeps a history of A.D.T.

Examples:

| Traffic Volume | Code |
| --- | --- |
| 540 | 000540 |
| 1560 | 001560 |
| 24000 | 024000 |
| 893 | 000893 |

## ITEM #7 - AVERAGE DAILY TRAFFIC YEAR [4 digits]

Code the four (4) digits of the year of the Traffic Count used.  (Or Year Estimated) ADT needs to be within last 4 years. If the Highway System Code = 0, leave this item blank.

## ITEM #8 - AVERAGE DAILY TRUCK TRAFFIC [5 digits]

Code a five (5) digit figure to indicate the amount of truck traffic for the inventory route.  Right justify filling with leading zeros where necessary.  For physically separate parallel structures, code one-half the traffic for each bridge.  If Item #6 is blank leave this item blank.

Truck traffic where this item is concerned does not include vans, pick-up trucks, other light delivery trucks, school buses or recreational vehicles.

Show the actual number of trucks, if not available, estimate as closely as possible.

If a bridge is closed code the actual ADT before the closure occurred.

## ITEM #9 - DIRECTION OF TRAFFIC [1 digit]

Code the direction of traffic of the inventory route identified in Item #3 as a one (1) digit number using one (1) of the codes below.  If the bridge is under a railroad don’t use code 0, but code for route under the railroad.

| Code | Description |
| --- | --- |
| 0 | Highway traffic not carried |
| 1 | One way traffic |
| 2 | Two way traffic |
| 3 | One lane bridge for two way traffic |

## ITEM #10 - TEMPORARY STRUCTURE DESIGNATION [1 digit]

Code this item to indicate situations where temporary structures or conditions exist. This item should be coded N if not applicable.

| Code | Description |
| --- | --- |
| Y | Temporary structure or conditions exist |
| N | Not Applicable |

Temporary structure(s) or conditions which are required to facilitate traffic flow.  This may occur either before or during the modification or replacement of a structure found to be deficient.  Such condition includes the following:

1.	Bridge shored up, including additional temporary supports.

2.	Temporary repairs made to keep a bridge open.

3.	Temporary structures, temporary run-around or bypasses.

4.	Other temporary measures, such as barricaded traffic lanes to keep the bridge open.

Any repaired structure or replacement structure, which is expected to remain in place without further project activity other than maintenance, for a significant period of time, shall not be considered temporary.  Under such conditions, that structure regardless of its type shall be considered the minimum adequate to remain in place and be evaluated accordingly.

If this item is coded Y, then all data recorded for the structure shall be for the condition of the structure without temporary measures, except for the following items which shall be for the temporary structure:

| Item | Description |
| --- | --- |
| 66 | (on Inspection Report Form BR-86) – Structure Open, Posted, or Closed to Traffic |
| 154 | Minimum Horizontal Clearance On Bridge |
| 77 | Minimum Vertical Clearance Under Bridge |
| 78 | Minimum Lateral Under clearance on Right (Cardinal) |
| 78 | Minimum Lateral Under clearance on Left (Non-Cardinal) |
| 67 | Minimum Vertical Clearance On Bridge |

This item must be coded for all bridges.

## ITEM #11 - DESIGNATED NATIONAL NETWORK [1 digit]

The national network for trucks includes most of the Interstate System and those portions of the Federal Aid Primary System Identified in the Code of Federal Regulations (23 CFR 658). The national network for trucks is available for use by commercial motor vehicles of the dimensions and configurations described in these regulations.  For the inventory route identified in Item #3, indicate conditions using one of the following codes:

| Code | Description |
| --- | --- |
| N | The inventory route is not part of the national network for trucks. |
| Y | The inventory route is part of the national network for trucks. |

Central Office shall code this item until further notice.  All agencies except the State shall code this item N.

## ITEM #12 - PARALLEL STRUCTURES [1 digit]

This item indicates situations where separate structures carry the traffic on the route in opposite directions over the same feature.  Use the appropriate code when the route is a county road or city street.  If the route is under a railroad, pipeline etc., or there is no parallel structure code N.

| Code | Description |
| --- | --- |
| R | Structure carrying east bound or north bound lanes and there is a corresponding structure carrying traffic in the opposite direction. Cardinal Direction |
| L | Structure carrying west bound or south bound lanes and there is a corresponding structure carrying traffic in the opposite direction. Non-Cardinal Direction |
| N | No parallel structure |

This item must be coded.

ITEM #13 - [BLANK]

FORM #BR87 - CARD B

## ITEM #14 - NATIONAL HIGHWAY SYSTEM (NHS) [1 digit]

This code shall reflect a structure on the designated NHS as reported to the FHWA to meet the NHS "Interim System" description in Section 1006(a) of the 1991 ISTEA.  For the Inventory Route (Item #3), indicate whether the route is on the NHS or not on that system.  To see a map of the NHS Routes visit the link below:

www.fhwa.dot.gov/hep10/nhs

| Code | Description |
| --- | --- |
| N | NHS bridges excluding Congressional Corridor bridges |
| H | NHS bridges that are also Congressional Corridor bridges |
| C | Non-NHS bridges that are also Congressional Corridor bridges |
| X | Non-NHS bridges excluding Congressional Corridor bridges |

## ITEM #15 - MACRO LEVEL CORRIDOR [1 digit]

This item indicates whether or not the Inventory route (Item #3) is on a Macro Level Corridor route. These routes have been determined by Access Ohio.

http://www.dot.state.oh.us/maps/Pages/MacroCorridorMap.aspx

| Code | Description |
| --- | --- |
| N | Structure is not a Macro Level Corridor |
| Y | Structure is a Macro Level Corridor |

## ITEM #16 - FUNCTIONAL CLASSIFICATION [2 digits]

Use the appropriate functional classification code listed below.  This item cannot be coded without first referencing the applicable Functional Classification Map.  These maps are available through the Department of Transportation District Planning Offices or on the Planning website.

http://www.dot.state.oh.us/DIVISIONS/TRANSSYSDEV/PROGRAMMGT/FUNCTIONALCLASS/Pages/FunctionalClassificationMaps.aspx

If System Code = 0, code this item 00.

A bridge shall be coded Rural if not inside a designated Urban area.

| Code | Description |
| --- | --- |
| 00 | Park or Other Public Reservation Road (0 System Code) |

Rural

| Code | Description |
| --- | --- |
| 01 | Principal Arterial - Interstate |
| 02 | Principal Arterial – Other |
| 06 | Minor Arterial |
| 07 | Major Collector |
| 08 | Minor Collector |
| 09 | Local |

Urban

| Code | Description |
| --- | --- |
| 11 | Principal Arterial – Interstate |
| 12 | Principal Arterial - Other Freeways or Expressways |
| 14 | Other Principal Arterial |
| 16 | Minor Arterial |
| 17 | Collector |
| 19 | Local |

ITEM #17 & #18 – [BLANK]

## ITEM #19 - STRAHNET HIGHWAY DESIGNATION [1 digit]

Strategic Highway Corridor Network (STRAHNET) is a system of highways which are strategically important to the defense of the United States.  It includes the Interstate Highways and other non-interstate highways.  STRAHNET connectors are roads that connect military installations and ports of embarkation to the STRAHNET.  Indicate the STRAHNET highway situation, using one of the following codes:

| Code | Description |
| --- | --- |
| 1 | Strahnet Interstate Bridges |
| 2 | Strahnet Non-interstate bridges |
| 3 | Strahnet Non-interstate connector bridges |
| 0 | Non-Strahnet bridges |

All counties and municipalities shall code this item 0.

http://www.fhwa.dot.gov/planning/nhs/maps/oh/oh_ohio.pdf

ITEM #20 & #21 - [BLANK]

## ITEM #22 - INTERSECTED ROUTE (5 digits)

Code same as for Item #3 with respect to the Intersected route.

## ITEM #23 - FEATURE(S) INTERSECTED [24 digits]

Code same as for Item #4 with respect to the Intersected route.

## ITEM #24 - INTERSECTED ROUTE BRIDGE NUMBER [5 digits]

Code same as for Item #5 with respect to the Intersected route.

Note:	If the coding agency numbers only the bridge (Roadway on the bridge) and not the underpass (Roadway under the bridge) then it is necessary to create a number for the underpass, preferably based on mileage along the intersected route.  This is necessary only for structures separating the grades of highways or streets. If a bridge is not coded on the intersecting route, a structure will not be recognized at that location on the route.

## ITEM #25 – INTERSECTED ADT [6 digits]

Code same as for Item #6 with respect to the Intersected route.

## ITEM #26 - INTERSECTED ADT YEAR [2 digits]

Code same as for Item #7 with respect to the Intersected route.

FORM #BR87 - CARD C

## ITEM #27 - INTERSECTED TRUCK ADT [5 digits]

Code same as for Item #8 with respect to the Intersected route.

## ITEM #28 - INTERSECTED NATIONAL HIGHWAY SYSTEM (NHS) [1 digit]

Code same as for Item #14 with respect to the Intersected route.

## ITEM #29 - INTERSECTED MACRO LEVEL CORRIDOR (1digit)

Code same as for Item #15 with respect to the Intersected route.

## ITEM #30 - INTERSECTED FUNCTIONAL CLASSIFICATION [2 digits]

Code same as for Item #16 with respect to the Intersected route.

ITEM #31 – [BLANK]

## ITEM #32 - INTERSECTED DIRECTION OF TRAFFIC (1 digit)

Code same as for Item #9 with respect to the Intersected route.

## ITEM #33 - INTERSECTED DESIGNATED NATIONAL NETWORK [1 digit]

Code same as for Item #11 with respect to the Intersected route.

## ITEM #34 - INTERSECTED PARALLEL STRUCTURES [1 digit]

Refer to Item #12 with respect to the Intersected route.

ITEM #35 – [BLANK]

## ITEM #36 - INTERSECTED STRAHNET HIGHWAY DESIGNATION [1 digit]

Code same as for Item #19 with respect to the Intersected route.

ITEM #37 - [BLANK]

## ITEM #38 - BYPASS LENGTH [2 digits]

This item must be coded by all agencies for each structure inventoried.  The bypass or detour length should be determined with respect to the route carried by the bridge, regardless of how the structure appears on the Inventory Route.  If a ground level bypass is available at the structure site for the route over the structure, code the detour length as zero.  If the bridge is one of twin bridges and is not at an interchange code 01 to indicate that the other twin bridge can be used as a temporary bypass.  In other cases, indicate the detour distance or length to the nearest mile using the shortest temporary “Alternate" route having comparable structures.  The detour length should be coded to represent only the total additional travel (see Appendix "J") for a vehicle which would result from closing of the bridge.  The factor to consider when determining if a bypass is available at the site is the potential for moving vehicles, including military vehicles, around the structure.  This is particularly true when the structure is in an interchange.  For instance, a bypass likely would be available in the case of diamond interchanges where there are ramps available or other interchanges where the positioning and layout of the ramps and service roads are such that they could be used without difficulty to get around the structure.  Use 00 for railroad, pedestrian walk, pipeline, etc., since they carry non highway traffic.  It will appear on the inventory route as an underpass only, considered readily by passable at all times.  For detour lengths exceeding 61 miles or more code “99”.  Use "XX" coding for structures located on any route which has no outlet (dead end) and no detour possible.

| Examples | Code |
| --- | --- |
| Culvert | 00 |
| Diamond interchange, structure by passable | 00 |
| Cloverleaf, not by passable, 8 mile detour | 08 |
| Structure over river, 121 mile detour | 99 |
| Railroad structure over highway (Inventory route), by passable | 00 |
| Structure over stream, on local road, low traffic, no detour possible | XX |

ITEM #38 – BYPASS LENGTH

## ITEM #39 - COORDINATES [16 digits]

As many more mapping applications are using BMS data, it is imperative to code the bridge coordinates accurately.

A.	Latitude (XX degrees, XX minutes, XX.XX seconds) (8 digits)

Code the Latitude of the rear or beginning point of each structure in the direction of inventory.  Code the Latitude in degrees, minutes, and seconds to the nearest hundredth of a second.

Example:  35 degrees 27' 32.50"	Code 35273250

Ohio’s Limits:	38 degrees, 23 minutes, 10.80 seconds

41 degrees, 58 minutes, 43.20 seconds

B.	Longitude (XX degrees, XX minutes, XX.XX seconds) (8 digits)

Code the Longitude as instructed by "A" above for Latitude.  Code the Longitude in degrees, minutes, and seconds to the nearest hundredth of a second.

Example:  81 degrees 5' 45.18"	Code 81054518

Ohio’s Limits: 	80 degrees, 30 minutes, 04.80 seconds

84 degrees, 50 minutes, 15.60 seconds

## ITEM #40 - TOLL [1 digit]

Code this item with respect to the route on the bridge.  Use the proper one (1) digit numeric code from the list below:

| Code | Description |
| --- | --- |
| 1 | Toll Bridge, Tolls are paid specifically to use the structure. |
| 2 | On Toll road, the structure carries a toll road, that is, tolls are paid to use the facility, which includes both the highway and structure. |
| 3 | On Free road, the structure is toll free and carries a toll free highway. (Default value) |
| N | Not Applicable (i.e. railroad, pedestrian bridges, etc.) |

## ITEM #41 - DATE BUILT [8 digits]

Code the actual day, month and year the bridge was completed, if unknown code 07/01/1900.

| #### Examples | #### Code |
| --- | --- |
| #### Built June 30, 1928 | #### 06301928 |
| #### If you cannot estimate the year built. | #### 07011900 |

## ITEM #42 - DATE OF MAJOR REHABILITATION [8 digits]

Code the date of the latest month, day and year of major reconstruction (rehabilitation) of the structure.

Replacement of an entire deck or work of a greater magnitude should be termed "Major Reconstruction".  Beyond these guidelines "Major Reconstruction" will be a judgmental determination, by a qualified person in the agency submitting the change sheet.

Some types of work not to be considered as major reconstruction are listed below:

- Safety feature replacement or upgrading (for example, bridge rail, approach or impact attenuators).

- Painting of structural steel.

- Overlaying of bridge deck as part of a larger highway re-surfacing project (for example overlay carried across bridge deck for surface uniformity without additional bridge work).

- Utility work.

- Emergency repair to restore structure integrity to the previous status following an accident.

- Retrofitting to correct a deficiency which does not substantially alter physical geometry or increase the load carrying capacity.

- Work performed to keep a bridge operational while plans for complete rehabilitation or replacement are under preparation (for example, adding a substructure element or extra girder).

| Examples | Code |
| --- | --- |
| Built May 1, 1898 Major Reconstruction October 10, 1949, July 1, 1964 | 07011964 |
| If there has never been Major Reconstruction on the bridge. | 00000000 |

## ITEM #43 - NUMBER OF LANES [4 digits]

Code the number of through marked lanes carried by the structure and being crossed over by the structure as a four (4) digit number composed of two (2) segments of two (2) digits each.  The first segment from left shall contain the number of lanes ON the structure.

The number of lanes should be coded right justified in each segment with leading zero(s) coded as required.  If there is no highway over or under a structure, code all zeros in the appropriate segment. Where there are no lane markings on a roadway, code number of lanes as intended to carry through the traffic.

Include all lanes carrying highway traffic (i.e., cars, trucks, busses) which are striped or otherwise operated as full width traffic lanes for the entire length of the structure or under the structure.  This should include any full width merge lanes and ramp lanes, and shall be independent of directional usage (i.e., a one lane bridge carrying two-way directional traffic is still considered to carry only one lane on the structure).

When an inventory route is over or under a non-highway obstruction (railroad, pedestrian, pipeline, etc.), code 00 in the appropriate segment for the non-highway obstruction.  For example, when the inventory route is “under” the structure carrying railroad tracks, code 00 for the railroad bridge over if there are no highway lanes on the obstructing structure.

| Examples | Code |
| --- | --- |
| 16 lanes on and 0 lanes under the structure | 1600 |
| 4 lanes on and 2 lanes under the structure | 0402 |
| Railroad or pedestrian on, 4 lanes under | 0004 |

If a double deck bridge is coded as one structure, code total number of lanes on both decks as noted in the examples below:

| Examples | Code |
| --- | --- |
| 5 lanes on double deck each direction, 2 lanes under | 1002 |

## ITEM #44 – DEGREE OF HORIZONTAL CURVE [4 digits]

This item must be coded for all structures, where the route carried by the structure is located within a horizontal curve.  Code this item to the nearest degree and minute.

If the structure is entirely or primarily on a tangent, leave this item blank.  For structures on a curve code the curve.  For structures on a spiral code the spiral as 9999.  And for non-tangent structures of unknown curvature code them 0000.

| Curve | Code |
| --- | --- |
| 18 degrees - 30' | 1830 |
| 25 degrees - 06' - 35" | 2507 |
| Spiral | 9999 |
| Curve unknown | 0000 |

## ITEM #45 - SKEW [2 digits]

The skew angle is the angle between the centerline of a pier or abutment or a pipe and a line normal to the roadway centerline.  When plans are available, the skew angle can be taken directly from the plans.  If no plans are available, the angle is to be field measured, if possible.

The skew should be coded to the nearest degree. When the structure is on a curve or if the skew varies for some other reason, the average skew should be coded, if reasonable.  Otherwise, code "99" to indicate a major variation in skews of sub-structure units.  If the structure is not skewed code zeros.  Always code a two (2) digit numeric using leading zeroes where necessary.

| Skew | Code |
| --- | --- |
| 10 degrees | 10 |
| 8 degrees | 08 |
| 29 degrees | 29 |

## ITEM #46 – UTILITIES (1 digit)

This item will indicate the type of utilities carried by the bridge.

| Item # | Type |  |
| --- | --- | --- |
| 46A | Electric | 1 digit |
| 46B | Gas | 1 digit |
| 46C | Sanitary Sewer | 1 digit |
| 46D | Telephone | 1 digit |
| 46E | T.V. Cable | 1 digit |
| 46F | Water | 1 digit |
| 46G | Other | 1 digit |

If the bridge carries a utility code the proper utility listed above; Code N if it does not.

Code other if a bridge carries a utility other than those listed above (i.e. steam line, coal slurry, petroleum line, pulp line, etc.).  Code a “U“ if Unknown.

ITEM #47 - [BLANK]

## ITEM #48 - DESIGN LOADING [1 digit]

Code for all structures the live load for which the structure was designed.  For most of the structures the Design Loading can be taken directly from the plans.  Classify any nonstandard loading, when feasible, using the nearest equivalent of the (AASHTO) H-loadings given below:

| Code | Description |
| --- | --- |
| 1 | H10 |
| 2 | H15 |
| 3 | HS15 |
| 4 | H20 |
| 5 | HS20 |
| 6 | HS20-44 & Alternate Military Loading |
| 7 | HS25 |
| 8 | HL93 |
| P | Pedestrian |
| R | Railroad |
| 0 | Other (includes railroad bridges w/track removed) |
| U | Unknown |

FORM #BR87 - CARD D

## ITEM #49 - APPROACH ROADWAY WIDTH [3 digits]

This item should be coded for all structures with reference only to the roadway carried by the structure.  Code a three (3) digit number to the nearest foot which represents the normal width of the roadway approaching the structure.  This dimension will include the widths of the usable shoulders. For paved approach medians, the median width at the normal point should be included in this dimension. For multiple lane divided highways, where separate parallel bridges carry each direction of traffic, code only that portion, of the approach roadway width which applies to each bridge.  Code the shoulders and directional pavement, ignoring unpaved median widths.  When there is a variation between the approaches at either end of the structure, record and code the most restrictive of the approach conditions.  If item does not apply, code all zeros.

| LEFT |  | RIGHT |  |
| --- | --- | --- | --- |
| SHOULDER\ROADWAY | MEDIAN | ROADWAY\SHOULDER | CODE |
| 4.0\00.0 | 00.0 | 16.0\06.0 | 026 |
| 6.0\00.0 | 00.0 | 36.0\12.0 | 054 |
| 12.0\48.0 | 00.0 | 48.0\12.0 | 120 |
| 10.0\24.0 | 16.0 | 36.0\10.0 | 096 |

## ITEM #50 - BRIDGE ROADWAY WIDTH (CURB TO CURB)

[4 digits]

The information to be coded in this item is the distance between curbs or sidewalks on the structure roadway. A four (4) digit code should be used to represent the distance to the nearest tenth of a foot (decimal point is implied).

If a median (level or raised) exists on the structure, the distance will be between the outside curbs of the structure; i.e., the distance will include both roadways and the median width.

The measurement should be exclusive of flared areas for ramps: i.e., it should be the minimum or nominal width. When a curb (9 inches or less in width) is used, code the measurement to face of parapet, guardrail, or railing.

For structures carrying non-highway traffic (railroads, pedestrian, buildings, conveyers, etc.) and structures with no deck such as most culverts, code all zeros, right justify all entries and fill with leading zeros where necessary.

Where deflector type parapets are the restraining elements, the bridge roadway width is measured toe/toe of parapets.

This item must be equal to or less than Deck Width (Out/Out).

Examples

| ##### Bridge Roadway Width | ##### Code |
| --- | --- |
| ##### 36.0’ | ##### 0360 |
| ##### 110.13’ | ##### 1101 |
| ##### 66.37’ | ##### 0664 |

## ITEM #51 - DECK WIDTH (OUT/OUT) [4 digits]

Code a four (4) digit number to show the out/out width of the deck to the nearest tenth of a foot (decimal point is implied). If the structure is a through structure, the number to be coded will represent the lateral clearance between superstructure members.  The measurement should be exclusive of flared areas for ramps, i.e., it should be the minimum width.

For structures with no deck such as most culverts, code all zeros; right justify all entries and fill with leading zeros where necessary.

This item must be equal to or greater than bridge roadway width.

Examples

| ##### Deck Width (OUT/OUT) | ##### Code |
| --- | --- |
| ##### 36.0’ | ##### 0360 |
| ##### 121.5’ | ##### 1215 |
| ##### 29.69’ | ##### 0297 |

## ITEM #52 – BRIDGE MEDIAN TYPE [3 digits]

This item must be coded for each structure with respect to the route on the structure. Code three (3) separate digits as follows:

| 1st Digit | 2nd Digit | 3rd Digit |
| --- | --- | --- |
| 1 - Level Median | 1 - Steel Barrier (Rail) | 1 - Open Joint |
| 2 - Raised Median | 2 - Concrete Barrier (Rail) | 2 - Filled Joint |
| N - None | 3 - 32” Deflector Type (New Jersey Shape) | N - No Joint |
|  | 4 - 42” Deflector Type (New Jersey Shape) |  |
|  | 5 - 50” Deflector Type (New Jersey Shape) |  |
|  | 6 - 32” Deflector Type (General Motors Shape) |  |
|  | 7 - 36” Deflector Type (New Jersey Shape) |  |
|  | N - Non Barrier |  |
|  | 0 - Other (Barrier) |  |

All digit locations must be filled with appropriate codes.  If no median exists code “NNN".

All culverts with fill and grass median code "NNN".

## ITEM #53 - BRIDGE MEDIAN CODE [1 digit]

Indicate if the median is non-existent, open or closed.  The median is closed when the area between the two (2) roadways at the structure is bridged over and is capable of supporting traffic.  All bridges that carry either one-way traffic or two-way traffic separated only by a centerline will be coded N for no median.

This item must be filled with the appropriate code from the list below:

| Code | Description |
| --- | --- |
| 1 | Open median |
| 2 | Closed median (no barrier) |
| 3 | Closed median with non-mountable barriers |
| N | No median |

ITEM #53 - BRIDGE MEDIAN

## ITEM #54 - CURBS OR SIDEWALKS [2*3 digits]

Code the widths of the right and left curbs or sidewalks to the nearest tenth of a foot.  This essentially is a six (6) digit number, with the leftmost three (3) digits representing the left sidewalk or curb and the rightmost three (3) digits representing the right sidewalk or curb. "Left" and "Right" should be determined on the basis of the established or cardinal direction of the route.  Right justify and fill with leading zeros where necessary in each subsection.  (Sidewalks are greater than two feet (2') in width.)

| Left Side | Right Side | Code |
| --- | --- | --- |
| None | 8.3’ | 000083 |
| 10.0’ | 4.1’ | 100041 |
| 12.1’ | 11.5’ | 121115 |

Code all zeros if no sidewalks or curbs exist.

## ITEM #55 - TYPE OF CURB OR SIDEWALKS [2*2 digits]

Code the most appropriate type curb(s) or sidewalk(s) on the bridge (at both the right and left sides of the structure).  Two digits are to be coded for the curb or sidewalk on each side of the deck.  The first two digits from left represent the curb or sidewalk on the left side of the deck and the last two digits on right represent the curb or sidewalk on the right side of the deck (Right & left sides of deck looking in the direction of the route).

Code the first digit of each two digit group to represent the material and the second digit to represent the type of curb or sidewalk.  If no curb or sidewalks are present code this item “NN”.

| First Digit Code | Material |
| --- | --- |
| 1 | Concrete |
| 2 | Steel |
| 3 | Timber |
| N | None |
| 0 | Other |

| Second Digit Code | Type |
| --- | --- |
| 1 | Safety Curb (2’ or less width) |
| 2 | Sidewalk (greater than 2’ in width) |
| 3 | Felloe Guards |
| 4 | Open Grid |
| 5 | Filled Grid |
| 6 | Check Plate |
| N | None or N/A (RR, Pedestrian, etc.) |
| 0 | Other |

| Code | Example |
| --- | --- |
| 1211 | Concrete safety curb on right side and concrete sidewalk on left side. |
| NN11 | No curb or sidewalk on left; concrete safety curb on right |
| 11NN | Concrete curb on left only |

## ITEM #56 - FLARED [1 digit]

This item is coded to indicate whether or not the width of the structures varies.  Generally, such variance will result from ramps converging with or diverging from the through lanes on the structure, but there may be other causes. Minor flares at ends of structures should be ignored.

This item must be coded.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No flare or not applicable |

## ITEM #57 - COMPOSITE STRUCTURE [1 digit]

This item will reflect whether a structure is built composite (i.e. beams w/shear connectors) by using the following codes.  Also considered to be composite are structures with steel beams encased in concrete and jack arch bridges.

Do not code bridges with composite material here.

| Code | Description |
| --- | --- |
| Y | Composite Construction |
| N | Non-composite Construction |
| X | Not Applicable |
| U | Unknown |

If the bridge carries non-highway traffic (i.e. railroad, pipeline, pedestrian) over an agency's highway or if the structure is a culvert, code X.

This item must be coded for all structures being "Added" to the file.

## ITEM #58 - BRIDGE RAILING TYPE [1 digit]

Code a one (1) digit numeric for the type of bridge railing as follows:

| Code | Description |
| --- | --- |
| 1 | Reinforced Concrete Parapet |
| 2 | Reinforced Concrete and Steel |
| 3 | Reinforced Concrete Safety Curb and Parapet with Aluminum Railing |
| 4 | Reinforced Concrete Post and Steel Panel |
| 5 | Reinforced Concrete Post and Concrete Panel |
| 6 | Steel Post and Steel Panel (Decorative) |
| 7 | Steel Guardrail on Steel, Concrete or Timber Posts |
| 8 | Timber |
| 9 | Twin Steel Tube Bridge Railing |
| N | None |
| 0 | Other |
| A | Tubular Backup |
| B | Tri-Beam |
| C | 32” Deflector Type Parapet (New Jersey Shape) |
| D | 42” Deflector Type Parapet (New Jersey Shape) |
| E | 50” Deflector Type Parapet (New Jersey Shape) |
| F | 32” Deflector Type Parapet (General Motors Shape) |
| G | 36” Deflector Type Parapet (New Jersey Shape) |
| H | 57” Deflector Type Parapet (Single Slope) |
| I | 42” Deflector Type Parapet (Single Slope) |

When more than one (1) type of railing exists on the bridge, code the item for the predominate type. If railing type cannot be determined code "0".  If no railing exists on the structure; such as for all culverts; code N.

This item must be coded for all structures.

## ITEM #59 - DECK DRAINAGE [1 digit]

Code a one (1) digit numeric for the drainage system listed below which most nearly describes that on the structure.

| Code | Description |
| --- | --- |
| 1 | Over the side (without drip strip) |
| 2 | Opening thru curbs or wheel guards |
| 3 | Scuppers and downspouts |
| 4 | Inlets with drain pipes |
| 5 | Drainage trough under open joints |
| 6 | Over the side (with drip strip) |
| N | None |
| 0 | Other (Natural-off the bridge ends) |

If no deck exists, such as for most culverts, code N.

This item must be coded.

## ITEM #60 - DECK TYPE [1 digit]

Code a one (1) digit numeric to describe the deck type as follows:

| Code | Description |
| --- | --- |
| 1 | Reinforced concrete (Prestressed Slabs, Precast Slabs, Prestressed Box Beams, Concrete Rigid Frames) |
| 2 | Laminated timber strip |
| 3 | Timber plank |
| 4 | Steel grid – filled |
| 5 | Steel grid – open |
| 6 | Corrugated steel |
| 7 | Steel buckle plate |
| 8 | Steel checker plate |
| N | None |
| 0 | Other |
| A | Jack Arch |
| B | Slab prestressed in both directions |
| C | Slab prestressed longitudinally only |
| D | Slab prestressed transversely only |
| E | Prestressed slab on beams |

Where no deck or floor slab exists, such as for culverts, code item as N.  When more than one type of floor slab exists over the full length of the bridge, code the item for the predominate type. If a pre-dominate type cannot be determined, code zero "0".

## ITEM #61 - DECK PROTECTION [2 digits]

Use 1st Digit for External Deck Protection (Type of Membrane Waterproofing)

| Code | Description |
| --- | --- |
| 1 | Built-up (Type “D”, Layers of fiberglass and coal tar) |
| 2 | Performed Fabric (Type III Waterproofing) |
| 3 | Epoxy |
| U | Unknown |
| N | None or Not Applicable (applies only to structure with no deck i.e. culverts, filled arches) |
| 0 | Other |

Use 2nd Digit for Internal Deck Protection

| Code | Description |
| --- | --- |
| 1 | Epoxy Coated Reinforcing (Top Mat) |
| 2 | Galvanized Reinforcing |
| 3 | Other Coated Reinforcing Bar |
| 4 | Cathodic Protection |
| 6 | Polymer Impregnated |
| 7 | **Internally Sealed |
| 8 | Epoxy Coated Reinforcing (Both Mats) |
| N | None or Not Applicable (applies only to structures with no deck i.e. culverts, prestressed box beams). |
| 0 | Other |
| U | Unknown |

**Internally sealed:  Wax beads mixed with concrete when deck is poured. After concrete cures, deck is heated by covering with electric blankets and wax beads melt and fill the capillary openings.

## ITEM #62 - WEARING SURFACE

This item must be coded.

A.	Type [1 digit]

The type of wearing surface material on the structure shall be coded using one of the choices given below to represent the wearing surface material.  Where there are multiple layers of wearing material of various types on the structure, code the top or the exposed layer.  All of the following codes are considered to be overlays except codes 2 (Integral Concrete), 7 (Timber) and 8 (Gravel).

For open Steel Grid Decks code 0 for wearing surface.

| Code | Description |
| --- | --- |
| 1 | Concrete (Separate) - Overlay |
| 2 | Integral Concrete (Monolithic) - Not an Overlay |
| 3 | Latex Modified Concrete (LMC) - Overlay |
| 4 | Dense Concrete (IOWA System) - Overlay |
| 5 | Epoxy Overlay - Overlay |
| 6 | Bituminous (Asphaltic Concrete) - Overlay |
| 7 | Timber - Not an Overlay |
| 8 | Gravel - Not an Overlay |
| 0 | Other |
| N | Not Applicable (Culvert under fill etc.) - Not an Overlay |
| A | Super-plasticized Dense Concrete (SDC) - Overlay |
| B | Chip & Seal - Overlay |
| C | Micro-silica Modified Concrete - Overlay |

B.	Thickness [2 digits]

Whether the wearing surface material indicated in part "A" is a separate wearing surface or monolithic concrete, code a two (2) digit numeric to indicate the total thickness rounded to the nearest inch.  Leave this portion of item blank if not applicable and item will default to zeros. Where there are multiple layers of wearing materials of different types on the structure code the total thickness of all materials including any monolithic concrete.  Right justify and use leading zeros where necessary.

## ITEM #63 - STRUCTURE TYPE

This item must be coded.

This item must be coded for all structures.  It allows the coder to define the structure as to the main spans and approach spans.

A.  Main Spans

1.	Number [3 digits]

Indicate with a three (3) digit code the number of spans in the main or major unit. This part of the item will include all spans of most bridges, the major unit only of the sizable structure, or a unit of material or design, different from that of the approach spans. All entries must be right justified with leading zeros where necessary.

Example:  2 spans - "002", 10 spans - "010".

2.	Type [3 digits]

Indicate by a three (3) digit code the type of the spans under the classification of "Main Spans" as defined above.  This classification denotes the material, the overall type of construction, and the special design configuration of the main supporting members of the superstructure.  Code using the tabulation and list of permissible combinations shown below in Appendix "C".

| 1st DIGIT | MATERIAL | 2nd DIGIT | TYPE | 3rd DIGIT | DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete | 1 | Slab | 1 | Simple |
| 2 | Prestressed Concrete | 2 | Beam | 2 | Continuous |
| 3 | Steel | 3 | Box Beam | 3 | Deck |
| 4 | Timber | 4 | Truss | 4 | Thru |
| 5 | Stone | 5 | Arch | 5 | Filled |
| 6 | Aluminum | 6 | Girder (Floor System) | 6 | Orthotropic |
| 7 | Cast Iron | 7 | Frame | 7 | Movable – Lift |
| 8 | Wrought Iron | 8 | Suspension | 8 | Movable – Bascule |
| 9 | Composite | 9 | Culvert | 9 | Movable – Swing |
| 0 | Other | 0 | Other | 0 | Other |
|  |  | A | Cable – Stayed | A | Pony (Truss) |
|  |  | B | Tunnel |  |  |
| N | None | N | None | N | None |

All structures with more than two (2) lines of beams or girders (rolled or built-up) without a floor system (i.e. floor beams, stringers, etc.) shall be called beam - type bridges. Structures with a floor system are to be classified as girder type bridges.

All girder type and truss type bridges shall be described as either thru or deck.  No simple or continuous descriptions are permissible for these types of structures.

A pony truss is a through truss which has no top chord lateral bracings due to its short height, generally less tall than a through truss.

If a three or four sided precast concrete box carries twenty-four (24) inches or less of fill *code as a 171 (Concrete Rigid Frame Simple Span);  If it carries more than twenty-four (24) inches of fill code as a 195 (Concrete Culvert Filled).

All structures designed as culverts and carrying earth fill, regardless of overall span length shall be coded as X95.

Culvert type bridges are structures which convey water or form a passageway through an embankment and are designed to support super-imposed loads of earth or other fill material plus a live load.  Generally, prefabricated or corrugated metal structures 10’ span or greater are considered culvert type bridges.  Masonry arches with integral spandrel walls, sidewalks, and railings, greater than 10' span shall not be construed as culvert type structures.

* Note - Fill is measured from top of Culvert to C/L of grade.

B.	Approach Spans

1.	Number [3 digits]

Code same as for Part "A.1."

2.	Type [3 digits]

Code same as for Part "A.2."

If there are differences of material and construction among these spans, judgment will have to be exercised in choosing the one most representative type for coding.

Note: 	This item allows up to two (2) types of spans to be coded.  If there is one (1) type for all spans, code only the Main Span portion.  If there are no Approach Spans code “NNN” for None in Type and zeros in the Number portion of this item.

ITEM #64 - [BLANK]

## ITEM #65 - MAXIMUM SPAN LENGTH [4 digits]

This item must be coded.

The length of the maximum (longest) span only should be coded for this item. (This item is not for coding total length of all spans.)

Code a four (4) digit number to represent the measured length (center to center of bearings) to the nearest foot.  Right justify and use leading zeros where necessary.

| Length of Maximum Span | Code |
| --- | --- |
| 50 Feet | 0050 |
| 117 Feet | 0117 |
| 1,050 Feet | 1050 |

## ITEM #66 - OVERALL STRUCTURE LENGTH [6 digits]

This item must be coded.

Code a six (6) digit number to represent the total length of the structure to the nearest foot. This length should be measured back to back of back walls of the abutments, between expansion joints at abutments or between paving notches.  When taking the measurements from the plans, use the Bridge Limits.  Right justify and use leading zeros where necessary.

| Overall Length | Code |
| --- | --- |
| 50 Feet | 000050 |
| 5,421 Feet | 005421 |
| 333 Feet | 000333 |
| 101, 235 Feet | 101235 |

## ITEM #67 - MINIMUM VERTICAL CLEARANCE ON BRIDGE [2*4 digits]

This item must be coded.

This item includes a four (4) digit coding representing the Minimum Vertical Clearance from the roadway or railroad track on the structure to the underside of the superstructure. For highways on the structure, code the minimum clearance for the main lane openings in each direction of travel (Cardinal and Non-Cardinal).  For undivided highways with bi-directional traffic on a structure, code the minimum clearance under the subtitle “Cardinal Opening”.  For highways with one-directional traffic on the structure, code the minimum clearance under the appropriate subtitle.  The item should be recorded in feet and inches, rounded down to the inch.  In each subsection (Feet, inches) right justify the coded values and fill with leading zeros where necessary.  All zeros (0) should be coded for any portion of the item not applicable.  Code the entire item with zeros for structures which do not carry a highway.

ITEM #67 - MINIMUM VERTICAL CLEARANCE ON BRIDGE

ITEM #68 - [BLANK]

## ITEM #69 - NBIS BRIDGE LENGTH [1 digit]

This item must be coded.

Does this structure meet or exceed the minimum length specified to be designed as a bridge for National Bridge Inspection Standards purposes?  The following definition of a bridge is used by AASHTO and is given in the NBIS, 23CFR650.3:

A structure including supports erected over a depression or an obstruction, such as water, highway, railway, and having a track or passageway for carrying traffic or other moving loads, and having an opening measured along the center of the roadway of more than 20 feet between under copings of abutments or spring lines of arches, or extreme ends of openings for multiple boxes; it may also include multiple pipes, where the clear distance between openings is less than half of the smaller contiguous openings.

NOTE:  NBIS Bridge Length is different from overall structure length.  For single span structures with no bearings use clear span.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No |

ITEM #69 - NBIS BRIDGE LENGTH

FORM #BR87 - CARD E

## ITEM #70 - SUBSTRUCTURE

This item is intended to describe the substructure and therefore must be coded for all bridges on the inventory.  For culverts only code if there is a discernible difference between the culvert and the abutment. (Large bridges with unusual configuration may be subdivided into more than one structure and thus may have only one abutment or no abutments.)

A.	 Abutments [2*1 digit]

Code the material and type of each abutment.  The coding locations are self-explanatory on the form.  Particular attention must be paid to the terms "Rear" and Forward".  The rear abutment is nearest the established beginning of the route.  If the structure has no abutments (Multiple pipe culverts etc.) code all N's in part "A".  If the structure has only, one (1) abutment (Example:  long ramp type bridges connected to a main structure but inventoried as a separate bridge) code all N's in the non-applicable portion "Rear" or "Forward".

| Code | 1st Digit Material | Code | 2nd Digit Type |
| --- | --- | --- | --- |
| 1 | Stone | 1 | Gravity |
| 2 | Concrete | 2 | Cantilever |
| 3 | Concrete and Stone | 3 | Solid Wall |
| 4 | Timber | 4 | Cellular or “U” |
| 5 | Steel | 5 | Stub Gravity |
| 6 | Steel and Timber | 6 | Stub – Capped Pile (Single Row Piles) |
| 7 | Steel and Concrete | 7 | Integral |
| N | None | 8 | Pedestal |
| 0 | Other | 9 | Stub – Capped Pile (Multiple Row Piles) |
|  |  | 0 | Other |
|  |  | N | None |
|  |  | A | Proprietary Wall w/Stub Type Abutments |
|  |  | B | Capped Pile Bent |
|  |  | C | Cap & Column |
|  |  | D | Semi - Integral |

Note - See Appendix E for sketches of the typical abutment types listed above.

B.	Piers

This item allows a four (4) digit coded description of the piers.  Code all N's if the structure has no piers.  The code sheet allows coding of up to three (3) types of piers, grouped by material and construction types.

1.	Predominate [4 digits]

Code in the first two (2) digits the number of the predominate type of piers up to "99" using leading zeros where necessary.  Example:  5 piers -   Code "05".  In the 3rd and 4th digit code the material and the type of construction for the pier or group of piers. Codes for 3rd and 4th digits are as follows:

| Code | 3rd Digit Material | Code | 4th Digit Type |
| --- | --- | --- | --- |
| 1 | Stone | 1 | Gravity |
| 2 | Concrete | 2 | Cantilever (Tee) Open Panel |
| 3 | Concrete and Stone | 3 | Cantilever (Tee) Solid Panel |
| 4 | Timber | 4 | Open Column |
| 5 | Steel | 5 | Capped Column |
| 6 | Steel and Timber | 8 | Capped Pile |
| 7 | Steel and Concrete | 0 | Other |
| N | None | A | Solid Wall |
| 0 | Other | B | Tower |
|  |  | N | None |

Note - See Appendix "G" for sketches typical of Pier types listed above.

2.	Other [4 digits]

Code as for Part "1", if there is a 2nd pier or group of piers that reasonably fit a material and type description different than that of Part “1".

3.	Other [4 digits]

Code as for Parts "1" and "2", if a 3rd pier or group of piers differ sufficiently in material and construction to justify a 3rd listing.

Note: 	If a structure has only one (1) or two (2) piers or groups of piers of different types, code N in the remaining positions of Part "B" (Piers).  If more than three (3) different type piers or groups of piers exist, the third (3rd) group coded under the heading "Other" should include the two (2) or more groups with the least count of units per group.  The total number of piers coded in all three (3) listings must equal the actual total number of piers for the structure.

## ITEM #71 - FOUNDATIONS [5 digits]

This item must be coded.

From the list below show the type foundation for the rear and forward abutments.  If there is only one (1) abutment or no abutments, code N's in the appropriate spaces.  Also code the type foundation for each pier.  If there is only one (1) type for all piers, code it in the predominate type and code remaining spaces N's.  You may code up to three (3) types of foundations for piers.  If there are no piers code all N's.

For three sided precast concrete structures, code as spread footings (Code 4) or pile type foundation accordingly.  For four-sided boxes code foundations as “N” for none.

| Code | Type |
| --- | --- |
| 1 | Steel H Piles (Other size) |
| 2 | Cast-in-Place Reinforced Concrete Piles (Other diameter) |
| 3 | Drilled Shafts |
| 4 | Spread Footing |
| 5 | Timber Piles |
| 6 | Rock |
| 7 | Steel H Piles (HP 10 x 42) |
| 8 | Steel H Piles (HP 12 x 53) |
| 9 | Steel H Piles (HP 14 x 73) |
| 0 | Other |
| U | Unknown (Older Bridge being added to the file and foundations are unknown). |
| N | None (such as most Culverts) (Code all N’s) |
| A | Cast-in-Place Reinforced Concrete Piles (12” diameter) |
| B | Cast-in-Place Reinforced Concrete Piles (14” diameter) |
| C | Cast-in-Place Reinforced Concrete Piles (16” diameter) |

This item must be coded for any applicable portion for all new structures being "Added" to the file.

If on a rare occasion an old bridge is being "added" to the file and the foundations are unknown, code U's for the appropriate portion of the foundations.

## ITEM #72 - AVERAGE LENGTH ABUTMENT FOUNDATIONS  [2 digits]

Code to the nearest foot the average actual length of the abutment piles, drilled shafts, etc.

If no foundations are present or if there are spread footings, footings on rock or culverts, code zeros.

If the foundations under the abutments are unknown (coded as “U”), leave blank and item will default to zeros.  If the length of abutment foundations exceeds 99 feet, code 99.

## ITEM #73 - AVERAGE LENGTH PIER FOUNDATIONS[2 digits]

Code to the nearest foot the average actual length of the pier piles, drilled shafts, etc.  For capped pile piers, include exposed length plus embedment length.

If no foundations are present or if there are spread footings, or footings on rock, code zeros.

The foundations under the piers are unknown (coded as “U”), leave blank and the item will default to zeros.  If the length of pier foundations exceeds 99 feet, code 99.

## ITEM #74 - SCOUR CRITICAL SUSCEPTIBILITY [1 digit]

This item must be coded.

Using a single digit code as indicated below to identify the current status of the bridge regarding its vulnerability to scour.  Evaluations shall be made by hydraulic/geotechnical/structural engineers. Guidance on conducting a scour evaluation is included in the FHWA Technical Advisory T5140.23 titled, “Evaluating Scour at Bridges.”  Detailed engineering guidance is provided in the Hydraulic Engineering Circular 18 titled, “Evaluating Scour at bridges.”  Whenever a rating factor of 2 or below is determined for this item, the rating factor for Item 42 B Substructure and other affected items (i.e., load ratings, superstructure rating) should be revised to be consistent with the severity of observed scour and resultant damage to the bridge.  A plan of action should be developed for each scour critical bridge (see FHWA Technical Advisory T 5140.23, HEC 18 and HEC 23). A scour critical bridge is one with abutment or pier foundation rated as unstable due to (1) observed scour at the bridge site (rating factor of 2, 1, or 0) or (2) a scour potential as determined from a scour evaluation study (rating factor of 3).  It is assumed that the coding of this item has been based on an engineering evaluation, which included consultation of the NBIS filed inspection findings.

| Code | Description |
| --- | --- |
| N | Bridge not over waterway. |
| U | Bridge with unknown foundation that has not been evaluated for scour. Until risk can be determined, a plan of action should be developed and implemented to reduce the risk to users from a bridge failure during and immediately after a flood event (see HEC 23). |
| T | Bridge over tidal waters that have not been evaluated for scour, but considered low risk. Bridge will be monitored with regular inspection cycle and with appropriate underwater inspections until an evaluation is performed (Unknown foundations in tidal waters should be coded U.) |
| 9 | Bridge foundations (including piles) on dry land well above flood water elevations. |
| 8 | Bridge foundations determined to be stable for the assessed or calculated scour condition. Scour is determined to be above top of footing (Example A) by assessment (i.e., bridge foundations are on rock formations that have been determined to resist scour during the service life of the bridge), by calculation or by installation of properly designed countermeasures (see HEC 23). |
| 7 | Countermeasures have been installed to mitigate an existing problem with scour and to reduce the risk of bridge failure during a flood event. Instructions contained in a plan of action have been implemented to reduce the risk to users from a bridge failure during or immediately after a flood event. |
| 6 | Scour calculation/evaluation has not been made. (Use only to describe case where bridge has not yet been evaluated for scour potential.) |
| 5 | Bridge foundations determined to be stable for assessed or calculated scour conditions. Scour is determined to be within the limits of footing or piles (Example B) by assessment (i.e., bridge foundations are on rock formations that have been determined to resist scour during the life of the bridge), by calculations or by installation of properly designed countermeasures (see HEC 23). |
| 4 | Bridge foundations determined to be stable for assessed or calculated scour conditions; field review indicates action is required to protect exposed foundations (see HEC 23). |
| 3 | Bridge is scour critical; bridge foundations determined to be unstable for assessed or calculated scour conditions: - Scour within limits of footing or piles. (Example B) - Scour below spread footing base or pile tips. (Example C) |
| 2 | Bridge is scour critical; field review indicates that extensive scour has occurred at bridge foundations, which are determined to be unstable by: - A comparison of calculated scour and observed scour during the bridge inspection, or - An engineering evaluation of the observed scour condition reported by the bridge inspector during bridge inspection on BR-86. |
| 1 | Bridge is scour critical; field review indicates that failure of piers/abutments is imminent. Bridge is closed to traffic. Failure is imminent based on: - A comparison of calculated and observed scour during the bridge inspection, or - An engineering evaluation of the observed scour condition reported by the bridge inspector. |
| 0 | Bridge is scour critical. Bridge has failed and is closed to traffic |

## ITEM #75 - TYPE OF CHANNEL PROTECTION [1 digit]

Indicate the method, if any, used to protect the bridge and upstream channel banks from scour and other degradation caused by stream action.  Vegetation is a form of channel protection.

Select from the list below, if there is no protection code N.  For structures not over a waterway code X.

| Code | Description |
| --- | --- |
| 1 | Concrete (cast-in-place) |
| 2 | Stone |
| 3 | Sheet Piling |
| 4 | Piling |
| 5 | Rip Rap (dumped rock or rock channel protection) |
| 6 | Gabions (wire mesh baskets filled with stone) |
| 7 | Fabric bags filled with concrete or sand |
| N | None |
| 0 | Other (grass, bushes, trees) |
| X | Not Applicable |
| A | Precast concrete (panels) |
| B | Earthen Dikes |
| U | Unknown |

This item must be coded.

ITEM #76 - [BLANK]

## ITEM #77 - MINIMUM VERTICAL CLEARANCE UNDER BRIDGE [2*4 digits]

This item must be coded for all bridges with roadway/railway underneath.

This item includes a four (4) digit coding representing the Minimum Vertical Clearance from the roadway or railroad track beneath the structure to the underside of the superstructure. For divided highways under a structure, code the minimum clearance for the main lane openings in each direction of travel (Cardinal and Non-Cardinal).  For undivided highways with bi-directional traffic under a structure, code the minimum clearance under the subtitle “Cardinal Opening”.  For highways with one-directional traffic under a structure, code the minimum clearance under the appropriate subtitle.  If the structure is over a railroad or railroads, record the minimum clearance over any track under the sub-title "Cardinal Opening".  The item should be recorded in feet and inches, rounded down to the inch.  In each subsection (Feet, inches) right justify the coded values and fill with leading zeros where necessary.  All zeros (0) should be coded for any portion of the item not applicable.  Code the entire item with zeros for structures which are not over a highway or railroad.

## ITEM #78 - MINIMUM LATERAL UNDERCLEARANCE (TO EDGE OF LANE) [2*6 digits]

If the feature beneath the structure is either a railroad or highway, code a three (3) digit number to represent the minimum lateral clearance on the right and another three (3) digit number for the clearance on the left under the subtitle “Cardinal Opening”.  If the feature is not a railroad or highway leave blank and item will default to zeros.  In the case of dual or divided highways requiring two (2) or more openings under the structure, record the clearances on the left and the right for the cardinal opening and the same for the Non-Cardinal opening (Main lanes only for each direction of travel.)  For structures with railroads underneath, regardless of number of tracks or number of openings, record and code the absolute minimum lateral clearances (Right and Left) without regard to the opening or openings in which they occur.  Lateral clearances should be measured from the right or left edge of the lane or from the centerline (between the rails) of the track (if single) and the centerline of the outside track (if multiple) to the nearest substructure unit (or toe of slope steeper than 3 to 1) on the right and to the nearest substructure unit or median barrier on the left.  All measurements should be made normal to the centerline of the main or thru lanes or normal to the centerline of the main line track or tracks.  For the purposes of this item, the term "Left" refers to the left hand edge or median side of the lanes on a divided highway while facing in the direction of traffic flow, while the term "Right" refers to the right hand edge or outside edge of the lanes while facing in the direction of traffic.  For a non-divided single highway the terms refer to the left and right hand sides respectively, while facing in the cardinal direction of the route.  For railroads,” Right will be assumed to be the side to the tracks nearest the forward end of the bridge and the left side is assumed to be nearest the rear end of the bridge.  Code these dimensions to the nearest tenth of a foot.  If there is only a single roadway (non-divided) or track thru a single opening under the structure, code the lateral clearances under the heading "Cardinal Opening", and code all zeros under Non-Cardinal direction.  In each subsection right justify all entries and fill with leading zeros where necessary.

ITEM #79 – [BLANK]

## ITEM #80 - MINIMUM LATERAL CLEARANCE ON BRIDGE (TO EDGE OF LANE) [2 x 6 digits]

The lateral clearance on the bridge should be the minimum distance measured to nearest tenth of one foot from the outside of the painted edge line, on either edge of the pavement, to the nearest restriction (i.e., curb, railing, median, parapet, etc.) on the corresponding side of bridge.

If the surface of the traveled roadway on a structure has no painted edge line, then measure from the edge of the approach traveled roadway surface extended.

Use edge of shoulder as nearest restriction if no other restriction exists.  For structure buried under fill or guardrail measure to the edge of shoulder as well as the nearest restriction.  Use edge of lane to curb (gutter) 6" or higher as lateral clearance.

In the case of a divided highway crossing the structure, code the clearance on the right and the clearance on the left for both the cardinal and Non-Cardinal Openings. When two (2) or more non-divided lanes cross a structure, code only the right and left clearances in the cardinal opening. Code all zeros in the Non-Cardinal opening.

For the purposes of this item the term "Left" refers to the left edge or median side of the pavement on a divided highway when facing the direction of traffic, and the term "Right" refers to the right edge of the outside edge of the pavement when facing in the direction of traffic.  For a non-divided highway the terms refer to the left and right edges of the pavement while facing in the cardinal direction of the route.

If a structure carries non-highway traffic such as railroads, pedestrian, bicycle, etc. code all zeros.

All entries in each subsection should be right justified and filled with leading zeros where necessary. If the approach roadway width is more than the pavement width of the roadway and there is no painted edge lines leave item blank and item will default to zeros.  This item must be coded for all bridges.

## ITEM #81 - VERTICAL CLEARANCE AT VERTICAL LIFT BRIDGE [3 digits]

Record to the nearest foot (rounding down) the minimum vertical clearance imposed at the site as measured above a datum that is specified on a navigation permit issued by a control agency. Code this item only for vertical lift bridges in the dropped or closed position, otherwise leave blank and item will default to zeros.

| Vertical Clearance | Code |
| --- | --- |
| 20.7 | 020 |
| 24.2 | 024 |

## ITEM #82 - BLANK

FORM #BR87 - CARD F

## ITEM #83 - LOAD RATING [5 digits]

This item must be coded.

Ohio Percent of Legal Load - Maximum Allowable 150% [Code 150] [3 digits]

This portion of Item #83 applies only to structures carrying highway traffic.  Enter the calculated or estimated percent of Ohio Legal Load on record.  Always code the figures to the nearest 5 percent.  If the structure carries other than public motor vehicular traffic (e.g., railroad, pedestrian walk, pipe line, taxiway, etc.) or if structure is closed or if item does not apply, code all zeros.

| Example | Code |
| --- | --- |
| 125 % | 125 |
| 65 % (Post 35%) | 065 |
| *For structures rated at greater than 150% of Legal Load | 150* |

Rating Year [2 digits]

Code the last two (2) digits of the year in which the structures load ratings coded in parts 1, 2, and 3 were determined.  The most recent ratings should always be coded.  If the bridge is closed, code year closed.

Other Structures

Structures other than that carrying highway traffic such as railroad, pedestrian, temporary, taxiway and utility structures shall be coded according to the examples below:

| Type | Operating Load Factor | Inventory Load Factor | %Legal Load |
| --- | --- | --- | --- |
| Railroad Bridge | 100 | 100 | 000 |
| Railroad Bridge (W/tracks removed) | 100 | 100 | 000 |
| Pedestrian Bridge | 000 | 000 | 000 |
| Pipeline (over Hwy) | 000 | 000 | 000 |
| Closed Bridge * | 000 | 000 | 000 |
| Temporary Bridge ** | 000 | 000 | 100 |
| Shored-up Bridge *** | 030 | 030 | 000 |
| Structure under large fill (Not affected by live load). **** | 299 | 299 | 150 |
| Aircraft Taxiway Bridge | 800 | 800 | 000 |

* Closed” should be included in “Features Intersected” item.

** “Y” must be coded in Item #10

*** Load capacity without shoring.

**** When there is sufficient fill over a structure such that, according to the AASHTO design specification, that structure would have insignificant live load effect.

## ITEM #84 - METHOD OF RATING [1 digit]

This item must be coded.

This item will indicate the method of analysis for the load rating (Items #92 & #93) of the structure.

Code the appropriate description from the list below.

| Code | Description |
| --- | --- |
| 0 | Field evaluation and documented engineering judgment |
| 1 | Allowable Stress Rating (ASR) or Working Stress Rating (WSR) using HS20 loading |
| 2 | Load Factor Rating (LFR) |
| 3 | Load & Resistance Factor Rating (LRFR) using HL93 loadings |
| 4 | Combination of Methods |
| 5 | Engineering Judgment – Superstructure (Default) |
| 6 | Load Testing |
| 7 | Engineering Judgment – Substructure |
| 8 | Not Used |
| 9 | Not Used |
| A | Not Used |
| B | Not Used |
| C | Not Used |
| D | Assigned Load Factor Rating (LFR) using HS20 loading |
| E | Not Used |
| F | Assigned Load & Resistance Factor Rating (LRFR) using HL93 loading |
| X | Not Applicable (RR bridges, buildings, Non-highway, etc.) |

Code 0 is to be used when the load rating is determined by field evaluation and documented engineering judgment, typically done when plans are not available or in cases of severe deterioration. Field evaluation and engineering judgment ratings must be documented.

Code 5 is to be used when the bridge has not been load rated or load rating documentation does not exist.

## ITEM #85 - SOFTWARE OF RATING ANALYSIS [1 digit]

This item must be coded.

Code the appropriate description from the list below:

| Code | Description |
| --- | --- |
| 1 | BARS |
| 2 | BRASS |
| 3 | VIRTIS |
| 4 | DESCUS |
| 5 | LARSA Or Other Finite Element (FE) Program |
| 6 | In-House Program/Spreadsheet |
| 7 | Combination |
| 8 | Other program |
| 9 | Manual Calculations |
| 0 | No calculations were done for load rating (Default) |

## ITEM #86 - STREAM VELOCITY [3 digits]

Record the one-hundred year velocity of the stream under the bridge to the nearest one-tenth of a foot per second.

This figure is generally found on the "Site Plan" of the "Design Plans".

This item must be coded for all applicable structures being "Added" to the file.

If not applicable code N's.  If stream velocity is unknown code U=s.  If left blank item will default to zeros.

## ITEM #87 – PLAN INFORMATION AVAILABLE [1 digit]

This item must be coded.

Code this item to indicate if plans/information is available for Load Rating of the bridge.

| Code | Description |
| --- | --- |
| 0 | No Plans or information available for load rating analysis |
| 1 | Plan information available for load rating analysis (Default) |
| 2 | Field measured information for load rating analysis |
| 3 | Field testing information |
| N | Not Applicable |

## ITEM #88 - WATERWAY ADEQUACY [1 digit]

This item appraises the waterway opening with respect to passage of flow through the bridge. The following codes shall be used in evaluating waterway adequacy.  Site conditions may warrant higher or lower ratings than indicated by the table (e.g., flooding of an urban area due to a restricted bridge opening).

| Where overtopping frequency information is available, the descriptions given in the table for chance of overtopping mean the following: |  |
| --- | --- |
| Chance of overtopping | Description |
| Remote | Greater than 100 years |
| Slight | 11 to 100 years |
| Occasional | 3 to 10 years |
| Frequent | Less than 3 years |
| Adjectives describing traffic delays mean the following: |  |
| Adjective | Description |
| Insignificant | Minor inconvenience. Highway passable in a matter of hours. |
| Significant | Traffic delays of up to several days. |
| Severe | Long term delays to traffic with resulting hardship. |

| Functional Classification |  |  |  |
| --- | --- | --- | --- |
| Principal Arterial -Interstates, Freeways, or Expressways | Other Principal and Minor Arterials and Major Collectors | Minor Collectors, Locals | Description |
| N | N | N | Bridge not over a waterway. |
| 9 | 9 | 9 | Bridge deck and roadway approaches above flood water elevations (high water). Chance of overtopping roadway approaches is remote. |
| 8 | 8 | 8 | Bridge deck above roadway approaches, slight chance of overtopping roadway approaches. |
| 6 | 6 | 7 | Slight chance of overtopping bridge deck and roadway approaches. |
| 4 | 5 | 6 | Bridge deck above roadway approaches. Occasional overtopping of roadway approaches with insignificant traffic delays. |
| 3 | 4 | 5 | Bridge deck above roadway approaches. Occasional overtopping of roadway approaches with significant traffic delays. |
| 2 | 3 | 4 | Occasional overtopping of bridge deck and roadway approaches with significant traffic delays. |
| 2 | 2 | 3 | Frequent overtopping of bridge deck and roadway approaches with significant traffic delays. |
| 2 | 2 | 2 | Occasional or frequent overtopping of bridge deck and roadway approaches with severe traffic delays. |
| 0 | 0 | 0 | Bridge closed. |

## ITEM #89 - APPROACH ALIGNMENT [1 digit]

Record the appropriate code from the table below about the condition of the approach alignment.

For example, if the highway section requires substantial speed reduction due to vertical or horizontal alignment, and roadway approach to the bridge requires only a very minor additional speed reduction at the bridge, the appropriate code would be a 6.  This concept shall be used at each bridge site.

Speed reductions necessary because of structure width and not alignment shall not be considered in evaluating this item.

| Code | Description |
| --- | --- |
| N | Not Applicable (Conveyor, Pedestrian, Railroad) |
| 9 | Condition superior to present desirable criteria |
| 8 | Condition equal to present desirable criteria |
| 7 | Condition better than present minimum criteria |
| 6 | Condition equal to present minimum criteria |
| 5 | Condition somewhat better than minimum adequacy to tolerate being left in place as is |
| 4 | Condition meeting minimum tolerable limits to be left in place as is |
| 3 | Basically intolerable condition requiring high priority of repair |
| 2 | Basically intolerable condition requiring high priority of replacement |
| 0 | Immediate replacement necessary to put back in service |

## ITEM #90 - PROPOSED IMPROVEMENTS

If there are no proposed improvements planned for the bridge leave the entire item blank.  You must code all or none of the item.

### Type of Work [2 digits]

The information to be coded for this item will be the type of work proposed to be accomplished at the site to provide the service needed.

It shall be coded for all structures that are eligible for HBRRP funds. (80% or less sufficiency, Structurally Deficient or Functionally Obsolete and > 20 feet clear span) It also may be coded for other structures.

| Code | Description |
| --- | --- |
| 31 | Replacement of bridge or other structure because of substandard load carrying capacity or substandard bridge roadway geometry. |
| 32 | Replacement of bridge or other structure because of relocation of road. |
| 33 | Widening of existing bridge or other major structure without deck rehabilitation or replacement includes culvert lengthening. |
| 34 | Widening of existing bridge with deck rehabilitation or replacement. |
| 35 | Bridge rehabilitation because of general structure deterioration or inadequate strength. |
| 36 | Bridge deck rehabilitation with only incidental widening. |
| 37 | Bridge deck replacement with only incidental widening. |
| 38 | Other structural work. |

### Method [1 digit]

The method of improvement shall be coded using one of the following codes to indicate whether the proposed work is to be done by contract or by force account:

| Code | Description |
| --- | --- |
| 1 | Work to be done by contract |
| 2 | Work to be done by agency’s forces |

Examples:

| Code |  |
| --- | --- |
| 311 | A bridge is to be replaced by contract because it has deteriorated to the point that it can no longer carry legal loads. The same code should be used if the bridge is replaced because it is now too narrow or the original design was too light to accommodate today’s legal loads. |
| 321 | A bridge is to be replaced because the roadway must be straightened to eliminate a dangerous curve. The work will be done by contract. |
| 361 | A bridge deck is to be rehabilitated by contract and a safety curb to be removed which results in incidental widening of 2 feet. |
| 371 | A bridge deck is to be replaced by contract and the deck cantilever overhang extended 2 feet, which is the maximum that can be done without adding another line of stringers or girders to the superstructure. |

### Length of Improvements [6 digits]

Code a six (6) digit number that represents the length of the bridge portion only of the proposed improvement including any necessary approach work, to the nearest foot.  This length may be less than the full length of the structure if part of the bridge is to be improved.

For improvements on buried structures, use the proposed length measured along the centerline of the barrel regardless of the depth below grade.  The measurement should be made between the inside faces of the top parapet or edge stiffening beam of the top slab.

| Length of Improvement | Code |
| --- | --- |
| 250 feet | 000250 |
| 1,200 feet | 001200 |

For substructure or channel work only, code the length of superstructure over, or supported by, the substructure or channel.

Typically, a replacement bridge is longer than the existing bridge.  Nationwide averages for the increase in bridge length with replacement as a function of the existing length are given in the figure on the following page.  Where site-specific data is lacking, these factors are suggested for estimating the length of replacement bridges.  For exceedingly long bridges (i.e., 1000 feet or more) the length expansion factor approaches 1.0.

Length of Structure Improvement [6 digits]

INCREASED LENGTH OF REPLACED BRIDGES

- Bridge Improvement Costs [6 digits]

Code a six (6) digit number to represent the cost of the proposed bridge or major structure improvements in thousands of dollars.  This cost shall include only bridge construction costs, EXCLUDING roadway, right of way, detour, demolition, preliminary engineering, etc.  The year of bridge improvements costs should be coded in the Year of Improvement Cost Estimate.  Do not use this item for estimating maintenance costs.

This item must be coded for all bridges eligible for the Highway Bridge Replacement and Rehabilitation Program (HBRRP).  It may be coded for other bridges at the option of the highway agency.

| Bridge Improvement Costs | Code |
| --- | --- |
| $55,850 | 000056 |
| 250,000 | 000250 |
| $7,451,233 | 007451 |

### Roadway Improvement Costs [6 digits]

Code a six (6) digit number to represent the cost of the proposed roadway improvement in thousands of dollars.  This shall include only roadway construction costs, excluding bridge, right-of-way, detour, extensive roadway realignment costs, preliminary engineering, etc.  The base year for the costs should be coded in the Year of Improvement Cost Estimate.  Do not use this item for estimating maintenance costs.

This item must be coded for bridges eligible for the Highway Bridge Replacement and Rehabilitation Program (HBRRP).  It may be coded for other bridges at the option of the highway agency.

In the absence of a procedure for estimating roadway improvement costs, a guide of 10 percent of the bridge costs is suggested.

### Total Project Costs [6 digits]

Code a six (6) digit number to represent the total project cost in thousands of dollars, including incidental costs not included in Bridge Improvement Costs and Roadway Improvement Costs.  This item should include all costs normally associated with the proposed bridge improvement project. The Total Project Cost will therefore usually be greater than the sum of the Bridge Improvement Costs and Roadway Improvement Costs.  Code the total costs in the Total Project Costs.  Do not use this item for coding maintenance costs.

This item must be coded for bridges eligible for the Highway Bridge Replacement and Rehabilitation Program.  It may be coded for other bridges at the option of the highway agency.

In the absence of a procedure for estimating the total project cost, a guide of 10 percent of the bridge cost is suggested.

### Year Of Improvement Cost Estimate [2 digits]

Record the year that the costs of work estimated in Bridge Improvement Cost, Roadway Improvement Cost, and Total Project Cost were based upon.  This date and the data provided for Bridge Improvement Costs, Roadway Improvement Costs and Total Project Costs must be current; that is, Year of Improvement Cost Estimate shall be no more than 8 years old.  Code the last two (2) digits of the year so recorded.

| Year of Cost Estimate | Code |
| --- | --- |
| 1988 costs | 88 |
| 2010 costs | 10 |

## ITEM #91 - DESIGNATED INSPECTION FREQUENCY [2 digits]

Code two (2) digits to represent the number of months between designated inspections of the structure.  Leading zeros shall be coded.  This interval is usually determined by the individual in charge of the inspection program.  For posted, under strength bridges, this interval could be substantially less than the 12 month standard.  The designated inspection interval may vary from inspection to inspection depending on the condition of the bridge at the time of inspection.  The default value is 12 months.

| Code | Description |
| --- | --- |
| 01 | Posted bridge with heavy truck traffic and questionable structural details to be inspected each month |
| 12 | Bridge is scheduled to be inspected every 12 months |

It should be noted that bridges will also require special non-scheduled inspections after unusual physical traumas such as floods, earthquakes, fires or collisions.  These special inspections may range from a very brief visual examination to a detailed in-depth evaluation depending upon the nature of the trauma.  For example:  where a substructure pier or abutment is struck by an errant vehicle, in most cases only a visual examination of the bridge is necessary.  After major collisions or earthquakes, in-depth inspections may be warranted as directed by the engineer in overall charge of the program.  After and during severe floods, the stability of the substructure of bridges may have to be determined by probing, underwater sensors or other appropriate measures.  Underwater inspection by divers may be required for some scour critical bridges immediately after floods.  See Item #74 - Scour Critical Susceptibility.

## ITEM #92 – OPERATING LOAD RATING FACTOR [5 digits]

A.  OPERATING RATING FACTOR [4 digits]

This item must be coded.

Code the Operating Load Rating Factor rounded to the nearest third decimal point as a four (4) digit number without a decimal point (all four digits to be coded).  Maximum value of Rating Factor that can be coded is 9.999.  There is an implied decimal point after three digits from right.

Examples:

| Operating Load Rating Factor | Code |
| --- | --- |
| 1.50 | 1500 |
| 1.35 | 1350 |
| 0.85 | 0850 |
| 0.25 | 0250 |
| 9.99 | 9990 |
| 0 | 0000 |
| 0.083 | 0083 |

If the bridge is closed and/or will not carry any live load, the Rating Factor shall be coded as 0000.

On new and other highway structures, if a load rating analysis has not been performed, the Operating Load Rating Factor may be coded as 1.25.

The use or presence of temporary bridge requires special consideration in coding.  In such cases, since there is no permanent bridge, Operating Load Rating Factor should be coded as 000.

A bridge shored up or repaired on a temporary basis is considered a temporary arrangement bridge and the Rating Factor should be coded as if the temporary shoring were not in place.

Operating RF cannot be less than Inventory RF.

Examples:

| Type of Bridge | Code |
| --- | --- |
| Temporary bridge | 000 |
| Shored-up bridge** | 010 |
| Structures under sufficient fill that live load on structure is insignificant | 999 |

**Rating Factor without shoring.

B.  OPERATING RATING LOAD [1 digit]

This item must be coded for all bridges.

Code the type of load used in the operating rating calculations from the following table:

| Type of Load | Code |
| --- | --- |
| Not Used | 0 |
| Not Used | 1 |
| HS20 Loading | 2 |
| HL93 Loading | 3 |
| Not Used | 4 |
| Special Maintenance Loading | 5 |
| Railroad Loading | 7 |
| Pedestrian Loading | 8 |
| Special Loading | 9 |

Other Structures

Structures other than that carrying highway traffic such as railroad, pedestrian, temporary, taxiway and utility structures shall be coded according to the examples below:

| Type | Operating Load Factor | Inventory Load Factor | %Legal Load |
| --- | --- | --- | --- |
| Railroad Bridge | 100 | 100 | 000 |
| Railroad Bridge (W/tracks removed) | 100 | 100 | 000 |
| Pedestrian Bridge | 000 | 000 | 000 |
| Pipeline (over Hwy) | 000 | 000 | 000 |
| Closed Bridge * | 000 | 000 | 000 |
| Temporary Bridge ** | 000 | 000 | 100 |
| Shored-up Bridge *** | 030 | 030 | 000 |
| Structure under large fill (Not affected by live load). **** | 299 | 299 | 150 |
| Aircraft Taxiway Bridge | 800 | 800 | 000 |

* Closed” should be included in “Features Intersected” item.

** “Y” must be coded in Item #10

*** Load capacity without shoring.

**** When there is sufficient fill over a structure such that, according to the AASHTO design specification, that structure would have insignificant live load effect.

## ITEM #93 –INVENTORY LOAD RATING FACTOR [5 digits]

A.  INVENTORY RATING FACTOR [4 digits]

This item must be coded.

The Inventory Load Rating Factor represents the load level which can safely utilize an existing structure for an indefinite period of time for unlimited cycles of traffic.  Inventory rating level is the same as design level.

Code the Inventory Load Rating Factor as a four (4) digit number without a decimal point (all four digits to be coded).  Maximum value of Rating Factor that can be coded is 9.999.  There is an implied decimal point after three digits from right.

Examples:

| Inventory Load Rating Factor | Code |
| --- | --- |
| 1.50 | 1500 |
| 1.35 | 1350 |
| 0.85 | 0850 |
| 0.254 | 0254 |
| 9.999 | 9999 |
| 0 | 0000 |

If the bridge is closed and/or will not carry any live load, the Rating Factor shall be coded as 0000.

On new and other highway structures, if a load rating analysis has not been performed, the Inventory Load Rating Factor may be coded as 1.000

The use or presence of a temporary bridge requires special consideration in coding.  In such cases, since there is no permanent bridge, Inventory Load Rating Factor should be coded as 0000.

A bridge shored up or repaired on a temporary basis is considered a temporary arrangement bridge and the Rating Factor should be coded as if the temporary shoring were not in place.

Inventory RF cannot be greater than Operating RF.

Examples:

| Type of Bridge | Code |
| --- | --- |
| Temporary bridge | 0000 |
| Shored-up bridge** | 0100 |
| Structures under sufficient fill that live load on structure is insignificant | 9999 |

**Rating Factor without shoring.

B.  INVENTORY RATING LOAD [1 digit]

This item must be coded for all bridges.

Code the type of load used in the inventory rating calculations from the following table:

| Type of Load | Code |
| --- | --- |
| Not Used | 0 |
| Not Used | 1 |
| HS20 Loading | 2 |
| HL93 Loading | 3 |
| Not Used | 4 |
| Special Maintenance Loading | 5 |
| Railroad Loading | 7 |
| Pedestrian Loading | 8 |
| Special Loading | 9 |

For Other structures, see Item 92.

## ITEM #94 - INSPECTION PROGRAM RESPONSIBILITY [3digits]

The agency or agencies legally responsible should be coded.  The name should already be available from existing records in accordance with the Ohio Revised Code or by legal agreement.  Code a three (3) digit numeric or alphabetic code to specify agency or agencies responsible for the Inspection Program from the table below, where the agency with the left-most code is the one with primary responsibility.  When more than one agency has equal responsibility, code them in the hierarchy of State, Ohio Turnpike, Federal, County, City, railroad and other or private.

| Code | Description |
| --- | --- |
| 1 | Ohio State Transportation Department |
| 2 | Other Ohio State Agency (e.g., Ohio Turnpike Commission) |
| 3 | County Agency |
| 4 | City or other Local Agency |
| 5 | Federal Agency (other than listed) |
| 6 | Railroad |
| 7 | Private (other than railroad) |
| 8 | Not Used |
| 9 | Not Used |
| A | Ohio Department of Natural Resources (ODNR) |
| B | Ohio Conservancy District |
| C | Ohio Park District |
| D | Ohio Institution |
| E | Local Transit Authority |
| F | Township |
| G | Local Toll Authority |
| H | Bureau of Indian Affairs |
| I | U.S. Forest Service |
| J | National Park Service |
| K | Bureau of Land Management |
| L | Bureau of Reclamation |
| M | Military Reservation/Corps of Engineers |
| N | Indian Tribal Government |
| O | Air Force |
| P | Navy/Marines |
| Q | Army |
| R | NASA |
| S | Not Used |
| T | Not Used |
| U | Unknown |
| V | Not Used |
| W | Not Used |
| X | None |
| Y | Not Used |
| Z | Border State |

Examples:

| Responsible Agency or Agencies | Code |
| --- | --- |
| ODOT Responsibility | 1XX |
| Municipality Responsibility | 4XX |
| ODOT and County Responsibility | 13X |
| Border State Responsibility | ZXX |
| ODOT and Border State Responsibility | 1ZX |
| County, Municipality & Railway Responsibility | 346 |
| Railway and City Responsibility | 64X |
| Ohio Turnpike & ODOT Responsibility | 21X |

## ITEM #95 - MAJOR MAINTENANCE PROGRAM RESPONSIBILITY [3 digits]

The agency or agencies legally responsible should be coded.  The name should already be available from existing records in accordance with the Ohio Revised Code or by legal agreement. Code a three (3) digit numeric or alphabetic code to specify agency or agencies responsible for the Major Maintenance Program from the table below, where the agency with the left-most code is the one with primary responsibility.  When more than one agency has equal responsibility, code them in the hierarchy of State, Ohio Turnpike, Federal, County, City, railroad and other or private.

| Code | Description |
| --- | --- |
| 1 | Ohio State Transportation Department |
| 2 | Other Ohio State Agency (e.g., Ohio Turnpike Commission) |
| 3 | County Agency |
| 4 | City or other Local Agency |
| 5 | Federal Agency (other than listed) |
| 6 | Railroad |
| 7 | Private (other than railroad) |
| 8 | Not Used |
| 9 | Not Used |
| A | Ohio Department of Natural Resources (ODNR) |
| B | Ohio Conservancy District |
| C | Ohio Park District |
| D | Ohio Institution |
| E | Local Transit Authority |
| F | Township |
| G | Local Toll Authority |
| H | Bureau of Indian Affairs |
| I | U.S. Forest Service |
| J | National Park Service |
| K | Bureau of Land Management |
| L | Bureau of Reclamation |
| M | Military Reservation/Corps of Engineers |
| N | Indian Tribal Government |
| O | Air Force |
| P | Navy/Marines |
| Q | Army |
| R | NASA |
| S | Not Used |
| T | Not Used |
| U | Unknown |
| V | Not Used |
| W | Not Used |
| X | None |
| Y | Not Used |
| Z | Border State |

Examples:

| Responsible Agency or Agencies | Code |
| --- | --- |
| ODOT Responsibility | 1XX |
| Municipality Responsibility | 4XX |
| ODOT and County Responsibility | 13X |
| Border State Responsibility | ZXX |
| ODOT and Border State Responsibility | 1ZX |
| County, Municipality & Railway Responsibility | 346 |
| Railway and City Responsibility | 64X |
| Ohio Turnpike & ODOT Responsibility | 21X |

## ITEM #96 – ROUTINE MAINTENANCE RESPONSIBILITY [3digits]

The agency or agencies legally responsible should be coded.  The name should already be available from existing records in accordance with the Ohio Revised Code or by legal agreement.  Code a three (3) digit numeric or alphanumeric code to specify agency or agencies responsible for the routine maintenance from the table below, where the agency with the left-most code is the one with primary responsibility.  When more than one agency has equal responsibility, code them in the hierarchy of State, Ohio Turnpike, Federal, County, City, railroad and other or private.

| Code | Description |
| --- | --- |
| 1 | Ohio State Transportation Department |
| 2 | Other Ohio State Agency (e.g., Ohio Turnpike Commission) |
| 3 | County Agency |
| 4 | City or other Local Agency |
| 5 | Federal Agency (other than listed) |
| 6 | Railroad |
| 7 | Private (other than railroad) |
| 8 | Not Used |
| 9 | Not Used |
| A | Ohio Department of Natural Resources (ODNR) |
| B | Ohio Conservancy District |
| C | Ohio Park District |
| D | Ohio Institution |
| E | Local Transit Authority |
| F | Township |
| G | Local Toll Authority |
| H | Bureau of Indian Affairs |
| I | U.S. Forest Service |
| J | National Park Service |
| K | Bureau of Land Management |
| L | Bureau of Reclamation |
| M | Military Reservation/Corps of Engineers |
| N | Indian Tribal Government |
| O | Air Force |
| P | Navy/Marines |
| Q | Army |
| R | NASA |
| S | Not Used |
| T | Not Used |
| U | Unknown |
| V | Not Used |
| W | Not Used |
| X | None |
| Y | Not Used |
| Z | Border State |

Examples:

| Responsible Agency or Agencies | Code |
| --- | --- |
| ODOT Responsibility | 1XX |
| Municipality Responsibility | 4XX |
| ODOT and County Responsibility | 13X |
| Border State Responsibility | ZXX |
| ODOT and Border State Responsibility | 1ZX |
| County, Municipality & Railway Responsibility | 346 |
| Railway and City Responsibility | 64X |
| Ohio Turnpike & ODOT Responsibility | 21X |

## ITEM #97 - [BLANK]

## ITEM #98 - TYPE SERVICE [2 digits]

This item is intended to show the type of service on the bridge and the type of service under the bridge.  The service types for this item will be indicated by a two (2) digit code.

This first (1st) digit for the service on the bridge is as follows:

| Code | Description |
| --- | --- |
| 1 | Highway |
| 2 | Railroad |
| 3 | Pedestrian and/or bikeway |
| 4 | Highway/Railroad |
| 5 | Highway/pedestrian |
| 6 | Overpass structure at an interchange or second level of a multilevel interchange |
| 7 | Third level (Interchange) |
| 8 | Fourth level (Interchange) |
| 9 | Building or plaza |
| 0 | Other |
| A | Railroad (Abandoned w/rails removed) |

The second (2nd) digit will indicate the type of service under the bridge.

Code 9 (Relief) for a railroad, under a highway with the rails removed

| Code | Description |
| --- | --- |
| 1 | Highway, with or without pedestrian |
| 2 | Railroad |
| 3 | Pedestrian and/or bikeway |
| 4 | Highway/railroad |
| 5 | Waterway |
| 6 | Highway/waterway |
| 7 | Railroad/waterway |
| 8 | Highway/waterway/railroad |
| 9 | Relief (includes Railroad with tracks removed) |
| 0 | Other |

## ITEM # 99 – [BLANK]

## ITEM # 100 – [BLANK]

FORM #BR87 - CARDS G

Card G – PE Number (6 digits); First Name (20 Characters); and Last Name (20 Characters) of Rating Engineer

FORM #BR87 - CARDS H & I – REMARKS

Cards H & I - Retired Bridges, list SFN (s) that will replace this bridge.

Cards H & I - Add Bridges – SFN (s) that were replaced by this bridge.

FORM #BR87A - CARD J

## ITEM #101 - LOCATION OF STRUCTURE [25 digits]

This item gives a description of the bridge location.  The location should be keyed to some recognizable feature on the official highway map, (county map for county, township or municipal structures), such as road junctions and topographical features (using meaningful abbreviations when necessary).

Examples

6 MI. S. of SCIOTO RIVER

3.5 MI. S. of JCT SR 89

Item shall be left justified with no trailing zeros and must be coded for all structures.

## ITEM #102 - FACILITY CARRIED BY STRUCTURE [18 digits]

The facility being carried by the structure shall be recorded and coded.  This item shall be left justified without trailing zeros.  Meaningful abbreviations may be used when necessary.

Examples:

County Road 450

USR 66

Main Street

C & O Railroad (appropriate only when route is under railroad)

Pedestrian Bridge (appropriate only when route is under pedestrian bridge)

## ITEM #103 - ROUTE ON THE BRIDGE [2 digits]

Code a two (2) digit numeric for the agency whose route is carried by the bridge:

| Code | Description |
| --- | --- |
| 00 | Other |
| 10 | State (ODOT) (Toll Free) |
| 11 | ODNR |
| 12 | State (Other) (Toll Free) |
| 20 | Federal Domain |
| 30 | Toll Bridge (State) |
| 31 | Toll Bridge (Private) |
| 40 | County |
| 41 | Municipal |
| 42 | Township |
| 43 | Park District |
| 44 | Conservancy District |
| 99 | Non highway traffic on bridge (i.e. pedestrian, railroad, pipeline etc.) |

Do not code Turnpike as a toll road on the bridge.

## ITEM #104 - ROUTE UNDER THE BRIDGE [2 digits]

Code the Agency of the route which goes under the bridge and use the same codes used in Item #103.  For non-highway service under the bridge code the item 99.

## ITEM #105 - BORDER BRIDGE [5 digits]

Use this item to indicate structures crossing borders of States.  Code a five (5) digit number composed of two (2) segments specifying the responsibility for improvements to the existing structure when it is shared with a neighboring state.  Code the first three (3) digits using state codes listed below to code the neighboring state of the border bridge..  Code the fourth and fifth digits with the percentage of total deck area of the existing bridge that the neighboring State is responsible for funding.  (Maintenance agreements do not apply.)

If a neighboring State codes the structure and accepts 100% of the responsibility, FHWA still requires Ohio agencies to code a record for the structure.  Code the fourth and fifth digits as “99” to represent that the neighboring state has full responsibility for the structure.

If structure is not on a border, leave blank.

| Segment | Description | Length |
| --- | --- | --- |
| A | Neighboring State Code | 3 digits |
| B | Percent Responsibility | 2 digits |

| Code | State |
| --- | --- |
| 185 | Indiana |
| 214 | Kentucky |
| 265 | Michigan |
| 423 | Pennsylvania |
| 543 | West Virginia |

| Example |  |
| --- | --- |
| Code | Description |
| 21445 | A structure connects Ohio with Kentucky and Kentucky is responsible for funding 45 percent of future improvement costs. |
| 21499 | A structure connects Ohio with Kentucky and Kentucky is responsible for funding 100 percent of future improvement costs. |

## ITEM #106 - BORDER BRIDGE STRUCTURE NUMBER [15 digits]

Code the neighboring State's fifteen (15) digit National Bridge Inventory structure number for any structure noted in Item #105 - Border Bridge.  This number must match exactly the neighboring State's submitted NBI structure number.  The entire fifteen (15) digit field must be accounted for including zeros and blank spaces whether they are leading, trailing, or embedded in the fifteen (15) digit field.  If Item #105 is blank, this item must be blank.

## ITEM #107 – [BLANK]

## Item #108 – [BLANK]

FORM #BR87A - CARD K

## ITEM #109 - APPROACH GUARDRAIL TYPE [1 digit]

Using one of the codes below indicate the type guardrail on approach to the bridge.  Code X when bridge carries non-highway traffic (i.e. railroads, pedestrians, conveyor belts, etc.). Use this item to code railing on a culvert (X9X).

| Code | Type |
| --- | --- |
| 1 | Steel Beam |
| 2 | Flexible Steel Plate |
| 3 | Steel Cable |
| 4 | Steel Tape |
| 5 | Timber Rail |
| 6 | Timber & Steel Cable |
| 7 | Concrete Deflector Parapet |
| N | None |
| 0 | Other |
| X | Not applicable |

## ITEM #110 - APPROACH PAVEMENT WEARING SURFACE TYPE [1 digit]

Code the appropriate wearing surface for the approach pavement from the following list:

| Code | Description |
| --- | --- |
| 1 | Concrete |
| 2 | Bituminous |
| 3 | Brick |
| 4 | Gravel |
| 0 | Other |
| N | None or Not Applicable |

When the bridge carries non-highway traffic (i.e. railroads, pedestrians, conveyor belts, etc.) code N.

## ITEM #111 - APPROACH PAVEMENT GRADE [1 digit]

Note and rate the effect the grade of the approach pavement has on the impact to the bridge and safety to the vehicle.

| Code | Description |
| --- | --- |
| 1 | Good |
| 2 | Fair |
| 3 | Poor |
| 4 | Critical |
| N | None or Not Applicable |

When the bridge carries non-highway traffic (i.e. Railroad, Pedestrian, Conveyor Belts, etc.) code N.

## ITEM #112 - APPROACH SLAB [1 digit]

This item indicates whether a bridge has at least one approach slab.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No |
| U | Unknown |

For structures which carry non-highway traffic and most Culverts code N.

## ITEM #113 - LENGTH OF APPROACH SLAB [3 digits]

If Item #112 is coded Y then code the length of the approach slab to the nearest foot.  If Item #112 is coded N leave blank and item will default to zeros.

## ITEM #114 - #118 BLANK

## ITEM #119 - DATE OF WEARING SURFACE [8 digits]

Code eight (8) digits for the month, day and year of the most recent wearing surface application (i.e. concrete overlays, bituminous, etc.).  If not applicable leave item blank.

## ITEM #120 – BLANK

## ITEM #121 - MAIN MEMBER TYPE [1 digit]

This item must be coded for any structure whose main span is of the girder, beam or slab type regardless of material.

Select the appropriate code from the following list.  If item is not applicable code N.

| Code | Type |
| --- | --- |
| 1 | Rolled Steel |
| 2 | Riveted Built-Up Steel |
| 3 | Welded Built-Up Steel |
| 4 | Concrete Tee Beam |
| 5 | Concrete Girder |
| 6 | Prestressed Concrete Box Beam |
| 7 | Prestressed Concrete I Beam |
| 8 | Timber |
| 9 | Segmental Box Girder |
| 0 | Other (Concrete Rigid Frame) |
| A | Channel Beam |
| B | Cast-In-Place Concrete Box Beam |
| C | Slab |
| N | Not Applicable (Culverts, Trusses, Arches, etc.) |

## ITEM #122 - MOMENT PLATES [1 digit]

If the structure has moment plates, code the appropriate response from the listing below.  If there are no moment plates code N.

Code “X” may be coded when a non-highway (railroad, pedestrian; bikeway; coal haul beltway, etc.) passes over a roadway.

| Code | Description |
| --- | --- |
| 1 | Moment Plates (Welded) |
| 2 | Moment Plates (Riveted or Bolted) |
| N | No Moment Plates |
| X | Not Applicable |

This item must be coded for every new structure being "Added" to the database.

## ITEM #123 – [BLANK]

## ITEM #124 - BEARING DEVICE [2 digits]

Code a one (1) digit numeric which most closely describes the superstructure bearing device system. You may code either one (1) or two (2) types from the list below:

| Code | Description |
| --- | --- |
| 1 | Rollers |
| 2 | Rockers & Bolsters |
| 3 | Sliding (Bronze) |
| 4 | Elastomeric (Plain) |
| 5 | Pot |
| 6 | Spherical |
| 7 | Disc |
| 8 | Fixed Arch-Rib |
| N | None |
| 0 | Other |
| A | Sliding (Other) |
| B | Fixed |
| C | Elastomeric (laminated) |
| D | Integral & semi-integral abutment bearings |

For integral & semi-integral type abutments code D.  For structures which have no superstructure, such as culverts, code N.

ITEM #125 - [BLANK]

## ITEM #126 - NAVIGATION

This item is only for structures over a waterway and therefore should be left blank if the entire item does not apply.  If left blank, controlled portion will default to X and Vertical and Horizontal Clearance will default to zeros.

A.	Controlled [1 digit]

Indicate for this item whether or not navigation control exists.  The determination of whether or not a water course is navigable is made by the U.S. Coast Guard or U.S. Army Corps of Engineers, whoever has the authority or control.

| Code | Description |
| --- | --- |
| Y | Yes it is a navigable stream |
| N | No it is not a navigable stream |
| X | N/A |

B.	Vertical Clearance [3 digits]

If Part "A" has been coded "Y", record to the nearest foot the minimum vertical clearance imposed at the site as measured above a datum that is specified on navigation permits issued by a control agency.  This measurement will show the clearance that is allowable for navigation purposes.  In the case of a bascule bridge, the vertical clearance should be measured with the bridge in the closed position (i.e. open to vehicular traffic).  If Part "A" of this item is coded "N", or if a permit has not been issued to establish the clearances, code all zeros to indicate not applicable.  Clearance shall be added right-justified with leading zeros.

Examples

| Actual | Record | Code |
| --- | --- | --- |
| 150.0 | 150 | 150 |
| 20.6 | 21 | 021 |

C.	Horizontal Clearance [4 digits]

If Part "A" has been coded "Y"; this item must be coded for the minimum horizontal clearance to the nearest foot.  This measurement should be that shown on a navigation permit and may be less than the structure allows.  If Part “A” of this item is coded “N”, or if a permit has not been issued to establish the clearances, code all zeros to indicate not applicable.  Clearance should be added right-justified with leading zeros.

Examples

| Actual | Code |
| --- | --- |
| 95 ft. | 0095 |
| 538 ft. | 0538 |
| 1200 ft. | 1200 |

## ITEM #127 - LENGTH OUT/OUT OF CULVERT [4 digits]

This item shall be used to record the out/out length of circular or elliptical pipes, barrels, closed boxes and culvert type bridges, etc. to the nearest foot.  The length shall be measured along the axis from inlet to outlet at invert.  If there is a bend or curve along the length, measure the length along the bend or the curve.

This item must be recorded for all structure types coded as X9X.  Code the length right-justified with leading zeros.  For all other structure types, leave this item blank.

Example

| Length | Code |
| --- | --- |
| 127’ | 0127 |

Length Out/Out of Culvert is measured differently than Overall Structure Length.

ITEM #128 – [BLANK]

## ITEM #129 - DEPTH OF FILL OVER CULVERT [2 digits]

- Code to the nearest foot the depth of fill over a structure type (Item #63) that is coded as a culvert (X95).

- If depth of fill exceeds 99’ code 99.

- If bridge type is other than XX5 leave this item blank and item will default to zeros.

The depth of the fill should be measured from the top of the structure to the top of the pavement.  DO NOT subtract for the depth of the pavement.  If the depth of fill over the structure varies, measure the depth of fill at the center of the structure at centerline of the paved surface on the structure.

This item must be coded for all filled culverts (structure type X95).

Examples

| Depth of Fill | Code |
| --- | --- |
| 4.83 ft. | 05 |
| 10.16 ft. | 10 |
| 105 ft. | 99 |

## ITEM #130 - HEADWALLS OR ENDWALLS [1 digit]

Code the material type of headwall or end-wall for culvert type structures from the list below:

| Code | Type |
| --- | --- |
| 1 | Concrete |
| 2 | Stone |
| 3 | Metal |
| 4 | Wood |
| 5 | Gabion |
| N | None or Not Applicable (Not a Culvert) |
| 0 | Other |

Leave this item blank if the structure type is other than X95 and it will default to N.

## ITEM #131 - CULVERT/BRIDGE TYPE (1 digit)

If the structure is coded as a culvert X9X or rigid frame X7X in Item #63, then this item must reflect the appropriate code from the following list.  For a combination of more than one type, code the most appropriate type under the pavement.

If structure is not a Culvert or Frame type code N.

| Code | Description |
| --- | --- |
| 1 | Slab Top |
| 2 | 4-Sided Box (Concrete Cast-In-Place), structure material type must be coded as “1” |
| 3 | 3-Sided Frame (Concrete Cast-In-Place), structure material type must be coded as “1” |
| 4 | Pipe-Circular |
| 5 | Pipe-Arch (also includes stone) |
| 6 | Pipe-Elliptical |
| 7 | Arch (Multi-Plate) |
| 8 | 4-Sided Box (Concrete Precast), structure material type must be coded as “1” |
| 9 | 3-Sided Frame (Concrete Precast), structure material type must be coded as “1” |
| 0 | Other |
| A | 4-Sided Box (Aluminum) |
| B | 4-Sided Box (Other) |
| C | 3-Sided Frame (Aluminum) |
| D | Conspan (Concrete pre-cast) |
| E | Bebo-type (Concrete pre-cast) |
| F | Metal Arch on concrete pedestal wall & footing |
| N | Not a Culvert or Rigid Frame |

ITEM #132 - #134 – [BLANK]

## ITEM #135 – PINS, HANGERS, & HINGES [1 digit]

Code the type of hinges on the bridge.  Code N for none if not applicable.

| Code | Description |
| --- | --- |
| 1 | Pins & Hangers |
| 2 | Pins, Pin Plates |
| 3 | Seated Hinges (Rollers, Plates, Pot Bearings) |
| 4 | Other (includes Strut) |
| N | Not Applicable (structures with no hinges) |

ITEM #136 & #137 - [BLANK]

## ITEM #138 - TYPE OF MAIN LONGITUDINAL BRIDGE MEMBERS [1 digit]

Code the type of main bridge member of which the bridge is constructed, from the list below:

| Code | Description |
| --- | --- |
| 1 | Two Girder Bridge |
| 2 | Three Girder Bridge |
| 3 | Four or More Girder Bridge |
| 4 | Two Trusses (Welded) |
| 5 | Two Trusses (Riveted) |
| 6 | Three or More Trusses (Welded) |
| 7 | Three or More Trusses (Riveted) |
| 0 | Other |
| A | Two or more Steel Arches (Welded) |
| B | Two or more Steel Arches (Riveted) |
| C | Two Concrete Arches |
| D | Three Concrete Arches |
| E | Four or more Concrete Arches |
| F | Jack Arch |
| G | One Concrete Arch |
| N | Not Applicable (i.e. Culvert, Beam, Slab, etc.) |

Note:	See Item #63 A2 for definition of girder.

## ITEM #139 - FRAMING TYPE [1 digit]

Code the appropriate framing type for prestressed concrete I-beam, steel beam or steel girder bridges.

| Code | Description |
| --- | --- |
| 1 | Curved Beams/Girders |
| 2 | Dog-legged Beams/Girders |
| 3 | Flared Beams/Girders |
| 4 | Straight Beams/Girders |
| 0 | Other |
| N | None or Not Applicable |

Code “N” if it is not applicable (concrete slabs, timber, culverts, trusses, etc.)

ITEM #140 – [BLANK]

## ITEM #141 - STRUCTURAL STEEL

Leave this item blank if bridge has no structural steel (e.g., concrete slab, culverts, etc.) and the item will default to all zeros in Pay Weight and all N's for the rest of the item.

A.	Member [1 digit]

Record the appropriate code for the predominant type of structural steel (beams, girders, cross frames, etc.) used for a bridge.

| Code | Type |
| --- | --- |
| 1 | A588 (Weathering Steel) |
| 2 | A572 |
| 3 | A441 |
| 4 | A440 |
| 5 | A373 |
| 6 | A242 |
| 7 | A36 |
| 8 | A7 |
| 9 | A6 |
| 0 | Other |
| A | Wrought Iron |
| B | Hybrid (A572 or A588 flanges with A36 webs) |
| C | Hybrid (A709 grade 70W flanges with 50W webs) |
| D | A709 Grade 50 |
| E | A 709 Grade 50W |
| N | None |
| U | Unknown |

B.	Bridge Railing [1 digit]

Use the codes in part (A) to list the appropriate material for the bridge railing. (Guardrail not counted as bridge railing).  Any structure type "inventoried" as a culvert (X9X) shall not show bridge railing.

C. 	Structural Steel Pay Weight [8 digits]

Record the actual pay weight of the structural steel, for the bridge, in pounds.  When the actual pay weight is unknown, code the estimated pay weight to the nearest pound. Right justify using leading zeros.

D.	Prime Coat Location [1 digit]

Indicate where the prime coat of paint was applied to structural steel from the list of codes below:

| Code | Location |
| --- | --- |
| 1 | Shop |
| 2 | Field |
| 3 | Combination (Shop & Field) |
| U | Unknown |
| N | None (i.e. steel = A588, unpainted) |

E.	Protective Coating System (PCS) [1 digit]

Use the appropriate code from the list below to indicate the protective coating system.  This item (when applicable) should be coded for all structures.

| Code | Type Protection |
| --- | --- |
| 1 | Red lead |
| 2 | Unpainted Weathered Steel |
| 3 | Paint System A |
| 4 | Paint System B |
| 5 | Paint System OZEU |
| 6 | Galvanized |
| 7 | Metalized (Alum/Zinc) |
| 8 | Paint System A with intermediate tie coat |
| 9 | Paint System IZEU |
| 0 | Other Paint |
| A | EEU |
| B | Epoxy – Urethane sealers |
| C | Non – Epoxy sealers |
| N | None or Not Applicable |
| U | Unknown Sealant |

This item (when applicable) should be coded for all new structures being "Added" to the file.

FORM #BR85A - CARD L

## ITEM #142 - NAME OF FABRICATOR [14 digits]

List the name of the fabricator for the structural steel (i.e. beams, girders etc.) for the original bridge. Leave blank if not applicable.

Left justify and use meaningful abbreviations if necessary.

## ITEM #143 - NAME OF CONTRACTOR [14 digits]

This item will show the prime contractor for the original construction of the bridge. Leave blank if not applicable.

Left justify and use meaningful abbreviations, if necessary.

## ITEM #144 - OHIO ORIGINAL CONSTRUCTION PROJECT NUMBER [4 & 2 digits]

Code the number of the Ohio Project which the structure was originally constructed.  Right justify the project number in the first four (4) digits using leading zeros where necessary. Code the last two (2) digits of the project year in the last two (2) positions.  Agencies other than the State may code this item using any numeric system which fits within the six (6) digit field provided.  If non-state agencies choose not to code or the State project number is unknown, leave this item blank.

Note:	 See also Appendix F

## ITEM #145 - RAILROAD IDENTIFICATION [2 digits]

Code the appropriate railroad identification number for the railroad going over or under the bridge.  If not applicable leave blank and item will default to NN.

See Appendix K for railroad identification list.

ITEM #146 & #147 – [BLANK]

## ITEM #148 - DECK CONCRETE TYPE [1 digit]

Code the appropriate type concrete for the deck from the list below:

| Code | Description |
| --- | --- |
| 1 | Class C |
| 2 | Class S Superstructure |
| 3 | Class S Superstructure using shrinkage compensating cement |
| 4 | Class S Superstructure with Type 2 cement |
| 5 | High Performance Concrete Superstructure, Type 3 |
| 6 | High Performance Concrete Superstructure, Type 4 |
| 7 | Super-plasticized Concrete |
| 8 | Pre-cast Concrete (5,000 psi minimum) |
| 0 | Other |
| A | QSC1-Substructure concrete |
| B | QSC2-Superstructure concrete |
| C | QSC3-Project specific |
| N | None or Not Applicable |
| U | Unknown |

For Concrete Rigid Frames (171) code 8.  For culverts (X9X), railroad structures, pipelines, etc., code N.

ITEM #149 & #150 – [BLANK]

## ITEM #151 - STANDARD DRAWING NUMBER [11 digits]

Code the Standard Drawing Number for the bridge superstructure.  Left justify and do not fill with trailing zeros.

## ITEM #152 - DRAINAGE AREA [3 digits]

Code the drainage area to the nearest square mile for all the bridges over waterways.  If the drainage area on plans is given in acres, use the equation shown below to find square miles.

No. of Acres/640 = sq. mile

Code the drainage area right justified with leading zeros.  Code 001 if area is less than 1 square mile. If the drainage area is unknown code all U's.  However every effort should be made to determine the drainage area.

If not applicable code N's (i.e. structures over roadways, railroads, etc.).  If number of square miles exceeds 999 code all 9's.

Examples

| Drainage area | Code |
| --- | --- |
| 29.8 sq. miles | 030 |
| 12.1 sq. miles | 012 |

FORM #BR87A - CARD M

## ITEM #153 – WORK HISTORY

This item is to be filled in for each repair done to a particular bridge in order to keep a history of work on the bridge.  All work, whether it is major or minor (i.e. wearing surface, spot painting, etc.) should be recorded, regardless if it is done by contract or state forces.  When coding this item Start Date, End Date and Type Work must be coded.

Note:	Paint Year can only be changed by coding Bridge Work Type - 020 and entering a Start Date and End Date.  Paint Year can only be DELETED by Central Office.

Bridge painting is defined as part of a Protective Coating System (PCS).  Enter the Bridge Work Type (BWT) Code 020, a Start Date and an End Date for the bridge paint work.  BWT Code 020 shall only be used when the whole structural steel members are being painted.  DO NOT use BWT code 020 when only ends of a beam (e.g., ends of an A588 steel beam) are being painted.

- Bridge Work Type [3 digits]

This item should be coded with the three (3) digit work code that most closely fits the repair work done to the bridge, from the list below:

| General |  |
| --- | --- |
| Code | Description |
| 001 | Structure (New) |
| 002 | Structure (Replacement) |
| 003 | Superstructure (Replacement) |
| 004 | Deck Replacement |
| 005 | Structure Widening |
| 006 | Graffiti Removal |
| 007 | Movable Bridge (fix cost) |
| 008 | Movable Bridge (repair) |
| 009 | Collision Damage |
| 010 | Concrete Patching (non-deck) |
| 011 | Bridge Inspection (consultant) |
| 012 | Bridge Analysis |
| 013 | Structural Removal |
| 014 | Heat Straightening after an under-bridge hit |
| 015 | Raising Bridge |

| Superstructure |  |
| --- | --- |
| Code | Description |
| 020 | Painting Structural Steel |
| 021 | Spot Painting Structural Steel |
| 022 | Structure Steel Repair |
| 023 | Fatigue Retrofit |
| 024 | Pin and Hanger Retrofit |
| 025 | Bearing Reset/Replaced |
| 026 | Overhead Concrete Spall Removal |
| 027 | Bridge Diapering Installation |
| 028 | Drainage System Cleaned/Repaired |
| 029 | Back-wall Replacement/Repair |
| 039 | Superstructure (Other) |

| Deck Surface |  |
| --- | --- |
| Code | Description |
| 040 | Deck Overlay (Concrete) |
| 041 | Deck Overlay (Asphaltic Concrete) |
| 042 | Deck Overlay (Asphaltic Concrete with Waterproofing) |
| 043 | Deck Overlay (Epoxy) |
| 044 | Deck Overlay (Other) |
| 045 | Deck Skid Resistance Retrofit |
| 046 | Deck Cathodic Protection Installation |
| 047 | Deck and Abutment Seat Cleaning |
| 048 | Expansion Joint Repair/Replacement |
| 049 | Deck Patching (Concrete) |
| 050 | Deck Patching (Asphalt) |
| 051 | Deck Sealing |
| 052 | Spall Removal |
| 059 | Deck Surface (Other) |

| Deck Other |  |
| --- | --- |
| Code | Description |
| 060 | Sidewalk Repair/Replacement |
| 061 | Bridge Railing Upgrade/Repair |
| 062 | Fence Installation/Repair |
| 063 | Bridge Light Installation/Repair |

| Substructure |  |
| --- | --- |
| Code | Description |
| 080 | Foundation Stabilization |
| 081 | Channel Drift Removal |
| 082 | Slope Repair & Protection Installation |
| 083 | Scour Prevention and Correction |
| 084 | Pile Encasement |
| 085 | Pier Replacement/Repair |
| 086 | Abutment Replacement/Repair |
| 087 | Channel Clean Out |
| 088 | Substructure Sealing |
| 089 | Semi-Integral Abutment Conversion |
| 099 | Substructure (Other) |

| Approach |  |
| --- | --- |
| Code | Description |
| 100 | Approach Slab Replacement/Repair |
| 101 | Approach Roadway Grade Profile Correction |
| 102 | Approach Railing Repair |
| 103 | Pressure Relief Joint Installation |
| 119 | Approach (Other) |

| Structure Type Culvert |  |
| --- | --- |
| Code | Description |
| 120 | Culvert Invert Repair |
| 121 | Tunnel Liner Installation |
| 122 | Culvert Liner Repair |
| 139 | Culvert (Other) |

- Work Start Date [6 digits]

Record the month, day and year the most recent repair work started.

| Date 06-23-1995 | 062395 |
| --- | --- |

- Work End Date [6 digits]

Record the month, day and year the most recent repair of the structure was finished.

| Date 10-10-1995 | 101095 |
| --- | --- |

- Purchase Order Number [8 digits]

If the repairs were done by "purchase order", list the correct "purchase order number" in this item.

- Project Number [4 digits]

This project number is for the current or most recent repair job on the bridge, sold by state contract, and becomes part of the work history file for the bridge.

Code the three (3) or four (4) digit number under which this project was sold.  Right justify and use leading zeros where necessary.  If there has been no repair on the structure by contract, then leave the item blank.

- Project Year [2 digits]

If there is a project number coded then code the last two (2) digits of the year of the most recent repair project sold.  Otherwise leave this item blank.

- Project I.D. Number [9 digits]

Record the "Project I.D." for the most recent repair done to the bridge.  Right justify using leading zeros.

- Total Work Costs [11 digits]

Record the total cost in dollars of the most recent repairs or work to the bridge.  Right justify using leading zeros.

## ITEM #154 - TOTAL MINIMUM HORIZONTAL CLEARANCE ON BRIDGE [6 digits]

This item refers to the bridge roadway horizontal clearance on the bridge.  Code the available clearance measured normal to the centerline of roadway between restrictive features such as curbs, railings, sidewalks, wheel guards, raised medians and other structural features limiting the roadway surface width in the cardinal and non-cardinal directions.  Raised Button-Type medians and small raised lane channeling curbs, etc. are not considered restrictions.

If more than one roadway exists, a measurement should be made for each, but only the horizontal clearance of the mainline in each direction of travel shall be coded.

The term “Cardinal” in the item heading refers to the “Primary” direction of the route on the bridge (direction in which the route is officially measured for a particular agency’s records).

The purpose of this item is to give the largest available horizontal clearance for the movement of wide loads on the bridge. If a single or non-divided roadway is present, the clearance figure shall be coded in the space for the “Cardinal” opening and all zeros shall be coded under Non-Cardinal opening.  For culvert type structures, which appear on the route as a bridge but actually have no deck, the coded clearance should show the full width of the pavement plus shoulders except where this dimension is limited by guardrail, culvert end-walls, etc.  All clearances are to be coded in feet to the nearest tenth of a foot (with decimal point implied).

CLEARANCE A > B	ITEM #154

B > A              ITEM #154 = B

## ITEM #155 - PRACTICAL MAXIMUM VERTICAL CLEARANCE ON BRIDGE [4 digits]

The information to be coded for this item is the maximum vertical clearance on the bridge.  It is to be coded in feet and inches.  The minimum clearance for a ten-foot width of pavement traveled part of the roadway (shoulders paved or unpaved and median areas not included) where the clearance is greatest should be coded.

For structures having multiple openings on the bridge, vertical clearance for each mainline opening should be measured and recorded, but only the greatest of the minimum vertical clearances for two or more openings should be coded on the inventory form regardless of the direction of travel. Where no restrictions exist, code “9999”.

## ITEM #156 - TOTAL MINIMUM HORIZONTAL CLEARANCE UNDER BRIDGE [6 digits]

This item refers to the bridge roadway horizontal clearance under the bridge.  Code the available clearance measured normal to the centerline of roadway between restrictive features such as curbs, railings, sidewalks, wheel guards, raised medians and other structural features limiting the roadway surface width in the cardinal and non-cardinal directions.  Raised Button-Type medians and small raised lane channeling curbs, etc. are not considered restrictions.

If more than one roadway exists, a measurement should be made for each, but only the horizontal clearance of the mainline in each direction of travel shall be coded.

The term “Cardinal” in the item heading refers to the “Primary” direction of the route under the bridge (direction in which the route is officially measured for a particular agency’s records).

The purpose of this item is to give the largest available horizontal clearance for the movement of wide loads under the bridge. If a single or non-divided roadway is present, the clearance figure shall be coded in the space for the “Cardinal” opening and all zeros shall be coded under Non-Cardinal opening.  For culvert type structures which have no deck, the coded clearance should show the full width of the pavement plus shoulders except where this dimension is limited by guardrail, culvert end-walls, etc.  All clearances are to be coded in feet to the nearest tenth of a foot (with decimal point implied).

CLEARANCE A > B	ITEM #156

B > A              ITEM #156 = B

## ITEM #157 - PRACTICAL MAXIMUM VERTICAL CLEARANCE UNDER BRIDGE [4 digits]

The information to be coded for this item is the maximum vertical clearance under the bridge.  It is to be coded in feet and inches.  The minimum clearance for a ten-foot width of pavement traveled part of the roadway (shoulders paved or unpaved and median areas not included) where the clearance is greatest should be coded.

For structures having multiple openings under the bridge, vertical clearance for each mainline opening should be measured and recorded, but only the greatest of the minimum vertical clearances for two or more openings should be coded on the inventory form regardless of the direction of travel. Where no restrictions exist, code “0000”.

ITEM #158 – [BLANK]

FORM # BR87A - CARD N

## ITEM #159 - TEMPORARY BRIDGE CONDITION [4 * 1 digit]

Code Y or N for each of the following temporary conditions:

Barrier

Debris Net

Shored

Sub decking

| Code | Description |
| --- | --- |
| Y | Yes the condition exists |
| N | No the condition does not exist |

ITEM #160 – [BLANK]

## ITEM #161 - SPECIAL FEATURES [6*1 digit]

This item will indicate the type of special features carried by the bridge.  Code Y or N for each of the following special features carried by the bridge:

Lighting

Fencing

Glare Screen

Splash Guard

Catwalks

Other

If the bridge carries a special feature code Y in the appropriate space.  Code N if a special feature does not exist.  If a bridge carries a feature other than those listed above code as "other".

## ITEM #162 - HEIGHT OF FENCING ON BRIDGE [2 digits]

Code a two (2) digit number in feet to represent the height of fencing on the bridge.  Measure from the top of the parapet to the highest point on the fencing regardless if the fence is vertical or curved.

Code the height of the fence to the nearest foot using a leading zero.  Leave blank if not applicable and item will default to zeros.

## ITEM #163 - NOISE BARRIER WALLS [1 digit)]

Code this item to indicate if Noise Barrier Walls are present on the bridge.

| Code | Description |
| --- | --- |
| Y | The bridge does have Noise Barrier Walls |
| N | The bridge does not have Noise Barrier Walls |

ITEM #164 – (BLANK)

## ITEM #165 - POST TENSIONED [1 digit]

Code the item to indicate if the bridge is post tensioned.

| Code | Description |
| --- | --- |
| Y | Bridge is Post Tensioned |
| N | Bridge is not Post Tensioned |

ITEM #166 – [BLANK]

## ITEM #167 - SCENIC RIVER [1 digit]

Code the item to indicate if the river under the bridge is designated as a Scenic River.

| Code | Description |
| --- | --- |
| Y | Waterway is classified as Scenic River |
| N | Waterway is not classified as Scenic River |

ITEM #168 – [BLANK]

## ITEM #169 - EXPANSION JOINT TYPE [3 digits]

You may code up to three (3) different types of expansion joints for a bridge.  Code N in all non-applicable sections.

| Code | Description |
| --- | --- |
| 1 | Metal Finger |
| 2 | Sliding Metal Plate Angle |
| 3 | Compression Seal |
| 4 | Poured |
| 5 | Open (Armored) |
| 6 | Open (Unarmored) |
| 7 | Steel Reinforced Elastomeric |
| 8 | Elastomeric Strip Seal |
| N | None |
| 0 | Other |
| A | Modular |
| B | Polymer modified expansion device |

ITEM #170 – [BLANK]

## ITEM #171 - JOINT RETROFIT [6 digits]

This item may be coded for existing expansion joints coded in Item #169, for up to three (3) expansion joints, which have been retrofitted with a replacement joint system.  The following codes may be used for RETRO 1, 2 or 3.

| Code | Description |
| --- | --- |
| 1 | Elastomeric strip seal device |
| 2 | Polymer modified expansion device |
| 3 | Compression seal device |
| 4 | Modular type expansion device |
| 5 | Hot poured joint sealer |
| 6 | Silicon joint sealer |
| 0 | Other |
| N | None or Not Applicable |
| U | Unknown |

TROUGH 1, 2 or 3 may be coded as Y for yes or N for no to define the presence of an elastomeric trough installed as part of the expansion joint system retrofit.  Code all N's if not applicable.

ITEM #172 – [BLANK]

## ITEM #173 - HAUNCHED GIRDER [1 digit]

This item indicates if the bridge has a haunched girder.

| Code | Description |
| --- | --- |
| Y | Bridge contains a haunched girder |
| N | Bridge does not contain a haunched girder |

## ITEM #174 - HAUNCHED GIRDER DEPTH [4 digits]

Measure and record to the nearest inch the depth of the haunch (depth over pier or abutment) of the girder if Item #173 is coded Y.  If Item #173 is coded N leave this item blank and item will default to zeros.

| Examples |  |
| --- | --- |
| Haunched Girder Depth | Code |
| 6 ft. 10 inches | 0610 |
| 5 ft. 2 inches | 0502 |

## ITEM #175 - MAIN MEMBER DEPTH [4 digits]

Measure and record the vertical height of the main member of the structure to the nearest inch (i.e. girder, beam, slab, lower chord, etc.).

If the main member is a haunched girder record the height of the girder to the nearest inch at mid-span.

| Examples |  |
| --- | --- |
| Main Member Depth | Code |
| 3 ft. 4 inches | 0304 |
| 27 inches | 0203 |

ITEM #176 & #177 – [BLANK]

## ITEM #178 - PILE LOG [1 digit]

This item will indicate if a pile log has been maintained for the structure.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No |

## ITEM #179 - ABUTMENT/PILE LOAD TESTING [1 digit]

Code Y if dynamic or static load testing has been performed on the abutments and/or piers.  Code N if no testing has been performed code N.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No |

ITEM #180 – [BLANK]

FORM #BR85A - CARD O

## ITEM #181 - NON-CARDINAL RAMP

This item is for recording under clearances and roadway widths for ramps entering or exiting under a bridge on one or both sides of a mainline.  Leave item blank if there is no ramp and item will default to zeros.

- Minimum Vertical Under clearance [4 digits]

This item includes a four (4) digit coding representing the Minimum Vertical Clearance from the roadway of the ramp beneath the structure to the underside of the superstructure.  The item should be recorded in feet and inches, rounded down to the inch.  In each subsection (feet, inches) right justify the coded values and fill with leading zeros where necessary.

- Minimum Lateral Under Clearance to Edge of Lane [2 * 3 digits]

If the feature beneath the structure is a ramp, code a three (3) digit number to represent the Minimum Lateral clearance on the right and another number for the clearance on the left.  Code each under the appropriate heading on the code sheet.  Lateral clearances should be measured from the right or left edge of the lane to the nearest substructure unit (Or toe of slope steeper than 3 to 1) on the right and to the nearest substructure unit or median barrier.  All measurements should be made normal to the centerline of the ramp. The term "Left" refers to the left hand edge of the lane on a ramp while facing in the direction of traffic flow, while the term "Right" refers to the right hand edge of the lane while facing in the direction of traffic.  Code these dimensions to the nearest tenth of a foot.  In each subsection right justify all entries and fill with leading zeros where necessary.

- Ramp Roadway Width [3 digits]

Measure and record the roadway width of the ramp between edges of pavement to the nearest tenth of a foot.  Use leading zeros where necessary.

| Example |  |
| --- | --- |
| Roadway width of Ramp | Code |
| 24 ft. | 240 |
| 18 ft. 5 inches | 184 |

## ITEM #182 - CARDINAL RAMP

This item is for recording under clearances and roadway widths for ramps entering or exiting under a bridge on one or both sides of a mainline.  Leave item blank if there is no ramp and item will default to zeros.

- Minimum Vertical Under clearance [4 digits]

This item includes a four (4) digit coding representing the Minimum Vertical Clearance from the roadway of the ramp beneath the structure to the underside of the superstructure.  The item should be recorded in feet and inches, rounded down to the inch.  In each subsection (feet, inches) right justify the coded values and fill with leading zeros where necessary.

- Minimum Lateral Under clearance to Edge of Lane [2 * 3 digits]

If the feature beneath the structure is a ramp, code a three (3) digit number to represent the Minimum Lateral clearance on the right and another number for the clearance on the left.  Code each under the appropriate heading on the code sheet.  Lateral clearances should be measured from the right or left edge of the lane to the nearest substructure unit (on toe of slope steeper than 3 to 1) on the right and to the nearest substructure unit or median barrier.  All measurements should be made normal to the centerline of the ramp.  The term "Left" refers to the left hand edge of the lane on a ramp while facing in the direction of traffic flow, while the term "Right" refers to the right hand edge of the lane while facing in the direction of traffic.  Code these dimensions to the nearest tenth of a foot.  In each subsection right justify all entries and fill with leading zeros where necessary.

- Ramp Roadway Width [3 digits]

Measure and record the roadway width of the ramp between edges of pavement to the nearest tenth of a foot.  Use leading zeros where necessary.

| Example |  |
| --- | --- |
| Roadway width of Ramp | Code |
| 24 ft. | 240 |
| 18 ft. 5 inches | 185 |

ITEM #183 – [BLANK]

## ITEM #184 - SIGN ON/UNDER BRIDGE [2 digits]

This item is composed of two (2) parts:  The first column will determine if there are signs attached to the bridge that deal with the traffic on the bridge.  The second column will indicate if there are signs attached to the structure that affect traffic under the bridge.

Code the item “Y” or “N” for each area to indicate the presence of a traffic sign.

| Code | Description |
| --- | --- |
| Y | Yes |
| N | No |

## ITEM #185 - WATER FLOW DIRECTION  [1 digit]

The default value of this item is “N”.  Code this item to indicate the direction of water flow under the bridges over waterways.  The direction of water flow shall be determined while standing on the bridge facing the forward direction of the route (cardinal direction); if water flows from your left to your right, Code “R” if water flows from your right to your left, Code “L”.  Code “N” if there is negligible water flow as in a lake or pond, etc.

## ITEM #186 - SLOPE PROTECTION [1 digit]

Indicate the method, if any, used to protect the areas under the bridge from erosion and other degradation.  Natural protection (grass, bushes, trees) are to be coded N.  If channel extends all the way to the abutments, there is no slope protection.  The dominant slope protection must be coded for all structures.  (See also Item #75 – Channel Protection, which is different from this item.)

Select from the list below:

| Code | Description |
| --- | --- |
| 1 | Concrete (cast-in-place) |
| 2 | Stone (No. 1 Aggregate) |
| 3 | Rip Rap (dumped rock or rock channel protection) |
| 4 | Gabions (wire mesh baskets filled with stone) |
| 5 | Fabric bags filled with concrete |
| 0 | Other |
| A | Reinforced Cast-In-Place Concrete Wall |
| B | Soldier Pile and Lagging Wall |
| C | Mechanically Stabilized Earth (MSE) Wall |
| D | Soil Nail Wall |
| E | Tie Back Wall |
| F | Sheet Pile Wall |
| G | Cribbed Wall |
| H | Concrete Block Wall |
| I | Stone Wall |
| S | Soil |
| N | None |

ITEM #187 – [BLANK]

## ITEM #188 - FRACTURE CRITICAL INSPECTION

A fracture critical bridge is defined as having one (1) or more fracture critical members.

A fracture critical member is a member (usually a tension member) or component, whose failure will result in a sudden collapse of all or part of the structure.

Welded structures are particularly susceptible to fracture criticality as are bridges containing only two (2) main members (non-redundant).

This item consists of three (3) parts.  In the first part, first column, indicate if a fractural critical member is present on the bridge by coding the fracture critical switch to “Y” or “N”.

Fracture Critical Switch [1 digit]

| Code | Description |
| --- | --- |
| Y | Yes, there is a fracture critical member on the bridge. |
| N | No, there is no fracture critical member on the bridge. |

- Inspection Frequency Months [2 digits]

In the second part, in next two (2) columns, code the frequency of inspection in months.

If the fracture critical switch is coded Y the fracture critical inspection frequency must be coded for the structure.

Code right justified a two (2) digit number indicating the number of months between inspections.

If fracture critical switch is coded N leave inspection frequency months blank and they will default to zeros.

- Date of Inspection [6 digits]

Code the most recent date of fracture critical inspection for the structure including month, day and year.

If fracture critical inspection is coded N leave this item blank.

| Most recent date of Fracture Critical Inspection | Code |
| --- | --- |
| July 7, 2003 | 070703 |
| 10/17/1998 | 101798 |

## ITEM #189 - UNDERWATER INSPECTION

This item shall be coded for all bridges over waterways.

- Diving Inspection Switch [1 digit]

If a wading inspection cannot be done, a dive inspection must be done.  A wading inspection is when the substructure units and the waterway are evaluated using a probing rod, sounding rod or line, waders, and possibly a boat.

| Code | Description |
| --- | --- |
| Y | Bridge to be inspected by diving |
| N | Bridge does not need inspection by diving |

Code N for structures not over water.

- Diving Inspection Frequency [2 digits]

If the diving inspection switch is coded Y the diving inspection frequency in months must be coded for the structure.

Code right justified a two (2) digit number indicating the number of months between diving inspections.

| Code | Frequency of Diving Inspection |
| --- | --- |
| 06 | Every 6 months |
| 60 | 5 years |
| 24 | 2 years |

If Diving inspection switch is coded N leave this item blank and it will default to zeros.

- Date of Diving Inspection [6 digits]

Code the most recent date of diving inspection for the structure including month, day and year.

If the diving inspection switch is coded N leave this date blank and it will default to zeros.

| Code | Date of Last Diving Inspection |
| --- | --- |
| 072103 | 07/21/03 |
| 050105 | May 1, 2005 |

- Probed [1 digit]

If the bridge is probed code the item Y.  All bridges over water don=t have to be probed if the elements are out of the water.

- Frequency of Probing [2 digits]

If the Probed Switch is coded “Y”, code the frequency of probing inspection in months.  This item cannot have a frequency coded greater than 12 months.

If the probe switch is coded N this item must be blank.

## ITEM #190 - INSPECTION FROM SNOOPER [1 digit]

Code this item “Y” or “N” to indicate if a Snooper is required to inspect the bridge.

| Code | Description |
| --- | --- |
| Y | Needs snooper to inspect bridge |
| N | Does not need snooper to inspect bridge |

## ITEM #191 - INSPECTION FROM BOAT [1 digit]

Code this item “Y” or “N” to indicate if a boat is required to inspect the bridge.

| Code | Description |
| --- | --- |
| Y | Needs boat to inspect bridge |
| N | Does not need boat to inspect bridge |

ITEM #192 – BLANK

## ITEM #193 - SPECIAL INSPECTION [1 digit]

The inspection of a structure with a critical feature that needs inspected more frequently than the regular twelve month cycle.

- Special Inspection Switch [1 digit]

| Code | Description |
| --- | --- |
| Y | Special Inspection needed |
| N | No Special Inspection needed |

- Special Inspection Frequency [2 digits]

Designate the inspection interval in months for any structure that has a critical feature that needs inspected more frequently than the regular twelve month cycle. Code the frequency of special inspection in months.  If the special inspection frequency switch is coded “Y” the special inspection frequency must be coded.

| Example: | A bridge that has been temporarily shored and needs to be inspected every 3 months code 03. |
| --- | --- |

Since Fracture Critical, Scour Critical bridges and Snooper Inspection are covered elsewhere in this manual, disregard them when coding this item.

If there are no features that are critical other than fracture or scour critical, Code N in the Special Inspection Switch, and the special inspection frequency will default to zeros.

- Date of Special Inspection [6 digits]

Code the most recent date of special inspection for the structure including month, day and year.

If the special inspection switch is coded N leave this date blank and it will default to zeros.

| Code | Date of Special Inspection |
| --- | --- |
| 072103 | 07/21/03 |
| 050105 | May 1, 2005 |

## ITEM #194 – SEISMIC SUSCEPTIBLE[1 digit]

Code this item according to the following table:

| Code | Description |
| --- | --- |
| 1 | Unreinforced masonry (laid-up stone abutments and piers) |
| 2 | Simple span approaches to large truss bridges |
| 3 | Prestressed I or box beams if not tied together over the piers or pinned into the piers and abutments |
| 4 | Non-redundant steel pier caps which are not tied to either the pier columns or other bridge superstructure |
| 5 | Multiple column type piers with no pier cap |
| 6 | Bridges with centerline of bearing 4” or less to face of abutment or pier |
| 7 | Bridge with seated hinges (not pins and hangers) |
| N | Not applicable |

## APPENDIX A

Ohio County Code Numbers

| Code | County | District | Code | County | District |
| --- | --- | --- | --- | --- | --- |
| 01 | Adams | 09 | 02 | Allen | 01 |
| 03 | Ashland | 03 | 04 | Ashtabula | 04 |
| 05 | Athens | 10 | 06 | Auglaize | 07 |

| 07 | Belmont | 11 | 08 | Brown | 09 |
| --- | --- | --- | --- | --- | --- |
| 09 | Butler | 8 |  |  |  |

| 10 | Carroll | 11 | 11 | Champaign | 07 |
| --- | --- | --- | --- | --- | --- |
| 12 | Clark | 07 | 13 | Clermont | 08 |
| 14 | Clinton | 08 | 15 | Columbiana | 11 |
| 16 | Coshocton | 05 | 17 | Crawford | 03 |
| 18 | Cuyahoga | 12 |  |  |  |

| 19 | Darke | 07 | 20 | Defiance | 01 |
| --- | --- | --- | --- | --- | --- |
| 21 | Delaware | 06 |  |  |  |

| 22 | Erie | 03 |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 23 | Fairfield | 05 | 24 | Fayette | 06 |
| --- | --- | --- | --- | --- | --- |
| 25 | Franklin | 06 | 26 | Fulton | 02 |

| 27 | Gallia | 10 | 28 | Geauga | 12 |
| --- | --- | --- | --- | --- | --- |
| 29 | Greene | 08 | 30 | Guernsey | 05 |

| 31 | Hamilton | 08 | 32 | Hancock | 01 |
| --- | --- | --- | --- | --- | --- |
| 33 | Hardin | 01 | 34 | Harrison | 11 |
| 35 | Henry | 02 | 36 | Highland | 09 |
| 37 | Hocking | 10 | 38 | Holmes | 11 |
| 39 | Huron | 03 |  |  |  |

APPENDIX A

(Continued)

Ohio County Code Numbers

| Code | County | District | Code | County | District |
| --- | --- | --- | --- | --- | --- |
| 40 | Jackson | 09 | 41 | Jefferson | 11 |

| 42 | Knox | 05 |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 43 | Lake | 12 | 44 | Lawrence | 09 |
| --- | --- | --- | --- | --- | --- |
| 45 | Licking | 05 | 46 | Logan | 07 |
| 47 | Lorain | 03 | 48 | Lucas | 02 |

| 49 | Madison | 06 | 50 | Mahoning | 04 |
| --- | --- | --- | --- | --- | --- |
| 51 | Marion | 06 | 52 | Medina | 03 |
| 53 | Meigs | 10 | 54 | Mercer | 07 |
| 55 | Miami | 07 | 56 | Monroe | 10 |
| 57 | Montgomery | 07 | 58 | Morgan | 10 |
| 59 | Morrow | 06 | 60 | Muskingum | 05 |

| 61 | Noble | 10 |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 62 | Ottawa | 02 |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 63 | Paulding | 01 | 64 | Perry | 05 |
| --- | --- | --- | --- | --- | --- |
| 65 | Pickaway | 06 | 66 | Pike | 09 |
| 67 | Portage | 04 | 68 | Preble | 08 |
| 69 | Putnam | 01 |  |  |  |

| 70 | Richland | 03 | 71 | Ross | 09 |
| --- | --- | --- | --- | --- | --- |

APPENDIX A

(Continued)

Ohio County Code Numbers

| Code | County | District | Code | County | District |
| --- | --- | --- | --- | --- | --- |
| 72 | Sandusky | 02 | 73 | Scioto | 09 |
| 74 | Seneca | 02 | 75 | Shelby | 07 |
| 76 | Stark | 04 | 77 | Summit | 04 |

| 78 | Trumbull | 04 | 79 | Tuscarawas | 11 |
| --- | --- | --- | --- | --- | --- |

| 80 | Union | 06 |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 81 | Van Wert | 01 | 82 | Vinton | 10 |
| --- | --- | --- | --- | --- | --- |

| 83 | Warren | 08 | 84 | Washington | 10 |
| --- | --- | --- | --- | --- | --- |
| 85 | Wayne | 03 | 86 | Williams | 02 |
| 87 | Wood | 02 | 88 | Wyandot | 01 |

## APPENDIX B

List of Counties by District

Showing Alphabetic and Numeric

County Codes

| CODE | DISTRICT 1 | COUNTY | CODE | DISTRICT 2 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 02 | Allen | ALL | 26 | Fulton | FUL |
| 20 | Defiance | DEF | 35 | Henry | HEN |
| 32 | Hancock | HAN | 48 | Lucas | LUC |
| 33 | Hardin | HAR | 62 | Ottawa | OTT |
| 63 | Paulding | PAU | 72 | Sandusky | SAN |
| 69 | Putnam | PUT | 74 | Seneca | SEN |
| 81 | Van Wert | VAN | 86 | Williams | WIL |
| 88 | Wyandot | WYA | 87 | Wood | WOO |

| CODE | DISTRICT 3 | COUNTY | CODE | DISTRICT 4 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 03 | Ashland | ASD | 04 | Ashtabula | ATB |
| 17 | Crawford | CRA | 50 | Mahoning | MAH |
| 22 | Erie | ERI | 67 | Portage | POR |
| 39 | Huron | HUR | 76 | Stark | STA |
| 47 | Lorain | LOR | 77 | Summit | SUM |
| 52 | Medina | MED | 78 | Trumbull | TRU |
| 70 | Richland | RIC |  |  |  |
| 85 | Wayne | WAY |  |  |  |

| CODE | DISTRICT 5 | COUNTY | CODE | DISTRICT 6 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 16 | Coshocton | COS | 21 | Delaware | DEL |
| 23 | Fairfield | FAI | 24 | Fayette | FAY |
| 30 | Guernsey | GUE | 25 | Franklin | FRA |
| 42 | Knox | KNO | 49 | Madison | MAD |
| 45 | Licking | LIC | 51 | Marion | MAR |
| 60 | Muskingum | MUS | 59 | Morrow | MRW |
| 64 | Perry | PER | 65 | Pickaway | PIC |
|  |  |  | 80 | Union | UNI |

APPENDIX B

(Continued)

List of Counties by District

Showing Alphabetic and Numeric

County Codes

| CODE | DISTRICT 7 | COUNTY | CODE | DISTRICT 8 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 06 | Auglaize | AUG | 09 | Butler | BUT |
| 11 | Champaign | CHP | 13 | Clermont | CLE |
| 12 | Clark | CLA | 14 | Clinton | CLI |
| 19 | Darke | DAR | 29 | Greene | GRE |
| 46 | Logan | LOG | 31 | Hamilton | HAM |
| 54 | Mercer | MER | 68 | Preble | PRE |
| 55 | Miami | MIA | 83 | Warren | WAR |
| 57 | Montgomery | MOT |  |  |  |
| 75 | Shelby | SHE |  |  |  |

| CODE | DISTRICT 9 | COUNTY | CODE | DISTRICT 10 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 01 | Adams | ADA | 05 | Athens | ATH |
| 08 | Brown | BRO | 27 | Gallia | GAL |
| 36 | Highland | HIG | 37 | Hocking | HOC |
| 40 | Jackson | JAC | 53 | Meigs | MEG |
| 44 | Lawrence | LAW | 56 | Monroe | MOE |
| 66 | Pike | PIK | 61 | Noble | NOB |
| 71 | Ross | ROS | 82 | Vinton | VIN |
| 73 | Scioto | SCI | 84 | Washington | WAS |

| CODE | DISTRICT 11 | COUNTY | CODE | DISTRICT 12 | COUNTY |
| --- | --- | --- | --- | --- | --- |
| 07 | Belmont | BEL | 18 | Cuyahoga | CUY |
| 10 | Carroll | CAR | 28 | Geauga | GEA |
| 15 | Columbiana | COL | 43 | Lake | LAK |
| 34 | Harrison | HAS |  |  |  |
| 38 | Holmes | HOL |  |  |  |
| 41 | Jefferson | JEF |  |  |  |
| 79 | Tuscarawas | TUS |  |  |  |

## APPENDIX C

STRUCTURE TYPE CODING

The following list of structure type code combinations established in accordance with the three (3) digit coding scheme shown in Item #63 Part A(2) and used also in Part B(2) includes a word description for the bridge type opposite each code shown.  All bridges inventoried must be type coded using the applicable code combination from this list.  Code combinations not shown may not be used as they will be rejected as an error by the computer.

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 000 | Other | Other | Other |
| 055 | Other | Arch | Filled |
| 095 | Other | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 100 | Concrete | Other | Other |
| 105 | Concrete | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 110 | Concrete | Slab | Other |
| 111 | Concrete | Slab | Simple |
| 112 | Concrete | Slab | Continuous |
| 115 | Concrete | Slab | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 120 | Concrete | Beam | Other |
| 121 | Concrete | Beam | Simple |
| 122 | Concrete | Beam | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 130 | Concrete | Box Beam | Other |
| 131 | Concrete | Box Beam | Simple |
| 132 | Concrete | Box Beam | Continuous |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 150 | Concrete | Arch | Other |
| 153 | Concrete | Arch | Deck |
| 154 | Concrete | Arch | Thru |
| 155 | Concrete | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 160 | Concrete | Girder | Other |
| 163 | Concrete | Girder | Deck |
| 164 | Concrete | Girder | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 170 | Concrete | Frame | Other |
| 171 | Concrete | Frame | Simple |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 190 | Concrete | Culvert | Other |
| 195 | Concrete | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 1A0 | Concrete | Cable Stayed | Other |
| 1A4 | Concrete | Cable Stayed | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 1B0 | Concrete | Tunnel | Other |
| 1B4 | Concrete | Tunnel | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 200 | Prestressed Concrete | Other | Other |
| 205 | Prestressed Concrete | Other | Filled |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 210 | Prestressed Concrete | Slab | Other |
| 211 | Prestressed Concrete | Slab | Simple |
| 212 | Prestressed Concrete | Slab | Continuous |
| 215 | Prestressed Concrete | Slab | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 220 | Prestressed Concrete | Beam | Other |
| 221 | Prestressed Concrete | Beam | Simple |
| 222 | Prestressed Concrete | Beam | Continuous |
| 225 | Prestressed Concrete | Beam | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 230 | Prestressed Concrete | Box Beam | Other |
| 231 | Prestressed Concrete | Box Beam | Simple |
| 232 | Prestressed Concrete | Box Beam | Continuous |
| 235 | Prestressed Concrete | Box Beam | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 250 | Prestressed Concrete | Arch | Other |
| 253 | Prestressed Concrete | Arch | Deck |
| 254 | Prestressed Concrete | Arch | Thru |
| 255 | Prestressed Concrete | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 260 | Prestressed Concrete | Girder | Other |
| 261 | Prestressed Concrete | Girder | Simple |
| 262 | Prestressed Concrete | Girder | Continuous |
| 263 | Prestressed Concrete | Girder | Deck |
| 264 | Prestressed Concrete | Girder | Thru |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 295 | Prestressed Concrete | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 300 | Steel | Other | Other |
| 305 | Steel | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 320 | Steel | Beam | Other |
| 321 | Steel | Beam | Simple |
| 322 | Steel | Beam | Continuous |
| 326 | Steel | Beam | Orthotropic |
| 327 | Steel | Beam | Movable – Lift |
| 328 | Steel | Beam | Movable – Bascule |
| 329 | Steel | Beam | Movable - Swing |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 330 | Steel | Box Beam | Other |
| 331 | Steel | Box Beam | Simple |
| 332 | Steel | Box Beam | Continuous |
| 336 | Steel | Box Beam | Orthotropic |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 340 | Steel | Truss | Other |
| 343 | Steel | Truss | Deck |
| 344 | Steel | Truss | Thru |
| 347 | Steel | Truss | Movable – Lift |
| 348 | Steel | Truss | Movable – Bascule |
| 349 | Steel | Truss | Movable – Swing |
| 34A | Steel | Truss | Pony |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 350 | Steel | Arch | Other |
| 353 | Steel | Arch | Deck |
| 354 | Steel | Arch | Thru |
| 355 | Steel | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 360 | Steel | Girder | Other |
| 363 | Steel | Girder | Deck |
| 364 | Steel | Girder | Thru |
| 366 | Steel | Girder | Orthotropic |
| 367 | Steel | Girder | Movable – Lift |
| 368 | Steel | Girder | Movable – Bascule |
| 369 | Steel | Girder | Movable - Swing |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 370 | Steel | Frame | Other |
| 371 | Steel | Frame | Simple |
| 372 | Steel | Frame | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 380 | Steel | Suspension | Other |
| 383 | Steel | Suspension | Deck (Truss) |
| 384 | Steel | Suspension | Thru (Truss) |
| 386 | Steel | Suspension | Orthotropic |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 390 | Steel | Culvert | Other |
| 395 | Steel | Culvert | Filled |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 3A0 | Steel | Cable Stayed | Other |
| 3A4 | Steel | Cable Stayed | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 3B0 | Steel | Tunnel | Other |
| 3B4 | Steel | Tunnel | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 400 | Timber | Other | Other |
| 405 | Timber | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 410 | Timber | Slab | Other |
| 411 | Timber | Slab | Simple |
| 412 | Timber | Slab | Continuous |
| 415 | Timber | Slab | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 420 | Timber | Beam | Other |
| 421 | Timber | Beam | Simple |
| 422 | Timber | Beam | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 440 | Timber | Truss | Other |
| 443 | Timber | Truss | Deck |
| 444 | Timber | Truss | Thru |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 450 | Timber | Arch | Other |
| 453 | Timber | Arch | Deck |
| 454 | Timber | Arch | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 460 | Timber | Girder | Other |
| 463 | Timber | Girder | Deck |
| 464 | Timber | Girder | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 470 | Timber | Frame | Other |
| 471 | Timber | Frame | Simple |
| 472 | Timber | Frame | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 480 | Timber | Suspension | Other |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 490 | Timber | Culvert | Other |
| 495 | Timber | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 500 | Stone | Other | Other |
| 505 | Stone | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 511 | Stone | Slab | Simple |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 550 | Stone | Arch | Other |
| 555 | Stone | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 590 | Stone | Culvert | Other |
| 595 | Stone | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 5B0 | Stone | Tunnel | Other |
| 5B4 | Stone | Tunnel | Thru |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 600 | Aluminum | Other | Other |
| 605 | Aluminum | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 620 | Aluminum | Beam | Other |
| 621 | Aluminum | Beam | Simple |
| 622 | Aluminum | Beam | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 630 | Aluminum | Box Beam | Other |
| 631 | Aluminum | Box Beam | Simple |
| 632 | Aluminum | Box Beam | Continuous |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 640 | Aluminum | Truss | Other |
| 643 | Aluminum | Truss | Deck |
| 644 | Aluminum | Truss | Thru |
| 647 | Aluminum | Truss | Movable – Lift |
| 648 | Aluminum | Truss | Movable – Bascule |
| 649 | Aluminum | Truss | Movable – Swing |
| 64A | Aluminum | Truss | Pony |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 650 | Aluminum | Arch | Other |
| 653 | Aluminum | Arch | Deck |
| 654 | Aluminum | Arch | Thru |
| 655 | Aluminum | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 660 | Aluminum | Girder | Other |
| 663 | Aluminum | Girder | Deck |
| 664 | Aluminum | Girder | Thru |
| 666 | Aluminum | Girder | Orthotropic |
| 667 | Aluminum | Girder | Movable – Lift |
| 668 | Aluminum | Girder | Movable – Bascule |
| 669 | Aluminum | Girder | Movable - Swing |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 670 | Aluminum | Frame | Other |
| 671 | Aluminum | Frame | Simple |
| 672 | Aluminum | Frame | Continuous |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 680 | Aluminum | Suspension | Other |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 690 | Aluminum | Culvert | Other |
| 695 | Aluminum | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 700 | Cast Iron | Other | Other |
| 705 | Cast Iron | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 720 | Cast Iron | Beam | Other |
| 721 | Cast Iron | Beam | Simple |
| 722 | Cast Iron | Beam | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 740 | Cast Iron | Truss | Other |
| 743 | Cast Iron | Truss | Deck |
| 744 | Cast Iron | Truss | Thru |
| 747 | Cast Iron | Truss | Movable – Lift |
| 748 | Cast Iron | Truss | Movable – Bascule |
| 749 | Cast Iron | Truss | Movable – Swing |
| 74A | Cast Iron | Truss | Pony |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 750 | Cast Iron | Arch | Other |
| 755 | Cast Iron | Arch | Filled |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 760 | Cast Iron | Girder | Other |
| 763 | Cast Iron | Girder | Deck |
| 764 | Cast Iron | Girder | Thru |
| 767 | Cast Iron | Girder | Movable – Lift |
| 768 | Cast Iron | Girder | Movable – Bascule |
| 769 | Cast Iron | Girder | Movable - Swing |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 770 | Cast Iron | Frame | Other |
| 771 | Cast Iron | Frame | Simple |
| 772 | Cast Iron | Frame | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 790 | Cast Iron | Culvert | Other |
| 795 | Cast iron | Culvert | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 800 | Wrought Iron | Other | Other |
| 805 | Wrought Iron | Other | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 820 | Wrought Iron | Beam | Other |
| 821 | Wrought Iron | Beam | Simple |
| 822 | Wrought Iron | Beam | Continuous |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 840 | Wrought Iron | Truss | Other |
| 843 | Wrought Iron | Truss | Deck |
| 844 | Wrought Iron | Truss | Thru |
| 847 | Wrought Iron | Truss | Movable – Lift |
| 848 | Wrought Iron | Truss | Movable – Bascule |
| 849 | Wrought Iron | Truss | Movable – Swing |
| 84A | Wrought Iron | Truss | Pony |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 850 | Wrought Iron | Arch | Other |
| 855 | Wrought Iron | Arch | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 860 | Wrought Iron | Girder | Other |
| 863 | Wrought Iron | Girder | Deck |
| 864 | Wrought Iron | Girder | Thru |
| 867 | Wrought Iron | Girder | Movable – Lift |
| 868 | Wrought Iron | Girder | Movable – Bascule |
| 869 | Wrought Iron | Girder | Movable - Swing |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 880 | Wrought Iron | Suspension | Other |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 890 | Wrought Iron | Culvert | Other |
| 895 | Wrought Iron | Culvert | Filled |

APPENDIX C

(Continued)

STRUCTURE TYPE CODING

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 910 | Composite | Slab | Other |
| 911 | Composite | Slab | Simple |
| 912 | Composite | Slab | Continuous |
| 913 | Composite | Slab | Deck |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 920 | Composite | Beam | Other |
| 921 | Composite | Beam | Simple |
| 922 | Composite | Beam | Continuous |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 970 | Composite | Frame | Other |
| 971 | Composite | Frame | Simple |
| 975 | Composite | Frame | Filled |

| Code | Material | Type | Description |
| --- | --- | --- | --- |
| 995 | Composite | Culvert | Filled |

## APPENDIX D

GEOGRAPHIC CODING FOR CITIES AND PLACES IN OHIO

Federal Information Processing Standards (FIPS)

ADAMS COUNTY

| CHERRY FORK | 13834 | MANCHESTER | 47012 |
| --- | --- | --- | --- |
| PEEBLES | 61420 | ROME | 68196 |
| SEAMAN | 71206 | WEST UNION | 84294 |
| WINCHESTER | 85876 | BRATTON TWP | 08350 |
| BRUSH CREEK TWP | 09722 | FRANKLIN TWP | 28196 |
| GREEN TWP | 31668 | JEFFERSON TWP | 38486 |
| LIBERTY TWP | 43036 | MANCHESTER TWP | 47026 |
| MEIGS TWP | 48888 | MONROE TWP | 51268 |
| OLIVER TWP | 58366 | SCOTT TWP | 71052 |
| SPRIGG TWP | 74069 | TIFFIN TWP | 76768 |
| WAYNE TWP | 82012 | WINCHESTER TWP | 85890 |

ALLEN CONTY

| BEAVERDAM | 04752 | BLUFFTON | 07426 |
| --- | --- | --- | --- |
| CAIRO | 10884 | DELPHOS | 21602 |
| ELIDA | 24808 | FORT SHAWNEE | 27944 |
| HARROD | 34118 | LAFAYETTE | 41118 |
| LIMA | 43554 | SPENCERVILLE | 74034 |
| AMANDA TWP | 01602 | AMERICAN TWP | 01756 |
| AUGLAIZE TWP | 02988 | BATH TWP | 04206 |
| JACKSON TWP | 37646 | MARION TWP | 47656 |
| MONROE TWP | 51282 | PERRY TWP | 61742 |
| RICHLAND TWP | 66614 | SHAWNEE TWP | 71955 |
| SPENCER TWP | 73982 | SUGAR CREEK TWP | 75199 |

ASHLAND COUNTY

| ASHLAND | 02568 | BAILEY LAKES | 03562 |
| --- | --- | --- | --- |
| HAYESVILLE | 34636 | JEROMESVILLE | 39060 |
| LOUDONVILLE | 45066 | MIFFLIN | 50036 |
| PERRYSVILLE | 62190 | POLK | 63996 |
| SAVANNAH | 70576 | CLEAR CREEK TWP | 15672 |
| GREEN TWP | 31682 | HANOVER TWP | 33236 |
| JACKSON TWP | 37660 | LAKE TWP | 41272 |
| MIFFLIN TWP | 50050 | MILTON TWP | 50610 |
| MOHICAN TWP | 51086 | MONTGOMERY TWP | 51688 |
| ORANGE TWP | 58562 | PERRY TWP | 61756 |
| RUGGLES TWP | 68966 | SULLIVAN TWP | 75357 |
| TROY TWP | 77532 | VERMILLION TWP | 79758 |

ASHTABULA COUNTY

| ANDOVER | 02050 | ASHTABULA | 02638 |
| --- | --- | --- | --- |
| CONNEAUT | 18350 | GENEVA | 29610 |
| GENEVA ON LAKE | 29652 | JEFFERSON | 38500 |
| NORTH KINGSVILLE | 56700 | ORWELL | 58856 |
| ROAMING SHORES | 67600 | ROCK CREEK | 67846 |
| ANDOVER TWP | 02064 | ASHTABULA TWP | 02652 |
| AUSTINBURG TWP | 03156 | CHERRY VALLEY TWP | 13890 |
| COLEBROOK TWP | 16560 | DENMARK TWP | 21672 |
| DORSET TWP | 22344 | GENEVA TWP | 29624 |
| HARPERSFIELD TWP | 33642 | HARTSGROVE TWP | 34300 |
| JEFFERSON TWP | 38514 | KINGSVILLE TWP | 40404 |
| LENOX TWP | 42742 | MONROE TWP | 51296 |
| MORGAN TWP | 52066 | NEW LYME TWP | 54964 |
| ORWELL TWP | 58870 | PIERPONT TWP | 62568 |
| PLYMOUTH TWP | 63772 | RICHMOND TWP | 66796 |
| ROME TWP | 68224 | SAYBROOK TWP | 70646 |
| SHEFFIELD TWP | 72053 | TRUMBULL TWP | 77686 |
| WAYNE TWP | 82040 | WILLIAMSFIELD TWP | 85372 |
| WINDSOR TWP | 85988 |  |  |

ATHENS COUNTY

| ALBANY | 01042 | AMESVILLE | 01784 |
| --- | --- | --- | --- |
| ATHENS | 02736 | BUCHTEL | 09834 |
| CHAUNCEY | 13778 | COOLVILLE | 18588 |
| GLOUSTER | 30674 | JACKSONVILLE | 38304 |
| NELSONVILLE | 53886 | TRIMBLE | 77406 |
| ALEXANDER TWP | 01112 | AMES TWP | 01770 |
| ATHENS TWP | 02750 | BERN TWP | 05942 |
| CANAAN TWP | 11220 | CARTHAGE TWP | 12336 |
| DOVER TWP | 22414 | LEE TWP | 42420 |
| LODI TWP | 44590 | ROME TWP | 68238 |
| TRIMBLE TWP | 77420 | TROY TWP | 77546 |
| WATERLOO TWP | 81777 | YORK TWP | 86982 |

AUGLAIZE COUNTY

| BUCKLAND | 09974 | CRIDERSVILLE | 19400 |
| --- | --- | --- | --- |
| MINSTER | 50918 | NEW BREMEN | 54194 |
| NEW KNOXVILLE | 54838 | ST MARYS | 69680 |
| UNIOPOLIS | 78848 | WAPAKONETA | 80766 |
| WAYNESFIELD | 82390 | CLAY TWP | 15448 |
| DUCHOUQUET TWP | 22722 | GERMAN TWP | 29848 |
| GOSHEN TWP | 30954 | JACKSON TWP | 37674 |
| LOGAN TWP | 44618 | MOULTON TWP | 52472 |
| NOBLE TWP | 55986 | PUSHETA TWP | 65018 |
| ST MARYS TWP | 69694 | SALEM TWP | 69806 |
| UNION TWP | 78204 | WASHINGTON TWP | 81074 |
| WAYNE TWP | 82054 |  |  |

BELMONT COUNTY

| BARNESVILLE | 03926 | BELLAIRE | 05074 |
| --- | --- | --- | --- |
| BELMONT | 05312 | BETHESDA | 06138 |
| BRIDGEPORT | 08560 | BROOKSIDE | 09316 |
| FAIRVIEW | 26348 | FLUSHING | 27552 |
| HOLLOWAY | 35924 | MARTINS FERRY | 48104 |
| MORRISTOWN | 52346 | POWHATAN POINT | 64542 |
| ST CLAIRSVILLE | 69526 | SHADYSIDE | 71640 |
| WILSON | 85834 | YORKVILLE | 87178 |
| COLERAIN TWP | 16602 | FLUSHING TWP | 27566 |
| GOSHEN TWP | 30968 | KIRKWOOD TWP | 40600 |
| MEAD TWP | 48580 | PEASE TWP | 61378 |
| PULTNEY TWP | 64962 | RICHLAND TWP | 66628 |
| SMITH TWP | 72736 | SOMERSET TWP | 72970 |
| UNION TWP | 78218 | WARREN TWP | 80864 |
| WASHINGTON TWP | 81088 | WAYNE TWP | 82068 |
| WHEELING TWP | 84602 | YORK TWP | 86996 |

BROWN COUNTY

| ABERDEEN | 00142 | FAYETTEVILLE | 26796 |
| --- | --- | --- | --- |
| GEORGETOWN | 29778 | HAMERSVILLE | 32984 |
| HIGGINSPORT | 35168 | MT ORAB | 52906 |
| RIPLEY | 67272 | RUSSELLVILLE | 69316 |
| ST MARTIN | 69666 | SARDINIA | 70534 |
| BYRD TWP | 10758 | CLARK TWP | 15224 |
| EAGLE TWP | 23072 | FRANKLIN TWP | 28210 |
| GREEN TWP | 31696 | HUNTINGTON TWP | 36834 |
| JACKSON TWP | 37688 | JEFFERSON TWP | 38528 |
| LEWIS TWP | 42910 | PERRY TWP | 61770 |
| PIKE TWP | 62596 | PLEASANT TWP | 63212 |
| SCOTT TWP | 71066 | STERLING TWP | 74559 |
| UNION TWP | 78232 | WASHINGTON TWP | 81102 |

BUTLER COUNTY

| COLLEGE CORNER | 16700 | FAIRFIELD | 25970 |
| --- | --- | --- | --- |
| HAMILTON | 33012 | JACKSONBURG | 38192 |
| MIDDLETOWN | 49840 | MILLVILLE | 50540 |
| MONROE | 51310 | NEW MIAMI | 55104 |
| OXFORD | 59234 | SEVEN MILE | 71444 |
| SHARONVILLE | 71892 | SOMERVILLE | 72998 |
| TRENTON | 77322 | FAIRFIELD TWP | 25984 |
| HANOVER TWP | 33250 | LEMON TWP | 42672 |
| LIBERTY TWP | 43050 | MADISON TWP | 46340 |
| MILFORD TWP | 50162 | MORGAN TWP | 52080 |
| OXFORD TWP | 59241 | REILY TWP | 66096 |
| ROSS TWP | 68616 | SAINT CLAIR TWP | 69498 |
| WAYNE TWP | 82082 | WEST CHESTER TWP | 83150 |

CARROLL COUNTY

| CARROLLTON | 12280 | DELLROY | 21560 |
| --- | --- | --- | --- |
| LEESVILLE | 42518 | MAGNOLIA | 46844 |
| MALVERN | 46998 | MINERVA | 50834 |
| SHERRODSVILLE | 72242 | AUGUSTA TWP | 03030 |
| BROWN TWP | 09400 | CENTER TWP | 12896 |
| EAST TWP | 23226 | FOX TWP | 28126 |
| HARRISON TWP | 33782 | LEE TWP | 42434 |
| LOUDON TWP | 45038 | MONROE TWP | 51324 |
| ORANGE TWP | 58576 | PERRY TWP | 61784 |
| ROSE TWP | 68420 | UNION TWP | 78260 |
| WASHINGTON TWP | 81116 |  |  |

CHAMPAIGN COUNTY

| CHRISTIANSBURG | 14296 | MECHANICSBURG | 48706 |
| --- | --- | --- | --- |
| MUTUAL | 53480 | NORTH LEWISBURG | 56770 |
| ST PARIS | 69708 | URBANA | 79072 |
| WOODSTOCK | 86478 | ADAMS TWP | 00212 |
| CONCORD TWP | 18126 | GOSHEN TWP | 30982 |
| HARRISON TWP | 33796 | JACKSON TWP | 37702 |
| JOHNSON TWP | 39228 | MAD RIVER TWP | 46760 |
| RUSH TWP | 69078 | SALEM TWP | 69820 |
| UNION TWP | 78274 | URBANA TWP | 79086 |
| WAYNE TWP | 82096 |  |  |

CLARK COUNTY

| CATAWBA | 12560 | CLIFTON | 16056 |
| --- | --- | --- | --- |
| DONNELSVILLE | 22288 | ENON | 25452 |
| LAWRENCEVILLE | 42210 | N HAMPTON | 56588 |
| NEW CARLISLE | 54334 | S CHARLESTON | 73124 |
| S VIENNA | 73796 | SPRINGFIELD | 74118 |
| TREMONT CITY | 77308 | BETHEL TWP | 06054 |
| GERMAN TWP | 29862 | GREEN TWP | 31703 |
| HARMONY TWP | 33586 | MADISON TWP | 46354 |
| MAD RIVER TWP | 46788 | MOOREFIELD TWP | 51912 |
| PIKE TWP | 62610 | PLEASANT TWP | 63226 |
| SPRINGFIELD TWP* | 74119 | CRYSTAL LAKES (CDP) | 19596 |

CLERMONT COUNTY

| AMELIA | 01742 | BATAVIA | 04150 |
| --- | --- | --- | --- |
| BETHEL | 06068 | CHILO | 14226 |
| FELICITY | 26880 | LOVELAND | 45108 |
| MILFORD | 50176 | MOSCOW | 52416 |
| NEVILLE | 53956 | NEW RICHMOND | 55384 |
| NEWTONSVILLE | 55664 | OWENSVILLE | 59220 |
| WILLIAMSBURG | 85288 | BATAVIA TWP | 04157 |
| FRANKLIN TWP | 28224 | GOSHEN TWP | 31010 |
| JACKSON TWP | 37716 | MIAMI TWP | 49322 |
| MONROE TWP | 51338 | OHIO TWP | 57960 |
| PIERCE TWP | 62540 | STONELICK TWP | 74825 |
| TATE TWP | 76155 | UNION TWP | 78288 |
| WASHINGTON TWP | 81130 | WAYNE TWP | 82110 |
| WILLIAMSBURG TWP | 85302 |  |  |

CLINTON COUNTY

| BLANCHESTER | 06908 | CLARKSVILLE | 15406 |
| --- | --- | --- | --- |
| LYNCHBURG | 45542 | MARTINSVILLE | 48118 |
| MIDLAND | 49896 | NEW VIENNA | 55748 |
| PORT WILLIAM | 64360 | SABINA | 69400 |
| WILMINGTON | 85792 | ADAMS TWP | 00226 |
| CHESTER TWP | 13974 | CLARK TWP | 15238 |
| GREEN TWP | 31710 | JEFFERSON TWP | 38542 |
| LIBERTY TWP | 43064 | MARION TWP | 47670 |
| RICHLAND TWP | 66642 | UNION TWP | 78302 |
| VERNON TWP | 79772 | WASHINGTON TWP | 81144 |
| WAYNE TWP | 82124 | WILSON TWP | 85820 |

COLUMBIANA COUNTY

| COLUMBIANA | 17036 | EAST LIVERPOOL | 23730 |
| --- | --- | --- | --- |
| EAST PALESTINE | 23940 | HANOVERTON | 33306 |
| LEETONIA | 42560 | LISBON | 44030 |
| MINERVA | 50834 | NEW WATERFORD | 55790 |
| ROGERS | 68084 | SALEM | 69834 |
| SALINEVILLE | 70100 | SUMMITVILLE | 75574 |
| WASHINGTONVILLE | 81732 | WELLSVILLE | 82740 |
| BUTLER TWP | 10562 | CENTER TWP | 12910 |
| ELKRUN TWP | 24906 | FAIRFIELD TWP | 25998 |
| FRANKLIN TWP | 28238 | HANOVER TWP | 33264 |
| KNOX TWP | 40824 | LIVERPOOL TWP | 44226 |
| MADISON TWP | 46368 | MIDDLETON TWP | 49784 |
| PERRY TWP | 61798 | SAINT CLAIR TWP | 69512 |
| SALEM TWP | 69848 | UNITY TWP | 78890 |
| WASHINGTON TWP | 81158 | WAYNE TWP | 82138 |
| WEST TWP | 82852 | YELLOW CREEK TWP | 86912 |

COSHOCTON COUNTY

| BALTIC | 03744 | CONESVILLE | 18266 |
| --- | --- | --- | --- |
| COSHOCTON | 18868 | NELLIE | 53816 |
| PLAINFIELD | 63044 | WARSAW | 81032 |
| WEST LAFAYETTE | 83608 | ADAMS TWP | 00240 |
| BEDFORD TWP | 04864 | BETHLEHEM TWP | 06152 |
| CLARK TWP | 15266 | CRAWFORD TWP | 19218 |
| FRANKLIN TWP | 28252 | JACKSON TWP | 37730 |
| JEFFERSON TWP | 38556 | KEENE TWP | 39634 |
| LAFAYETTE TWP | 41132 | LINTON TWP | 43960 |
| MILL CREEK TWP | 50274 | MONROE TWP | 51352 |
| NEWCASTLE TWP | 54376 | OXFORD TWP | 59248 |
| PERRY TWP | 61812 | PIKE TWP | 62624 |
| TIVERTON TWP | 76953 | TUSCARAWAS TWP | 77896 |
| VIRGINIA TWP | 80220 | WASHINGTON TWP | 81172 |
| WHITE EYES TWP | 84714 |  |  |

CRAWFORD COUNTY

| BUCYRUS | 10030 | CHATFIELD | 13694 |
| --- | --- | --- | --- |
| CRESTLINE | 19330 | GALION | 29162 |
| NEW WASHINGTON | 55776 | N ROBINSON | 56994 |
| TIRO | 76932 | AUBURN TWP | 02890 |
| BUCYRUS TWP | 10044 | CHATFIELD TWP | 13708 |
| CRANBERRY TWP | 19134 | DALLAS TWP | 19932 |
| HOLMES TWP | 35980 | JACKSON TWP | 37744 |
| JEFFERSON TWP | 38570 | LIBERTY TWP | 43078 |
| LYKENS TWP | 45500 | POLK TWP | 64010 |
| SANDUSKY TWP | 70366 | TEXAS TWP | 76463 |
| TOD TWP | 76967 | VERNON TWP | 79786 |
| WHETSTONE TWP | 84644 |  |  |

CUYAHOGA COUNTY

| BAY VILLAGE | 04416 | BEACHWOOD | 04500 |
| --- | --- | --- | --- |
| BEDFORD | 04878 | BEDFORD HEIGHTS | 04920 |
| BENTLEYVILLE | 05550 | BEREA | 05690 |
| BRATENAHL | 08336 | BRECKSVILLE | 08364 |
| BROADVIEW HTS | 09064 | BROOKLYN | 09246 |
| BROOKLYN HEIGHTS | 09274 | BROOK PARK | 09288 |
| CHAGRIN FALLS | 13358 | CLEVELAND | 16000 |
| CLEVELAND HEIGHTS | 16014 | CUYAHOGA HEIGHTS | 19806 |
| EAST CLEVELAND | 23380 | EUCLID | 25704 |
| FAIRVIEW PARK | 26446 | GARFIELD HEIGHTS | 29428 |
| GATES MILLS | 29498 | GLENWILLOW | 30632 |
| HIGHLAND HEIGHTS | 35252 | HIGHLAND HILLS | 35255 |
| HUNTING VALLEY | 36918 | INDEPENDENCE | 37240 |
| LAKEWOOD | 41664 | LINNDALE | 43918 |
| LYNDHURST | 45556 | MAPLE HEIGHTS | 47306 |
| MAYFIELD | 48468 | MAYFIELD HEIGHTS | 48482 |
| MIDDLEBURGH HTS | 49644 | MORELAND HILLS | 52052 |
| NEWBURGH HEIGHTS | 54250 | NORTH OLMSTED | 56882 |
| NORTH RANDALL | 56924 | NORTH ROYALTON | 57008 |
| OAKWOOD | 57750 | OLMSTED FALLS | 58422 |
| ORANGE | 58604 | PARMA | 61000 |
| PARMA HEIGHTS | 61028 | PEPPER PIKE | 61686 |
| RICHMOND HEIGHTS | 66894 | ROCKY RIVER | 68056 |
| SEVEN HILLS | 71416 | SHAKER HEIGHTS | 71682 |
| SOLON | 72928 | SOUTH EUCLID | 73264 |
| STRONGSVILLE | 75098 | UNIVERSITY HEIGHTS | 78932 |
| VALLEY VIEW | 79268 | WALTON HILLS | 80738 |
| WARRENSVILLE HTS | 80990 | WESTLAKE | 83622 |
| WOODMERE | 86394 | CHAGRIN FALLS TWP | 13372 |
| OLMSTED TWP | 58408 |  |  |

DARKE COUNTY

| ANSONIA | 02120 | ARCANUM | 02330 |
| --- | --- | --- | --- |
| BRADFORD | 08084 | BURKETTSVILLE | 10296 |
| CASTINE | 12504 | GETTYSBURG | 29974 |
| GORDON | 30912 | GREENVILLE | 32340 |
| HOLLANSBURG | 35896 | ITHACA | 37604 |
| NEW MADISON | 54978 | NEW WESTON | 55818 |
| NORTH STAR | 57064 | OSGOOD | 58912 |
| PALESTINE | 59598 | PITSBURG | 62890 |
| ROSSBURG | 68672 | UNION CITY | 78624 |
| VERSAILLES | 79912 | WAYNE LAKES | 82348 |
| YORKSHIRE | 87164 | ADAMS TWP | 00254 |
| ALLEN TWP | 01294 | BROWN TWP | 09414 |
| BUTLER TWP | 10576 | FRANKLIN TWP | 28266 |
| GREENVILLE TWP | 32354 | HARRISON TWP | 33810 |
| JACKSON TWP | 37758 | LIBERTY TWP | 43092 |
| MISSISSINAWA TWP | 50960 | MONROE TWP | 51366 |
| NEAVE TWP | 53732 | PATTERSON TWP | 61168 |
| RICHLAND TWP | 66656 | TWIN TWP | 77980 |
| VAN BUREN TWP | 79380 | WABASH TWP | 80248 |
| WASHINGTON TWP | 81186 | WAYNE TWP | 82152 |
| YORK TWP | 87010 |  |  |

DEFIANCE COUNTY

| DEFIANCE | 21308 | HICKSVILLE | 35098 |
| --- | --- | --- | --- |
| NEY | 55874 | SHERWOOD | 72256 |
| ADAMS TWP | 00268 | DEFIANCE TWP | 21322 |
| DELAWARE TWP | 21420 | FARMER TWP | 26614 |
| HICKSVILLE TWP | 35112 | HIGHLAND TWP | 35196 |
| MARK TWP | 47852 | MILFORD TWP | 50190 |
| NOBLE TWP | 56014 | RICHLAND TWP | 66670 |
| TIFFIN TWP | 76772 | WASHINGTON TWP | 81200 |

DELAWARE COUNTY

| ASHLEY | 02582 | COLUMBUS | 18000 |
| --- | --- | --- | --- |
| DELAWARE | 21434 | DUBLIN | 22694 |
| GALENA | 29148 | OSTRANDER | 58940 |
| POWELL | 64486 | SHAWNEE HILLS | 71976 |
| SUNBURY | 75602 | WESTERVILLE | 83342 |
| BERKSHIRE TWP | 05774 | BERLIN TWP | 05788 |
| BROWN TWP | 09428 | CONCORD TWP | 18140 |
| DELAWARE TWP | 21448 | GENOA TWP | 29694 |
| HARLEM TWP | 33516 | KINGSTON TWP | 40362 |
| LIBERTY TWP | 43106 | MARLBORO TWP | 47908 |
| ORANGE TWP | 58618 | OXFORD TWP | 59262 |
| PORTER TWP | 64178 | RADNOR TWP | 65312 |
| SCIOTO TWP | 70842 | THOMPSON TWP | 76617 |
| TRENTON TWP | 77336 | TROY TWP | 77560 |

ERIE COUNTY

| BAY VIEW | 04402 | BELLEVUE | 05228 |
| --- | --- | --- | --- |
| BERLIN HEIGHTS | 05900 | CASTALIA | 12476 |
| HURON | 37016 | KELLEYS ISLAND | 39662 |
| MILAN | 50134 | SANDUSKY | 70380 |
| VERMILION | 79716 | BERLIN TWP | 05802 |
| FLORENCE TWP | 27482 | GROTON TWP | 32578 |
| HURON TWP | 37030 | MARGARETTA TWP | 47572 |
| MILAN TWP | 50148 | OXFORD TWP | 59276 |
| PERKINS TWP | 61714 | VERMILION TWP | 79730 |

FAIRFIELD COUNTY

| AMANDA | 01630 | BALTIMORE | 03758 |
| --- | --- | --- | --- |
| BREMEN | 08392 | BUCKEYE LAKE | 09890 |
| CANAL WINCHESTER | 11332 | CARROLL | 12252 |
| COLUMBUS | 18000 | LANCASTER | 41720 |
| LITHOPOLIS | 44086 | MILLERSPORT | 50400 |
| PICKERINGTON | 62498 | PLEASANTVILLE | 63716 |
| REYNOLDSBURG | 66390 | RUSHVILLE | 69204 |
| STOUTSVILLE | 74916 | SUGAR GROVE | 75252 |
| THURSTON | 76764 | WEST RUSHVILLE | 84182 |
| AMANDA TWP | 01637 | BERNE TWP | 05956 |
| BLOOM TWP | 06950 | CLEAR CREEK TWP | 15686 |
| GREENFIELD TWP | 32060 | HOCKING TWP | 35812 |
| LIBERTY TWP | 43120 | MADISON TWP | 46382 |
| PLEASANT TWP | 63240 | RICHLAND TWP | 66684 |
| RUSH CREEK TWP | 69120 | VIOLET TWP | 80206 |
| WALNUT TWP | 80570 |  |  |

FAYETTE COUNTY

| BLOOMINGBURG | 07188 | JEFFERSONVILLE | 38920 |
| --- | --- | --- | --- |
| MILLEDGEVILLE | 50316 | NEW HOLLAND | 54726 |
| OCTA | 57918 | WASHINGTON CH | 81214 |
| CONCORD TWP | 18154 | GREEN TWP | 31724 |
| JASPER TWP | 38388 | JEFFERSON TWP | 38598 |
| MADISON TWP | 46396 | MARION TWP | 47684 |
| PAINT TWP | 59486 | PERRY TWP | 61826 |
| UNION TWP | 78316 | WAYNE TWP | 82166 |

FRANKLIN COUNTY

| BEXLEY | 06278 | BRICE | 08532 |
| --- | --- | --- | --- |
| CANAL WINCHESTER | 11332 | COLUMBUS | 18000 |
| DUBLIN | 22694 | GAHANNA | 29106 |
| GRANDVIEW HTS | 31304 | GROVE CITY | 32592 |
| GROVEPORT | 32606 | HARRISBURG | 33740 |
| HILLIARD | 35476 | LITHOPOLIS | 44086 |
| LOCKBOURNE | 44310 | MARBLE CLIFF | 47474 |
| MINERVA PARK | 50862 | NEW ALBANY | 53970 |
| OBETZ | 57862 | PICKERINGTON | 62498 |
| REYNOLDSBURG | 66390 | RIVERLEA | 67440 |
| UPPER ARLINGTON | 79002 | URBANCREST | 79100 |
| VALLEYVIEW | 79282 | WESTERVILLE | 83342 |
| WHITEHALL | 84742 | WORTHINGTON | 86604 |
| BLENDON TWP | 06922 | BROWN TWP | 09442 |
| CLINTON TWP | 16112 | FRANKLIN TWP | 28280 |
| HAMILTON TWP | 33026 | JACKSON TWP | 37772 |
| JEFFERSON TWP | 38612 | MADISON TWP | 46410 |
| MIFFLIN TWP | 50064 | NORWICH TWP | 57344 |
| PERRY TWP | 61840 | PLAIN TWP | 62974 |
| PLEASANT TWP | 63254 | PRAIRIE TWP | 64570 |
| SHARON TWP | 71787 | TRURO TWP | 77714 |
| WASHINGTON TWP | 81242 |  |  |

FULTON COUNTY

| ARCHBOLD | 02344 | DELTA | 21616 |
| --- | --- | --- | --- |
| FAYETTE | 26768 | LYONS | 45626 |
| METAMORA | 49238 | SWANTON | 75896 |
| WAUSEON | 81928 | AMBOY TWP | 01728 |
| CHESTERFIELD TWP | 14072 | CLINTON TWP | 16126 |
| DOVER TWP | 22442 | FRANKLIN TWP | 28294 |
| FULTON TWP | 29036 | GERMAN TWP | 29876 |
| GORHAM TWP | 30940 | PIKE TWP | 62638 |
| ROYALTON TWP | 68896 | SWAN CREEK TWP | 75861 |
| YORK TWP | 87024 |  |  |

GALLIA COUNTY

| CENTERVILLE | 13169 | CHESHIRE | 13932 |
| --- | --- | --- | --- |
| CROWN CITY | 19554 | GALLIPOLIS | 29204 |
| RIO GRANDE | 67258 | VINTON | 80178 |
| ADDISON TWP | 00422 | CHESHIRE TWP | 13946 |
| CLAY TWP | 15462 | GALLIPOLIS TWP | 29218 |
| GREEN TWP | 31738 | GREENFIELD TWP | 32074 |
| GUYAN TWP | 32760 | HARRISON TWP | 33824 |
| HUNTINGTON TWP | 36848 | MORGAN TWP | 52094 |
| OHIO TWP | 57974 | PERRY TWP | 61854 |
| RACCOON TWP | 65228 | SPRINGFIELD TWP | 74120 |
| WALNUT TWP | 80584 |  |  |

GEAUGA COUNTY

| AQUILLA | 02274 | BURTON | 10436 |
| --- | --- | --- | --- |
| CHARDON | 13554 | HUNTING VALLEY | 36918 |
| MIDDLEFIELD | 49700 | S RUSSELL | 73684 |
| AUBURN TWP | 02904 | BAINBRIDGE TWP | 03590 |
| BURTON TWP | 10464 | CHARDON TWP | 13561 |
| CHESTER TWP | 13988 | CLARIDON TWP | 15168 |
| HAMBDEN TWP | 32914 | HUNTSBURG TWP | 36946 |
| MIDDLEFIELD TWP | 49714 | MONTVILLE TWP | 51842 |
| MUNSON TWP | 53340 | NEWBURY TWP | 54292 |
| PARKMAN TWP | 59948 | RUSSELL TWP | 69232 |
| THOMPSON TWP | 76628 | TROY TWP | 77574 |

GREENE COUNTY

| BEAVERCREEK | 04720 | BELLBROOK | 05102 |
| --- | --- | --- | --- |
| BOWERSVILLE | 07930 | CEDARVILLE | 12784 |
| CENTERVILLE | 13190 | CLIFTON | 16056 |
| FAIRBORN | 25914 | HUBER HEIGHTS | 36610 |
| JAMESTOWN | 38374 | KETTERING | 40040 |
| SPRING VALLEY | 74216 | XENIA | 86772 |
| YELLOW SPRINGS | 86940 | BATH TWP | 04220 |
| BEAVERCREEK TWP | 04724 | CAESARCREEK TWP | 10856 |
| CEDARVILLE TWP | 12798 | JEFFERSON TWP | 38626 |
| MIAMI TWP | 49336 | NEW JASPER TWP | 54810 |
| ROSS TWP | 68630 | SILVERCREEK TWP | 72473 |
| SPRING VALLEY TWP | 74223 | SUGARCREEK TWP | 75201 |
| XENIA TWP | 86786 |  |  |

GUERNSEY COUNTY

| BYESVILLE | 10716 | CAMBRIDGE | 10996 |
| --- | --- | --- | --- |
| CUMBERLAND | 19694 | FAIRVIEW | 26348 |
| KIMBOLTON | 40264 | LORE CITY | 44954 |
| OLD WASHINGTON | 58226 | PLEASANT CITY | 63436 |
| QUAKER CITY | 65116 | SALESVILLE | 70072 |
| SENECAVILLE | 71360 | ADAMS TWP | 00282 |
| CAMBRIDGE TWP | 11003 | CENTER TWP | 12938 |
| JACKSON TWP | 37786 | JEFFERSON TWP | 38640 |
| KNOX TWP | 40838 | LIBERTY TWP | 43134 |
| LONDONDERRY TWP | 44716 | MADISON TWP | 46424 |
| MILLWOOD TWP | 50568 | MONROE TWP | 51380 |
| OXFORD TWP | 59290 | RICHLAND TWP | 66698 |
| SPENCER TWP | 73986 | VALLEY TWP | 79156 |
| WASHINGTON TWP | 81256 | WESTLAND TWP | 83664 |
| WHEELING TWP | 84616 | WILLS TWP | 85708 |

HAMILTON COUNTY

| ADDYSTON | 00436 | AMBERLEY | 01672 |
| --- | --- | --- | --- |
| ARLINGTON HTS | 02428 | BLUE ASH | 07300 |
| CHEVIOT | 14128 | CINCINNATI | 15000 |
| CLEVES | 16028 | DEER PARK | 21266 |
| ELMWOOD PLACE | 25186 | EVENDALE | 25802 |
| FAIRFAX | 25942 | FAIRFIELD | 25970 |
| FOREST PARK | 27706 | GLENDALE | 30380 |
| GOLF MANOR | 30786 | GREENHILLS | 32158 |
| HARRISON | 33838 | INDIAN HILL | 76582 |
| LINCOLN HEIGHTS | 43722 | LOCKLAND | 44366 |
| LOVELAND | 45108 | MADEIRA | 46312 |
| MARIEMONT | 47600 | MILFORD | 50176 |
| MONTGOMERY | 51716 | MT HEALTHY | 52752 |
| N COLLEGE HILL | 56322 | NEWTOWN | 55678 |
| NORTH BEND | 56182 | NORWOOD | 57386 |
| READING | 65732 | SHARONVILLE | 71892 |
| SILVERTON | 72522 | SPRINGDALE | 74104 |
| ST BERNARD | 69470 | TERRACE PARK | 76428 |
| WOODLAWN | 86366 | WYOMING | 86730 |
| ANDERSON TWP | 01980 | COLERAIN TWP | 16616 |
| COLUMBIA TWP | 16882 | CROSBY TWP | 19470 |
| DELHI TWP | 21504 | GREEN TWP | 31752 |
| HARRISON TWP | 33852 | MIAMI TWP | 49364 |
| SPRINGFIELD TWP | 74121 | SYCAMORE TWP | 75973 |
| SYMMES TWP | 76028 | WHITEWATER TWP | 84938 |

HANCOCK COUNTY

| ARCADIA | 02316 | ARLINGTON | 02400 |
| --- | --- | --- | --- |
| BENTON RIDGE | 05662 | BLUFFTON | 07426 |
| FINDLAY | 27048 | FOSTORIA | 28014 |
| JENERA | 38948 | MCCOMB | 45808 |
| MT BLANCHARD | 52598 | MT CORY | 52668 |
| RAWSON | 65634 | VAN BUREN | 79394 |
| VANLUE | 79534 | ALLEN TWP | 01308 |
| AMANDA TWP | 01644 | BIGLICK TWP | 06362 |
| BLANCHARD TWP | 06838 | CASS TWP | 12392 |
| DELAWARE TWP | 21462 | EAGLE TWP | 23086 |
| JACKSON TWP | 37800 | LIBERTY TWP | 43148 |
| MADISON TWP | 46438 | MARION TWP | 47698 |
| ORANGE TWP | 58632 | PLEASANT TWP | 63268 |
| PORTAGE TWP | 64066 | UNION TWP | 78330 |
| VAN BUREN TWP | 79408 | WASHINGTON TWP | 81284 |

HARDIN COUNTY

| ADA | 00198 | ALGER | 01210 |
| --- | --- | --- | --- |
| DUNKIRK | 22946 | FOREST | 27636 |
| HALE | 32837 | KENTON | 39886 |
| MCGUFFEY | 46046 | MT VICTORY | 53144 |
| PATTERSON | 61182 | RIDGEWAY | 67118 |
| BLANCHARD TWP | 06866 | BUCK TWP | 09848 |
| CESSNA TWP | 13316 | DUDLEY TWP | 22736 |
| GOSHEN TWP | 31024 | HALE TWP | 32837 |
| JACKSON TWP | 37814 | LIBERTY TWP | 43162 |
| LYNN TWP | 45598 | MARION TWP | 47712 |
| MCDONALD TWP | 45920 | PLEASANT TWP | 63282 |
| ROUNDHEAD TWP | 68784 | TAYLOR CREEK TWP | 76198 |
| WASHINGTON TWP | 81298 |  |  |

HARRISON COUNTY

| ADENA | 00464 | BOWERSTON | 07916 |
| --- | --- | --- | --- |
| CADIZ | 10800 | DEERSVILLE | 21294 |
| FREEPORT | 28798 | HARRISVILLE | 34090 |
| HOPEDALE | 36260 | JEWETT | 39172 |
| NEW ATHENS | 54068 | SCIO | 70814 |
| ARCHER TWP | 02358 | ATHENS TWP | 02764 |
| CADIZ TWP | 10814 | FRANKLIN TWP | 28308 |
| FREEPORT TWP | 28812 | GERMAN TWP | 29890 |
| GREEN TWP | 31766 | MONORE TWP | 51394 |
| MOOREFIELD TWP | 51940 | NORTH TWP | 56084 |
| NOTTINGHAM TWP | 57456 | RUMLEY TWP | 68994 |
| SHORT CREEK TWP | 72361 | STOCK TWP | 74724 |
| WASHINGTON TWP | 81312 |  |  |

HENRY COUNTY

| DESHLER | 21812 | FLORIDA | 27538 |
| --- | --- | --- | --- |
| HAMLER | 33096 | HOLGATE | 35854 |
| LIBERTY CENTER | 43414 | MALINTA | 46942 |
| MCCLURE | 45794 | NAPOLEON | 53550 |
| NEW BAVARIA | 54110 | BARTLOW TWP | 04052 |
| DAMASCUS TWP | 20016 | FLATROCK TWP | 27342 |
| FREEDOM TWP | 28700 | HARRISON TWP | 33866 |
| LIBERTY TWP | 43176 | MARION TWP | 47726 |
| MONROE TWP | 51408 | NAPOLEON TWP | 53564 |
| PLEASANT TWP | 63296 | RICHFIELD TWP | 66502 |
| RIDGEVILLE TWP | 67062 | WASHINGTON TWP | 81326 |

HIGHLAND COUNTY

| GREENFIELD | 32088 | HIGHLAND | 35210 |
| --- | --- | --- | --- |
| HILLSBORO | 35560 | LEESBURG | 42476 |
| LYNCHBURG | 45542 | MOWRYSTOWN | 53186 |
| SARDINIA | 70534 | SINKING SPRING | 72578 |
| BRUSH CREEK TWP | 09736 | CLAY TWP | 15476 |
| CONCORD TWP | 18168 | DODSON TWP | 22204 |
| FAIRFIELD TWP | 26026 | HAMER TWP | 32970 |
| JACKSON TWP | 37828 | LIBERTY TWP | 43190 |
| MADISON TWP | 46452 | MARSHALL TWP | 48034 |
| NEW MARKET TWP | 55048 | PAINT TWP | 59500 |
| PENN TWP | 61588 | SALEM TWP | 69862 |
| UNION TWP | 78344 | WASHINGTON TWP | 81340 |
| WHITE OAK TWP | 84826 |  |  |

HOCKING COUNTY

| BUCHTEL | 09834 | LAURELVILLE | 42084 |
| --- | --- | --- | --- |
| LOGAN | 44632 | MURRAY CITY | 53410 |
| BENTON TWP | 05578 | FALLS TWP | 26488 |
| GOOD HOPE TWP | 30842 | GREEN TWP | 31780 |
| LAUREL TWP | 42056 | MARION TWP | 47740 |
| PERRY TWP | 61868 | SALT CREEK TWP | 70142 |
| STARR TWP | 74405 | WARD TWP | 80780 |
| WASHINGTON TWP | 81354 |  |  |

HOLMES COUNTY

| BALTIC | 03744 | GLENMONT | 30506 |
| --- | --- | --- | --- |
| HOLMESVILLE | 35994 | KILLBUCK | 40180 |
| LOUDONVILLE | 45066 | MILLERSBURG | 50372 |
| NASHVILLE | 53634 | BERLIN TWP | 05830 |
| CLARK TWP | 15280 | HARDY TWP | 33460 |
| KILLBUCK TWP | 40194 | KNOX TWP | 40852 |
| MECHANIC TWP | 48692 | MONROE TWP | 51422 |
| PAINT TWP | 59514 | PRAIRIE TWP | 64584 |
| RICHLAND TWP | 66712 | RIPLEY TWP | 67286 |
| SALT CREEK TWP | 70156 | WALNUT CREEK TWP | 80626 |
| WASHINGTON TWP | 81368 |  |  |

HURON COUNTY

| BELLEVUE | 05228 | GREENWICH | 32368 |
| --- | --- | --- | --- |
| MILAN | 50134 | MONROEVILLE | 51618 |
| N FAIRFIELD | 56420 | NEW LONDON | 54908 |
| NORWALK | 57302 | PLYMOUTH | 63800 |
| WAKEMAN | 80458 | WILLARD | 85232 |
| BRONSON TWP | 09148 | CLARKSFIELD TWP | 15364 |
| FAIRFIELD TWP | 26040 | FITCHVILLE TWP | 27216 |
| GREENFIELD TWP | 32102 | GREENWICH TWP | 32382 |
| HARTLAND TWP | 34258 | LYME TWP | 45514 |
| NEW HAVEN TWP | 54712 | NEW LONDON TWP | 54922 |
| NORWALK TWP | 57316 | NORWICH TWP | 57358 |
| PERU TWP | 62246 | RICHMOND TWP | 66810 |
| RIDGEFIELD TWP | 67006 | RIPLEY TWP | 67300 |
| SHERMAN TWP | 72193 | TOWNSEND TWP | 77158 |
| WAKEMAN TWP | 80472 |  |  |

JACKSON COUNTY

| COALTON | 16434 | JACKSON | 37842 |
| --- | --- | --- | --- |
| OAK HILL | 57596 | WELLSTON | 82712 |
| BLOOMFIELD TWP | 07090 | COAL TWP | 16336 |
| FRANKLIN TWP | 28322 | HAMILTON TWP | 33040 |
| JACKSON TWP | 37856 | JEFFERSON TWP | 38654 |
| LIBERTY TWP | 43204 | LICK TWP | 43442 |
| MADISON TWP | 46466 | MILTON TWP | 50624 |
| SCIOTO TWP | 70856 | WASHINGTON TWP | 81382 |

JEFFERSON COUNTY

| ADENA | 00464 | AMSTERDAM | 01938 |
| --- | --- | --- | --- |
| BERGHOLZ | 05718 | BLOOMINGDALE | 07202 |
| DILLONVALE | 22022 | EMPIRE | 25368 |
| IRONDALE | 37422 | MINGO JUNCTION | 50904 |
| MT PLEASANT | 52976 | NEW ALEXANDRIA | 54012 |
| RAYLAND | 65662 | RICHMOND | 66824 |
| SALINEVILLE | 70100 | SMITHFIELD | 72760 |
| STEUBENVILLE | 74608 | STRATTON | 75000 |
| TILTONSVILLE | 76806 | TORONTO | 77112 |
| WINTERSVILLE | 86184 | YORKVILLE | 87178 |
| BRUSH CREEK TWP | 09750 | CROSS CREEK TWP | 19484 |
| ISLAND CREEK TWP | 37534 | KNOX TWP | 40866 |
| MOUNT PLEASANT TWP | 52990 | ROSS TWP | 68644 |
| SALEM TWP | 69876 | SALINE TWP | 70086 |
| SMITHFIELD TWP | 72767 | SPRINGFIELD TWP | 74122 |
| STEUBENVILLE TWP | 74615 | WARREN TWP | 80878 |
| WAYNE TWP | 82180 | WELLS TWP | 82698 |

KNOX COUNTY

| CENTERBURG | 13036 | DANVILLE | 20114 |
| --- | --- | --- | --- |
| FREDERICKTOWN | 28658 | GAMBIER | 29246 |
| GANN | 29288 | MARTINSBURG | 48090 |
| MT VERNON | 53102 | UTICA | 79114 |
| BERLIN TWP | 05844 | BROWN TWP | 09470 |
| BUTLER TWP | 10590 | CLAY TWP | 15504 |
| CLINTON TWP | 16140 | COLLEGE TWP | 16686 |
| HARRISON TWP | 33880 | HILLIAR TWP | 35462 |
| HOWARD TWP | 36526 | JACKSON TWP | 37870 |
| JEFFERSON TWP | 38668 | LIBERTY TWP | 43218 |
| MIDDLEBURY TWP | 49658 | MILFORD TWP | 50204 |
| MILLER TWP | 50330 | MONROE TWP | 51436 |
| MORGAN TWP | 52108 | MORRIS TWP | 52290 |
| PIKE TWP | 62652 | PLEASANT TWP | 63310 |
| UNION TWP | 78358 | WAYNE TWP | 82194 |

LAKE COUNTY

| EASTLAKE | 23618 | FAIRPORT HARBOR | 26306 |
| --- | --- | --- | --- |
| GRAND RIVER | 31234 | KIRTLAND | 40642 |
| KIRTLAND HILLS | 40670 | LAKELINE | 41398 |
| MADISON | 46480 | MENTOR | 49056 |
| MENTOR ON LAKE | 49098 | NORTH PERRY | 56910 |
| PAINESVILLE | 59416 | PERRY | 61882 |
| TIMBERLAKE | 76834 | WAITE HILL | 80402 |
| WICKLIFFE | 85036 | WILLOUGHBY | 85484 |
| WILLOUGHBY HILLS | 85512 | WILLOWICK | 85638 |
| CONCORD TWP | 18196 | LEROY TWP | 42812 |
| MADISON TWP | 46494 | PAINESVILLE TWP | 59430 |
| PERRY TWP | 61896 | NORTH MADISON (CDP) | 56812 |

LAWRENCE COUNTY

| ATHALIA | 02722 | CHESAPEAKE | 13904 |
| --- | --- | --- | --- |
| COAL GROVE | 16378 | HANGING ROCK | 33194 |
| IRONTON | 37464 | PROCTORVILLE | 64766 |
| SOUTH POINT | 73670 | AID TWP | 00562 |
| DECATUR TWP | 21084 | ELIZABETH TWP | 24822 |
| FAYETTE TWP | 26782 | HAMILTON TWP | 33054 |
| LAWRENCE TWP | 42154 | MASON TWP | 48174 |
| PERRY TWP | 61910 | ROME TWP | 68280 |
| SYMMES TWP | 76031 | UNION TWP | 78372 |
| UPPER TWP | 78974 | WASHINGTON TWP | 81396 |
| WINDSOR TWP | 86002 |  |  |

LICKING COUNTY

| ALEXANDRIA | 01154 | BUCKEYE LAKE | 09890 |
| --- | --- | --- | --- |
| GRANVILLE | 31402 | GRATIOT | 31458 |
| HANOVER | 33292 | HARTFORD | 34202 |
| HEATH | 34748 | HEBRON | 34790 |
| JOHNSTOWN | 39340 | KIRKERSVILLE | 40572 |
| NEW ALBANY | 53970 | NEWARK | 54040 |
| PATASKALA | 61112 | REYNOLDSBURG | 66390 |
| ST LOUISVILLE | 69652 | UTICA | 79114 |
| BENNINGTON TWP | 05494 | BOWLING GREEN TWP | 07944 |
| BURLINGTON TWP | 10366 | EDEN TWP | 24332 |
| ETNA TWP | 25690 | FALLSBURY TWP | 26530 |
| FRANKLIN TWP | 28336 | GRANVILLE TWP | 31416 |
| HANOVER TWP | 33299 | HARRISON TWP | 33894 |
| HARTFORD TWP | 34188 | HOPEWELL TWP | 36316 |
| JERSEY TWP | 39102 | LIBERTY TWP | 43232 |
| LICKING TWP | 43456 | MADISON TWP | 46508 |
| MARY ANN TWP | 48132 | MCKEAN TWP | 46116 |
| MONROE TWP | 51450 | NEWARK TWP | 54054 |
| NEWTON TWP | 55580 | PERRY TWP | 61924 |
| SAINT ALBANS TWP | 69456 | UNION TWP | 78400 |
| WASHINGTON TWP | 81410 |  |  |

LOGAN COUNTY

| BELLE CENTER | 05116 | BELLEFONTAINE | 05130 |
| --- | --- | --- | --- |
| DEGRAFF | 21378 | HUNTSVILLE | 36988 |
| LAKEVIEW | 41608 | QUINCY | 65200 |
| RIDGEWAY | 67118 | RUSHSYLVANIA | 69176 |
| RUSSELLS POINT | 69302 | VALLEY HI | 79226 |
| W LIBERTY | 83734 | W MANSFIELD | 83818 |
| ZANESFIELD | 88070 | BLOOMFIELD TWP | 07118 |
| BOKES CREEK TWP | 07552 | HARRISON TWP | 33908 |
| JEFFERSON TWP | 38682 | LAKE TWP | 41286 |
| LIBERTY TWP | 43246 | MCARTHUR TWP | 45682 |
| MIAMI TWP | 49378 | MONROE TWP | 51464 |
| PERRY TWP | 61938 | PLEASANT TWP | 63324 |
| RICHLAND TWP | 66740 | RUSHCREEK TWP | 69134 |
| STOKES TWP | 74780 | UNION TWP | 78414 |
| WASHINGTON TWP | 81424 | ZANE TWP | 88042 |

LORAIN COUNTY

| AMHERST | 01798 | AVON | 03352 |
| --- | --- | --- | --- |
| AVON LAKE | 03464 | ELYRIA | 25256 |
| GRAFTON | 31150 | KIPTON | 40544 |
| LAGRANGE | 41230 | LORAIN | 44856 |
| N RIDGEVILLE | 56966 | OBERLIN | 57834 |
| ROCHESTER | 67762 | S AMHERST | 73040 |
| SHEFFIELD | 72060 | SHEFFIELD LAKE | 72088 |
| VERMILION | 79716 | WELLINGTON | 82642 |
| AMHERST TWP | 01812 | BRIGHTON TWP | 08770 |
| BROWNHELM TWP | 09568 | CAMDEN TWP | 11010 |
| CARLISLE TWP | 12140 | COLUMBIA TWP | 16910 |
| EATON TWP | 24220 | ELYRIA TWP | 25270 |
| GRAFTON TWP | 31164 | HENRIETTA TWP | 34972 |
| HUNTINGTON TWP | 36876 | LAGRANGE TWP | 41244 |
| NEW RUSSIA TWP | 55446 | PENFIELD TWP | 61532 |
| PITTSFIELD TWP | 62960 | ROCHESTER TWP | 67776 |
| SHEFFIELD TWP | 72067 | WELLINGTON TWP | 82656 |

LUCAS COUNTY

| BERKEY | 05732 | HARBOR VIEW | 33376 |
| --- | --- | --- | --- |
| HOLLAND | 35882 | MAUMEE | 48342 |
| OREGON | 58730 | OTTAWA HILLS | 59010 |
| SWANTON | 75896 | SYLVANIA | 76022 |
| TOLEDO | 77000 | WATERVILLE | 81858 |
| WHITEHOUSE | 84770 | HARDING TWP | 33418 |
| JERUSALEM TWP | 39116 | MONCLOVA TWP | 51156 |
| PROVIDENCE TWP | 64836 | RICHFIELD TWP | 66516 |
| SPENCER TWP | 73990 | SPRINGFIELD TWP | 74123 |
| SWANTON TWP | 75903 | SYLVANIA TWP | 76025 |
| WASHINGTON TWP | 81438 | WATERVILLE TWP | 81872 |

MADISON COUNTY

| LONDON | 44674 | MIDWAY | 50008 |
| --- | --- | --- | --- |
| MT STERLING | 53046 | PLAIN CITY | 63030 |
| SOUTH SOLON | 73768 | WEST JEFFERSON | 83580 |
| CANAAN TWP | 11234 | DARBY TWP | 20142 |
| DEER CREEK TWP | 21154 | FAIRFIELD TWP | 26068 |
| JEFFERSON TWP | 38696 | MONROE TWP | 51478 |
| OAK RUN TWP | 57708 | PAINT TWP | 59528 |
| PIKE TWP | 62666 | PLEASANT TWP | 63338 |
| RANGE TWP | 65480 | SOMERFORD TWP | 72960 |
| STOKES TWP | 74784 | UNION TWP | 78428 |

MAHONING COUNTY

| ALLIANCE | 01420 | BELOIT | 05410 |
| --- | --- | --- | --- |
| CAMPBELL | 11066 | CANFIELD | 11360 |
| COLUMBIANA | 17036 | CRAIG BEACH | 19106 |
| LOWELLVILLE | 45178 | NEW MIDDLETOWN | 55118 |
| POLAND | 63954 | SALEM | 69834 |
| SEBRING | 71220 | STRUTHERS | 75126 |
| WASHINGTONVILLE | 81732 | YOUNGSTOWN | 88000 |
| AUSTINTOWN TWP | 03198 | BEAVER TWP | 04668 |
| BERLIN TWP | 05858 | BOARDMAN TWP | 07468 |
| CANFIELD TWP | 11374 | COITSVILLE TWP | 16476 |
| ELLSWORTH TWP | 25088 | GOSHEN TWP | 31038 |
| GREEN TWP | 31794 | JACKSON TWP | 37884 |
| MILTON TWP | 50638 | POLAND TWP | 63968 |
| SMITH TWP | 72740 | SPRINGFIELD TWP | 74124 |
| AUSTINTOWN (CDP) | 03184 | BOARDMAN (CDP) | 07454 |

MARION COUNTY

| CALEDONIA | 10954 | GREEN CAMP | 31948 |
| --- | --- | --- | --- |
| LA RUE | 41902 | MARION | 47754 |
| MORRAL | 52276 | NEW BLOOMINGTON | 54152 |
| PROSPECT | 64780 | WALDO | 80500 |
| BIG ISLAND TWP | 06348 | BOWLING GREEN TWP | 07958 |
| CLARIDON TWP | 15196 | GRAND PRAIRIE TWP | 31192 |
| GRAND TWP | 31178 | GREEN CAMP TWP | 31962 |
| MARION TWP | 47768 | MONTGOMERY TWP | 51730 |
| PLEASANT TWP | 63352 | PROSPECT TWP | 64794 |
| RICHLAND TWP | 66754 | SALT ROCK TWP | 70240 |
| SCOTT TWP | 71073 | TULLY TWP | 77742 |
| WALDO TWP | 80514 |  |  |

MEDINA COUNTY

| BRUNSWICK | 09680 | CHIPPEWA LAKE | 14282 |
| --- | --- | --- | --- |
| CRESTON | 19344 | GLORIA GLENS PARK | 30660 |
| LODI | 44604 | MEDINA | 48790 |
| RITTMAN | 67356 | SEVILLE | 71486 |
| SPENCER | 73992 | WADSWORTH | 80304 |
| WESTFIELD CENTER | 83468 | BRUNSWICK HILLS TWP | 09708 |
| CHATHAM TWP | 13750 | GRANGER TWP | 31374 |
| GUILFORD TWP | 32676 | HARRISVILLE TWP | 34104 |
| HINCKLEY TWP | 35644 | HOMER TWP | 36078 |
| LAFAYETTE TWP | 41174 | LITCHFIELD TWP | 44072 |
| LIVERPOOL TWP | 44240 | MEDINA TWP | 48804 |
| MONTVILLE TWP | 51856 | SHARON TWP | 71801 |
| SPENCER TWP | 73999 | WADSWORTH TWP | 80318 |
| WESTFIELD TWP | 83426 | YORK TWP | 87052 |

MEIGS COUNTY

| MIDDLEPORT | 49756 | POMEROY | 64024 |
| --- | --- | --- | --- |
| RACINE | 65256 | RUTLAND | 69358 |
| SYRACUSE | 76050 | BEDFORD TWP | 04906 |
| CHESTER TWP | 14016 | COLUMBIA TWP | 16924 |
| LEBANON TWP | 42336 | LETART TWP | 42868 |
| OLIVE TWP | 58282 | ORANGE TWP | 58646 |
| RUTLAND TWP | 69372 | SALEM TWP | 69904 |
| SALISBURY TWP | 70114 | SCIPIO TWP | 70996 |
| SUTTON TWP | 75847 |  |  |

MERCER COUNTY

| BURKETTSVILLE | 10296 | CELINA | 12868 |
| --- | --- | --- | --- |
| CHICKASAW | 14156 | COLDWATER | 16532 |
| FT RECOVERY | 27902 | MENDON | 49042 |
| MONTEZUMA | 51674 | ROCKFORD | 67874 |
| ST HENRY | 69540 | BLACK CREEK TWP | 06586 |
| BUTLER TWP | 10604 | CENTER TWP | 12952 |
| DUBLIN TWP | 22708 | FRANKLIN TWP | 28350 |
| GIBSON TWP | 30058 | GRANVILLE TWP | 31430 |
| HOPEWELL TWP | 36344 | JEFFERSON TWP | 38724 |
| LIBERTY TWP | 43260 | MARION TWP | 47782 |
| RECOVERY TWP | 65781 | UNION TWP | 78442 |
| WASHINGTON TWP | 81452 |  |  |

MIAMI COUNTY

| BRADFORD | 08084 | CASSTOWN | 12462 |
| --- | --- | --- | --- |
| COVINGTON | 19050 | FLETCHER | 27412 |
| HUBER HEIGHTS | 36610 | LAURA | 42028 |
| LUDLOW FALLS | 45374 | PIQUA | 62848 |
| PLEASANT HILL | 63534 | POTSDAM | 64430 |
| TIPP CITY | 76876 | TROY | 77588 |
| WEST MILTON | 83902 | UNION | 78470 |
| BETHEL TWP | 06110 | BROWN TWP | 09498 |
| CONCORD TWP | 18224 | ELIZABETH TWP | 24836 |
| LOST CREEK TWP | 44968 | MONROE TWP | 51492 |
| NEWBERRY TWP | 54138 | NEWTON TWP | 55594 |
| SPRING CREEK TWP | 74097 | STAUNTON TWP | 74475 |
| UNION TWP | 78456 | WASHINGTON TWP | 81466 |

MONROE COUNTY

| ANTIOCH | 02148 | BEALLSVILLE | 04542 |
| --- | --- | --- | --- |
| CLARINGTON | 15210 | GRAYSVILLE | 31542 |
| JERUSALEM | 39130 | LEWISVILLE | 42980 |
| MILTONSBURG | 50722 | STAFFORD | 74300 |
| WILSON | 85834 | WOODSFIELD | 86436 |
| ADAMS TWP | 00296 | BENTON TWP | 05606 |
| BETHEL TWP | 06124 | CENTER TWP | 12966 |
| FRANKLIN TWP | 28357 | GREEN TWP | 31808 |
| JACKSON TWP | 37898 | LEE TWP | 42448 |
| MALAGA TWP | 46928 | OHIO TWP | 57988 |
| PERRY TWP | 61952 | SALEM TWP | 69918 |
| SENECA TWP | 71349 | SUMMIT TWP | 75497 |
| SUNSBURY TWP | 75686 | SWITZERLAND TWP | 75945 |
| WASHINGTON TWP | 81480 | WAYNE TWP | 82208 |

MONTGOMERY COUNTY

| BROOKVILLE | 09358 | CARLISLE | 12168 |
| --- | --- | --- | --- |
| CENTERVILLE | 13190 | CLAYTON | 15644 |
| DAYTON | 21000 | ENGLEWOOD | 25396 |
| FARMERSVILLE | 26656 | GERMANTOWN | 29932 |
| HUBER HEIGHTS | 36610 | KETTERING | 40040 |
| MIAMISBURG | 49434 | MORAINE | 52010 |
| NEW LEBANON | 54852 | OAKWOOD | 57764 |
| PHILLIPSBURG | 62414 | RIVERSIDE | 67468 |
| SPRINGBORO | 74076 | TROTWOOD | 77504 |
| UNION | 78470 | VANDALIA | 79492 |
| VERONA | 79898 | W CARROLLTON | 83111 |
| BUTLER TWP | 10618 | CLAY TWP | 15518 |
| GERMAN TWP | 29904 | HARRISON TWP | 33922 |
| JACKSON TWP | 37912 | JEFFERSON TWP | 38738 |
| MIAMI TWP | 49392 | PERRY TWP | 61966 |
| WASHINGTON TWP | 81494 |  |  |

MORGAN COUNTY

| CHESTERHILL | 14086 | MALTA | 46970 |
| --- | --- | --- | --- |
| MCCONNELSVILLE | 45822 | STOCKPORT | 74748 |
| BLOOM TWP | 06964 | BRISTOL TWP | 08910 |
| CENTER TWP | 12980 | DEERFIELD TWP | 21182 |
| HOMER TWP | 36092 | MALTA TWP | 46984 |
| MANCHESTER TWP | 47040 | MARION TWP | 47796 |
| MEIGSVILLE TWP | 48930 | MORGAN TWP | 52122 |
| PENN TWP | 61602 | UNION TWP | 78484 |
| WINDSOR TWP | 86016 | YORK TWP | 87066 |

MORROW COUNTY

| CARDINGTON | 12084 | CHESTERVILLE | 14114 |
| --- | --- | --- | --- |
| EDISON | 24598 | FULTON | 29050 |
| MARENGO | 47558 | MT GILEAD | 52738 |
| SPARTA | 73950 | BENNINGTON TWP | 05508 |
| CANAAN TWP | 11248 | CARDINGTON TWP | 12098 |
| CHESTER TWP | 14030 | CONGRESS TWP | 18294 |
| FRANKLIN TWP | 28378 | GILEAD TWP | 30128 |
| HARMONY TWP | 33600 | LINCOLN TWP | 43680 |
| NORTH BLOOMFLD TWP | 56224 | PERRY TWP | 61980 |
| PERU TWP | 62260 | SOUTH BLOOMFLD TWP | 73061 |
| TROY TWP | 77602 | WASHINGTON TWP | 81508 |
| WESTFIELD TWP | 83454 |  |  |

MUSKINGUM COUNTY

| ADAMSVILLE | 00380 | DRESDEN | 22610 |
| --- | --- | --- | --- |
| FRAZEYSBURG | 28574 | FULTONHAM | 29064 |
| GRATIOT | 31458 | NEW CONCORD | 54446 |
| NORWICH | 57372 | PHILO | 62442 |
| ROSEVILLE | 68560 | S ZANESVILLE | 73894 |
| ZANESVILLE | 88084 | ADAMS TWP | 00310 |
| BLUE ROCK TWP | 07398 | BRUSH CREEK TWP | 09757 |
| CASS TWP | 12406 | CLAY TWP | 15532 |
| FALLS TWP | 26502 | HARRISON TWP | 33936 |
| HIGHLAND TWP | 35238 | HOPEWELL TWP | 36372 |
| JACKSON TWP | 37926 | JEFFERSON TWP | 38752 |
| LICKING TWP | 43470 | MADISON TWP | 46536 |
| MEIGS TWP | 48916 | MONORE TWP | 51506 |
| MUSKINGUM TWP | 53438 | NEWTON TWP | 55608 |
| PERRY TWP | 61994 | RICH HILL TWP | 66600 |
| SALEM TWP | 69932 | SALT CREEK TWP | 70170 |
| SPRINGFIELD TWP | 74125 | UNION TWP | 78498 |
| WASHINGTON TWP | 81522 | WAYNE TWP | 82236 |

NOBLE COUNTY

| BATESVILLE | 04192 | BELLE VALLEY | 05158 |
| --- | --- | --- | --- |
| CALDWELL | 10940 | DEXTER CITY | 21910 |
| SARAHSVILLE | 70520 | SUMMERFIELD | 75406 |
| BEAVER TWP | 04682 | BROOKFIELD TWP | 09162 |
| BUFFALO TWP | 10142 | CENTER TWP | 12994 |
| ELK TWP | 24878 | ENOCH TWP | 25438 |
| JACKSON TWP | 37940 | JEFFERSON TWP | 38766 |
| MARION TWP | 47810 | NOBLE TWP | 56028 |
| OLIVE TWP | 58310 | SENECA TWP | 71352 |
| SHARON TWP | 71826 | STOCK TWP | 74728 |
| WAYNE TWP | 82250 |  |  |

OTTAWA COUNTY

| CLAY CENTER | 15588 | ELMORE | 25144 |
| --- | --- | --- | --- |
| GENOA | 29722 | MARBLEHEAD | 47502 |
| OAK HARBOR | 57582 | PORT CLINTON | 64150 |
| PUT-IN-BAY | 65032 | ROCKY RIDGE | 68042 |
| ALLEN TWP | 01322 | BAY TWP | 04304 |
| BENTON TWP | 05620 | CARROLL TWP | 12266 |
| CATAWBA ISLAND TWP | 12588 | CLAY TWP | 15546 |
| DANBURY TWP | 20058 | ERIE TWP | 25578 |
| HARRIS TWP | 33726 | PORTAGE TWP | 64080 |
| PUT-IN-BAY TWP | 65046 | SALEM TWP | 69946 |

PAULDING COUNTY

| ANTWERP | 02204 | BROUGHTON | 09386 |
| --- | --- | --- | --- |
| CECIL | 12700 | GROVER HILL | 32620 |
| HAVILAND | 34552 | LATTY | 41986 |
| MELROSE | 49000 | OAKWOOD | 57792 |
| PAULDING | 61252 | PAYNE | 61322 |
| SCOTT | 71080 | AUGLAIZE TWP | 03002 |
| BENTON TWP | 05634 | BLUE CREEK TWP | 07370 |
| BROWN TWP | 09512 | CARRYALL TWP | 12308 |
| CRANE TWP | 19176 | EMERALD TWP | 25298 |
| HARRISON TWP | 33950 | JACKSON TWP | 37954 |
| LATTY TWP | 42000 | PAULDING TWP | 61266 |
| WASHINGTON TWP | 81536 |  |  |

PERRY COUNTY

| CORNING | 18770 | CROOKSVILLE | 19456 |
| --- | --- | --- | --- |
| GLENFORD | 30436 | HEMLOCK | 34888 |
| JUNCTION CITY | 39508 | NEW LEXINGTON | 54866 |
| NEW STRAITSVILLE | 55552 | RENDVILLE | 66222 |
| ROSEVILLE | 68560 | SHAWNEE | 71962 |
| SOMERSET | 72977 | THORNVILLE | 76680 |
| BEARFIELD TWP | 04584 | CLAYTON TWP | 15658 |
| COAL TWP | 16350 | HARRISON TWP | 33964 |
| HOPEWELL TWP | 36386 | JACKSON TWP | 37968 |
| MADISON TWP | 46550 | MONDAY CREEK TWP | 51198 |
| MONROE TWP | 51520 | PIKE TWP | 62680 |
| PLEASANT TWP | 63366 | READING TWP | 65760 |
| SALT LICK TWP | 70226 | THORN TWP | 76659 |

PICKAWAY COUNTY

| ASHVILLE | 02680 | CIRCLEVILLE | 15070 |
| --- | --- | --- | --- |
| COMMERCIAL POINT | 18070 | DARBYVILLE | 20212 |
| HARRISBURG | 33740 | NEW HOLLAND | 54726 |
| ORIENT | 58800 | SOUTH BLOOMFIELD | 73068 |
| TARLTON | 76148 | WILLIAMSPORT | 85414 |
| CIRCLEVILLE TWP | 15077 | DARBY TWP | 20156 |
| DEER CREEK TWP | 21168 | HARRISON TWP | 33978 |
| JACKSON TWP | 37982 | MADISON TWP | 46564 |
| MONROE TWP | 51534 | MUHLENBERG TWP | 53256 |
| PERRY TWP | 62008 | PICKAWAY TWP | 62484 |
| SALT CREEK TWP | 70184 | SCIOTO TWP | 70870 |
| WALNUT TWP | 80598 | WASHINGTON TWP | 81550 |
| WAYNE TWP | 82264 |  |  |

PIKE COUNTY

| BEAVER | 04696 | PIKETON | 62708 |
| --- | --- | --- | --- |
| WAVERLY | 81942 | BEAVER TWP | 04710 |
| BENTON TWP | 05648 | CAMP CREEK TWP | 11122 |
| JACKSON TWP | 37996 | MARION TWP | 47824 |
| MIFFLIN TWP | 50078 | NEWTON TWP | 55622 |
| PEBBLE TWP | 61392 | PEE PEE TWP | 61434 |
| PERRY TWP | 62022 | SCIOTO TWP | 70884 |
| SEAL TWP | 71171 | SUNFISH TWP | 75637 |
| UNION TWP | 78512 |  |  |

PORTAGE COUNTY

| AURORA | 03086 | BRADY LAKE | 08168 |
| --- | --- | --- | --- |
| GARRETTSVILLE | 29442 | HIRAM | 35658 |
| KENT | 39872 | MANTUA | 47180 |
| MOGADORE | 51058 | RAVENNA | 65592 |
| STREETSBORO | 75014 | SUGAR BUSH KNOLL | 75196 |
| TALLMADGE | 76106 | WINDHAM | 85946 |
| ATWATER TWP | 02862 | BRIMFIELD TWP | 08840 |
| CHARLESTOWN TWP | 13610 | DEERFIELD TWP | 21210 |
| EDINBURG TWP | 24584 | FRANKLIN TWP | 28392 |
| FREEDOM TWP | 28742 | HIRAM TWP | 35672 |
| MANTUA TWP | 47194 | NELSON TWP | 53858 |
| PALMYRA TWP | 59668 | PARIS TWP | 59822 |
| RANDOLPH TWP | 65452 | RAVENNA TWP | 65606 |
| ROOTSTOWN TWP | 68392 | SHALERSVILLE TWP | 71731 |
| SUFFIELD TWP | 75189 | WINDHAM TWP | 85960 |

PREBLE COUNTY

| CAMDEN | 11024 | COLLEGE CORNER | 16700 |
| --- | --- | --- | --- |
| EATON | 24234 | ELDORADO | 24766 |
| GRATIS | 31472 | LEWISBURG | 42938 |
| NEW PARIS | 55188 | VERONA | 79898 |
| W ALEXANDRIA | 82880 | W ELKTON | 83216 |
| W MANCHESTER | 83804 | DIXON TWP | 22106 |
| GASPER TWP | 29484 | GRATIS TWP | 31486 |
| HARRISON TWP | 33992 | ISRAEL TWP | 37590 |
| JACKSON TWP | 38010 | JEFFERSON TWP | 38780 |
| LANIER TWP | 41846 | MONROE TWP | 51548 |
| SOMERS TWP | 72964 | TWIN TWP | 77994 |
| WASHINGTON TWP | 81564 |  |  |

PUTNAM COUNTY

| BELMORE | 05396 | CLOVERDALE | 16266 |
| --- | --- | --- | --- |
| COLUMBUS GROVE | 18014 | CONTINENTAL | 18504 |
| DUPONT | 22974 | FT JENNINGS | 27818 |
| GILBOA | 30114 | GLANDORF | 30282 |
| KALIDA | 39536 | LEIPSIC | 42602 |
| MILLER CITY | 50358 | OTTAWA | 58982 |
| OTTOVILLE | 59052 | PANDORA | 59738 |
| WEST LEIPSIC | 83706 | BLANCHARD TWP | 06880 |
| GREENSBURG TWP | 32228 | JACKSON TWP | 38024 |
| JENNINGS TWP | 38976 | LIBERTY TWP | 43288 |
| MONROE TWP | 51562 | MONTEREY TWP | 51660 |
| OTTAWA TWP | 58996 | PALMER TWP | 59612 |
| PERRY TWP | 62036 | PLEASANT TWP | 63380 |
| RILEY TWP | 67174 | SUGAR CREEK TWP | 75206 |
| UNION TWP | 78526 | VAN BUREN TWP | 79450 |

RICHLAND COUNTY

| BELLVILLE | 05284 | BUTLER | 10632 |
| --- | --- | --- | --- |
| CRESTLINE | 19330 | GALION | 29162 |
| LEXINGTON | 42994 | LUCAS | 45276 |
| MANSFIELD | 47138 | ONTARIO | 58520 |
| PLYMOUTH | 63800 | SHELBY | 72102 |
| SHILOH | 72298 | BLOOMINGGROVE TWP | 07230 |
| BUTLER TWP | 10646 | CASS TWP | 12420 |
| FRANKLIN TWP | 28406 | JACKSON TWP | 38038 |
| JEFFERSON TWP | 38794 | MADISON TWP | 46578 |
| MIFFLIN TWP | 50092 | MONROE TWP | 51576 |
| PERRY TWP | 62050 | PLYMOUTH TWP | 63814 |
| SANDUSKY TWP | 70394 | SHARON TWP | 71830 |
| SPRINGFIELD TWP | 74128 | TROY TWP | 77616 |
| WASHINGTON TWP | 81578 | WELLER TWP | 82628 |
| WORTHINGTON TWP | 86618 |  |  |

ROSS COUNTY

| ADELPHI | 00450 | BAINBRIDGE | 03604 |
| --- | --- | --- | --- |
| CHILLICOTHE | 14184 | CLARKSBURG | 15336 |
| FRANKFORT | 28182 | KINGSTON | 40376 |
| SOUTH SALEM | 73698 | BUCKSKIN TWP | 10016 |
| COLERAIN TWP | 16630 | CONCORD TWP | 18238 |
| DEERFIELD TWP | 21224 | FRANKLIN TWP | 28420 |
| GREEN TWP | 31822 | HARRISON TWP | 34006 |
| HUNTINGTON TWP | 36890 | JEFFERSON TWP | 38808 |
| LIBERTY TWP | 43302 | PAINT TWP | 59542 |
| PAXTON TWP | 61308 | SCIOTO TWP | 70898 |
| SPRINGFIELD TWP | 74129 | TWIN TWP | 78008 |
| UNION TWP | 78540 |  |  |

SANDUSKY COUNTY

| BELLEVUE | 05228 | BURGOON | 10282 |
| --- | --- | --- | --- |
| CLYDE | 16308 | ELMORE | 25144 |
| FREMONT | 28826 | GIBSONBURG | 30072 |
| GREEN SPRINGS | 32256 | HELENA | 34860 |
| LINDSEY | 43904 | WOODVILLE | 86492 |
| BALLVILLE TWP | 03730 | GREEN CREEK TWP | 31990 |
| JACKSON TWP | 38052 | MADISON TWP | 46592 |
| RICE TWP | 66460 | RILEY TWP | 67188 |
| SANDUSKY TWP | 70408 | SCOTT TWP | 71087 |
| TOWNSEND TWP | 77162 | WASHINGTON TWP | 81592 |
| WOODVILLE TWP | 86506 | YORK TWP | 87080 |

SCIOTO COUNTY

| NEW BOSTON | 54166 | OTWAY | 59066 |
| --- | --- | --- | --- |
| PORTSMOUTH | 64304 | RARDEN | 65508 |
| S WEBSTER | 73824 | BLOOM TWP | 06992 |
| BRUSH CREEK TWP | 09764 | CLAY TWP | 15560 |
| GREEN TWP | 31836 | HARRISON TWP | 34020 |
| JEFFERSON TWP | 38815 | MADISON TWP | 46606 |
| MORGAN TWP | 52150 | NILE TWP | 55902 |
| PORTER TWP | 64206 | RARDEN TWP | 65522 |
| RUSH TWP | 69092 | UNION TWP | 78554 |
| VALLEY TWP | 79170 | VERNON TWP | 79828 |
| WASHINGTON TWP | 81606 |  |  |

SENECA COUNTY

| ATTICA | 02820 | BETTSVILLE | 06194 |
| --- | --- | --- | --- |
| BLOOMVILLE | 07286 | FOSTORIA | 28014 |
| GREEN SPRINGS | 32256 | NEW RIEGEL | 55398 |
| REPUBLIC | 66320 | TIFFIN | 76778 |
| ADAMS TWP | 00324 | BIG SPRING TWP | 06432 |
| BLOOM TWP | 07006 | CLINTON TWP | 16154 |
| EDEN TWP | 24346 | HOPEWELL TWP | 36400 |
| JACKSON TWP | 38066 | LIBERTY TWP | 43316 |
| LOUDON TWP | 45052 | PLEASANT TWP | 63394 |
| REED TWP | 65942 | SCIPIO TWP | 71024 |
| SENECA TWP | 71355 | THOMPSON TWP | 76632 |
| VENICE TWP | 79674 |  |  |

SHELBY COUNTY

| ANNA | 02092 | BOTKINS | 07832 |
| --- | --- | --- | --- |
| FT LORAMIE | 27832 | JACKSON CENTER | 38220 |
| KETTLERSVILLE | 40054 | LOCKINGTON | 44352 |
| PORT JEFFERSON | 64262 | RUSSIA | 69344 |
| SIDNEY | 72424 | CLINTON TWP | 16168 |
| CYNTHIAN TWP | 19834 | DINSMORE TWP | 22050 |
| FRANKLIN TWP | 28434 | GREEN TWP | 31850 |
| JACKSON TWP | 38080 | LORAMIE TWP | 44884 |
| MCCLEAN TWP | 46172 | ORANGE TWP | 58660 |
| PERRY TWP | 62064 | SALEM TWP | 69960 |
| TURTLE CREEK TWP | 77854 | VAN BUREN TWP | 79464 |
| WASHINGTON TWP | 81620 |  |  |

STARK COUNTY

| ALLIANCE | 01420 | BEACH CITY | 04458 |
| --- | --- | --- | --- |
| BREWSTER | 08504 | CANAL FULTON | 11304 |
| CANTON | 12000 | EAST CANTON | 23324 |
| EAST SPARTA | 24052 | HARTVILLE | 34328 |
| HILLS AND DALES | 35532 | LIMAVILLE | 43596 |
| LOUISVILLE | 45094 | MAGNOLIA | 46844 |
| MASSILLON | 48244 | MEYERS LAKE | 49294 |
| MINERVA | 50834 | NAVARRE | 53690 |
| NORTH CANTON | 56294 | WAYNESBURG | 82376 |
| WILMOT | 85806 | BETHLEHEM TWP | 06180 |
| CANTON TWP | 12014 | JACKSON TWP | 38094 |
| LAKE TWP | 41314 | LAWRENCE TWP | 42168 |
| LEXINGTON TWP | 43022 | MARLBORO TWP | 47936 |
| NIMISHILLEN TWP | 55944 | OSNABURG TWP | 58926 |
| PARIS TWP | 59850 | PERRY TWP | 62078 |
| PIKE TWP | 62694 | PLAIN TWP | 62988 |
| SANDY TWP | 70436 | SUGAR CREEK TWP | 75208 |
| TUSCARAWAS TWP | 77910 | WASHINGTON TWP | 81634 |

SUMMIT COUNTY

| AKRON | 01000 | BARBERTON | 03828 |
| --- | --- | --- | --- |
| BOSTON HEIGHTS | 07790 | CLINTON | 16182 |
| CUYAHOGA FALLS | 19778 | FAIRLAWN | 26166 |
| GREEN | 31860 | HUDSON | 36651 |
| LAKEMORE | 41454 | MACEDONIA | 45976 |
| MOGADORE | 51058 | MUNROE FALLS | 53312 |
| NEW FRANKLIN | 54562 | NORTHFIELD | 56448 |
| NORTON | 57260 | PENINSULA | 61574 |
| REMINDERVILLE | 66152 | RICHFIELD | 66530 |
| SILVER LAKE | 72494 | STOW | 74944 |
| TALLMADGE | 76106 | TWINSBURG | 78050 |
| BATH TWP | 04248 | BOSTON TWP | 07776 |
| COPLEY TWP | 18658 | COVENTRY TWP | 19036 |
| NORTHFLD CENTER TWP | 56490 | RICHFIELD TWP | 66544 |
| SAGAMORE HILLS TWP | 69428 | SPRINGFIELD TWP | 74130 |
| TWINSBURG TWP | 78064 |  |  |

TRUMBULL COUNTY

| CORTLAND | 18812 | GIRARD | 30198 |
| --- | --- | --- | --- |
| HUBBARD | 36582 | LORDSTOWN | 44912 |
| MCDONALD | 45934 | NEWTON FALLS | 55650 |
| NILES | 55916 | ORANGEVILLE | 58674 |
| W FARMINGTON | 83384 | WARREN | 80892 |
| YANKEE LAKE | 86856 | YOUNGSTOWN | 88000 |
| BAZETTA TWP | 04444 | BLOOMFIELD TWP | 07160 |
| BRACEVILLE TWP | 08056 | BRISTOL TWP | 08938 |
| BROOKFIELD TWP | 09190 | CHAMPION TWP | 13470 |
| FARMINGTON TWP | 26684 | FOWLER TWP | 28098 |
| GREENE TWP | 32046 | GUSTAVUS TWP | 32732 |
| HARTFORD TWP | 34230 | HOWLAND TWP | 36554 |
| HUBBARD TWP | 36596 | JOHNSTON TWP | 39298 |
| KINSMAN TWP | 40502 | LIBERTY TWP | 43344 |
| MECCA TWP | 48678 | MESOPOTAMIA TWP | 49210 |
| NEWTON TWP | 55636 | SOUTHINGTON TWP | 73397 |
| VERNON TWP | 79856 | VIENNA TWP | 80052 |
| WARREN TWP | 80906 | WEATHERSFIELD TWP | 82446 |

TUSCARAWAS COUNTY

| BALTIC | 03744 | BARNHILL | 03940 |
| --- | --- | --- | --- |
| BOLIVAR | 07594 | DENNISON | 21714 |
| DOVER | 22456 | GNADENHUTTEN | 30702 |
| MIDVALE | 49966 | MINERAL CITY | 50764 |
| NEWCOMERSTOWN | 54432 | NEW PHILADELPHIA | 55216 |
| PARRAL | 61056 | PORT WASHINGTON | 64346 |
| ROSWELL | 68742 | STONE CREEK | 74804 |
| STRASBURG | 74958 | SUGAR CREEK | 75210 |
| TUSCARAWAS | 77924 | UHRICHSVILLE | 78176 |
| ZOAR | 88168 | AUBURN TWP | 02918 |
| BUCKS TWP | 09988 | CLAY TWP | 15574 |
| DOVER TWP | 22470 | FAIRFIELD TWP | 26082 |
| FRANKLIN TWP | 28462 | GOSHEN TWP | 31066 |
| JEFFERSON TWP | 38836 | LAWRENCE TWP | 42182 |
| MILL TWP | 50232 | OXFORD TWP | 59304 |
| PERRY TWP | 62092 | RUSH TWP | 69106 |
| SALEM TWP | 69974 | SANDY TWP | 70443 |
| SUGAR CREEK TWP | 75217 | UNION TWP | 78568 |
| WARREN TWP | 80920 | WARWICK TWP | 81060 |
| WASHINGTON TWP | 81648 | WAYNE TWP | 82292 |
| YORK TWP | 87094 |  |  |

UNION COUNTY

| DUBLIN | 22694 | MAGNETIC SPRINGS | 46830 |
| --- | --- | --- | --- |
| MARYSVILLE | 48160 | MILFORD CENTER | 50218 |
| PLAIN CITY | 63030 | RICHWOOD | 66936 |
| UNIONVILLE CENTER | 78834 | ALLEN TWP | 01336 |
| CLAIBOURNE TWP | 15112 | DARBY TWP | 20170 |
| DOVER TWP | 22484 | JACKSON TWP | 38108 |
| JEROME TWP | 39046 | LEESBURG TWP | 42490 |
| LIBERTY TWP | 43358 | MILL CREEK TWP | 50288 |
| PARIS TWP | 59864 | TAYLOR TWP | 76194 |
| UNION TWP | 78582 | WASHINGTON TWP | 81662 |
| YORK TWP | 87122 |  |  |

VAN WERT COUNTY

| CONVOY | 18546 | DELPHOS | 21602 |
| --- | --- | --- | --- |
| ELGIN | 24794 | MIDDLE POINT | 49728 |
| OHIO CITY | 58002 | SCOTT | 71080 |
| VAN WERT | 79562 | VENEDOCIA | 79632 |
| WILLSHIRE | 85736 | WREN | 86632 |
| HARRISON TWP | 34034 | HOAGLIN TWP | 35756 |
| JACKSON TWP | 38122 | JENNINGS TWP | 38990 |
| LIBERTY TWP | 43372 | PLEASANT TWP | 63408 |
| RIDGE TWP | 66978 | TULLY TWP | 77756 |
| UNION TWP | 78596 | WASHINGTON TWP | 81676 |
| WILLSHIRE TWP | 85750 | YORK TWP | 87136 |

VINTON COUNTY

| HAMDEN | 32956 | MCARTHUR | 45696 |
| --- | --- | --- | --- |
| WILKESVILLE | 85190 | ZALESKI | 88028 |
| BROWN TWP | 09526 | CLINTON TWP | 16210 |
| EAGLE TWP | 23100 | ELK TWP | 24892 |
| HARRISON TWP | 34048 | JACKSON TWP | 38136 |
| KNOX TWP | 40880 | MADISON TWP | 46620 |
| RICHLAND TWP | 66768 | SWAN TWP | 75854 |
| VINTON TWP | 80192 | WILKESVILLE TWP | 85204 |

WARREN COUNTY

| BLANCHESTER | 06908 | BUTLERVILLE | 10674 |
| --- | --- | --- | --- |
| CARLISLE | 12168 | CORWIN | 18840 |
| FRANKLIN | 28476 | HARVEYSBURG | 34384 |
| LEBANON | 42364 | LOVELAND | 45108 |
| MAINEVILLE | 46872 | MASON | 48188 |
| MIDDLETOWN | 49840 | MONROE | 51310 |
| MORROW | 52374 | PLEASANT PLAIN | 63576 |
| SOUTH LEBANON | 73446 | SPRINGBORO | 74076 |
| WAYNESFIELD | 82418 | CLEAR CREEK TWP | 15700 |
| DEERFIELD TWP | 21238 | FRANKLIN TWP | 28490 |
| HAMILTON TWP | 33068 | HARLAN TWP | 33474 |
| MASSIE TWP | 48216 | SALEM TWP | 69988 |
| TURTLE CREEK TWP | 77868 | UNION TWP | 78610 |
| WASHINGTON TWP | 81690 | WAYNE TWP | 82306 |

WASHINGTON COUNTY

| BELPRE | 05424 | BEVERLY | 06222 |
| --- | --- | --- | --- |
| LOWELL | 45164 | LOWER SALEM | 45220 |
| MACKSBURG | 46158 | MARIETTA | 47628 |
| MATAMORAS | 48286 | ADAMS TWP | 00338 |
| AURELIUS TWP | 03072 | BARLOW TWP | 03898 |
| BELPRE TWP | 05438 | DECATUR TWP | 21098 |
| DUNHAM TWP | 22918 | FAIRFIELD TWP | 26096 |
| FEARING TWP | 26810 | GRANDVIEW TWP | 31276 |
| INDEPENDENCE TWP | 37268 | LAWRENCE TWP | 42196 |
| LIBERTY TWP | 43386 | LUDLOW TWP | 45360 |
| MARIETTA TWP | 47642 | MUSKINGUM TWP | 53452 |
| NEWPORT TWP | 55356 | PALMER TWP | 59626 |
| SALEM TWP | 70002 | WARREN TWP | 80934 |
| WATERFORD TWP | 81774 | WATERTOWN TWP | 81844 |
| WESLEY TWP | 82838 |  |  |

WAYNE COUNTY

| APPLE CREEK | 02232 | BURBANK | 10254 |
| --- | --- | --- | --- |
| CONGRESS | 18308 | CRESTON | 19344 |
| DALTON | 19974 | DOYLESTOWN | 22568 |
| FREDERICKSBURG | 28616 | MARSHALLVILLE | 48048 |
| MT EATON | 52682 | NORTON | 57260 |
| ORRVILLE | 58828 | RITTMAN | 67356 |
| SHREVE | 72396 | SMITHVILLE | 72788 |
| WEST SALEM | 84196 | WOOSTER | 86548 |
| BAUGHMAN TWP | 04276 | CANAAN TWP | 11276 |
| CHESTER TWP | 14044 | CHIPPEWA TWP | 14240 |
| CLINTON TWP | 16224 | CONGRESS TWP | 18322 |
| EAST UNION TWP | 24136 | FRANKLIN TWP | 28504 |
| GREEN TWP | 31878 | MILTON TWP | 50666 |
| PAINT TWP | 59556 | PLAIN TWP | 62995 |
| SALT CREEK TWP | 70198 | SUGAR CREEK TWP | 75231 |
| WAYNE TWP | 82320 | WOOSTER TWP | 86562 |

WILLIAMS COUNTY

| ALVORDTON | 01588 | BLAKESLEE | 06810 |
| --- | --- | --- | --- |
| BRYAN | 09792 | EDGERTON | 24486 |
| EDON | 24640 | HOLIDAY CITY | 35864 |
| MONTPELIER | 51772 | PIONEER | 62834 |
| STRYKER | 75140 | W UNITY | 84308 |
| BRADY TWP | 08140 | BRIDGEWATER TWP | 08630 |
| CENTER TWP | 13008 | FLORENCE TWP | 27530 |
| JEFFERSON TWP | 38864 | MADISON TWP | 46634 |
| MILL CREEK TWP | 50302 | NORTHWEST TWP | 57162 |
| PULASKI TWP | 64920 | SAINT JOSEPH TWP | 69638 |
| SPRINGFIELD TWP | 74131 | SUPERIOR TWP | 75819 |

WOOD COUNTY

| BAIRDSTOWN | 03646 | BLOOMDALE | 07062 |
| --- | --- | --- | --- |
| BOWLING GREEN | 07972 | BRADNER | 08112 |
| CUSTAR | 19750 | CYGNET | 19820 |
| FOSTORIA | 28014 | GRAND RAPIDS | 31206 |
| HASKINS | 34412 | HOYTVILLE | 36568 |
| JERRY CITY | 39074 | LUCKEY | 45332 |
| MILLBURY | 50260 | MILTON CENTER | 50708 |
| N BALTIMORE | 56154 | NORTHWOOD | 57190 |
| PEMBERVILLE | 61504 | PERRYSBURG | 62148 |
| PORTAGE | 64108 | RISING SUN | 67314 |
| ROSSFORD | 68686 | TONTOGANY | 77070 |
| W MILLGROVE | 83888 | WALBRIDGE | 80486 |
| WAYNE | 82334 | WESTON | 83972 |
| BLOOM TWP | 07020 | CENTER TWP | 13015 |
| FREEDOM TWP | 28756 | GRAND RAPIDS TWP | 31220 |
| HENRY TWP | 34986 | JACKSON TWP | 38164 |
| LAKE TWP | 41328 | LIBERTY TWP | 43400 |
| MIDDLETON TWP | 49812 | MILTON TWP | 50680 |
| MONTGOMERY TWP | 51744 | PERRY TWP | 62106 |
| PERRYSBURG TWP | 62162 | PLAIN TWP | 63002 |
| PORTAGE TWP | 64122 | TROY TWP | 77630 |
| WASHINGTON TWP | 81704 | WEBSTER TWP | 82544 |
| WESTON TWP | 83986 |  |  |

WYANDOT COUNTY

| CAREY | 12112 | HARPSTER | 33656 |
| --- | --- | --- | --- |
| KIRBY | 40558 | MARSEILLES | 47992 |
| NEVADA | 53942 | SYCAMORE | 75980 |
| UPPER SANDUSKY | 79044 | WHARTON | 84574 |
| ANTRIM TWP | 02190 | CRANE TWP | 19190 |
| CRAWFORD TWP | 19246 | EDEN TWP | 24360 |
| JACKSON TWP | 38178 | MARSEILLES TWP | 48006 |
| MIFFLIN TWP | 50106 | PITT TWP | 62904 |
| RICHLAND TWP | 66782 | RIDGE TWP | 66992 |
| SALEM TWP | 70016 | SYCAMORE TWP | 75987 |
| TYMOCHTEE TWP | 78141 |  |  |

## APPENDIX E

ABUTMENT TYPES

The following sketches show varying views or sections of some of the typical abutment types:

## APPENDIX F

ITEM #144 - OHIO ORIGINAL CONSTRUCTION PROJECT NO.

This item must be coded by all State agencies as follows:

A.	Structures sold and originally built under a Project Number.  Code the project number in the first four (4) positions of the item, right justified and using leading zeros where necessary.  Code the last two (2) digits of the year in which the project was sold in the last two (2) positions of the item.

B.	Structures built by Force Account (by Agency Forces).  Code the capital letters "FA" in the third and fourth character position of the item and code the last two (2) digits of the year in which the structure was built in the last two (2) positions of the item.

C. 	Structures built by non-state agencies, including those built by States other than Ohio and those constructed under private contracts.  Code an alphabetic abbreviation to represent the agency that constructed the bridge using all capital letters.  A combination of alphabetic and numeric characters may be used in this item to identify specific contracts by an agency.  Numerals, if used, may only be coded in the first four (4) character positions.  Coded abbreviations using less than six (6) characters must be right justified and filled with leading zeros.  See Appendix H for suggested abbreviation standards.

D.	Structures built by State agencies other than the Department of Transportation, such as the Ohio Turnpike Commission, the Conservancy Districts, the Department of Natural Resources, etc., should code a contract or project number using that agency's scheme of alphabetic and/or numeric characters.  If such a number is not available or impractical, code an abbreviation following the scheme described in Part "C" above.  See Appendix H for suggested standards.

E.	Structures for which either incomplete or no information is available.  Code the key word "UNKNWN" in the item.  Although it is recommended that all records submitted include the information required in this item non-state agencies (Counties, municipalities, etc.) are not required to code the item.  If these agencies decide to code the information, a uniform individualized system should be adopted by each agency.  If an agency has a bridge that was sold through ODOT "Contract Sales", they should code the item for that bridge as described.

## APPENDIX G

PIER TYPES

APPENDIX G

(Continued)

PIER TYPES

## APPENDIX H

SUGGESTED STANDARD CODINGS

For

Ohio Original Construction Project Numbers

The following list of abbreviations includes those for non-state and other (non-DOT) State agencies most commonly involved.  For agencies not listed, it will be necessary for the coder to create his own abbreviation using the same logic and being careful not to cause confusion by duplicating those shown in this list.

| NON-STATE AGENCY CONSTRUCTION |  |
| --- | --- |
| Structure built by | Code |
| Federal Public Work Programs such as “WPA” and “CCC” | “OOOFPW” |
| County Construction not sold under State contract system | “COUNTY” |
| Army Corps of Engineers | “OOOOCE” |
| Private Contract | “OOOOPC” |
| City Construction not sold under State contract system | “OOOCITY” |
| Other State DOT (Example: Florida DOT) | “FLADOT” |

| OTHER STATE AGENCY CONSTRUCTION |  |
| --- | --- |
| Structure built by | Code |
| Ohio Turnpike Commission (Using Contract Number) | “OOOOTP” “OO25TP” |
| State Public Works Program | “OOOSPW” |
| Ohio Building Authority | “OOOOBA” |
| Ohio Department of Natural Resources | “OOODNR” |
| Ohio Bridge Commission | “OOOOBC” |
| Ohio National Guard | “OOOONG” |
| Muskingum Conservancy District | “OOOMCD” |

## APPENDIX I

ODOT STANDARDIZED

CODINGS FOR ROUTE NUMBER SUFFIX'S

To be coded in last digit positions only of "ROUTE" portion of Item #3 and #22 when a single digit suffix is required to describe an official special category route.

| Suffix Code | Special Route Designation |
| --- | --- |
| “A” | Alternate |
| “B” | Bypass |
| “C” | Spur or Connector |
| “D” | Directional Alternate (1st within County) |
| “F” | Directional Alternate (2nd for same route within County) |
| “G” | Directional Alternate (3rd for same route within County) |
| “J” | Future |
| “T” | Temporary |
| “K” | Turnpike (special code to allow differentiation between I-80 & I-76 Ohio Turnpike and ODOT controlled I-80 & I-76) |
| “X” | Duplicate Route (usually old route still on System after new route of same number is open to traffic) |

## APPENDIX J

BYPASS (DETOUR) LENGTH

THE BYPASS (DETOUR) LENGTH (ADDITIONAL TRAVEL DISTANCE) FOR THE EXAMPLE SHOWN BELOW IS CALCULATED AS FOLLOWS:

LENGTH OF ROUTE BEING BYPASSED MEASURED BETWEEN DETOUR POINTS = 10.3 MILES.

LENGTH OF DETOUR ROUTE USED TO TEMPORARILY BYPASS THE PROBLEM SITE, MEASURED BETWEEN DETOUR POINTS = 19.6 MILES.

BYPASS (DETOUR) LENGTH = 19.6 MILES - 10.3 MILES = 9.3 MILES (CODE 09 MILES)

## APPENDIX K

RAILROAD IDENTIFICATION

| Code | Railroad Company |
| --- | --- |
| 01 | Ann Arbor Railroad |
| 02 | Akron Barberton Cluster Railroad |
| 03 | Ashtabula, Carson & Jefferson Railroad |
| 04 | Ashland Railway |

| Code | Railroad Company |
| --- | --- |
| 05 | Bessemer & Lake Erie Railroad Company |

| Code | Railroad Company |
| --- | --- |
| 10 | Central Railroad Company of Indiana |
| 11 | Columbus & Ohio River Railroad Company |
| 12 | Consolidated Railroad Corporation |
| 13 | CSX Transportation Inc. |
| 14 | Cincinnati Terminal Railway Company |
| 15 | Cuyahoga Valley Railway Company |

| Code | Railroad Company |
| --- | --- |
| 20 | Great Miami & Scioto Railway Company |
| 21 | Germantown Rail Siding Company |
| 22 | Grand Trunk & Western Railroad |

| Code | Railroad Company |
| --- | --- |
| 23 | Indiana & Ohio Central Railroad |
| 24 | Indiana Northeastern Railway Company |
| 25 | Indiana & Ohio Rail Corporation |
| 30 | Indiana & Ohio Railway |

| Code | Railroad Company |
| --- | --- |
| 31 | Lake Terminal Railroad |

APPENDIX K

(Continued)

RAILROAD IDENTIFICATION

| Code | Railroad Company |
| --- | --- |
| 32 | Norfolk Southern Corporation |
| 33 | Nimishillon & Tuscarawas Railway Company |
| 34 | Newburgh & South Shore Railway Company |

| Code | Railroad Company |
| --- | --- |
| 35 | Ohio-Rail Corporation |
| 40 | Ohio Central Railroad |
| 41 | Ohio Southern Railroad |

| Code | Railroad Company |
| --- | --- |
| 42 | Pittsburgh & Lake Erie Railroad Company |
| 43 | PL & W Railroad |

| Code | Railroad Company |
| --- | --- |
| 44 | R.J. Corman Railroad |
| 45 | River Terminal Railway Company |

| Code | Railroad Company |
| --- | --- |
| 50 | Spencerville & Elgin Railroad |

| Code | Railroad Company |
| --- | --- |
| 51 | Temperance Yard Corporation |

| Code | Railroad Company |
| --- | --- |
| 52 | Wheeling & Lake Erie Railway Company |
| 53 | The Warren & Trumbull Railroad Company |
| 54 | Wabash & Erie Railroad Company |

APPENDIX K

(Continued)

RAILROAD IDENTIFICATION

| Code | Railroad Company |
| --- | --- |
| 55 | Youngstown & Austintown Railroad Company |

| Code | Railroad Company |
| --- | --- |
| 90 | Greater Cleveland Regional Transit Authority (Transit) |
| 91 | Central Ohio Transit Authority (Transit) |
| 92 | Southwestern Ohio Regional Transit Authority |
| 93 | Akron Metro Regional Transit Authority |
| NN | None or Not Applicable |

## APPENDIX L

MPO CODE TABLE

ITEM #1

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| ADA | NN | NONE |
| ALL | 12 | TCC: Lima & Allen County |
| ASD | NN | NONE |
| ATB | NN | NONE |
| ATH | NN | NONE |
| AUG | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| BEL | 03 | BOMTS: Wheeling – Bridgeport |
| BRO | NN | NONE |
| BUT | 09 | OKI: Ohio Kentucky Indiana - Cincinnati |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| CAR | NN | NONE |
| CHP | NN | NONE |
| CLA | 14 | TCC: Springfield |
| CLE | 09 | OKI: Ohio Kentucky Indiana – Cincinnati |
| CLI | NN | NONE |
| COL | NN | NONE |
| COS | NN | NONE |
| CRA | NN | NONE |
| CUY | 08 | NOACA: Cleveland |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| DAR | NN | NONE |
| DEF | NN | NONE |
| DEL | 07 | MORPC: Mid Ohio - Columbus |

APPENDIX L

(continued)

MPO CODE TABLE

ITEM #1

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| ERI | 17 | ERPC – Erie County - Vermilion |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| FAI | 07 | MORPC: Mid Ohio – Columbus |
| FAI | NN | NONE |
| FAI | UU | UNKNOWN: Fairfield |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| FAY | NN | NONE |
| FRA | 07 | MORPC: Mid Ohio – Columbus |
| FUL | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| GAL | NN | NONE |
| GEA | 08 | NOACA: Cleveland |
| GRE | 11 | MVRPC: Miami Valley – Dayton |
| GUE | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| HAM | 09 | OKI: Ohio Kentucky Indiana – Cincinnati |
| HAN | NN | NONE |
| HAR | NN | NONE |
| HAS | NN | NONE |
| HEN | NN | NONE |
| HIG | NN | NONE |
| HOC | NN | NONE |
| HOL | NN | NONE |
| HUR | NN | NONE |

APPENDIX L

(continued)

MPO CODE TABLE

ITEM #1

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| JAC | NN | NONE |
| JEF | 02 | BHJTS: Steubenville – Weirton |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| KNO | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| LAK | 08 | NOACA: Cleveland |
| LAW | 05 | HAIATS: Huntington |
| LAW | NN | NONE |
| LAW | UU | UNKNOWN |
| LIC | 06 | LCATS: Newark – Heath |
| LIC | 07 | MORPC: Mid Ohio – Columbus |
| LIC | UU | UNKNOWN |
| LOG | NN | NONE |
| LOR | 08 | NOACA: Cleveland |
| LOR | 17 | ERPC – Vermilion |
| LUC | 15 | TMACOG – Toledo |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| MAD | NN | NONE |
| MAH | 04 | EDATA: Youngstown |
| MAR | NN | NONE |
| MED | 08 | NOACA: Cleveland |
| MEG | NN | NONE |
| MER | NN | NONE |
| MIA | 11 | MVRPC: Miami Valley – Dayton |
| MOE | NN | NONE |
| MOT | 11 | MVRPC: Miami Valley – Dayton |
| MRG | NN | NONE |
| MRW | NN | NONE |
| MUS | NN | NONE |

APPENDIX L

(continued)

MPO CODE TABLE

ITEM #1

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| NOB | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| OTT | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| PAU | NN | NONE |
| PER | NN | NONE |
| PIC | NN | NONE |
| PIK | NN | NONE |
| POR | 01 | AMATS: Akron |
| PRE | NN | NONE |
| PUT | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| RIC | 13 | TCC: Mansfield |
| ROS | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| SAN | NN | NONE |
| SCI | NN | NONE |
| SEN | NN | NONE |
| SHE | NN | NONE |
| STA | 10 | SCATS: Canton |
| SUM | 01 | AMATS: Akron |

APPENDIX L

(continued)

MPO CODE TABLE

ITEM #1

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| TRU | 04 | EDATA: Youngstown |
| TUS | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| UNI | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| VAN | NN | NONE |
| VIN | NN | NONE |

| COUNTY | MPO | DESCRIPTION |
| --- | --- | --- |
| WAR | 09 | OKI: Ohio Kentucky Indiana – Cincinnati |
| WAS | NN | NONE |
| WAS | UU | UNKNOWN |
| WAS | 16 | WWW: Parkersburg – Belpre |
| WAY | 01 | AMATS: Akron |
| WAY | NN | NONE |
| WAY | UU | UNKNOWN |
| WIL | NN | NONE |
| WOO | 15 | TMACOG: Toledo |
| WYA | NN | NONE |

## INDEX

ABUTMENT/PILE LOAD TESTING	220

ANALYZED BY	116

APPROACH ALIGNMENT	125

APPROACH GUARDRAIL	153

APPROACH PAVEMENT GRADE	155

APPROACH PAVEMENT WEARING SURFACE	154

APPROACH ROADWAY WIDTH	67

APPROACH SLAB	156

AVERAGE DAILY TRAFFIC	24

AVERAGE DAILY TRAFFIC YEAR	25

AVERAGE DAILY TRUCK TRAFFIC	26

AVERAGE LENGTH ABUTMENT FOUNDATIONS	101

AVERAGE LENGTH PIER FOUNDATIONS	102

BEARING DEVICE	164

BORDER BRIDGE	150

BORDER BRIDGE STRUCTURE NUMBER	151

BR 86	324

BR 86S	325

BR85	322

BR85A	323

BRIDGE MEDIAN	73

BRIDGE RAILING TYPE	81

BRIDGE ROADWAY WIDTH (CURB TO CURB)	68

BYPASS LENGTH	54

CARDINAL RAMP	223

CITY OR TOWN (FIPS CODE)	16

COMPOSITE STRUCTURE	80

COORDINATES	56

CULVERT/BRIDGE TYPE	172

CURBS & SIDEWALKS	75

DATE BUILT	58

DATE OF MAJOR REHABILITATION	59

DATE OF WEARING SURFACE	159

DECK CONCRETE TYPE	186

DECK DRAINAGE	82

DECK PROTECTION	84

DECK TYPE	83

DECK WIDTH (OUT/OUT)	69

DEGREE OF HORIZONTAL CURVE	61

DEPTH OF FILL OVER CULVERT	170

DESIGN LOADING	66

DESIGNATED INSPECTION FREQUENCY	131

DESIGNATED NATIONAL NETWORK	29

DIRECTION OF TRAFFIC	27

DRAINAGE AREA	189

EXPANSION JOINT	211

FACILITY CARRIED BY STRUCTURE	147

FEATURE(S) INTERSECTED	20, 39

FLARED	79

FOUNDATIONS	100

FRACTURE CRITICAL INSPECTION	229

FRAMING TYPE	177

FUNCTIONAL CLASSIFICATION	34

HAUNCH GIRDER DEPTH	216

HAUNCHED GIRDER	215

HEADWALLS OR ENDWALLS	171

HEIGHT OF FENCING ON BRIDGE	204

HINGES	174

INDEX	319

INSPECTION FROM BOAT	234

INSPECTION FROM SNOOPER	233

INSPECTION PROGRAM RESPONSIBILITY (3 digits)	136

INTERSECTED A.D.T. YEAR	42

INTERSECTED DESIGNATED NATIONAL NETWORK	49

INTERSECTED DIRECTION OF TRAFFIC	48

INTERSECTED FUNCTIONAL CLASSIFICATION	46

INTERSECTED MACRO LEVEL CORRIDOR	45

INTERSECTED NATIONAL HIGHWAY SYSTEM (NHS)	44

INTERSECTED PARALLEL STRUCTURES	50

INTERSECTED ROUTE	38

INTERSECTED ROUTE BRIDGE NUMBER	40

INTERSECTED STRAHNET HIGHWAY DESIGNATION	52

INTERSECTED TRUCK A.D.T.	43

INVENTORY LOAD RATING FACTOR	134

INVENTORY ROUTE	17

INVENTORY ROUTE BRIDGE NUMBER	21

ITEM #25 - INTERSECTED A.D.T.	41

ITEM #74 - SCOUR CRITICAL SUSCEPTIBILITY	103

JOINT RETROFIT	213

LENGTH OF APPROACH SLAB	157

LENGTH OUT/OUT OF CULVERT	168

LOAD RATING	117

LOCATION OF STRUCTURE	146

MACRO LEVEL CORRIDOR	33

MAIN MEMBER DEPTH	217

MAIN MEMBER TYPE	161

MAXIMUM SPAN LENGTH	89

MEDIAN TYPE (ON BRIDGE)	72

METHOD OF RATING	119

MINIMUM LATERAL CLEARANCE ON BRIDGE	114

MINIMUM LATERAL UNDERCLEARANCE	111

MINIMUM VERTICAL CLEARANCE ON BRIDGE	92

MINIMUM VERTICAL CLEARANCE UNDER BRIDGE	108

MOMENT PLATES	162

NAME OF CONTRACTOR	182

NAME OF FABRICATOR	181

NATIONAL HIGHWAY SYSTEM (NHS)	32

NAVIGATION	166

NBIS BRIDGE LENGTH	95

NOISE BARRIER WALLS	205

NON-CARDINAL RAMP	222

NUMBER OF LANES	60

OHIO ORIGINAL CONSTRUCTION PROJECT NUMBER	183

OHIO'S DESIGNATED MPOs	15

OPERATING LOAD RATING FACTOR (4 digits)	132

OVERALL STRUCTURE LENGTH	90

PARALLEL STRUCTURES	30

PILE LOG	219

PLAN INFORMATION AVAILABLE	122

POST TENSIONED	207

PRACTICAL MAXIMUM VERTICAL	199

PRACTICAL MAXIMUM VERTICAL CLEARANCE ON BRIDGE	196

PROPOSED IMPROVEMENTS	126

RAILROAD IDENTIFICATION	184

ROUTE ON THE BRIDGE	148

ROUTE UNDER THE BRIDGE	149

ROUTINE MAINTENANCE RESPONSIBILITY (3 digits	140

SCENIC RIVER	209

SIGN ON/UNDER BRIDGE	225

SKEW	62

SLOPE PROTECTION	227

SOFTWARE OF RATING ANALYSIS	120

SPECIAL FEATURES	203

SPECIAL INSPECTION	236, 237

STANDARD DRAWING NUMBER	188

STRAHNET HIGHWAY DESIGNATION	36

STREAM VELOCITY	121

STRUCTURAL STEEL	179

STRUCTURE TYPE	86

SUBSTRUCTURE	98

TEMPORARY BRIDGE CONDITION	201

TEMPORARY STRUCTURE DESIGNATION	28

TOLL	57

TOTAL MINIMUM HORIZONTAL CLEARANCE ON BRIDGE	194

TOTAL MINIMUM HORIZONTAL CLEARANCE UNDER BRIDGE	197

TYPE OF CHANNEL PROTECTION [1 digit}]	106

TYPE OF CURB OR SIDEWALKS	78

TYPE OF MAIN LONGITUDINAL BRIDGE MEMBERS	176

TYPE SERVICE	143

UNDERWATER INSPECTION	231

UTILITIES	64

VERTICAL CLEARANCE AT VERTICAL LIFT BRIDGE	115

WATER FLOW DIRECTION	226

WATERWAY ADEQUACY	123

WEARING SURFACE	85

WORK HISTORY	190

## BR87

## BR87A

## BR86

## BR86S